Lab: IPv4, Subnetting, VLSM and IPv6 Addressing

Understanding IPv4 and IPv6 addressing (word count 1000)

This part of lab worksheet require you research and write a report on “Features of IPv6”. You may discuss but not limited to structure, address types, advanced functionalities and security.

Subnetting and VLSM

NOTE: To complete this part you would need full understanding of Lab 10 to 14.

Using the Network address 172.16.2.0/24, VLSM the network to support the following requirements:

Table 1

| Subnet | Network Address | Subnet mask | First Address | Last Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| B | 172..16.2.0 | 255.255.248.0 | 172.16.3.1 | 172.16.7.254 | 172.16.7.255 |
| D | 172.16.8.0 | 255.255.254.0 | 172.16.8.1 | 172.16.9.254 | 172.16.9.255 |
| F | 172.16.10.0 | 255.255.255.0 | 172.16.10.2 | 172.16.10.254 | 172.16.10.255 |
| E | 172.16.11.0 | 255.255.255.192 | 172.16.11.1 | 172.16.11.62 | 172.16.11.63 |
| A | 172.16.11.64 | 255.255.255.224 | 172.16.11.65 | 172.16.11.94 | 172.16.11.95 |
| C | 172.16.11.96 | 255.255.255.248 | 172.16.11.97 | 172.16.11.102 | 172.16.11.103 |