| LLEOLIAD: Coleg Sir Gar LEFEL 6 CYFNOD ARHOLIADAU MAI 2024 | LOCATION: Coleg Sir Gar LEVEL 6 EXAMINATION PERIOD MAY 2024 |
| --- | --- |

CÔD Y MODWL: SGEE6001

MODULE CODE: SGEE6001

ELECTRONEG GRYM A GYRWYR

POWER ELECTRONICS AND DRIVES

amser: 2½ awr

TIME:  2½ HOURS

ATEBWCH pedwar CWESTIWN ALLAN O pump

answer FOUR quESTIONS out of FIVE

Equipment or materials required. Please select from the following:

Adnoddau neu ddefnyddiau sydd eu hangen.

Cyfrifiannell/Calculator

Question 1

In figure Q1 below, the battery voltage E = 24V, its capacity is 200Wh and the average charging current should be Idc = 5A. The primary input voltage Vp = 120V at 60HZ with the transformer having a turn ratio of  n = 2:1

a) Draw the relative output waveforms of Vs, Vs – E and iL, and use relevant equations to   	explain the operation of the circuit.

[9]

b) Calculate:

- The conduction angle of the diode					[6]

- The current limiting resistance value of R				[8]

- The PIV of the diode							[2]

R

iL

Vp

Vs = Vm Sin wt

E

D1

n : 1

Figure Q1

Total = 25 marks

Question 2

a)

(i) Explain with the aid of diagrams the methods of protection that can be       	   	     established for thyristors in power circuits.

[7]

(ii) From the protection circuit established in part (i), derive the equations to select 	      values for the RLC components, and explain the circuit operation to justify the 	      equations.

[12]

b) An SCR circuit is operated with a supply Vs of 400V, the dv/dt value is 500V/µs and di/dt

is 20 A/µs. Calculate the values of the RLC circuit parameters for a damping ratio of 0.5.

[6]

Total = 25 marks

Question 3

Figure Q3 shows the voltage for one phase of a three-phase balanced delta connected load. The maximum value is 240V with a load resistance and inductance per phase of 0.2Ω and 3mH respectively. The operating frequency of the inverter is 60HZ.

a) Using the parameters in the waveform in figure Q3:

(i) Derive the equation which gives the Fourier Series for the voltage up to the 7th 	     harmonic.																																[15]

(ii) Draw the spectrum representation for the voltages up to the 7th harmonic.

[3]

b) Determine the RMS phase current for each harmonic and the total RMS current up to the

7th harmonic

[7]

Figure Q3

Total = 25 marks

Question 4

A full wave-controlled bridge rectifier has a 120V source at 60HZ with R = 10Ω, L = 20mH and α = 600

a)

(i) Draw the circuit and waveforms for discontinuous current and output voltage

[7]

(ii) Derive an expression for the average voltage

[4]

b)

(i) Calculate the average voltage and average load current

[7]

(ii) Calculate the power absorbed by the load

[7]

Total = 25 marks

Question 5

a)

(i) Draw an approximation diagram of an equivalent circuit per phase of an induction    	     motor when referred to the stator.

[4]

(ii) Derive an expression for the stator current per phase

[4]

(iii) Derive an expression for the per phase torque produced

[4]

b) Using the formulae derived in part (a) and the parameters given below of a star connected three-

phase induction motor, calculate:

(i) Stator current

[8]

(ii) Total torque produced

[5]

Poles = 6

Stator line voltage = 415V@ 60Hz

Rotor speed = 1140rpm

Stator leakage impedance = (0.7 + j2.0)Ω/phase

Rotor standstill impedance (referred to stator) = (0.6+ j2.0) Ω/phase

No load current = (1.7 – j6.5) A/phase

Total marks = 25

Diwedd/End