Jacob Smelt

PIC 1 Code

main.c

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

#include <lcd.c>

void ep_w(char, char);      // Write data to a specified location

char read_buttons(void);    // Read button states

char ep_r(char);            // Reads a value from a specified address

char txbyte;                // Byte to be transmitted depending on what button has been pressed

char val = 0;

// Main

void main()

{

lcd_init();

char buts = 0;  // Variable for button status

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

while (TRUE)  //loop

{

buts = read_buttons();  // Read the current button state

// Switch case to handle actions based on which button is pressed

switch (buts)

{

case 1:  // Case 1:

{

txbyte = '1';       // Set transmit byte to '1'

}

break;

case 2:  // Case 2:

{

txbyte = '2';       // Set transmit byte to '1'

}

break;

case 4:  // Case 3:

{

txbyte = '4';       // Set transmit byte to '1'

}

break;

case 8:  // Case 4:

{

txbyte = '8';       // Set transmit byte to '1'

}

break;

default:  // No button pressed state

break;

}

ep_w(0x00, txbyte);       // Write txbyte to the EEPROM using the ep_w function defined below

lcd_gotoxy(1,1);

printf(lcd_putc,"%c",txbyte); // print the value of txbyte on the LCD screen

lcd_gotoxy(1,2);

delay_ms(250); // delay put in to give the other pic time to write the value in 0x01 however it doesnt always work, could put a longer delay in but then task 8 wouldnt be needed

val = ep_r(0x01); // read the value from 0x01 and put it in variable val

printf(lcd_putc,"%c",val); // print the value of val on the LCD screen

delay_ms(20);

if (val != txbyte)

{

lcd_gotoxy(4,2);

printf(lcd_putc,"Error");

}

else lcd_gotoxy(4,2);

printf(lcd_putc,"     ");

}

}

// Function to write a byte 'what' to a given address 'where' via I2C

void ep_w(char where, char what)

{

while (!input(hs));    // Wait until the 'hs' signal is high

output_low(hs);        // Lower the 'hs' signal

i2c_start();           // Start I2C communication

i2c_write(0xA0);    -   // Send the device address

i2c_write(where);      // Send the specified address to write to

i2c_write(what);       // Send the data to write

delay_ms(20);          // Delay to ensure completion

i2c_stop();            // Stop I2C communication

delay_ms(20);          // Delay for stability

input(hs);             // Restore 'hs' signal state

}

// Reads data from specified address over I2C

char ep_r(char where)

{

char data = 0;            // Data variable to hold read value

while (!input(hs));       // Wait until the line is available

output_low(hs);           // Pull line low to initiate communication

i2c_start();              // Start I2C communication

i2c_write(0xA0);          // Send read command

i2c_write(where);         // Send address to read from

delay_ms(20);             // Delay for device processing

i2c_start();              // Restart communication

delay_ms(20);             // Delay

i2c_write(0xA1);          // Send read command again

data = i2c_read(0);       // Read data

delay_ms(20);             // Delay

i2c_stop();               // End communication

delay_ms(20);             // Additional delay

input(hs);                // Release the line

return data;              // Return the read data

}

// Function to read the button states

char read_buttons(void)

{

char value = 0;           // Variable to hold button state

while (!input_a());       // Wait until button input is available

value = input_a();        // Read the button state

delay_ms(500);            // Delay to avoid bouncing effect

return (value);           // Return the button state

}

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define HS pin_B3

PIC 2 Code

main.c

#include <main.h>

#include <2408.C>

#define LCD_ENABLE_PIN PIN_B2

#define LCD_RS_PIN PIN_B0

#define LCD_RW_PIN PIN_B1

#define LCD_DATA4 PIN_B4

#define LCD_DATA5 PIN_B5

#define LCD_DATA6 PIN_B6

#define LCD_DATA7 PIN_B7

#include <lcd.c>

// variable declarations

char buts = 0;              // Holds current button state

char old_but = 0;           // Holds previous button state

char validationval = 0;

char validatedval = 0;

// Function declarations

char ep_r(char);            // Reads a value from a specified address

void ep_w(char where, char what);  // Writes a value to a specified address

void payload(void);          // Processes actions based on button presses

void validation(void);

// Main function

void main()

{

lcd_init();

setup_adc_ports(NO_ANALOGS, VSS_VDD);

init_ext_eeprom();

while (true)              // Main loop

{

buts = ep_r(0x00);    // Read current button state from address 0x00#

validationval = ep_r(0x00); // Read current button state from address 0x00 and place it into variable validationval

validation();

validatedval = ep_r(0x01); // Read current button state from address 0x01 and place it into variable validatedval

if (old_but != buts)  // Check if button state has changed

{

lcd_gotoxy(1,1); // display on line 1

printf(lcd_putc,"%c",buts); // print the value of buts on the LCD screen

lcd_gotoxy(1,2);			// display on line 2

printf(lcd_putc,"%c",validatedval);// print the value of validatedval on the LCD screen

ep_w(0x10, buts);         // Write the new button state to address 0x10

delay_ms(20);             // Wait for debounce

payload();                // Process button actions

}

old_but = buts;       // Update the previous button state

}

}

// Processes button actions and updates control bytes

void payload(void)

{

switch (buts)             // Determine action based on button press

{

case '1':             // If txbyte "1" present

output_A(0);

output_high(pin_A3);

break;

case '2':             // If txbyte "2" present

output_A(0);

output_high(pin_A2);

break;

case '4':             // If txbyte "4" present

output_A(0);

output_high(pin_A1);

break;

case '8':             // If txbyte "8" present

output_A(0);

output_high(pin_A0);

break;

}

}

// Writes data to specified address over I2C

void ep_w(char where, char what)

{

while (!input(hs));       // Wait until the line is available

output_low(hs);           // Pull line low to initiate communication

i2c_start();              // Start I2C communication

i2c_write(0xA0);          // Send write command

i2c_write(where);         // Send address to write to

i2c_write(what);          // Send data to write

delay_ms(20);             // Delay for device processing

i2c_stop();               // End I2C communication

delay_ms(20);             // Additional delay for safety

input(hs);                // Release the line

}

// Reads data from specified address over I2C

char ep_r(char where)

{

char data = 0;            // Data variable to hold read value

while (!input(hs));       // Wait until the line is available

output_low(hs);           // Pull line low to initiate communication

i2c_start();              // Start I2C communication

i2c_write(0xA0);          // Send read command

i2c_write(where);         // Send address to read from

delay_ms(20);             // Delay for device processing

i2c_start();              // Restart communication

delay_ms(20);             // Delay

i2c_write(0xA1);          // Send read command again

data = i2c_read(0);       // Read data

delay_ms(20);             // Delay

i2c_stop();               // End communication

delay_ms(20);             // Additional delay

input(hs);                // Release the line

return data;              // Return the read data

}

void validation(void)

{

ep_w(0x01,validationval);

}

main.h

#include <18F2525.h>

#device ADC=10

#FUSES NOWDT                 	//No Watch Dog Timer

#FUSES FCMEN                 	//Fail-safe clock monitor enabled

#FUSES IESO                  	//Internal External Switch Over mode enabled

#FUSES PUT                   	//Power Up Timer

#FUSES NOMCLR

#FUSES BROWNOUT              	//Reset when brownout detected

#FUSES BORV21                	//Brownout reset at 2.1V

#FUSES NOPROTECT             	//Code not protected from reading

#FUSES NOWRT                 	//Program memory not write protected

#FUSES NOEBTR                	//Memory not protected from table reads

#use delay(internal=8MHz)

#use STANDARD_IO( C )

#use i2c(Master,Fast,sda=PIN_C4,scl=PIN_C3,force_hw)

#define hs pin_B3

Comments

Smelt Jacob

1.. The simulation runs and puts the button number in the top of lcd 1 and writes the data to the address 0x00 in the memory,

2.. the value is recieved by pic 2 and the value is displayed on lcd 2 and written to address location 0x01

3.. Your program illuminates the leds

4.. Pic 1 reads back the  the value returned in memory from pic2 and all values agree

5.. All lines on the displays agree with the displayed leds.

6.. your handshaking works correctly			100%