| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2023 | mai 2023 |

HND Electrical and Electronic Engineering

MODULE CODE: GOEE4002

Dynamics Control and Instrumentation

TIME:  2 HOURS

answer all Four questions

Equipment or materials required – Calculators are allowed – Formulae sheet attached.

Adnoddau neu ddefnyddiau sydd eu hangen - taflen fformiwlâu ynghlwm

Question One (10 marks)

- A 50 N force is applied horizontally to a 20 kg block resting on a surface. What is the acceleration of the block if the force of friction is 10 N?

2 Marks

- A body travelling at an initial velocity of 25m/s is acted upon by a linear acceleration of 10m/s2.

3 Marks

Calculate:

a	Velocity after 5 seconds

b	Total distance travelled

- Mason’s Gain Rule is used to reduce the block diagram to a single transfer function as shown:

TF(s)=1∆inPi∆i

Give the definition of each variable in the formal above.

5 Marks

Question Two (10 Marks)

The figure below shows a step input:

Draw a system time/response graph for the step input shown. Define the following terms and show clearly on the graph:

- Rise time

- Settling time

- Steady state error

- Maximum overshoot

- Delay time

Question Three (20 marks, 5 marks each)

Three particles A, B and C have a motion which can be described by the following expressions: -

For particle A   Sa = 3t2 + 10t - 8

For particle B    Sb = 7t3 + 4t2 + 4

For particle C    Sc = 10t3 - 30t + 2

Where S = displacement in m, t = time in seconds.

Determine:

- The time when the velocity of particle B is three times that of particle A to 2 decimal places?

- The time when the acceleration of particle B is equal to that of particle C to 3 decimal places.

- The distance travelled by each particle in the period from t= 0 to t= 4 seconds

- The time required for practical B to reach the velocity of 10 m/s

Question Four (20 marks)

For the given system block diagram:

G3

G4

G55

H2

G2

G1

H1

Y(s)

R(s)

+

+

+

_

+

+

_

+

H3

G3

G4

G55

H2

G2

G1

H1

Y(s)

R(s)

+

+

+

_

+

+

_

+

H3

- Produce a Signal Flow Graph of the system from the block diagram showing all nodes, branches, and gains.

- Apply Masons Gain Formula to determine the transfer function Y(s)R(s) of the system.

Formulae

TF(s)=1∆inPi∆i

v=u+at

v2-u2=2as

s=ut+ 12 at²

s=u + v2 t

Good luck

End of the exam