Ioan Davies

Student Number: 2115169

Design & analysis of extended surfaces

ACMA6011

# Advanced Thermofluid Mechanics

# BEng (Hons) Mechanical Engineering – Part Time Provision

Table of Contents

Introduction	3

Application of Extended Surfaces	3

Fins in Automotive Design	3

Extended surfaces in Micro-Processors Design	5

Plotting the Ratio of Heat Transfer	8

Analysis of Temperature Distribution	9

Temperature distribution of original length rods	9

Corresponding heat loss of original length rods	10

Optimum length rods and associated heat loss	11

Temperature distribution of optimum length rods	12

Temperature distribution of accurate length rods	13

Conclusion	16

Bibliography	17

Figure 1: 1904 Royce motor car with tube-fin radiator.	4

Figure 2: Louvered fin design for automobile.	4

Figure 3: Various examples of micro-processor heat sinks.	6

Figure 4: Plot of ratio of heat transfer for finite and infinite length fins.	9

Figure 5: Plot of temperature distributions for the four different materials.	10

Figure 6: Plot of temperature distributions for the four different materials with corresponding optimised lengths.	13

Figure 7: Plot of temperature distributions for the four different materials with corresponding accurate lengths using adiabatic tip boundary condition.	15

Figure 8: Plot of temperature distributions for the four different materials with corresponding accurate lengths using infinite fin boundary condition.	16

Table 1: Material Properties for rods.	9

Table 2: Calculated m value for each material.	10

Table 3: Heat loss values for all materials.	11

Table 4: Optimum rod length for each material.	12

Table 5: Optimum length heat loss values for all materials.	12

Table 6: Accurate rod length for each material.	14

# Introduction

Heat management is critical in the design and operation of automotive and micro-processor electronic systems. Extended surfaces also described as fins are used to enhance the heat transfer and ensure components maintain their optimal operating temperatures. This report explores the historical applications of fins in automotive design and how extended surfaces have been incorporated into to the design of micro-processors. Highlighting the recent technological advances in extended surfaces and their perceived advantages. The report also demonstrates the practical application of extended surfaces through calculations. Using excel a plot has been produced to confirm the optimum fin length, followed by various calculations for four different materials such as temperature distribution, heat loss, optimum length for heat loss and temperature distribution for the optimum length.

# Application of Extended Surfaces

## Fins in Automotive Design

Fins which can also be described as extended surfaces are designed to increase the surface area for heat transfer between a solid and fluid, most commonly air or liquid. In automotive applications, fins are primarily used in cooling systems to dissipate the heat generated by the engine or other components that generate heat. By increasing the heat exchange capacity, the fins help to maintain a stable operating temperature for the engine, which is important for the efficiency and performance of the automobile.

The application of fins in automotive design can be traced back to the early 20th century, which coincides with the rise of the internal combustion engine. According to (Ruian Sherrock Auto Parts, 2021) in 1904 the first tube-fin radiator was installed to the front of a car, by Royce. These radiators with finned surfaces were a practical solution for cooling these combustion engines, increasing the surface area available for heat to dissipate. Which allows heat to transfer more efficiently from the engine coolant to the surrounding air.

Figure 1: 1904 Royce motor car with tube-fin radiator.

(Rolls-Royce Motor Cars, 2024)

During the 1950s, advancements in materials and manufacturing processes led to further improvements in the design and efficiency of fins Implemented into automotive design. One example of this is louvered fins used in automotive car radiators. Firstly, investigated  by (Kays & London, 1984) during the 1950s, louvered fins became very popular in automotive design. Louvered fins are usually slanted or angled strips that extend from the surface, sometimes with small slit-like features or tabs as shown in Figure 2. These louvers help to distribute the air flow further and creates a turbulent flow, which increases the air contact with the fin’s surfaces and therefore, enhancing heat transfer (Erbay, et al., 2016). These design improvements allowed automobile radiators to become more compact and efficient.

Figure 2: Louvered fin design for automobile.

(Yadav, et al., 2017)

Due to the oil crisis in the 1970s which led to an emphasis on fuel efficiency further influenced the design priorities of automobile manufacturers. With vehicles becoming more powerful and engine technology advancing the demand for more efficient cooling systems also grew. Innovations such as compact and high-performance radiators and the integration of finned components into other areas of the engine were also developed, such as oil coolers and intercoolers. Automotive intercoolers were first introduced in 1977 on the Porsche 911 (WeCanFigureThisOut, 2020). An intercooler uses a series of fins to dissipate heat and cool the compressed air from a turbocharger before it reaches the engine. Cooler air allows for more air to be forced into the engine and therefore, increase the power output and engine efficiency.

In recent decades, fins have been implemented to most engine components that require some form of cooling or heat dissipation. Fin designs have advanced significantly with the introduction of computational fluid dynamics (CFD) and modern material science. Modern automobile designs feature optimised fins to provide much improved cooling performance which in turn improves engine efficiency and performance. A good example of this in modern cars is finned heat exchangers for battery packs in electric or hybrid cars. As explained by (Senior Flexonics, 2024) high performing and compact fin designs due to CFD enhance the longevity and performance of battery systems, which dissipate heat generated when charging or discharging the batteries.

## Extended surfaces in Micro-Processors Design

Extended surfaces have also been incorporated into the design and operation of micro-processors. When the performance of micro-processors increases so does the heat generated and therefore, efficient cooling solutions are required maintain performance and prevent thermal damage. One common solution to micro-processor cooling is the use of heat sinks which use extended surfaces such as fins to dissipate heat. (Mahajan, et al., 2006) explains that intricate fin designs increase the surface area available for heat transfer and therefore, allows the heat sink to dissipate more heat and lower the temperature of the adjacent components on the micro-processor chip. An example of a micro-processor heat sink is shown in Figure 3.

Figure 3: Various examples of micro-processor heat sinks.

(Electronic Components, 2019)

Recent advances in extended surface design for micro-processors have focused on optimising the fin structures and incorporating new materials and technologies. One such advance is microchannel heat sinks. These heat sinks use extremely thin channels that increase the available surface area for cooling and enhance heat transfer from the micro-processor surface to the surrounding air (Yu, et al., 2024). These microchannel heat sinks significantly reduce the size in comparison to traditional heat sinks as they provide the same surface area.

Another example of recent advancements is hybrid cooling solutions that combine air-cooled fin designs with liquid cooling solutions that use liquid coolants to absorb heat from the micro-processor and then dissipated by finned extended surfaces. (Wright, 2022) explains that water is capable of absorbing almost thirty times more heat compared to air and therefore, using liquid to absorb the heat and then dissipate through extended surfaces. This provides superiors cooling performance, especially in high power applications where lots of heat is generated.

Graphene is a material with exceptional thermal conductivity properties which has made it a promising material to be used in heat sink fins. (Scholarly Community Encyclopedia, 2024) suggests that applying a graphene coating to a copper material heat sink of a micro-processor significantly increased the heat dissipation and cooling of the system compared to traditional fin materials such as aluminium. Graphene coated fins significantly improve the heat transfer rates of extended surfaces, providing more efficient cooling for micro-processors.

Another recent advance in extended surfaces design is 3D-printed heat sinks. Advances in 3D printing technology have allowed highly complex fin geometries to be manufactured. This means that optimal geometries from CFD analysis can now be implemented into physical components and tailor them to specific thermal management needs, which further enhance the cooling performance. (Manaserh, et al., 2022) provides an optimised 3D printed heat sink solution for electronics that use guide vanes to target heat hotspots and channel the heat away from these areas. These guide vanes would not be possible to manufacture using traditional machining methods.

The advantage of Improved and optimised fin geometries and material coatings significantly increase heat dissipation, allowing micro-processors to operate at the higher performance levels they are now capable of without the risk of overheating. Another advantage associated with optimised fin designs is the ability to produce more compact cooling design solutions. Optimising the design by increasing the surface area allows the overall size of the design to decrease. Freeing up space in electronic devices in which micro-processors are used or reducing the overall size of the housing.

Another advantage of recent advances in extended surfaces is scalability. Modern cooling solutions and fin design can be scaled to meet the needs of various thermal applications, whether its small consumer electrical items such as phones or laptops to large data centres. In addition, improving the thermal management of micro-processors also reduces the risk of overheating and thermal damage. Therefore, improving the heat transfer of extended surfaces enhances the reliability and lifespan of the micro-processor by reducing components exposure to prolonged temperature increases.

# Plotting the Ratio of Heat Transfer

To plot the ratio of heat transfer for a fin with an adiabatic tip Qfinite with the heat transfer rate for an infinitely long fin Q∞ against a function of mL, we can use the following relationship:

QfiniteQ∞=tanh⁡(mL)

Equation 1: ratio of heat transfer rate for a fin with an adiabatic tip to infinitely long fin.

Figure 4: Plot of ratio of heat transfer for finite and infinite length fins.

The plot in Figure 1 shows that for small values of mL, tanh⁡(mL) increases almost linearly as the curve rises sharply. This indicates that as the fin length increases the heat transfer rate also increases significantly. As mL continues to increase,  tanh⁡(mL) begins to level out as it approaches the value of one in which mL is equal to 2.65. Further increases in fin length produce very little additional heat transfer beyond this point as the curve flattens out. Therefore, the plot confirms that mL is almost equal to 2.65 and is the optimal fin length as it provides nearly maximum possible heat transfer without the need for an infinitely long fin.

# Analysis of Temperature Distribution

## Temperature distribution of original length rods

To determine the temperature distribution along the rods, the theory of heat conduction in cylindrical rods must be used. For a very long rod (to a length of 400mm) with one end maintained at a constant temperature of 120C and the surface exposed to ambient air of 20C, the temperature distribution along the length of the rod can be calculated by;

Tx=T∞+(Tb-T∞)e-mx

Where, m=hPkA

Diameter, d=0.00476m

Parameter m can be simplified to:

m=4hkd

Using the following material properties:

| Material | Heat Transfer Coefficient, h (W/m^2 K) | Thermal Conductivity, k (W/m^2 K) |
| --- | --- | --- |
| 2024 Aluminium Alloy | 30 | 186 |
| Brass (70 Cu/30 Zn) | 30 | 99 |
| Copper | 15 | 398 |
| 304 Stainless Steel | 7.5 | 16 |

Table 1: Material Properties for rods.

By calculating m for each material and plugging the values back into the temperature distribution equation for a length range of 0 to 400mm in steps of 10mm we can plot the temperature distributions on the same axes for direct comparison as shown in Figure 5.

| 2024 Aluminium Alloy | Brass (70 Cu/30 Zn) | Copper | 304 Stainless Steel |
| --- | --- | --- | --- |
| 11.6421 | 15.9577 | 5.6277 | 19.8471 |

Table 2: Calculated m value for each material.

Figure 5: Plot of temperature distributions for the four different materials.

From the plot in Figure 5 we can see that Brass and 304 Stainless Steel exhibit steep decreases in temperature initially, gradually levelling out to ambient temperature of 20C at roughly 200mm. Indicating they both retain less heat along the rod’s length due to their lower thermal conductivity properties. 2024 Aluminium Alloy shows a more gradual temperature decrease, reaching ambient temperature at a length of roughly 310mm, which corresponds to its greater thermal conductivity properties. Copper shows a much more gradual curve even when compared to 2024 Aluminium Alloy due to its excellent thermal conductivity properties, in which it does not reach ambient temperature in a length of 400mm.

### Corresponding heat loss of original length rods

To determine the heat loss of each rod, the rate of heat transfer from the rod to the surrounding air must first be calculated. The rate of heat transfer can be calculated using the following;

Q= hPkAc Tb- T∞

Where for a cylindrical rod:

Perimeter, P= πd

P= π×0.00476

P= 0.0150m

Cross-sectional Area, A=πd24

A=π×0.0047624

A=0.0000178m

Tb=120°∁

T∞=20°∁

By plugging in the respective values of h and k each material into the rate of heat transfer equation and using the values calculated for Perimeter and Cross-sectional Area the values of heat loss in Table 2 are produced.

| Heat Loss (W) |  |  |  |
| --- | --- | --- | --- |
| 2024 Aluminium Alloy | Brass (70 Cu/30 Zn) | Copper | 304 Stainless Steel |
| 3.86 | 2.81 | 3.99 | 0.57 |

Table 3: Heat loss values for all materials.

## Optimum length rods and associated heat loss

Given that the optimum fin length corresponds to the parameter mL≈ 2.65, the actual length of the rod can be found using the following equation:

L=2.65m

Where,

m=4hkd

| Optimum Rod Length, L (m) |  |  |  |
| --- | --- | --- | --- |
| 2024 Aluminium Alloy | Brass (70 Cu/30 Zn) | Copper | 304 Stainless Steel |
| 0.228 | 0.166 | 0.471 | 0.134 |

Table 4: Optimum rod length for each material.

Therefore, to find the heat loss using the optimum lengths of the rods, the following formula for the heat transfer rate of a fin of finite length with insulated tip:

Q= hPkAc Tb- T∞tanh(mL)

Where,

tanh(2.65) ≈0.99

The heat loss can then be recalculated for each material.

| Optimum Length Heat Loss (W) |  |  |  |
| --- | --- | --- | --- |
| Aluminium Alloy | Brass (70 Cu/30 Zn) | Copper | 304 Stainless Steel |
| 3.81 | 2.78 | 3.95 | 0.56 |

Table 5: Optimum length heat loss values for all materials.

All the new heat loss values using the optimised rod lengths are slightly lower than the original heat loss values. This can be explained by the optimised fin lengths providing slightly more efficient thermal performance. Which demonstrates the practical application of optimising the fin length to balance heat transfer efficiency and material usage. However, the heat loss values being very similar is expected for a very long fin or rod, as the heat loss becomes independent of the length beyond a specific point and eventually approaching the same values.

## Temperature distribution of optimum length rods

The temperature distribution along the optimum length of the rod can be calculated by;

Tx= T∞+ (Tb- T∞)cosh⁡[mL-x]cosh⁡(mL)

By using the values for m in Table 2 for each material and optimum rod length values from Table 4, then plugging the values back into the temperature distribution equation for a length range of 0 to 400mm, in steps of 10mm, we can plot the temperature distributions on the same axes for direct comparison as shown in Figure 6.

Figure 6: Plot of temperature distributions for the four different materials with corresponding optimised lengths.

The new optimised lengths have significantly reduced their lengths of the rods when compared to the original temperature distribution in Figure 5, which affects the gradients of the subsequent curves accordingly. This is with the exception of the Copper rod in which the optimum length has increased from the original 400mm rod length. This is due to coppers superior thermal conductivity properties allowing more heat to be distributed further down the length of the rod. Therefore, each materials thermal distribution curve reflects their thermal conductivity and heat transfer coefficient properties. With the shorter rods having steeper temperature drops.

## Temperature distribution of accurate length rods

To determine a more accurate length for each of the rods in which the temperature is very close to the fluid temperature, we can use the following condition;

e-mL<0.01

Solving for L,

-mL<ln⁡(0.01)

L>ln(0.01)-m

L>-4.605-m

L>4.605m

Using this formula the accurate rod length for each material can be calculated as shown in Table 6.

| Accurate Rod Length, L (m) |  |  |  |
| --- | --- | --- | --- |
| 2024 Aluminium Alloy | Brass (70 Cu/30 Zn) | Copper | 304 Stainless Steel |
| 0.396 | 0.289 | 0.818 | 0.232 |

Table 6: Accurate rod length for each material.

The temperature distribution along the accurate length of the rod can be calculated by using the equation for an adiabatic tip boundary condition;

Tx= T∞+ (Tb- T∞)cosh⁡[mL-x]cosh⁡(mL)

By using the values for m in Table 2 for each material and accurate rod length values from Table 6, then plugging the values back into the temperature distribution equation in steps of 10mm, we can plot the temperature distributions on the same axes for direct comparison as shown in Figure 7.

Figure 7: Plot of temperature distributions for the four different materials with corresponding accurate lengths using adiabatic tip boundary condition.

The temperature distribution along the accurate length of the rod can be calculated by using the equation for an infinite fin boundary condition;

Tx=T∞+(Tb-T∞)e-mx

By using the values for m in Table 2 for each material and plugging the values back into the temperature distribution equation in steps of 10mm we can plot the temperature distributions on the same axes for direct comparison as shown in Figure 8.

Figure 8: Plot of temperature distributions for the four different materials with corresponding accurate lengths using infinite fin boundary condition.

By comparing the temperature distribution plots of the accurate length rods with the original length rods shown in Figure 5, the original length rods provided a much more gradual temperature distribution due to the longer estimated rod lengths. However, this may not be very efficient for practical cooling applications. When compared to the optimised rod length temperature distribution shown in Figure 6, there was a slightly faster temperature drop and compared similarly to the accurate length temperature distribution. Although the temperature distribution in Figure 6 does not accurately reflect the temperature close to the ambient temperature at the rods tip. As the accurate rod length shown in Figure 7 and 8 ensures efficient cooling across the rod length while maintaining accuracy of temperature close to the rods tip. As demonstrated the curves all end at the ambient temperature of 20C.

# Conclusion

The analysis of this report examined the temperature distribution along rods of various materials and determining the most accurate and practical predictions of their thermal behaviours. The analysis of the original length rods up to 400mm showed a gradual temperature decrease along the length of the rods for all materials. However, this is not practical for real application of cooling methods. Further analysis calculated the optimised lengths of the rods for more efficient cooling and demonstrated a steeper temperature gradient as the temperature decreases quicker over a shorter distance but required further refinement to ensure the tip temperature was accurately represented as it approached ambient temperature of 20C. The accurate length rods offered the best balance of efficient cooling and accurate temperature prediction at the rods tip. These findings highlight the importance of selecting appropriate rod lengths to balance practical cooling efficiency and predictions of thermal distribution behaviour.

# Bibliography

Electronic Components, 2019. Understanding heat sinks: Functions, types, & more. [Online] 
Available at: https://www.arrow.com/en/research-and-events/articles/understanding-heat-sinks-functions-types-and-more
[Accessed 12 June 2024].

Erbay, L. B., Dogan, B. & Ozturk, M. M., 2016. Comprehensive Study of Heat Exchangers with Louvered Fins. s.l.:Intechopen.

Kays, W. M. & London, A. L., 1984. Compact Heat Exchangers. 3rd Edition ed. London: McGraw Hill.

Mahajan, R., Chiu, C.-P. & Chrysler, G., 2006. Cooling a Microprocessor Chip. Chandler, Intel Corporation.

Manaserh, Y. et al., 2022. Multi-objective optimization of 3D printed liquid cooled heat sink with guide vanes for targeting hotspots in high heat flux electronics. International Journal of Heat and Mass Transfer, Volume 184.

Rolls-Royce Motor Cars, 2024. How Rolls met Royce. [Online] 
Available at: https://www.rolls-roycemotorcars.com/en_GB/inspiring-greatness/values/how-rolls-met-royce.html
[Accessed 11 June 2024].

Ruian Sherrock Auto Parts, 2021. The Evolution and Maturity of Automobile Radiator Layout. [Online] 
Available at: https://www.sheradiatortank.com/a/technology/Knowledge/2021/0315/AUTOMOBILE_RADIATOR_LAYOUT.html
[Accessed 10 June 2024].

Scholarly Community Encyclopedia, 2024. Graphene-Coating for Efficient Electronics Cooling, s.l.: Scholarly Community Encyclopedia.

Senior Flexonics, 2024. Electric Vehicle Thermal Management Solutions. [Online] 
Available at: https://www.seniorflexonics.com/electric-vehicle-thermal-management-solutions/
[Accessed 10 June 2024].

WeCanFigureThisOut, 2020. Intercooler. [Online] 
Available at: https://www.wecanfigurethisout.org/ENERGY/Web_notes/Energy_Consumption/Greener_Cars_and_Trucks_Supporting_Files/Intercooler%20-%20Wikipedia.pdf
[Accessed 10 June 2024].

Wright, G., 2022. Water Cooling. [Online] 
Available at: https://www.techtarget.com/searchdatacenter/definition/water-cooling#:~:text=Water%20cooling%2C%20also%20called%20liquid,30%20times%20faster%20than%20air.
[Accessed 12 June 2024].

Yadav, M. S., Giri, S. A. & Momale, V. C., 2017. Sizing analysis of louvered fin flat tube compact heat exchanger by genetic algorithm. Applied Thermal Engineering, Volume 125, pp. 1426-1436.

Yu, Z.-Q., Mo-Tong, L. & Bing-Yang, C., 2024. A comprehensive review on microchannel heat sinks for electronics cooling. International Journal of Extreme Manufacturing.