Site Waste Management Plan –

Paxton Street Car Park, Swansea

James Cullen (p151951)

Project Information

Project Name: Paxton Street office development.

Project Location: Local Authority Carpark, Paxton Street, Swansea, SA1 3SQ.

Site Location Description: Site currently in use as a public carpark operated by Swansea County Council and is bounded by Swansea civic centre, Marriott Hotel and Swansea Marina.  Access to location is via Oystermouth Road.

Project Cost (approximate): £20m.

Footprint Area: 900m₂.

Project Start Date: Summer 2021.

Project End Date: Spring 2022.

Client: City and County of Swansea Council.

Principle Contractor: Cullen Construction Ltd.

Document Version: 1 (March 2020).

Responsibilities

|  | Name | Company | Role | Contact Details |
| --- | --- | --- | --- | --- |
| Who is responsible for drafting the SWMP? | Joe Rodon | Cullen Construction LTD | Lead Consultant | Joe.Rodon@CullenC.co.uk 07896536719 |
| Who is responsible for implementing the SWMP | Ben Cabango | Cullen Construction LTD | Waste Coordinator | Ben.Cabango@CullenC.co.uk 07896536716 |
| Project Manager | Liam Cullen | Cullen Construction LTD | Project Manager | Liam.Cullen@CullenC.co.uk 07896536756 |
| Client Representative | Steve Cooper | City and County of Swansea Council | Contracts Manager | Steve.Cooper@Swancc.co.uk 07896536722 |

SWMP storage: 1) Electronic copy: City and County of Swansea Council intranet.

2) Electronic copy: Cullen Construction Ltd intranet.

3) Hard copy: Site office, Paxton Street, Swansea.

Declaration (Client and contractor to sign to agree to work collaboratively to keep waste to a minimum):

Client Representative:

Contractor Representative:

Legislation / Guidance applicable

This document has been compiled with reference to:

- Waste Framework Directive 2008/98 + 2018/851 update.

- Commission Decision 2014/955/EC about waste classification.

- Commission Notice on technical guidance on the classification of Waste (2018/C 124/01).

- The List of Wastes (LoW) (England) Regulations 2005 + 2015 update.

- Guidance on the classification and assessment of Waste. Technical Guidance WM3 (2015).

- Waste (England and Wales) Regulations 2011 (Ammended in 2012).

- Environmental Permitting (England and Wales) Regulations 2010.

- Hazardous Waste (England and Wales) Regulations 2005 (Ammended in 2009).

Construction Method

| Work Phase | Method of Construction | Materials Used |
| --- | --- | --- |
| 1)Site Preparation | Mechanical Excavation | N/A |
| 2)Foundation | Raft Foundation Installation | Concrete and Steel |
| 3)Basement | Diaphragm Wall Installation | Concrete and Steel |
| 4)Superstructure | Pre-cast fabricated frame Installation | Concrete and Steel |
| 5)Cladding | Pre-fabricated brick clay wall Installation | Brick and Mortar |

Site Preparation and waste produced

Mechanical Excavation of site required to a house foundation and 3 storey basement to a depth of roughly 11m:

| Waste produced by action | Waste Management Code | Waste Classification | Total Wastage Volume | Total Wastage Mass | Action for Material Produced |
| --- | --- | --- | --- | --- | --- |
| Soil | 17.05.04 | Inert | 9900m₃ | 12375 tonnes | 100% Re-use for site landscaping |

Foundation installation and waste produced

Roughly 1m thick raft foundation to be installed:

| Waste produced by action | Waste Management Code | Waste Classification | Total Wastage Volume | Total Wastage Mass | Action for Material Produced |
| --- | --- | --- | --- | --- | --- |
| Concrete | 17.01.01 | Inert | 36m₃ | 43.56 Tonnes | To be removed from site (and subsequently 100% recycled) |
| Steel | 17.04.05 | Non -hazardous | 8.57m₃ | 3.6 Tonnes | To be removed from site (and subsequently 100% recycled) |

Basement installation and waste produced

Diaphragm wall to be installed:

| Waste produced by action | Waste Management Code | Waste Classification | Total Wastage Volume | Total Wastage Mass | Action for Material Produced |
| --- | --- | --- | --- | --- | --- |
| Concrete | 17.01.01 | Inert | 10.2m₃ | 12.65 Tonnes | To be removed from site (and subsequently 100% recycled) |
| Steel | 17.04.05 | Non -hazardous | 12.57m₃ | 5.28 Tonnes | To be removed from site (and subsequently 100% recycled) |

Superstructure installation and waste produced

Roughly 200mm thick pre-fabricated concrete and steel frame be installed, 20 storeys’ high, 30m x 30m x 30m 30m:

| Waste produced by action | Waste Management Code | Waste Classification | Total Wastage Volume | Total Wastage Mass | Action for Material Produced |
| --- | --- | --- | --- | --- | --- |
| Concrete | 17.01.01 | Inert | 0m₃ | 0 Tonnes | n/a |
| Steel | 17.04.05 | Non -hazardous | 0m₃ | 0 Tonnes | n/a |

Cladding installation and waste produced

Pre-fabricated off-site brick cladding to be installed for 20 storeys’ high, 30m x 30m x 30m 30m:

| Waste produced by action | Waste Management Code | Waste Classification | Total Wastage Volume | Total Wastage Mass | Action for Material Produced |
| --- | --- | --- | --- | --- | --- |
| Brick | 17.01.02 | Inert | 0m₃ | 0 Tonnes | n/a |
| Mortar | 17.09.03 | Inert | 0m₃ | 0 Tonnes | n/a |

Overview of waste produced in total

| Work Phase | Waste Type Produced | Total Wastage Volume | Total Wastage Mass | Action for waste |
| --- | --- | --- | --- | --- |
| Site Preparation | Soil | 12375m₃ | 12375 tonnes | Re-use |
| Foundation | Steel | 8.57m₃ | 3.6 Tonnes | Removal from site |
|  | Concrete | 36m₃ | 43.56 Tonnes | Removal from site |
| Basement | Steel | 12.57m₃ | 5.28 Tonnes | Removal from site |
|  | Concrete | 10.2m₃ | 12.65 Tonnes | Removal from site |
| Superstructure | None | 0m₃ | 0 Tonnes | n/a |
| Cladding | None | 0m₃ | 0 Tonnes | n/a |
| Packaging | Various | 150 m₃ | 31.5 Tonnes | Removal from site |

Overview of actions and costs associated with waste being transported from site

All waste sorted on site into specific skips in readiness for removal by accredited carriers and disposers.

| Type of Waste | Total Volume produced | Total Mass Produced | To be Removed by | Cost |
| --- | --- | --- | --- | --- |
| Concrete | 46.2m₃ | 56.21 Tonnes | Skip Hire Swansea, Baldwin’s Crescent, Swansea, SA1 8QB | £1848 |
| Steel | 21.11m₃ | 8.88 Tonnes | Skip Hire Swansea, Baldwin’s Crescent, Swansea, SA1 8QB | £1108.80 |
| Packaging | 150 m₃ | 31.5 Tonnes | Skip Hire Swansea, Baldwin’s Crescent, Swansea, SA1 8QB | £5544 |

Total cost anticipated for waste removal: £8750.

Implementation of Waste Management Plan

- All operatives on site to be trained and regularly briefed in Waste Management and training records kept.

- Procedural documents to be disseminated to staff and displayed on notice boards.

- Records of all waste produced, its classification, its amount to be kept.

- Details of waste removal from site, when and by whom to be kept.

Monitoring of Waste Management

- Waste champion to constantly monitor waste management on site.

- All actions and orders to be recorded.

Review of Waste Management

- Whole scheme to be reviewed upon completion.

- Lessons learnt to be discussed and recorded in a register.

- Actual and forecast volumes and cost comparison.

Appendix

Excavation calculations

Excavation volume: 30m x 30m (site footprint) x 11m (depth of excavation) = 9900m₃.

9900 x 1.25 t / m₃ (soil bulking & conversion factor) = 12,375 Tonnes.

Foundation calculations

Foundation slab volume: 30m x 30m (site footprint) x 1m (slab thickness) = 900m₃ total volume.

30m x 30m (site footprint) = 900m₂ x 100kg/m₂ (concrete to steel ratio) = 90,000kg ÷ 1000 = 90 Tonnes.

90 Tonnes x 4% wastage = 3.6 Tonnes of steel wastage.

3.6 Tonnes ÷ 0.42 t / m₃ (steel bulking & conversion factor) = 8.57m₃ volume of steel wastage.

900m₃ (total volume) x 4% wastage = 36m₃ Volume wastage.

36m₃ x 1.24 (concrete bulk factor) = 43.56 Tonnes wastage.

Basement calculations

Diaphragm wall volume: 30m x 4 (perimeter) x 0.5m (wall thickness) x 11m (height) = 255m₃ total volume.

30m x 4 (perimeter) = 120m x 11m (height) = 1320m₂ total surface area of diaphragm wall.

1320m₂ x 100kg/m₂ (concrete to steel ratio) = 132,000kg ÷ 132 Tonnes.

132 Tonnes x 4% wastage = 5.28 Tonnes of wasted steel.

5.28 Tonnes ÷ 0.42 t / m₃ (steel bulking & conversion factor) = 12.57m₃ volume of steel wastage.

255m₃ (total volume) x 4% wastage = 10.2m₃ volume wastage.

10.2m₃ x 1.24 t / m₃ (concrete bulking & conversion factor) = 12.65 Tonnes wastage.

Superstructure calculations

Concrete and Steel pre-fabricated steel frame required: 30m x 4 = 120m Perimeter.

120m x 66.6m (building height) = 7992m₂ total area of frame required.

7992m₂ x 0.2m (frame thickness) = 1598.4m₃ total volume of frame required.

Cladding calculations

Brick and Mortar pre-fabricated cladding required: 32m x 4 (Rough Perimeter of building frame) = 128m

128m x 66.6m (building height) = 8524.8m₂ total area of cladding required.

8524.8m₂ x 0.1m (cladding thickness) = 852.48m₃ total volume of cladding required.

Packaging calculations

Various materials from derived from resources delivered to site roughly totalling 150 m₃.

150m₃ x 0.21 t / m₃ (packaging bulking & conversion factor) = 31.5 Tonnes of packaging wastage.

Totals:

Steel volume wastage: 8.57m₃ + 12.57m₃ = 21.11m₃.

Steel mass wastage: 3.6 Tonnes + 5.28 Tonnes = 8.88 Tonnes.

Concrete volume wastage: 36m₃ + 10.2m₃ = 46.2m₃.

Concrete mass wastage: 43.56 Tonnes + 12.65 Tonnes = 56.21 Tonnes.

Number of skips (volume: 10.7m₃) required

Skip Hire Swansea, Baldwin’s Crescent, Swansea, SA1 8QB charges £369.60 per skip removal.

Concrete total volume wastage: 46.2m₃.

46.2m₃ ÷ 10.07m₃ = 4.59 (5 skipfulls).

5 x £369.60 = £1848.

Steel total volume wastage: 21.11m₃.

21.11m₃ ÷ 10.07m₃ = 2.09 (3 skipfulls).

3 x £369.60 = £1108.80.

Packaging total volume wastage: 150 m₃.

150m₃ ÷ 10.07m₃ = 14.90 (15 skipfulls).

15 x £369.60 = £5544.

5 + 3 + 15 = 23 skips total.

Therefore £369.60 x 23 skips = £8500.80.