Risk Assessment – large calibre L118 105 mm Gun.

The number of staff typically operating the gun at one time will be 2 or 3.

| Hazard | How could the risk cause harm | What and who are in harm’s way | Control measures | Pre-mitigation risk | Resididual risk |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Severity | Likelihood | Rating |
| Noise | During the gun firing the sound wave overpressure could cause damage to peoples hearing Leading to permanent hearing damage / deafness | Staff in a relatively close (30 m) vicinity | - Staff within the proximity must wear ear defenders to the correct specification (EN 352) [1] - Operators of the gun must perform a visual check that staff close are wearing ear protection before the round is loaded into the barrel - Staff responsible for using the gun must ensure that a minimum number of people are present for using the gun. But not so few that the use of the weapon is hindered - Only fire the gun, when necessary, as instructed by a chain of command - When using propellant increments always ensure that the minimum number of increments are used for the task at hand - Noise danger training / briefing to new members of staff taking up the role | High | D | 3 | Medium (A) |
| Slips, trips and falls | While moving charges to and from the gun / storage bay. General operations around the gun. Leading to bumps, grazes, rarely broken bones | Staff operating the gun | - A ‘site’ layout map should be generated, and all equipment should be stored in the correct location when not in use - If a staff member sees something in the wrong location, and it is obvious that it is not being used, they must move it back to the correct location - When setting up the ‘site’ every effort will be made to minimize objects in a walking area. For example, racking and trolleys should be on the side, and not in the middle of the walkway | High | C | 4 | Medium (A) |
| Manual Handling | Staff could incorrectly lift objects causing injuries to various parts of the body. Additionally, staff could drop heavy loads onto their feet | Staff carrying ammunition to load the gun | - Staff must have undergone the necessary manual handling training as guided by the health and safety at work act [1] - Staff must wear safety boots that conform to the EN ISO 20345 standard [3] - Ensure that all lifts are done in accordance with the lifting and lowering chart shown in figure 1 below. - Any lifts that exceed the guidance in figure 1 shall be carried out by 2 or more people, depending on the total weight of the unit - Staff must have completed the necessary manual handling training | High | C | 3 | Medium (A) |
| Pinch Points | Staff could get fingers / limbs trapped in moving parts of the gun system | Members of staff responsible for loading the gun with ammunition | - When staff are rapidly loading / firing the gun, they must ensure that the loader has all body parts clear of the gun before firing. This can be achieved verbally, or with a signal of two hands in the air - The person firing the gun must check the loader for ‘hands clear’ signal before firing the gun and sending the round downrange - Loaders must be careful to not get fingers / hands trapped under the round when loading. This can be mitigated by the loader having proper training on the correct technique - The gun must be well maintained to ensure all parts are located in the proper position. This will mitigate against parts being in unexpected places, increasing the risk of pinch points | High | D | 3 | Medium (A) |
| Shock wave from gun firing | Death or serious injury to staff. Potential loss of hearing | Staff in the vicinity of the gun firing | - Staff operating the gun, and controlling the area in front of the gun must ensure that no one is in front of the gun (up to 50 m). Within this 50 m distance the risk to staff increases greatly - Only authorised staff must be in proximity of the gun when it is firing, keeping the number as low as practicable - The firer should perform a quick visual check before sending the round downrange, to ensure there is no one in front - The gun should be positioned in a location where the is considerable distance between it and the next location people are likely to be operating | High | E | 2 | Medium (A) |
| Unintentional detonation of round in gun | Death or serious injury to personnel nearby caused by the explosion of the round, fragments created by the round | Staff operating the gun | - The gun barrel must be well maintained to ensure no build up of propellant components - Staff must have the appropriate training in order to be part of the gun team - The gun must only operated according to the standard operating procedure. E.g. not firing the gun too quickly which will allow the barrel to sufficiently cool - Ensure only ammunition that is compatible with the gun is used - Regular gun checks shall be performed to check for cracks / wear and tear - Only the maximum number of people deemed necessary will be around the gun during firing. This minimizes the number of potential casualties | High | E | 1 | Medium (A) |

Figure 1 - Lifting Limits [4]

Overall, this activity, when considering only these risks can be considered a Medium (A) risk activity.

In the real world there are likely to be far more hazards present, and so the overall risk rating may change.

The risk assessment and mitigations have successfully reduced the overall risk of the activity from high to medium.

Risk Matrix and Criterion

|  |  | Probability of likelihood |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 Very Low | 2 Low | 3 Medium | 4 High | 5 Very High |
| Severity | (E) Critical | Medium (A) | Medium (A) | High (R) | High (R) | High (R) |
|  | (D) Severe | Low (Y) | Medium (A) | Medium (A) | High (R) | High (R) |
|  | (C) Major | Low (Y) | Medium (A) | Medium (A) | Medium (A) | High (R) |
|  | (B) Moderate | Very Low (G) | Low (Y) | Medium (A) | Medium (A) | Medium (A) |
|  | - Minor | Very Low (G) | Very Low (G) | Low (Y) | Low (Y) | Medium (A) |

G = Local line management review and authorisation

Y = Local line management review and authorisation

A = Local line management review, HSE review, senior authorisation

R = HS&E review required, top level or delegated authority authorisation

Severity

Critical – fatality / multiple life changing injury to 3 or more people or 5 mil + damage

Severe – life changing injury, damage 1 – 5 mil

Major - acute injury that can be recovered from, damage 100k – 1 mil

Moderate – injury requiring medical attention, damage 50k - 100 k

Minor – first aid injury, damage between 1 – 50k

Likelihood

5 = 1 in 10

4 = 1 in 100

3 = 1 in 1000

2 = 1 in 10000

1 = 1 in 1 million

ALARP Justification – 1497 words in text below

ALARP Introduction

ALARP, or As Low As Reasonably Practicable is a practice which takes into account the risks of an activity, and weighs it against various factors that are required to control it – examples include cost, time and number of people present for a task. It is a framework set out by the Health and Safety Executive (HSE) [5]. As following ALARP is a fairly subjective process, it is important to learn from previous incidents, so you can easily recognise potential mistakes that you will make – allowing you to better follow the industry best practice. ALARP comes from the Health and Safety at Work act, 1974, which forms the legislative backbone for many of the health and safety framework regulations that cross multiple industries and professions. If a company is following the ALARP principle, then it is critical that all staff members are aware of the method behind it. This is because working safely is everyone’s responsibility, so if everyone is aware of the ALARP principle, then they can all work towards it and maintain a work environment that is as safe as possible.

Chosen Hazard

The next section of this short ALARP justification report will focus in on one of the hazards that was identified in the risk assessment above. The report will be based on the ALARP argument structure, which will include the following subheading structures:

- Risk identification

- The necessity for taking the risk

- The duty holder

- Existing mitigation

- Relevant legislation

- Relevant, recognized good practice

The hazard that I have chosen to focus in on is noise. This is because it is a well-established hazard, that needs to be considered in many different industries – it is not specific to firing the large calibre L118 105 mm gun. The unique environment of a military situation does provide its own unique risks and consideration, which will be discussed in the report.

Risk ID

As noise can be directly measured, it can be considered as a quantitative risk. This makes prescribing control measures easier in some respects, because as long as you have a number to work towards, you can design PPE, and process around keeping the number to a minimum. Sound can be measured using the decibel or dB scale. It is logarithmic and works by assuming 0 dB is complete silence. The logarithmic scale then means that an increase to 10 dB will equate to a sound 100 times more intense for that at 0 db. 20 dB is therefore 1000 times more intense than 0 dB, and the scale proceeds in this way. It has been found that frequent exposure to sound greater than 85 dB can lead to permanent hearing damage [6]. A 105mm Howitzer can reach up to 147.5 dB [7], which is significantly higher than the 85 dB limit. This therefore poses a considerable risk which the army (in this case) have a legal obligation to try and mitigate.

Why Risk Needs to Be Taken

During conflict, using long range weapons to defeat / pin back enemy targets is of great value. The large calibre L118 105 mm gun provides this long-range support, which will in turn allow infantry to push further, or provides a strong defence against in coming assaults. From this perspective, the use of the artillery will likely outweigh the risks of deciding to not use it, as this may give enemy forces a significant advantage. This in some way is an ALARP justification for using the weapon system, as deploying them when required is a practicable solution to the problem. To summarise, deploying the weapons outweighs the potential risk of death if they are not being used, and therefore defines why the risk needs to be taken.

The Duty Holder

In the air force, duty holders are people appointed to be responsible for the safety, and ensuring that the Risk to Life (RtL) follows an ALARP principle [8]. It is feasible that the army adopts a similar process, which has not been published online. As with most big organisations, the person chiefly responsible for the safety is the person in the most senior position – i.e. the general. The generals will delegate the authority down the chain of command so it may be the squad leader / ranking officer working with the gun that is the duty holder in the case of the large calibre L118 105 mm gun. The duty holder will be supported by past evidence and experience to determine what is ALARP in this situation. For example, a 3-man operating team may be the limit to ensure swift operation, with the minimum number of people in close proximity.

Existing Mitigation

The existing mitigation is the basis of the contents in the appropriate segment of the risk assessment in the previous section. The mitigations that were identified are labelled 1 – 6.

- Staff within the proximity must wear ear defenders to the correct specification (EN 352) [1]

- Operators of the gun must perform a visual check that staff close are wearing ear protection before the round is loaded into the barrel

- Staff responsible for using the gun must ensure that a minimum number of people are present for using the gun. But not so few that the use of the weapon is hindered

- Only fire the gun, when necessary, as instructed by a chain of command

- When using propellant increments always ensure that the minimum number of increments are used for the task at hand

- Noise danger training / briefing to new members of staff taking up the role

Some of these mitigations, such as 1 aim to reduce the overall risk by reducing the severity of the risk itself. Wearing ear defenders decreases the amount of sound energy that reaches in inner ear, and so reduces the severity of the effect of the sound. For example in this case, the severity will be reduced from severe (complete deafness) to major (temporary tinnitus), thus reducing the overall risk. The second mitigation whereby the staff member operating the gun must perform a quick visual check for the people that they can clearly see – making sure they are wearing the PPE is a mitigation that aims to reduce both the likelihood and severity of the risk. If before every time the gun is fired there is a check that everyone is wearing the PPE, then this will reduce the likelihood of risk occurring. Actually wearing reduces the severity, as previously discussed. Mitigation 4, only using the gun when necessary to do so reduces only the likelihood of harm occurring. If the staff were to be irresponsible and use the gun to show other people how it works, then this increases the number of times the gun is fired and so in turn increases the number of times the risk can cause the damage. The controls for this hazard were devised by a knowledge of noise regulations, and by applying some common sense for the best ways to mitigate the risks. For example, only firing the gun when commanded is an obvious way to reduce the likelihood of an accident.

To maintain being ALARP it is important to not overprescribe control measures. Something that was considered, but deemed grossly disproportionate would be to redesign the whole gun system to make it quieter. If the mitigations stated are followed that it should provide a good degree of protection to the risk – redesigning the gun would be too costly for a small benefit, and so would fail the cost benefit analysis.

Relevant Legislation

- The Control of Noise at Work Regulations 2005 – the specific laws regarding protecting staff from noise.

- The Health and Safety at Work Act 1974 – how and why the employer is held responsible for the safety of staff.

- It is likely that there is additional military legislation that covers operational procedure, however this is not open-source information.

Relevant Recognised Good Practice

According to the U.S DoD, hearing loss is the most common medical issue among veterans [9] with 2.7 millions veterans receiving benefits when the article was published. Clearly this is an important issue that needs to be controlled. One of the manufacturers of hearing PPE was sued by the US government for a multimillion-dollar settlement due to providing defective earplugs [10]. This is an important lesson learned that the PPE should be checked to ensure it is functioning as intended. Doing this shows good practice at mitigating the risk.

In The Future

With advances in technology, it is possible that the artillery gun system will eventually become fully automated (loading, targeting, and firing). This therefore will likely completely remove the noise hazard from the risk assessment for the system – as no one will have to be in close proximity while it is firing. However, it is important to remember that injury to humans is not the only form of risk. As the cost of an automated system goes up, so does the financial risk incurred if there was an accident that destroyed the equipment.

# References

| [1] | WISEWORKSAFE, "Undersanding EN 352 - Hearing Protection Standard," 26 March 2019. [Online]. Available: https://www.wiseworksafe.com/blog/view/understanding-en-352-hearing-protection-standard. [Accessed 06 July 2023]. |
| --- | --- |
| [2] | HSE, Manual handling at work: A brief gude, London: TSO, 2020. |
| [3] | Uvex, "Standards and directives for safety footwear," Uvex, [Online]. Available: https://www.uvex-safety.co.uk/en/knowledge/safety-standards/safety-footwear/. [Accessed 06 July 2023]. |
| [4] | Simpro, "Manual handling in the workplace," Simpro Handling Equipment, [Online]. Available: https://simpro.world/connect/blog/manual-handling-in-the-workplace. [Accessed 06 July 2023]. |
| [5] | Health and Safety Executive, "ALARP "at a glance"," HSE, [Online]. Available: https://www.hse.gov.uk/enforce/expert/alarpglance.htm. [Accessed 07 July 2023]. |
| [6] | P. Rabinowitz, "Noise-Induced Hearing Loss," American Family Physician, pp. 2749-2756, 01 May 2000. |
| [7] | M. Salsbury, "Feasibility of an Artillery Silencer," US Army, 1969. |
| [8] | UK Government, Regulatory Article 1020, London: UK Government. |
| [9] | C. Michelle, "Top Medical Issue for Veterans: Hearing Loss," AARP, 23 March 2018. |
| [10] | Sound Relief Hearing Centre, "The Critical Importance of Effective Military Hearing Protection," Sound Relief Hearing Centre, [Online]. Available: https://www.soundrelief.com/the-critical-importance-of-effective-military-hearing-protection/. [Accessed 12 July 2023]. |