| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2021 |  |

MODULE CODE: ACHA5015

Stress Analysis & CAE

Exam

TIME:  2 HOURS

answer ALL questions

Equipment or materials required – Calculator

A ‘formula sheet’ is provided for reference.

Assume ‘g’  =  9.81 m/s2.

and  1 bar  =  105 N/m2 (Pa)

ANSWER ALL QUESTIONS

QUESTION ONE

3.5 kN

1500 N/m

1.5 m

A 1.5 m long cantilever beam has a point load of 3.5 kN on the free end of the beam & also carries a uniformly distributed load of 1500 N/m along its entire length. The beam has a flexural stiffness of 2.2 x 106 Nm2.

Calculate.

- The deflection at the free end of the beam due to the loads.

(7 Marks)

- How much the point load may be increased before the deflection exceeds 5mm

(3 Marks)

QUESTION TWO

A brake master cylinder has an outer diameter of 45mm, and the inner diameter is 35mm, determine the circumferential ‘hoop’ stress at the outer surface when the fluid pressure reaches 115 Bar.

- State the boundary conditions to be applied

(3 Marks)

- Calculate the constants, a and b, in Lame’s Equations for this application

(5 Marks)

- Calculate the corresponding circumferential stresses at the inner and outer radii

(2 marks)

QUESTION THREE

A material the following strain values at a certain point

- the ‘x’ direction of -10 x 10-6

- the ‘y’ direction of +13 x 10-6.

The material also has the following properties

- Poisson’s ratio of 0.20

- Modulus of Elasticity of 165 GPa.

- Calculate the corresponding stresses in each direction at this point in the material and state whether they are compressive or tensile.

(5 marks)

- If at a second point in the material the stresses are found to be 20MPa (tensile) in the ‘x’direction and -15 MPa (compressive) in the ‘y’ direction, calculate the corresponding strains.

(5 marks)

QUESTION FOUR

A vehicle fire extinguisher may be modelled as a thin-walled pressure vessel. If it has a diameter of 200mm, is 390mm in length and is made from 1.5mm thick sheet steel, calculate the maximum pressure allowable if the stress in the material must not exceed 100MPa? Take the longitudinal and circumferential joint efficiencies are 90% and 60% respectively.

(10 marks)

QUESTION FIVE

A cube, made from an Isotropic material, has the following parameters –

- Side length of 350mm

- External fluid pressure of 50 MPa.

- Poissons Ratio is 0.3

- Modulus of Elasticity of 165.0 GPa

- Calculate the bulk modulus, Volumetric Strain, and the Change in Volume

(8 marks)

- Explain what is meant by the term ‘orthotropic material’ and give an example material which exhibits orthotropic characteristics.

(2 marks)

Diwedd/End

Mechanical Principles Formula Sheet 1

Two-dimensional stress and strain equations:

Three-dimensional strain equations:

Volumetric Strain:

Relationships between elastic constants:

Stress and strains in thin walled pressure vessels:

Cylinders					Spheres

Stresses in thick-walled cylinders: (Lame’s Equations)

Where a and b are found by applying boundary conditions

Loaded beams : Standard Cases

Cantilever Beams:

Simply Supported Beams: