University of Wales Trinity Saint David

Wales Institute of Art and Science

Sustainability Challenges in the Motorsport Industry with A Specific Focus on Competition Vehicle Propulsion Legislation

Automotive Materials and Sustainability – ACHA6012

Jake Lawrence - 1450966

03/2024

- Contents

Tables and Figures	i

1	Introduction	1

2	Literature Review	2

2.1	Environmental Impact of Motorsport	2

2.2	Environmental Impact of Competition Vehicle Propulsion	4

3	Sustainable Practices and Legislation	5

4	Future Outlook	7

4.1	Alternative Fuels	7

4.2	Electrification	8

4.3	Additional Methods	8

5	Conclusion	9

6	References	10

## Tables and Figures

Table 1 - Scope 1, 2, and 3 Emissions (UN Global Compact Network UK, 2024)	2

Table 2 - Alternative Fuels Advantages and Disadvantages	7

# Introduction

Motorsport has historically been a driving force in technological innovation, driving improvements in a range of technological sectors including composite material manufacture, hybrid drivetrains and energy storage systems in addition to its contributions to the automotive industry such as improvements to vehicle performance, aerodynamics, safety, and efficiency. Drawing in large global manufacturers, motorsport provides a fast-paced dynamic environment that uses competition to drive innovation and encourages manufacturers to invest capitol and world-leading personnel into the development of new technologies, the direction of which can be controlled through championship regulations (Skeete, 2019). As such motorsport is an important contribution to the development of a sustainable future. However, as described by Standaert and Jarvenpaa (2016), younger generations of people are more environmentally conscious and may consider the inherent carbon emissions of current fossil-hydrocarbon fuelled Internal Combustion Engine (ICE) based motorsports to be at odds with the idea of a sustainable future. Therefore, there is increased scrutiny on the environmental effect of vehicle propulsion and as such it is important for the motorsport industry to be self-critical and reduce, offset, or eliminate carbon emissions in order to prove its commitment to a sustainable future and justify its place as a proving ground for future technology. This has led to an increasing use of hybrid systems and biofuels in order to meet the energy requirements of competition vehicles while reducing the quantity fossil-hydrocarbon fuels consumed (Turner and Pearson, 2008).

The motorsport industry as a whole covers a wider range of disciplines and events, each posing different sustainability challenges from multiple factors, for example land usage and destruction of natural habitats in addition to the traditionally considered factors of competition vehicle emissions, resource depletion, waste generation and pollution. The range of event types is extensive, from large international track events with large spectator crowds for example, the Formula 1 (F1) Championship and the World Endurance Championship (WEC) to the World Rally Championship (WRC) and Extreme E that take place in remote locations around the globe. WRC and Extreme E require transport of equipment and the construction of temporary work areas in natural environments and as such pose unique sustainability challenges. A large section of motorsport consists of smaller scale events including, track and rally-based club racing, hill climbs, sprints, drag racing and drift racing events which, while attracting smaller crowds and teams than the international events, have a significantly larger number of vehicles competing each year in total.

This report aims to cover the environmental effect of current methods of vehicle propulsion, sustainable practices, and sustainability legislation in Motorsport, in addition to identifying issues and challenges related to achieving a sustainable future and propose strategies for future improvements to sustainability. A brief general view of the sustainability challenges and environmental impacts of the motorsport industry will also be included to contextualise sustainability challenges facing the industry and the impact of vehicle propulsion on the sustainability of motorsport as a whole.

# Literature Review

## Environmental Impact of Motorsport

The environmental impact of Motorsport is wide reaching in terms of its effect on the environment and the range of industrial sectors that it involves. For a team to successfully compete at a global level, there must be collaboration between industries such as manufacturers, freight companies, fuel suppliers, event organisers, tyre suppliers and energy providers. All these areas of the Motorsport support chain have an impact on its carbon footprint and the environment at large which must be considered when analysing the sustainability of the sport. For example, when evaluating the carbon emissions involved in a large-scale event such as an F1 race, the majority of the emissions come from off-track elements such as the transport needs of large groups of event attendees and competitors, the transportation of team equipment and vehicles between events, and the energy needs of teams and spectators. The challenge of creating sustainable events involves collaboration between all stakeholders. Stakeholders in this case referring to all parties involved in or effected by an event such as local governments, residents, event organisers, teams and governing bodies (Bjerke and Naess, 2021). As further explored by Mourao (2018), the main risk to the environment associated with large events like an F1 race is carbon emissions, specifically carbon dioxide (CO₂). Reduction in carbon emissions is a key factor in improving sustainability, this is due to the fact that carbon dioxide acts as a Greenhouse Gas (GHG) and is a significant contributing factor to global warming (Choudhry et al. 2023). As explored by Waldhoff et al. (2014), GHG emissions can damage the health of people through factors such as an increase in the severity of respiratory disorders, in addition to the damage the rise in sea levels in combination with the increase in temperature will cause to the environment (Khojasteh et al., 2021) and the global economy (Abbass et al., 2022). When analysing the carbon footprint of a series, team or other organisation, emissions are split into three categories; Scope 1, 2 and 3 as described in the Table 1 (below).

Table 1 - Scope 1, 2, and 3 Emissions (UN Global Compact Network UK, 2024)

| Scope | Emissions Included |
| --- | --- |
| 1 | Direct greenhouse gas emissions created from sources the organisation owns or operates such as vehicle emissions, manufacturing emissions and emissions from energy production |
| 2 | Indirect greenhouse gas emissions created from sources of purchased energy such as electricity. Therefore, this includes emissions from powerplants or other energy providers. |
| 3 | All other indirect emissions created by an organisations activity which are not included in scope 1 and 2. This usually includes upstream and downstream activities. Emissions that are included in this category are related to employee travel, transport and disposal of waste, and recycling. |

In addition to carbon emissions, competition vehicles produce a range of environmentally harmful substances that contribute to pollution, soil contamination and water contamination. These can include microplastics from tyres, oil residues, and fuel residues that can be washed into streams and rivers leading to increased water contamination. Microplastic build up in waterways poses a significant threat to the health of wildlife and humans (Ziajahromi et al. 2020). This build up in microplastics in aquatic life could potentially pose a risk to the food security of countries that rely on aquatic life for sustenance (De-la-Torre, 2020). This example shows the interconnected nature of sustainability challenges, the importance of evaluating sustainability outside of its immediate local effect and the wide-ranging long-term consequences of unsustainable practices. Noise pollution must also be a consideration when analysing the environmental impact of motorsport as this can negatively affect both humans (Jhanwar, 2016) and wildlife (Senzaki, Kadoya and Francis, 2020).

In terms of resource depletion, motorsport consumes a range of natural resources including fossil-hydrocarbon fuels, oil-based lubricants, metals, plastics, and lithium. Hydrocarbon fuels are consumed by not only the competition vehicles themselves but are also often required to meet equipment transport, power generation and spectator transport needs. Metals and plastics are finite resources which are used in the construction of vehicles and equipment. Managing the use and recycling of metals and other raw materials is a key factor in creating a sustainable future where raw material demand is expected to grow (Dewulf et al. 2021).

Lithium is often required in the manufacture of lightweight energy storage systems used in hybrid drivetrains that have become increasingly common in recent years in an effort to reduce carbon emissions from fuel consumption. However, the mining of lithium causes damage to the local environment through water depletion, soil contamination and habitat destruction (Petavratzi et al., 2022). Additionally, the limited number of countries that produce lithium may cause security concerns for countries that cannot produce their own supply as lithium becomes an increasingly vital resource (Graham, Rupp and Brungard, 2021).

In regard to waste generation, motorsport generates a significant amount of waste including waste vehicle components, tyres, oil and lubricant waste, equipment packaging waste and food waste. All of which must be disposed of sustainably. The waste generation issues in motorsport are exacerbated by the drive for performance and the desire for competitiveness leading to frivolous use of materials and components if left unregulated. The quantity of components, for example engine components and gearboxes, and tyres used per event or per championship season is regulated in large series with the enforcement of grid penalties. This is in order to limit waste and cost as shown in section 28.2 (engine components), section 29.2  (gearboxes), section 30.2 (tyres) of the F1 2024 sporting regulations (Fédération Internationale de l’Automobile, 2024). However, these components and tyres need to be recycled in order to ensure sustainability, which can pose challenges (Lee, Hu and Lim, 2021). Composite materials, such as carbon fibre, commonly used in structural components also need to be recycled but require energy intensive processes to do so (Borjan, Knez and Knez, 2021).

## Environmental Impact of Competition Vehicle Propulsion

When evaluating the environmental impact of vehicle propulsion, it is important to consider the series in question. In a large-scale event like an F1 race, the majority of emissions come from off track activities. Whereas in club-level racing the majority of the event emissions come directly from competition vehicle propulsion as there is significantly less energy requirements and transport needs due to minimal attendance by spectators. As such in club level racing vehicle propulsion is the main contributing factor to emissions (Runicles, 2023). Therefore, when looking at motorsport as a whole, emissions from vehicle propulsion represents a larger section of the sports carbon footprint than would be observed when analysing the carbon footprint of large events such as F1 and WEC. In addition to the previously described effects of carbon emissions, the front-facing nature of competition vehicle emissions makes the reduction in the use of unsustainable fossil-hydrocarbon fuels a key issue for motorsport. Additionally, competition vehicles are the biggest contributor to noise pollution in motorsport which can negatively effect the well-being of local inhabitants and other Noise Sensitive Receivers such as schools, libraries and hospitals (Pascale et al., 2022).

The combustion of fossil hydrocarbon fuels can produce other harmful emissions in addition to carbon dioxide emissions due to incomplete combustion. These emissions can include Nitrogen Monoxide & Dioxide (NOx), Carbon Monoxide (CO), Unburnt Hydrocarbons, Sulphur Oxides (SOx) and Particulate Matter (PM) as described by Ashok et al. (2022). As temperatures in the cylinder rise the production of NOx increases (Flynn et al., 2000). The release of NOx into the atmosphere can cause the production of ground-level-ozone which can be damaging to human health by exacerbating the symptoms of pre-existing heart and lung conditions in addition to damage caused to crops and ecosystems through oxidative damage (Department for Environment, Food & Rural Affairs, 2023). NOx and SOx in the atmosphere can dissolve into water causing acid rain that is harmful for plant life and can be damaging to structures by damaging corrosion resistant coatings, such as paint, in addition to causing corrosion itself (Semakula, 2018).

When analysing the wastage produced from vehicle propulsion it is important to recognise waste produced during the whole cycle for example drilling waste, refinery sludge and packaging materials used during the production process. Spillage and leaks during handling, transportation and refilling must also be considered in addition to disposal of fuel containers and residues.

# Sustainable Practices and Legislation

Legislation related to motorsport is governed at various levels. At a global level motorsport is regulated by the United Nations (UN), which covers all sporting events, and the FIA, which covers official world championships for example F1, WEC and WRC. The championship stakeholders themselves also create their own sustainability strategies for example F1 aims to reach net zero by 2030 across scope 1, 2 and 3 carbon emissions. The current carbon footprint is estimated to be 256,551 tonnes of CO2 per year as stated in the F1 sustainability strategy (Formula One, 2019). At a national level the sport is governed by the relevant national body, in the UK this organisation is Motorsport UK (MSUK). MSUK regulates motorsports competitions ranging from club-level events to national professional championships. In addition to these regulators, the International Organisation for Standardisation (ISO) provides international recognised standards that allow teams, manufactures and organisations to prove the sustainability of their operations and ensure that partners meet the same standards. A key example of this is ISO 14001 (ISO, 2015), which provides a framework for organisations to implement Environmental Management Systems (EMS).

Broadly speaking, sporting events fall under the UN’s 17 Sustainable Development Goals (SDG). The 17 SDG set targets for global sustainability with a target date of 2030, described in detail by the United Nations Development Programme (2024). These goals cover all aspects of society however this report focuses on environmental sustainability. The UN encourages stakeholders and sporting organisations to take responsibility for their carbon footprint through providing guidance and its Sports for Climate Action (SCA) framework. Signatories to this framework have a commitment to reaching a target of halving emissions by 2030 and achieving net-zero by 2040. Net-zero refers to offsetting carbon emissions produced through methods such as planting woodlands or carbon capture.

The FIA can influence the sustainability of the series that it governs through championship regulations, encouraging teams to develop specified areas of technology with the incentive of a performance advantage. For example, in 2014 the FIA specified that a hybrid powertrain could be fitted to F1 vehicles in order to reduce the fuel consumed to meet energy needs as well as encourage development of energy storage systems and electrical propulsion systems. The change to F1 regulations in 2014 encouraged teams and industry partners to invest in reaching the goal of 50% brake thermal efficiency and as such provided advancements in knowledge and technology that improved the sustainability of the motorsport and automotive sectors (Elmagdoub and Samuel, 2021). The 2014 F1 regulations are described in detail by the FIA (2014). Similar regulations were introduced in WEC and WRC, notably with the LMP1 class in 2014, the Le Mans Hypercar (Hypercar) class in 2021, the Le Mans Daytona Hybrid (LMDh) class in 2022 and WRCs Rally1 class in 2022. These changes are influential as between WEC and WRC regulation changes have created an investment of research and personnel into hybrid propulsion systems from global manufacturers including Ferrari, Lamborghini, Toyota, Peugeot, BMW, Porche, Ford, and Hyundai. The FIA has established a three star rating system that aims to evaluate sustainability performance encompassing all activities linked to an event (Balestrieri et al., 2023). This system is designed to help develop a best practice framework to encourage stakeholders to prove their sustainable credentials through carbon offsetting and emissions reduction. A requirement for obtaining this is compliance with ISO 14001. Which includes monitoring GHG emissions, implementing pollution control and adopting sustainable waste management practices as described on the Aston Martin Formula One (AMF1) website (AMF1, 2023).  There has also been an increasing focus on the use of biofuels to reduce the reliance on fossil-hydrocarbons. For example, F1 introduced the use of E10 fuel for the 2023 season as well as a move to E55 fuel for use in Formula 2 and Formula 3 containing 10% and 55% ethanol by volume respectively.

MSUK requires venues to prove their sustainability in order to host an official event, in addition to this MSUK offer consultation and advice to venues and other partners to help improve sustainability and reduce carbon emissions. MSUK follows the United Nations Framework Convention on Climate Change (UNFCCC) and collaborates with the FIA to set sustainability goals as described in the MSUK Sustainability Strategy (MSUK, 2021). Current goals include halving carbon emissions by 2030 and achieving net zero by 2040 through avoiding unnecessary emissions, reducing unavoidable emissions and offsetting the remaining emissions as further described on the MSUK website (MSUK, 2024).

Increasing electrification as a method of reducing environmental impact in the motorsport industry can be seen in series like Formula E and Extreme E where electrification is at the forefront of the championships identity. Electrified racing series and the addition of hybrid systems to other series, such as WRC, has made valuable contribution towards the development of Electric Vehicles (EVs) by providing technical knowledge and demonstrating the capabilities of EVs to a large audience (Weggemans, 2021). Additionally, Extreme E events exclusively take place in locations under threat from environmental damage in an effort to raise awareness environmental damage and sustainability issues. The impact of this series is described in the Extreme E Sustainability Report (Extreme E, 2023).

# Future Outlook

## Alternative Fuels

There is an increasing focus on the use of sustainable produced biofuel for use as an alternative to fossil-hydrocarbons. Biofuel can increase energy independence as fuels can be produced domestically (Bielski et al., 2021) in addition to reducing emissions produced from transporting fuel internationally for use in events. Table 2 (below) summarises alternate fuel options.

Table 2 - Alternative Fuels Advantages and Disadvantages

| Fuel | Advantages | Disadvantages |
| --- | --- | --- |
| Bioethanol | - Can be produced sustainably and renewably (Bibi et al., 2017). - High octane, allowing for a higher compression ratio and therefore increased ICE efficiency (Leone et al., 2015). - Oxygen content encourages complete combustion therefore reducing emissions (Alhassan, Mshelia and Yusuf, 2021). | - May require replacement of rubber components and seals however many series already use an ethanol blend and as such have already made these modifications. - May cause increased engine wear from corrosion without development of ethanol-resistant engine oil (Khuong et al., 2017). - May pose a risk to global and national food security as farmers switch from growing food to crops for bioethanol production (Kurowska et al., 2020). |
| Biomethanol | - Can be sustainably and renewable produced (Gautam et al., 2020). - High oxygen content. - High octane allowing for a higher compression ratio. | - Low calorific value. - More volatile than ethanol requiring additional safety for transportation. |
| Hydrogen | - Production is sustainable and renewable. - Reduction in CO2 and CO emissions produced from combustion (Tian et al., 2022). | - Challenging to store with no clear consensus on method (Usman, 2022). Storage usually requires a pressure vessel that can pose a danger of rupturing during a crash. - Increase in NOx emissions (Shadidi, Najafi and Yusaf, 2021). |

The main focus of current plans is the use of bioethanol, for example F1 plans to switch to full bioethanol use for its 2026 regulations as described on the Formula 1 website (Barretto, 2022), additionally these regulations include a heavier reliance on energy storage systems and hybrid systems (50% vehicle power). Bioethanol can utilise existing fuel transport infrastructure with little modification when compared to hydrogen and biomethanol. WEC plans to introduce a hydrogen class to encourage investment and technological development however the viability of long-term implementation of hydrogen as a transport in other sectors is still unknown due to NOx production and storage issues. The introduction of this class has been pushed back to 2027 due to concerns over the feasibility for manufactures to create competitive vehicles before this date (Watkins, 2023). As such the use of hydrogen is not feasible in the immediate future without further development.

## Electrification

Electricity generation is a bigger contributor to global pollution than automobiles (Santos et al., 2021) and therefore the switch to fully Electric Vehicles (EVs) will require development of clean electricity production methods in order to be effective in reducing emissions (Sheng et al., 2021). This poses a question over the practicalities of reducing emissions through electrification as in 2018, 85% of electricity is still produced using coal, oil and gas (Kober et al., 2020) and energy demand is expected to rise due to the adoption and planned reliance on EVs in the automotive sector (Muratori et al., 2021). As described by Liu et al. (2023) energy demands are expected to grow by 80% due to population growth and as a result energy-related CO2 emissions are expected to grow by 70% causing irreversible damage to the climate system. The reliance on Lithium-ion batteries for energy storage will damage the environment through the previously mentioned effects of lithium mining. As such a full switch to electric in motorsport would put additional stress on global energy production as well as directly causing environmental damage. The switch to electric powertrains would also cause backlash from fans as engine noise is considered to be a part of the sport. Additionally some tracks are banning hybrid and electric vehicle due to safety concerns associated with battery fires (Smith, 2023).

## Additional Methods

Other methods of reducing emissions could also be introduced to championship regulations such as water injection into the cylinder in order to reduce cylinder temperatures and as such reduce NOx production (Fratita et al., 2021). This method would also provide the teams with a performance incentive to invest technological research into this area as the method of water injection can also be used to increase power produced by an engine. The addition of water injection on the 2015 BMW M4 GTS enabled a power increase of 50hp (Fratita, Popescu, Martins, Brito, Costa, et al., 2021). The use of water injection can reduce the need for over fuelling to control cylinder temperatures and therefore can reduce emissions of unburnt hydrocarbons (Cordier et al., 2019).

It should be noted that F1 currently has a ban on Exhaust Gas Recirculation Systems (EGR). An EGR system would provide little benefit during most conditions as competition vehicles run as close to full load as possible during a race. However, EGR can reduce NOx and PM production within part-load operation (Ice, 2010), for example a traction-limited acceleration zone.

# Conclusion

The environmental damage caused by motorsport cannot be ignored, however the industry is taking steps to reduce its environmental impact through sustainability strategies at all levels of legislation. It is important for motorsport to justify its place as a development ground for technology as this would provide knowledge an understanding that can be applied to the sustainability of other sectors. The electrification of motorsport poses further challenges and as such the best method for reducing carbon emissions is the use of alternate fuels for vehicle propulsion. Specifically, bioethanol due to the discussed benefits as well as existing knowledge on its production and use in ICE vehicles. However, this could create issues for other aspects of society where an increasing demand for bioethanol cash-crops will encourage more arable land use, therefore limiting land available for food production which may be difficult to control through legislation without the use of incentives for food producers. Additional methods such as water injection and EGR can be added to championship regulations to encourage teams to develop this technology as a method of emission reduction.

# References

Abbass, K. et al. (2022) ‘A review of the global climate change impacts, adaptation, and sustainable mitigation measures’, Environmental Science and Pollution Research, 29(28), pp. 42539–42559. Available at: https://doi.org/10.1007/s11356-022-19718-6.

Alhassan, A.G., Mshelia, R.B. and Yusuf, R. (2021) ‘Review of the use of bioethanol as alternative fuel for internal combustion engines’, Acta Technica Corviniensis - Bulletin of Engineering, 14(2), pp. 139–144. Available at: https://www.researchgate.net/publication/353669542.

AMF1 (2023) AMF1 achieves ISO 14001 Environmental Management Standard. Available at: https://www.astonmartinf1.com/en-GB/news/feature/amf1-achieves-iso14001-environmental-management-standard.

Ashok, B. et al. (2022) ‘NOx Emission Control Technologies in Stationary and Automotive Internal Combustion Engines’, in B. Ashok (ed.) NOx Emission Control Technologies in Stationary and Automotive Internal Combustion Engines. Amsterdam: Matthew Deans, pp. 1–38. Available at: https://doi.org/10.1016/c2020-0-01213-2.

Balestrieri, S. et al. (2023) ‘ACI for Safety and Sustainability in Motorsport’, Transportation Research Procedia, 69, pp. 871–877. Available at: https://doi.org/10.1016/j.trpro.2023.02.247.

Barretto, L. (2022) Formula 1 on course to deliver 100% sustainable fuels for 2026. Available at: https://www.formula1.com/en/latest/article.formula-1-on-course-to-deliver-100-sustainable-fuels-for-2026.1szcnS0ehW3I0HJeelwPam.html.

Bibi, R. et al. (2017) ‘Algal bioethanol production technology: A trend towards sustainable development’, Renewable and Sustainable Energy Reviews, 71, pp. 976–985. Available at: https://doi.org/10.1016/j.rser.2016.12.126.

Bielski, S. et al. (2021) ‘Importance of agriculture in creating energy security—A case study of Poland’, Energies, 14(9). Available at: https://doi.org/10.3390/en14092465.

Bjerke, R. and Naess, H.E. (2021) ‘Toward a co-Creation framework for developing a green sports event brand: the case of the 2018 Zürich E Prix’, Journal of Sport and Tourism, 25(2), pp. 129–154. Available at: https://doi.org/10.1080/14775085.2021.1895872.

Borjan, D., Knez, Ž. and Knez, M. (2021) ‘Recycling of carbon fiber-reinforced composites— difficulties and future perspectives’, Materials, 14(15). Available at: https://doi.org/10.3390/ma14154191.

Choudhry, T. et al. (2023) ‘Carbon emissions, environmental distortions, and impact on growth’, Energy Economics, 126. Available at: https://doi.org/10.1016/j.eneco.2023.107040.

Cordier, M. et al. (2019) ‘Water injection to improve direct injection spark ignition engine efficiency’, SAE Technical Papers, 2019-April(April). Available at: https://doi.org/10.4271/2019-01-1139.

De-la-Torre, G.E. (2020) ‘Microplastics: an emerging threat to food security and human health’, Journal of Food Science and Technology, 57(5), pp. 1601–1608. Available at: https://doi.org/10.1007/s13197-019-04138-1.

Department for Environment Food & Rural Affairs (2023) ‘Emissions of air pollutants in the UK – Nitrogen oxides (NOx)’. Available at: https://www.gov.uk/government/statistics/emissions-of-air-pollutants/emissions-of-air-pollutants-in-the-uk-nitrogen-oxides-nox.

Dewulf, J. et al. (2021) ‘Towards sustainable resource management: identification and quantification of human actions that compromise the accessibility of metal resources’, Resources, Conservation and Recycling, 167. Available at: https://doi.org/10.1016/j.resconrec.2021.105403.

Elmagdoub, A. and Samuel, S. (2021) ‘A Case for Technology - Forcing Transformative Changes in the F1 Power Unit’, SAE Technical Papers [Preprint], (2021). Available at: https://doi.org/10.4271/2021-01-0371.

Extreme E (2023) Sustainability Report - Season Two. Available at: https://www.extreme-e.com/sustainability/report.pdf.

Federation Internationale de l’Automobile (2014) 2014 FORMULA ONE TECHNICAL REGULATIONS. Available at: https://argent.fia.com/web/fia-public.nsf/A0425C3A0A7D69C0C12578D3002EBECA/$FILE/2014_F1_TECHNICAL_REGULATIONS_-_Published_on_20.07.pdf.

Fédération Internationale de l’Automobile (2024) 2024 FORMULA ONE SPORTING REGULATIONS. Available at: https://www.fia.com/sites/default/files/fia_2024_formula_1_sporting_regulations_-_issue_1_-_2023-09-26.pdf.

Flynn, P.F. et al. (2000) ‘The inevitability of engine-out NOx emissions from spark-ignited and diesel engines’, Proceedings of the Combustion Institute, 28(1), pp. 1211–1218. Available at: https://doi.org/10.1016/s0082-0784(00)80332-x.

Formula One (2019) Formula One Sustainability Strategy. Available at: https://corp.formula1.com/wp-content/uploads/2019/11/Environmental-sustainability-Corp-website-vFINAL.pdf.

Fratita, M., Popescu, F., Martins, J., Brito, F.P. and Costa, T. (2021) ‘Water injection as a way for pollution control’, Energy Reports, 7, pp. 543–549. Available at: https://doi.org/10.1016/j.egyr.2021.07.099.

Fratita, M., Popescu, F., Martins, J., Brito, F.P., Costa, T., et al. (2021) ‘Water injection in spark ignition engines—Impact on engine cycle’, Energy Reports, 7, pp. 374–379. Available at: https://doi.org/10.1016/j.egyr.2021.07.113.

Gautam, P. et al. (2020) ‘Bio-methanol as a renewable fuel from waste biomass: Current trends and future perspective’, Fuel, 273. Available at: https://doi.org/10.1016/j.fuel.2020.117783.

Graham, J.D., Rupp, J.A. and Brungard, E. (2021) ‘Lithium in the green energy transition: The quest for both sustainability and security’, Sustainability (Switzerland), 13(20). Available at: https://doi.org/10.3390/su132011274.

Ice, J.D. (2010) Investigation of EGR and Fuel Property Effects on Advanced Combustion Strategies Using In-cylinder Combustion Analyses. West Virginia University. Available at: https://researchrepository.wvu.edu/etd/4607/.

ISO (2015) ISO 14001:2015 Environmental Management Systems. Available at: https://www.iso.org/obp/ui/en/#iso:std:iso:14001:ed-3:v1:en.

Jhanwar, D. (2016) ‘Noise Pollution: A Review’, Journal of Environment Pollution and Human Health, Vol. 4, 2016, Pages 72-77, 4(3), pp. 72–77. Available at: http://pubs.sciepub.com/jephh/4/3/3/index.html%0Ahttp://pubs.sciepub.com/jephh/4/3/3/abstract.html.

Khojasteh, D. et al. (2021) ‘Sea level rise impacts on estuarine dynamics: A review’, Science of the Total Environment, 780. Available at: https://doi.org/10.1016/j.scitotenv.2021.146470.

Khuong, L.S. et al. (2017) ‘Effect of gasoline-bioethanol blends on the properties and lubrication characteristics of commercial engine oil’, RSC Advances, 7(25), pp. 15005–15019. Available at: https://doi.org/10.1039/c7ra00357a.

Kober, T. et al. (2020) ‘Global energy perspectives to 2060 – WEC’s World Energy Scenarios 2019’, Energy Strategy Reviews, 31. Available at: https://doi.org/10.1016/j.esr.2020.100523.

Kurowska, K. et al. (2020) ‘Food security in the context of liquid biofuels production’, Energies, 13(23). Available at: https://doi.org/10.3390/en13236247.

Lee, S.Y., Hu, J. and Lim, M.K. (2021) Maximising the circular economy and sustainability outcomes: An end-of-life tyre recycling outlets selection model, International Journal of Production Economics. Available at: https://doi.org/10.1016/j.ijpe.2020.107965.

Leone, T.G. et al. (2015) ‘The Effect of Compression Ratio, Fuel Octane Rating, and Ethanol Content on Spark-Ignition Engine Efficiency’, Environmental Science and Technology, 49(18), pp. 10778–10789. Available at: https://doi.org/10.1021/acs.est.5b01420.

Liu, H. et al. (2023) ‘A Perspective on the Overarching Role of Hydrogen, Ammonia, and Methanol Carbon-Neutral Fuels towards Net Zero Emission in the Next Three Decades’, Energies, 16(1). Available at: https://doi.org/10.3390/en16010280.

Mourao, P.R. (2018) ‘Smoking gentlemen-How formula one has controlled CO2 emissions’, Sustainability (Switzerland), 10(6). Available at: https://doi.org/10.3390/su10061841.

MSUK (2021) ‘Motorsport UK Sustainability Strategy’. Available at: https://www.motorsportuk.org/wp-content/uploads/2021/11/Motorsport-UK-Sustainability-Strategy-2021-2030.pdf.

MSUK (2024) Motorsport UK - Offsetting. Available at: https://www.motorsportuk.org/about-us/environmental-sustainability/environmental-resources/offsetting/ (Accessed: 8 March 2024).

Muratori, M. et al. (2021) ‘The rise of electric vehicles-2020 status and future expectations’, Progress in Energy, 3(2). Available at: https://doi.org/10.1088/2516-1083/abe0ad.

Pascale, A. et al. (2022) ‘Acoustic Indicators for Motor Race Noise: Definition, Validation, and Tests in Simulated and Real Races’, Applied Acoustics, 200. Available at: https://doi.org/10.1016/j.apacoust.2022.109071.

Petavratzi, E. et al. (2022) ‘The impacts of environmental, social and governance (ESG) issues in achieving sustainable lithium supply in the Lithium Triangle’, Mineral Economics, 35(3–4), pp. 673–699. Available at: https://doi.org/10.1007/s13563-022-00332-4.

Runicles, J. (2023) Girls on Track UK Webinar: Sustainability in Motorsport. Motorsport UK. Available at: https://www.youtube.com/watch?v=HI5htGsE-dA.

Santos, N.D.S.A. et al. (2021) ‘Internal combustion engines and biofuels: Examining why this robust combination should not be ignored for future sustainable transportation’, Renewable and Sustainable Energy Reviews, 148, p. 111292.

Semakula, S. (2018) ‘the Effects of Exhaust Gas Recirculation ( Egr ) on the Performance of Diesel Engine’. Available at: https://core.ac.uk/download/pdf/268999168.pdf#page=150.

Senzaki, M., Kadoya, T. and Francis, C.D. (2020) ‘Direct and indirect effects of noise pollution alter biological communities in and near noise-exposed environments’, Proceedings of the Royal Society B: Biological Sciences, 287(1923). Available at: https://doi.org/10.1098/rspb.2020.0176.

Shadidi, B., Najafi, G. and Yusaf, T. (2021) ‘A review of hydrogen as a fuel in internal combustion engines’, Energies, 14(19). Available at: https://doi.org/10.3390/en14196209.

Sheng, M.S. et al. (2021) ‘Well-to-wheel analysis of greenhouse gas emissions and energy consumption for electric vehicles: A comparative study in Oceania’, Energy Policy, 158. Available at: https://doi.org/10.1016/j.enpol.2021.112552.

Skeete, J.P. (2019) ‘The obscure link between motorsport and energy efficient, low-carbon innovation: Evidence from the UK and European Union’, Journal of Cleaner Production, 214, pp. 674–684. Available at: https://doi.org/10.1016/j.jclepro.2019.01.048.

Smith, S.C. (2023) Why racetracks are giving hybrids and EVs the cold shoulder, Hagerty Media. Available at: https://www.hagerty.com/media/motorsports/why-racetracks-are-giving-hybrids-and-evs-the-cold-shoulder/ (Accessed: 8 March 2024).

Standaert, W. and Jarvenpaa, S.L. (2016) ‘Formula E: Next generation motorsport with next generation Fans’, 2016 International Conference on Information Systems, ICIS 2016 [Preprint].

Tian, Z. et al. (2022) ‘Numerical comparative analysis on performance and emission characteristics of methanol/hydrogen, ethanol/hydrogen and butanol/hydrogen blends fuels under lean burn conditions in SI engine’, Fuel, 313. Available at: https://doi.org/10.1016/j.fuel.2021.123012.

Turner, J.W.G. and Pearson, R.J. (2008) ‘The application of energy-based fuel formulae to increase the efficiency relevance and reduce the CO2 emissions of motor sport’, SAE Technical Papers [Preprint]. Available at: https://doi.org/10.4271/2008-01-2953.

UN Global Compact Network UK (2024) Scope 3 Emissions. Available at: https://www.unglobalcompact.org.uk/scope-3-emissions/ (Accessed: 7 March 2024).

United Nations Development Programme (2024) UNDP Sustainable Development Goals, United Nations Development Programme. Available at: https://www.undp.org/sustainable-development-goals.

Usman, M.R. (2022) ‘Hydrogen storage methods: Review and current status’, Renewable and Sustainable Energy Reviews, 167. Available at: https://doi.org/10.1016/j.rser.2022.112743.

Waldhoff, S. et al. (2014) ‘The marginal damage costs of different greenhouse gases: An application of FUND’, Economics, 8. Available at: https://doi.org/10.5018/economics-ejournal.ja.2014-31.

Watkins, G. (2023) Hydrogen class introduction at Le Mans set for delay, 2027 ‘more realistic’, Autosport. Available at: https://www.autosport.com/le-mans/news/hydrogen-class-introduction-at-le-mans-set-for-delay-2027-more-realistic/10544641/.

Weggemans, B. (2021) ‘The Role of Electric Motorsport in the Sustainable Mobility Transition’.

Ziajahromi, S. et al. (2020) ‘Microplastic pollution in a stormwater floating treatment wetland: Detection of tyre particles in sediment’, Science of the Total Environment, 713. Available at: https://doi.org/10.1016/j.scitotenv.2019.136356.