Heavy gauge process enhancement

by

Grace Williams

Supervisor: Stalin Jacob

Project submitted as part of the requirements for the award of

BEng (Hons) Electrical and Electronics Engineering

April 2025

DECLARATION

I, Grace Williams declare that I am the sole author of this Project; that all references cited have been consulted; that I have conducted all work of which this is a record, and that the finished work lies within the prescribed word limits.

This work has not previously been accepted as part of any other degree submission.

FORM OF CONSENT

I Grace Williams hereby consent that my Project, submitted in candidature for the B.Eng. (Hons) Electrical and Electronics Engineering degree, if successful, may be made available for inter-library loan or photocopying (subject to the law of copyright), and that the title and abstract may be made available to outside organisations.

# Abstract

This report outlines the design and implementation of process improvements to the heavy gauge de-coiling line. These improvements are aimed at reducing roller pitting and enhancing operator safety during the roller cleaning procedures.

The primary objective was to minimise moisture and debris on the steel surface, which is the contributing factor to the roller pitting and infusions. The solution must allow the existing line to remain the same, incorporate the safety zones and be commissioned only in the down periods where production is not running. To achieve this, air knives were installed to effectively remove contaminants. A safer cleaning procedure was also developed with the use of carpet rollers and software modifications to incorporate safety.

The project encountered minor challenges, including the replacement of an isolator during the blower commissioning. However, these challenges were resolved efficiently with minimal cost and time implications.

Overall, the solutions implemented improved the product quality, reduced the maintenance required to the rollers due to pitting and also enhanced the operator safety with regards the cleaning procedure, therefore aligning with the project’s objectives.

Keywords: Reduced roller pitting, Air Knives, Safety.

# Acknowledgements

Anthony Dobbs (tdobbs@iac-ltd.co.uk) (Service Director)

Andrew Duggan (Andy.Duggan@tatasteeleurope.com) (TATA Engineering Manager)

Rhys Howells-Smith (rho@iac-ltd.co.uk) (Service Technician)

Stalin Jacob Wilson Packia Dhas ( s.wpackiadhas@uwtsd.ac.uk) (University Project Supervisor)

All members mentioned above helped me to achieve this project and make it a success. Without the guidance and support of these people, the project would not have been possible.

IAC have given me the support and resources needed throughout the whole project so that it could be achieved to a high standard and keep the customer satisfied with the result. IAC have taught me the key skills required to become an automation and control engineer over the 7 years of employment so that I can complete jobs professionally and to a high standard.

Contents

Abstract	3

Acknowledgements	4

List of figures	6

1.	Introduction	8

2.	Research	10

2.1.	Pitting initiation and surface contaminants	10

2.2.	Techniques to remove moisture and debris from materials	11

2.2.1.	Vibrating Dryers	11

2.2.2.	In- line heaters and ovens	11

2.2.3.	Air knives	12

3.	Background and development methodology	13

3.1.	Feasibility site visit	13

3.2.	Specification	13

3.3.	Job Quotation	14

3.4.	Planning	14

3.5.	Safety	15

3.5.1.	IAC standards	15

3.5.2.	IAC procedures	15

3.5.3.	Job method statement	16

4.	Design	17

4.1.	Hardware	17

4.1.1.	Air Knife hardware	17

4.1.2. Cleaning mode hardware	18

4.2.	Drawings	19

4.3.	Software	19

4.3.1.	Blower control software	19

4.3.2.	Cleaning mode software modifications	21

5.	Testing	29

6.	Implementation	31

6.1.	Fitting of components for air knife	31

6.2.	Commissioning air knife	33

6.3.	Fitting of components for cleaning mode	35

6.4.	Commissioning cleaning mode components	37

7.	Evaluation	41

7.1.	Issues encountered	41

7.2.	Effectiveness of project	42

8.	Conclusion and recommendations	44

9.	Reflection	46

10. 	References and Bibliography	47

11.	Appendices	50

Appendix 1- Quote and FDS for Cleaning modes	50

Appendix 2 – Quote and FDS for air knife	63

Appendix 3 – IA Work instructions (IAC E100)	71

Appendix 4 – TATA permits to work	74

Appendix 5 – Job method statement and risk assessment	76

Appendix 6- Excell document containing  I/O	86

Appendix 7- Modified pages of drawings	91

Appendix 8 – FAT document	101

Appendix 9 – Program blocks for cleaning mode and blower control functions	123

# List of figures

Figure 1- Vibrating Dryer……………………………………………………………………………………………..…...11

Figure 2- In line oven……………………………………………………………………………………………………..….11

Figure 3- Air Knife system…………………………………………………………………………………………….……12

Figure 4- line layout and safety zones……………………………………………………….……………………….13

Figure 5- Gantt chart…………………………………………………………………………………………….……………14

Figure 6 - Updated Gantt chart…………………………………………………………………………….…………….15

Figure 7- IAC procedures ………..………………………………………………………………………………………...16

Figure 8- Motor rating plate ………………………………………………………………………………………….…..17

Figure 9- FC210 Logic ……………………………………………………………………………………………………..….20

Figure 10- DB217 logic……………………..……………………………………………………………………………..….21

Figure 11- FC228 network 7 request manual raise…………………………………………………………...…23

Figure 12- FC228 network 12 disable auto positioning………………………………………………………..23

Figure 13- FC226 network 10 exit screws raise……………………………………………………………….……25

Figure 14- FC136 network 11 jog backwards……………………………………………………………….……….26

Figure 15- FC212 network 3…………………………………………………………………………………………………27

Figure 16- FC212 network 13………………………………………………………………………………………………27

Figure 17- FC132 network 11	…………………………………………………………………………………………..28

Figure 18- Bench testing set up at IAC ………………………………………………………………………………30

Figure 19- Bench test wiring for inverter ………………………………………………………………………….30

Figure 20- Desk push button location	 ……………………………………………………………………………….31

Figure 21- mounted inverters for air knife blowers ………………………………………………………….	.32

Figure 22- Air knife blower ……………………………………………………………………………………………….32

Figure 23- Air knife positioned on the line ………………………………………………………………………..32

Figure 24- VAT table for blowers ………………………………………………………………………………………34

Figure 25- Carpet roller with pins ……………………………………………………………………………………..36

Figure 26- Limit switches at the top of leveller 1……………………………………………………………….36

Figure 27- P15 panel with cleaning mode ………………………………………………………………………...36

Figure 28- P01 panel with cleaning mode …………………………………………………………………………36

Figure 29- Remote I/O of P15 panel …………………………………………………………………………………37

Figure 30- Wiring of E19 panel in sub-station …………………………………………………………………..37

Figure 31- E20 panel in substation with zone relay …………………………………………………………..39

Figure 32- Leveller 1 screen for auto positioning ………………………………………………………………40

Figure 33- Leveller 2 screen for auto positioning  ……………………………………………………………..40

Figure 34- Straightener screen for auto positioning ………………………………………………………….40

Figure 35- Email to TATA recommending new risk assessment …………………………………………42

Figure 36- FC207 new runstp commands ………………………………………………………………………….43

Figure 37- control room desk panel with start top buttons removed ………………………………..43

# Introduction

Heavy Gauge is a de-coiling line in the steelworks industry owned by TATA steel. It uncoils heavy gauges of steel and cuts it to a specific size required by the customer. It has three stages of levelling throughout the process to ensure that the final product produced is as flat as possible, and its surface brushing stage ensures that the product is as clean as possible [1].

In order to maintain the rate of production and product quality, steel manufacturing operations require a high degree of efficiency and precision. One of the key challenges that is faced at the heavy gauge de-coiling line is the pitting effect on steel rollers, caused by damp and contaminated raw material. This issue not only contributes to the quality issues faced with the final product, but also has a large impact on increasing downtime, material wastage and additional maintenance tasks being required. Addressing this problem is crucial for improving the overall efficiency and reliability of the process line and also reducing the costs associated with the wasted material and loss of production due to maintenance.

The aim of this project is to implement solutions to mitigate the pitting effect on the steel rollers by incorporating additional process applications, and subsequently modifying the PLC programme to suit. The primary objectives of this project are;

- To enhance the process line by ensuring that any raw material is dry before entering the straightening stage at the beginning of the process.

- The rollers on the straightener and levellers can easily be cleaned to remove contaminants.

- The cleaning task must be made safe so that an operator can carry out the task.

By having these objectives, the project aims to reduce the pitting on the rollers, minimise downtime, improve the product quality and therefore subsequently increase the profitability of the process.

This report details the methods taken to achieve these improvements. A systematic approach using a waterfall style project management has been used to ensure that each phase is completed before the next one starts. This report also considers the key operational constraints faced and overcome throughout the project including the cost effectiveness, safety compliance, and the minimal disruption to the process.

The successful implementation of these solutions should result in the accomplishment of overcoming a persistent problem for this de-coiling line. The outcome is evaluated by TATA personnel who compares the level of pitting on steel before and after any modifications have taken place. This is then used to evaluate if the solutions have worked effectively to meet the desired objectives.

# Research

For a thorough understanding of the cause and effect of pitting on steel rollers and to find potential solutions to overcome this issue, existing literature and research should be considered. This section explores the previous studies that have been carried out on pitting initiation on steel and also various drying techniques that can be used to help overcome this issue.

## Pitting initiation and surface contaminants

Pitting is a form of surface degradation that causes a cavity effect on steel [2]. It is a significant concern in steel rolling applications as it degrades the roller surface, potentially leading to product defects.

The likelihood of pitting is inversely proportional to the smoothness of the material surface [3]. Surface contaminants such as water, oil or debris (including wear products), play a crucial role in initiating pitting corrosion. S, Way suggests that the pressure from these contaminants on rolling steel is likely to be the cause of the pitting formation [4],[5]. This happens due to the contaminants disturbing the passivation layer on the steel surface, creating anodic and cathodic areas that facilitates corrosion [6],[7],[8].

Fluctuations in temperature can also increase pitting. Lower temperatures in particular contributes to this due to the precipitation that the lower temperatures lead to. Dampness is a significant factor in steel corrosion as moisture is an electrolyte, accelerating the electrochemical reactions involved in pitting. The presence of chlorines in the water can further intensify the pitting corrosion.

Furthermore, the cleanliness of the material remains a factor as inclusions are less frequent on cleaner steels [9]. Inclusions act as initiation sites for corrosion as they disrupt the uniformity of the steels microstructure and create localised areas of differing electrochemical potential.

Research conducted indicates that removing these surface contaminants could significantly prolong the life of steel rollers by reducing the occurrence of pitting and inclusions. This removal of contaminants could then lead to improved product consistency and less maintenance downtime.

## Techniques to remove moisture and debris from materials

Different techniques over the years have been tried and tested for removing moisture and debris from materials. Depending on the type of application will depend on the effectiveness of the technique.

### Vibrating Dryers

Vibrating dryers use mechanical vibration to move moisture and debris from the material surface [10]. They are most commonly seen used for granular materials and can be effective in removing loose contaminants [11]. This has not been recorded as an effective method in large heavy material however as the vibrations have a smaller effect on larger materials and would require far more power to cause the vibrations under heavy loads.

Figure 1- Vibrating Dryer [12]

### In- line heaters and ovens

In-line heaters and oven systems use heat to evaporate moisture from the material surface. They are commonly seen used in the steel industry and are suitable for applications where the material can withstand elevated temperatures without causing material degradation[13]. This method however is expensive to run and would require a complete re-design of the process layout in order to fit it in.

Figure 2- In line oven [14]

### Air knives

Air knives use a high velocity jet of air to blow off moisture and debris from the material surface [15]. They are commonly used in continuous process lines and can be effective in removing surface contaminants without direct contact [16].

Figure 3- Air Knife system [17]

For the required application, as TATA have specified that they do not want the layout of the line to change, only one method is practicable out of the three. This is the air knives method. This method is the only practical solution as it does not require any of the process to be re-arranged. Air knives would simply be placed on to the existing system for it to work. Furthermore, this method has evidence to support the success with this type of application, as it has been used in other areas of the TATA plant effectively.

The other methods mentioned would not have been effective for this application as applying heat to the process, like with an in-line heater or oven, could change the steel properties. Vibrating dryers would also not be appropriate for this application as there is little evidence to support that they would work for such large and heavy products, as all research points to them being used for small granular materials [18],[19].

# Background and development methodology

## Feasibility site visit

Prior to the project being accepted, a site visit is conducted to see what possibilities there are to overcome the issue that the customer is having and if a solution is practicable. During this site visit a site survey is carried out. This site survey consists of getting software backups, taking note of all the equipment the process has in the electrical panels, and also taking a copy of any relevant drawings to the process. By carrying out a site survey, it means that a good understanding can be had on what is possible and means that the job can be quoted accurately.

Figure 4- line layout and safety zones

## Specification

In most cases, the specification is given by the customer and this is what IAC quote and work to. As the customer was unsure of the best solution to this issue, no specification was given. This meant that a functional design specification (FDS) was written by IAC to be able to quote to and for the customer to be able to accept in order to know what can be achieved. This also safeguards IAC so that the customer agrees in advance what needs to be carried out and that they are happy with it. This FDS is written in the quote and can be seen in Appendix 1 and Appendix 2.

## Job Quotation

The quote was written according to the specification.  This quote includes any hardware costs, milage to and from site, time for engineering and commissioning and also expenses. The quote for the cleaning mode came to £8,901 and the quote for the Air knives came to £2,868 totalling £11,769. (Seen in Appendix 1 and Appendix 2)

Before the job can commence, the customer needs to agree with the quote and send a purchase order for it so that the job is authorised and IAC insurance will cover it.

## Planning

The customer only wanted the solution to be implemented on the down periods where the line wasn’t running, and therefore it wouldn’t interfere with production. This meant that planning needed to be carefully organised to ensure that it would be able to be carried out with minimum disruption for the customer. An estimation for the time frame was originally drafted on a Gantt chart and discussed with the customer to ensure that it would also work for them. Figure 2 Gantt chart is the initial chart used. In order to ensure that each task in the project was fully commissioned prior to the line starting again, it was completed in two stages. The first stage being the air knife system and the second stage the cleaning mode. Figure 6 is the amended Gantt chart used after TATA had made a request for the project to be brought forward.

Figure 5 Gantt chart

Figure 6- Updated Gantt chart

## Safety

In order to carry out the project safely and correctly, safety standards and procedures must be followed. These standards and procedures are from reputable sources such as the IOS and international organisation for standards and British standards. Internal IAC procedures must also be followed in order to ensure quality assurance is carried out.

### IAC standards

- IEC 61508 – Functional Safety of Electrical/Electronic/ Programmable Electronic Safety Related Systems

- BS EN 60204-1 2006 Safety of Machinery – Electrical equipment of machines Part 1: general requirements

- BS EN 13849-1 2008 Safety of Machinery – Safety-related parts of control systems Part 1: general principles for design

- BS EN 13849-2 2012 Safety of Machinery – Safety-related parts of control systems Part 2: validation

### IAC procedures

Figure 4-IAC procedures shows a list of procedures IAC follows in order to carry out a project correctly from beginning to end.  IAC work instructions used for engineering can be seen in Appendix 3. These work instructions detail how a project should be conducted in order to meet the quality standards set and approved.

Figure 7-IAC procedures

TATA have their own set of standards and procedures that need to be followed whilst working on any TATA project. For safe entry to site, an Induction needs to be carried out. The induction goes through all the safety for the site including the emergency numbers, and explains the risks and hazards that can occur. In order for any contractor to be able to work on their own without a TATA employee supervising them at all times, a PICOWP (Person in charge of working party) course needs to be taken. This course goes through site safety and permits to work, amongst other various documentation.

### Job method statement

Before starting any work on site at heavy gauge, a job method statement must be carried out. A job method statement is a health and safety document that details the tasks going to be carried out and the risks associated with them. The document identifies the potential risks and hazards that can arise when carrying out the work, and lists the necessary precautions that have to be taken to compete the task safely. TATA steel requires a job method statement from any contracting companies prior to any work being carried out. This is so that TATA can clearly identify the scope of work and write permits accordingly.

The job method statement contains risk assessments, PPE required, tools or materials that are going to be used and contact details for personnel who will be working on the job. This can be seen in Appendix 5.

# Design

The design phase of this project focused on how the solutions found can be integrated with the existing heavy gauge process line in order to reduce the pitting and enhance the cleaning procedure. This involved not only looking at the wiring schematics and hardware but also the software side with the PLC. The aim was to seamlessly integrate the new solutions to the process line, while adhering to TATA’s requests and keeping disruption to a minimum.

## Hardware

The hardware selection was chosen based on a number of factors, with the primary focus on choosing the most suitable equipment for the application. Customer preference and the existing equipment across plant were also key considerations with the choice of hardware.

### Air Knife hardware

The customer had specifically requested the use of the Solvair brand air knives based on the fact they were already in use across other areas of plant. This familiarity that TATA had with these air knives ensured that the maintenance engineers were comfortable with the operation of them and more importantly, that spare parts was already available in the stores. The PX2000 series Air blower was the Solvair product used for the air knives. This is an ultra-high performance centrifugal blower that uses a belt driven system with an automatic tensioner [20]. These air knives operate with the use of a Siemens 1AV3163A 1LE10031DA334NB4-Z motor, rated at 15kW. The motor details for the air knife were used in order to select the correct rated drive for the control.

Figure 8- Motor rating plate

A Siemens drive was initially considered to be used, due to the widespread use throughout the heavy gauge process line. An Invertek Optidrive E3 inverter however, was chosen over a siemens one due to a number of factors; the Optidrive E3 inverter was significantly more cost effective, had a shorter delivery time and had all the required functionalities for the operation [21]. This drives straight forward design should make it easy for the TATA engineers to be able to work with and understand if it faulted in the future. Furthermore, Invertek is a Welsh company, so choosing this drive helps with promoting the local infrastructure and meeting sustainability goals.

The existing PLC fortunately had sufficient spare I/O (inputs and outputs) to accommodate the new functions of the air knife and cleaning mode. This removed the need to purchase additional I/O modules. To ensure that all the available I/O was correctly identified, a comprehensive spreadsheet was completed (see Appendix 6). This spreadsheet listed all the spare I/O based on the existing drawings, and was also cross referenced in the PLC to ensure accuracy.

The push buttons used for the “start”, “stop”, as well as the indication lamps was taken from the TATA stores in order to keep the control desk consistent with existing components. This ensures that not only will the desk look more uniformed, it also means that the N/O and N/C contacts will be able to be easily identified in the stores as they will all be the same for all the desk panel. This then means that these components will be able to be quickly changed out in the future with a replacement in the case of failure.

### 4.1.2. Cleaning mode hardware

The carpet rolls used for the cleaning mode application was custom made by TATA in order for them to be able to fit exactly in the required space. This was also done as it is a bespoke and new design that has not been seen carried out anywhere else on the market so could not easily be purchased. The existing carpet material that was used previously was used to attach to these rollers. By reusing this material, it cut cost for the customer and also guaranteed for the customer that the material already worked for the required purpose.

The limit switches placed upon the top of the straightener and the levellers, and also the pin sensors for the carpet rollers, used the inductive proximity sensor IME18-12NPSZW2S [22]. This sensor was used due to plant familiarity and also because it was an appropriate, cost-effective product to carry out the job. Additionally, multiple of these sensors was already kept in stores as spares, so this allowed for consistency with proximity sensors across plant. In addition to the limit switches, the existing encoder will also be used to restrict the upward and downward movement of the leveller and straightener, preventing the possibility of mechanical failure from overtravel. This means that there will be two methods to stop movement so it has a backup if one method was to fail.

The beacon for the identification of the cleaning mode was an RS Pro Red flashing beacon [23]. This was selected as it was magnetic so could be moved around the panel for easy visibility if required and was within the correct operating voltage.

## Drawings

The quote for this project stipulated that the full drawings would not be updated but any modifications to the drawings carried out will be done so by hand where required. The comprehensive spreadsheet with the spare I/O was used along with the inverter manual to help determine where drawings will need to be modified. Any modifications carried out on the drawings was done in red pen for easy identification and was named accordingly for simple cross referencing. (see Appendix 7 for drawing modifications).

## Software

IAC have a service contract with the heavy gauge process line. As a result, software backups for the process are securely stored on the IAC server. Fortunately, a fully commented program was found on the IAC server for the process, and was used to upload the PLC to in order to ensure it was up to date. The commented program significantly makes the job easier for the software design, as it is more straightforward to understand the operation and where the necessary modifications need to be made. This is due to the program having comments and labels for each part, so it doesn’t require constantly referring to drawings to know what each I/O is for, and it can be easily understood what each memory bit is doing. As the PLC is a Siemens S7300 range, Siemens Simatic Manager is the PLC software that is used. A special Siemens A2 adapter is used in order to get online to the PLC over the Profibus DP port.

### Blower control software

New blocks was created for the blower function for better program structure. Function call FC207 is a new function block created that uses ladder logic to manage the blower control for the air knives. FC207 is responsible for calling and parameterising the blower control function call FC210.

The “start blower” push button located on the operator’s desk is read in FC210 to initiate the Invertek drives. The start command however is only executed if the safety zone 1 is healthy, the drives “ready to run” signal is present, and the emergency stop (E-stop) circuit is not broken. The “stop blower” push button located on the operator’s desk, the E-stop circuit or the safety zone will cause the drive to stop running. While the drive is running, a green indication light on the operator’s desk should be illuminated to provide visual conformation that the drive is in operation.

The run command is dependent on Safety Zone 1, both in software and hardware. The zone will need to be healthy to have the output from the PLC. It also goes through a relay [HO1-A3121] that requires the 24V given from zone 1 in order for the relay to operate.

FC210 is used to start and stop the inverters and also control the speed. It also gives the indication lamp output for the desk panel for when the inverter is running. This can be seen in  Figure 9- FC210 Logic.

Figure 9- FC210 Logic

The three operational speeds are written in DB217. If the line is stopped and the blower has not been started then speed 1 will be selected. This speed has been chosen to be 0. If the blowers have been started but the line is not in run, then speed 2 will be selected. Speed 2 has been set at 50% which is equivalent to approximately 24.6Hz. If the blowers and the line is in run then speed 3 is selected. This has been set to 100% which is approximately 49.2Hz.

Motor speed= 2955/60 seconds = 49.25Hz

These speeds were chosen as when the line is idle, the noise coming from the air knife is wanted to be kept to a minimum via the slower speed. This is so that the policy for the operators with regards to hearing protection will not need to change. While the line is running, the control room has sound proofing for the operators so the sound is not a concern at that time.

Figure 10- DB217 logic

### Cleaning mode software modifications

The new I/O used for the cleaning mode was selected by seeing what I/O was spare in the excel document written up at the start of the project (see appendix 6). The I/O used for the cleaning mode is shown below. This also identifies what panel the I/O is wired into; P01 and P15 being the small operator panels near the machines with remote I/O and E19 in the substation.

Leveller 1 I/O

P15; I241.7 leveller 1 cleaning mode input (N/C)

E19; 3B – I57.0 leveller 1 max down limit exit (N/C)

E19; 7B – I57.1 leveller 1 max down limit entry (N/C)

E19; 5B – I57.2 leveller pin 1 drive side (N/O)

E19; 6B- I57.3 leveller pin 2 operator side (N/O)

Leveller 2 I/O

P15 – I243.7 leveller 2 cleaning mode input (N/C)

E19; 3 – I57.4 leveller 2 max down limit entry (N/C)

E19; 7 – I57.5 leveller 2 max down limit exit (N/C)

E19; 5 – I57.6 leveller pin 1 drive side (N/O)

E19; 6- I57.7 leveller pin 2 operator side (N/O)

Straightener I/O

P01- I132.0 straightener cleaning mode input (N/C)

E19; 7 – I56.4 straightener max down limit (N/C)

E19; 5 – I56.5 straightener pin 1 drive side (N/O)

E19; 6- I56.6straightener pin 2 operator side (N/O)

##### Leveller 1 and 2 software

Both leveller 1 and leveller 2 operate under the same principals and therefore the programming was almost identical. Function call FC228 for leveller 1 and FC239 for leveller 2 was modified by adding the cleaning mode input in network 7 and 8. This was done so that it would stop the function of the manual raise and lower command from the control room, meaning only the P15 panel can perform the operation as this was the local panel to the machine area.

The full program can be seen in Appendix 9. Figure 8 shows the leveller 1 raise modification in network 7.

Figure 11- FC228 network 7 request manual raise

Network 12 was modified in FC228 for leveller 1 and FC239 for leveller 2 in order to stop the auto positioning being enabled while cleaning mode is active. This was done by resetting the HMI command for auto positioning while the cleaning mode was active. This way the auto position HMI command will never be able to be set when cleaning mode is on. This is essential as otherwise it could be moved while in the cleaning mode and cause issues.

Figure 12- FC228 network 12 disable auto positioning

Function call FC226 and FC237 was modified for leveller 1 and 2 for the raise and lower operation in the cleaning mode at the P15 panel. As the levellers have 4 encoders, two for the entry side and two for the exit, modifications needed to be carried out for each part. Network 5 and 6 was modified for the entry and network 10 and 11 for the exit. The two entry and two exit encoders were already linked together in a separate function call so this was not needed to be adjusted. The automatic operation and manual operation was then disabled in cleaning mode.

Whilst in cleaning mode, it was set to only allow leveller 1 to raise to 70mm and lower to -6 and leveller 2 to raise to 70mm and lower to -3. The customer requested these positions as it  would work with the size carpet material that was placed on each of the carpet rollers.  As a backup for the lower function there is also an ultimate limit set so that if the encoder was to fail there would be another means of stopping the lower movement when it reaches the limit, so that the machine would not be damaged. (ultimate limit set to -11.7 and 11 leveller 1 and -4 and -4.2 for leveller 2 ).

Figure 9 demonstrates the program for the raise function for the exit screws. It can be seen that the normal manual operation and the auto-positioning are no longer available while it is in the cleaning mode function, and that it must use the rung with the comparator block so that it is required to stop at the value set.

Figure 13- FC226 network 10 exit screws raise

Function call FC136 (leveller 1) and function call FC137 (leveller 2) was amended in network 10 and 11 for the forward and reverse operation in cleaning mode. While in cleaning mode the forward operation will no longer be able to work. The reverse operation will only be able to work at the P15 panel and not the control room. Additionally, the reverse operation will only be able to be selected if both pins are in position. If any of the pins are not in position, then the reverse function cannot be operated.  This is done so that the rolls of carpet material cannot be unwound by accident. It is also because they will only be able to scour the material surface in order to clean when there is tension on the carpet.

Figure 14- FC136 network 11 jog backwards

##### 4.2.2.3. Straightener software

The straightener operations are more basic compared to the levellers’ operations so less of the program was required to be amended. For example, the straightener does not have entry screws and exit screws like the levellers, so it therefore has less encoders required for the raise and lower function.

Function call FC212 was modified for the control of the raise and lower operation in the cleaning mode at the P01 panel. These modifications were carried out in network 3 and 4. The automatic operation and manual operation was disabled while it is in cleaning mode and it has been written so that it will only raise to 90mm and lower to -33 in this function, as specified by the customer. As a backup for the stopping of the lower function, an ultimate limit was set so that if the encoder failed, there would be another means of stopping the lower movement when it reaches the limit. (ultimate limit set to -36).

Figure 15 shows how the limit was set for the straightener raise operation.

Figure 15- FC212 network 3

Function call FC212 network 13 was added in order to stop auto positioning being enabled on the control room HMI while cleaning mode is active. This was done by resetting the HMI command for auto positioning while the cleaning mode was active, seen in figure 16.

Figure 16- FC212 network 13

Network 10 and 11 in FC132 had additional inputs added to them to accommodate the forward and reverse operation in cleaning mode. While in cleaning mode the forward operation will no longer be able to work. The reverse operation will only be able to operate if both pins are in position so that the sensors are made. Figure 17 shows the reverse operation.

Figure 17- FC132 network 11

# Testing

Bench testing was conducted at IAC for the air knife blowers in order to verify that both software and hardware functioned as expected prior to installation and commission. This included making sure that the correct wiring configuration and parameters was entered for the drive so that it will work in the desired mode. This was possible as the necessary testing equipment was available at IAC. The cleaning mode however could not be bench tested, as it required the actual machinery to confirm proper operation.

By thoroughly testing the blower functionality in advance, potential delays on-site were minimised, ensuring that valuable installation and commissioning time was not wasted. Since installation and commissioning time had to take place during limited production downtime, this preparation was essential.

The bench testing setup utilised a similar PLC and remote I/O to the one on site (S7-317-2DP). The hardware configuration of the PLC software was set up to accommodate the test PLC used. To prevent system faults, bus faults and alarms, testing was performed as a new project, where only the relevant I/O and function blocks were utilised for testing.

As part of the testing process, an emergency stop (E-stop) was wired into the circuit to simulate the real-world safety conditions and verify that the E-stop and safety zone functionality would operate correctly with the blower software. This also included checking the functionality of the safe torque off (STO) function utilised in the drive.

The motor used in testing was slightly different from the one installed on site with the air knifes, as that was all IAC had available. This test motor was rated at 15kW, like the one on site, so variations in motors was minimal and therefore didn’t matter for the tests being implemented. Testing with the motor gave confirmation that the program logic and wiring was configured and written correctly.  This difference in motors did however mean that the parameters in the inverter still needed to be adjusted later on to match the on-site motor, for example the motor rated speed was different.

Only the essential program blocks were included in the bench testing software. These were:

- OB1 (organisation block 1): the main execution block that initiates with the first PLC scan and then calls the necessary function blocks.

- FC207 (function call 207): responsible for calling the blower control block FC210.

- FC210 (function call 210): Contains the control block logic for the blower operation.

- DB217 (data block 217): stores speed setpoints for the blower so that it knows what speed to run the blowers at the different stages.

These specific blocks were tested as they directly related to the air knife operation. Loading the entire program for testing would have been impractical. This is because it would have generated numerous errors due to missing system connections and it would also have significantly increased set up time due to the size of the program.

By completing comprehensive bench testing prior to site installation, the risk of unforeseen issues that could possibly occur on site was greatly reduced, ensuring a smoother commissioning process.

Figure 18- Bench testing set up at IAC

Figure 19- Bench test wiring for inverter

# Implementation

The first thing that needed to be carried out on arrival to the process line was to sign a permit to work (Appendix 4). This meant that TATA had made the area safe to work in and knew of the presence of IAC carrying out the work for the project. Once the permit was signed, the Project could start to be  implemented on site.

## Fitting of components for air knife

The push buttons was fitted first in the control room desk. The holes to be used was already punched in the desk as they had blanking plates on so this made for easier and quicker installation.

Figure 20- Desk push button location

Although the fitting of component was being carried out on site, the IAC M100 seen in Appendix 3, was still followed for the wiring standard. Inside the desk panel, luckily the remote I/O was all wired to terminals and therefore the push buttons could easily be wired to the corresponding terminal for the I/O, making this process quicker. This wiring was carried out following the drawings marked up  prior to the site visit, which identifies what I/O is being used for each push button function (Appendix 7).

Once the desk was wired, the inverters in the substation were mounted and wired up. This included making room for the two inverters in the panel by adjusting sections of trunking that wasn’t being utilised. The amended drawings done prior to the site visit were used in order to wire the inverters up correctly. This was essential as the wiring configuration had already been confirmed to work through testing at IAC.

Figure 21- mounted inverters for air knife blowers

TATA was responsible for the fitting of the air knives on plant so powering on the inverters needed to wait until clearance was given that the motor tails were terminated and safe.

Figure 22- Air knife blower 		    Figure 23- Air knife positioned on the line

While waiting, a PLC upload was taken again to back up the existing software. This was done as insurance that if the old program needed to be put back in for whatever reason, the DB’s values were up to date.

Once the back-up was complete; FC210, FC207 and DB217 was downloaded into the program. OB1 was then downloaded in to the PLC to overwrite the existing block. This needed to be done in a specific order, as the PLC would have had a system fault occur otherwise. This system fault would have put the PLC into stop and would cause the safety system to fault, requiring a whole system safety reset.

## Commissioning air knife

TATA gave clearance that it was safe to power on the inverters and carry out the testing of them. The inverters were turned on and parameters for the motors on site was put into each of the drives. See table 1 for parameters.

Table 1- Drive parameters

| Drive parameters |  |
| --- | --- |
| P-01 maximum frequency | 50hz |
| P-02 minimum frequency | 0 |
| P-03 acceleration ramp time | default 5.0 |
| P-04 deceleration ramp time | default 5.0 |
| P-05 stopping mode | 0 (Ramp to stop) |
| P-06 energy optimiser | 0 (disabled) |
| P-07 motor rated voltage | 400V |
| P-08 motor rated current | 27.5A |
| P-09 motor rated frequency | 50hz |
| P-10 motor rated speed | 2955rpm |
| P-12 primary command source | 0 (terminal control) |
| P-13 operating mode | 2 (fan mode) |
| P-14 extended menu access code | 101 (enable extended parameter groups) |
| P-15 digital input function | 0 |
| P-16 analogue input signal format | U0-10 (Uni-polar 0 to 10 Volt Signal. The drive will remain at minimum speed (P-02) if the analogue reference after scaling and offset are applied is =<0.0%. 100% signal means the output frequency / speed will be the value set in P-0) |
| P-18 output relay function select | 1 (drive healthy. Logic 1 when the output frequency matches the setpoint frequency) |
| P-25 analogue output function select | 1 (drive healthy. Logic 1 when no fault conditions exist on the drive). |

In order to make testing easier, a variable table (VAT table) was created in the PLC with all the I/O that needed to be checked. This VAT table allowed for quick monitoring and easy forcing, without the need to go directly into the FCs or DBs that it is written to.

Figure 24- VAT table for blowers

The first step in the testing process was to verify if the drive ready signal were active. This signal confirms that the drive has power and is not in fault. While this signal is not currently utilised to its full potential, it has been included to notify the plc not to send the start signal without it. This signal can be used in the future if the customer would like notifications on the SCADA system. With this signal in place, the PLC will not illuminate the Drive running LED if the drive is not ready and the start button is pressed. This can be important as without it, the operator could press the start button and assume the drive is running. Without the ‘drive ready’ signal, the PLC would still issue the start command regardless of the drive’s actual status.

Once this start command was confirmed on the PLC by turning the drives on and off, the start and stop push buttons could be tested. Hand held receivers were used for easy communication between the control room and substation so that testing could be made easier. An engineer from TATA was in the control room carrying out the operations when instructed. When the start button was pressed, the LED on the desk illuminated as expected and the drive ran. The instruction to stop it was then given and it stopped as expected. This was then also done for the second blower control.

The Estop circuit was the next one to test. The start button was pressed again and the E-stop was pushed in to ensure that the blower stopped. Once this was confirmed, a similar test was carried out with the zone 1 safety. While the drive was running, the zone 1 gate was opened and the drive cut out as expected.

When all the basic operations were confirmed to be working, the speeds needed adjusting according to what the customer wanted. Once the desired speeds were inputted, the drives needed to be tested to ensure that they ran in the correct sequence according to the program. When the line was stopped, Speed 1 was at 0Hz. When the line was idle,  Speed 2 was at 25Hz (50%). When the line was running, speed 3 was at 50 Hz (100%).

Once the customer was happy with the set speeds and operation, the DBs was initialised so that if the customer was to power down the panel with the PLC in it, the speeds was set in the program. This was essential as without initialising the DB, the speed values were only forced and would not be retained if power was lost, as the force would then be removed.

## Fitting of components for cleaning mode

TATA personnel agreed to fit the carpet roll and the limit switches while IAC started with the wiring. This was so that additional paperwork wasn’t required for working at heights and so that the commission process could be done faster, allowing the line to be given back to production quicker.

Figure 25- Carpet roller with pins  	    Figure 26- Limit switches at the top of leveller 1

The small panels P01 and P15 on the process line was the first to have the cleaning mode added to them. The holes for the switches were pre punched and had a blanking plate on so was easy to remove and fit the switches.  The beacon was fitted on the top of the panel and a hole was punched for a gland to be inserted into the hole so that the cables could enter the panel. Lastly the wiring to the remote I/O in the panel was carried out. When the small panels were completed, the substation wiring commenced.

Figure 27 - P15 panel with cleaning mode         Figure 28- P01 panel with cleaning mode

Figure 29- Remote I/O of P15 panel		 Figure 30- Wiring of E19 panel in sub station

## Commissioning cleaning mode components

Prior to any changes being made to the PLC, once again an upload was taken. This was because the cleaning mode commissioning took place at a different time to the blowers, so the DB values could have changed in between this time. Once the backup was done, the edited blocks was downloaded into the PLC.

The cleaning mode was tested one machine at a time; leveller 1, leveller 2 and straightener. This made sure that each was functioning correctly before moving onto the next one, and followed the waterfall project management approach. The first to be tested was leveller 1. The edits made to each block relating to the cleaning mode was written down. This was important as it meant that each modification could be checked thoroughly, ensuring it was working as desired and nothing was overlooked.

The first test carried out for the cleaning mode was to make sure that the pins was functioning correctly. A hand-held receiver was used for communication so that someone could take the pins in and out on the process line and Someone could check the feedback in the sub-station. The pin operation was checked by testing voltage at the terminals so that it could be identified if it was wired at the normally closed contact (NC) as expected or normally open contact (NO). Once this was confirmed to be working, it was then checked in the PLC.

The next step was to see if the cleaning mode switch operated correctly. This was where the program had to be edited slightly for it to work as required. The reason for this was that the cleaning mode was originally written in the program as a NC contact, however operated as a NO, as that was the contact TATA provided from their stores. Once this was changed, it was checked again to make sure that the operation was working as it should.

Before any machinery started to be moved in commissioning, the E-stop buttons were tested to ensure that they were still functioning correctly. This was essential as safety should be the first thing tested to make sure that there is always a way to stop the machine if something was to fail.

The switches for the up and down operation whilst in cleaning mode was then tested. The down function was tested first to see if it would stop at the value set. The up value however hadn’t had the correct value set to it yet, as the position of the limit switches hadn’t been identified. This meant that the automatic stopping couldn’t be tested at that time, only the actual functionality of the up switch could be tested.

The reverse function was tested next. When the pins were removed the reverse operation would not run, so was working correctly. During this test, it was noticed that the machine operated whilst the gates was open. This issue needed to be addressed as it meant that the machine was not safe.

The new cleaning mode allowed the machine to go down automatically and could cause someone to be injured if the safety gates did not stop operation. This was corrected by putting the safety zone in the program to stop the machine running when the zone isn’t made, for example when the gates are open. The safety zone used Profisafe PLC cards so met safety requirements.  Profisafe output cards are fail safe switching outputs that give levels of safety to SIL 3 or PLe which are safety levels in the IEC62061 and ISO 13849-1 [24], [25],[26].

As this change was only software, and the actual output for the reverse was not on a safety PLC card, a wiring modification was made in the substation. This would mean that it would run through a relay and if the safety zone was lost, it would cause the voltage to drop off the run command.  Therefore the hardware safety was incorporated as well as software.

Figure 31- E20 panel in substation with safety zone relay

This was tested for the normal machine operation and the same safety Flaw happened. The TATA engineers were made aware of this, however advised IAC that this is how they need the normal operation to function and want it to remain the same. TATA confirmed that safe working procedures and risk assessments are in place that have taken account of the normal operation of the machine, including the fact that it runs with the safety gates open.

The next test was to make sure that the machine would not be able to be put into auto positioning whilst in cleaning mode. This was selected with a button on the SCADA. Figure 32, Figure 33 and Figure 34 show the auto position screen for each leveller and straightener. Once this step was confirmed, the value given for the maximum raise was entered into the program and downloaded. This was then tested to ensure that it stopped before the limit switch. When this was proven to be working, the rollers were manually raised to the limit switches to ensure that they are positioned correctly to stop the machine in the case of an encoder failure.

When the customer was satisfied with the cleaning mode of the one machine, the commissioning process was then repeated with the next machine. The factory acceptance test (FAT) can be seen in Appendix 8. This document contains the steps carried out in commissioning and the signatures of all party present during commissioning. The customers also needed to sign the FAT document to say they are happy with how the machine operates and that it meets the FDS agreed.

Figure 32- Leveller 1 screen for auto positioning

Figure 33- Leveller 2 screen for auto positioning

Figure 34- Straightener screen for auto positioning

# Evaluation

## Issues encountered

As with any project, issues are often encountered along the way. The first issue that occurred was that the inverter isolator snapped off while commissioning.  This happened when the invertor’s were quickly being turned off and on to test for the ‘drive ready’ signal. Due to the nature of the commissioning where quick access was required to the inverter terminals, the front cover was kept off and the isolator was being turned with an adjustable spanner. This unfortunately meant that at some point, too much force was applied to the isolator and it snapped in the off position. Because of this, the commissioning of the one blower was halted and only the other blower could be completely commissioned at that time. Fortunately, when the inverter case was opened, an ABB isolator was inside. This was then purchased from RS for next day delivery and commissioning could commence immediately after the part had arrived [27].

Although this caused some inconvenience, as it extended the commissioning of the blowers by an additional day, the issue was resolved quickly and with minimal consequences. The isolator cost less than £50, which had already been accounted for in the quote, as IAC include a mark up to cover potential discrepancies.

The second issue encountered was that the zones didn’t stop operation as expected for Leveller 1 and Leveller 2. This meant that additional software needed to be added for the cleaning mode operation, as it would have not have been safe to allow it to continue to work as it was. An email was written to TATA shown in Figure 35 to explain that IAC recommend a new risk assessment to be carried out. This is due to changes to the safety system being implemented, and therefore the current risk assessment that TATA are working to may not be valid anymore.

Figure 35- Email to TATA recommending new risk assessment

The last issue encountered was that the customer said the cleaning mode beacon on panels P01 and P15 was coming on when the machine was not in cleaning mode. After inspecting this issue it was found that the operators were leaving the switches in the on position after cleaning mode operation was complete. This was not a fault as it was simply that the switches for the cleaning mode needed to be returned to the neutral position after function is complete. The cleaning mode could only be implemented this way as otherwise additional remote I/O modules would have needed to have been purchased. This was documented in the FDS prior to implementation and TATA had agreed with this procedure. When this was explained, the customer was happy and said they would write out a cleaning mode operating procedure for the operators to follow in the future.

## Effectiveness of project

For a week after commissioning the full project, the customer monitored the operation and compared it with the result from coil runs prior to implementation. The feedback given was that it noticeably improved the amount of pitting on the steel rollers and less infusions appeared that required grinding off. The customer also gave feedback that the operators have been using the cleaning mode operation, and that the procedure had been safe, easy to follow and successful with each clean.

The only issue that TATA reported when monitoring over the course of a week was that one day an operator did not start the blowers at the start of the coil. This meant that the air knife was not on, and therefore the steel did not dry.

In order to overcome the potential for this to happen again, TATA requested to change the FDS. They wanted the air knives to start automatically when the line was initially turned on, instead of needing to be switched on.

TATA engineers proposed simply linking the start button out so that it simulates that the button is always pressed. This solution would work as a quick fix, but it is not ideal for a new piece of equipment which can easily have a better method carried out. A more permanent approach was implemented through modifying the new software. The start and stop button was removed from the desk panel and the Runstp command in the PLC so that as soon as the line had been started, the idle speed will start. This can be seen in Figure 36 and Figure 37. By doing it this way, it means that the I/O is available again for future modifications if required.

Figure 36- FC207 new runstp commands

Figure 37- control room desk panel with start top buttons removed

Ater a period of a few months, more conclusive data was able to be gathered on the effectiveness of this project. TATA informed IAC that they have been able to reduce the amount of cleans needed to be carried out with the cleaning mode from up to 3 times a day to only once every three/ four weeks. This means that the air knife system is effectively drying off the material and reducing the contaminants and pitting from happening on the rollers, meeting the aim of this project.

The cleaning mode is also reportedly working effectively with the operators carrying out the task. As the cleaning mode is consistent for each clean as the positions are programmed, TATA have been able to determine that the clean takes 30 minutes to fully complete. This means that scheduled cleans can happen at break times to not effect production, unlike before where it use to disrupt both production and engineers due to the inconsistency of the roller positions causing inconsistent cleaning times.

# Conclusion and recommendations

The project was carried out smoothly, with only minor issues occurring that was rectified quickly and efficiently. The time frame was brought forward by TATA as they needed the solution to be implemented far sooner than originally planned, due to pressures from higher management. This change in schedule was handled promptly by carefully planning when the software can be written and tested prior to fitting on site on the available down days.  This rearranged time frame did have an effect on a planned work schedule with another customer, however this was able to be adjusted to amicably accommodate both customers.

By completing the project in two stages, it meant that the customer was satisfied that they could see the continual progress of the project being carried out. This also meant that each stage could be done to a high quality as it could have the full attention required.

Time was taken when commissioning to ensure that the customer was assured that the project was meeting the needs required and that it all worked correctly. By carrying out a FAT document, the customer was able to clearly see everything had been tested and that it was completely in line with the FDS agreed with at the start of the project.

The Project was kept within the  price of the quote, even with the issue of needing to purchase a new isolator and an additional day on site to finish commissioning it. This meant that the quote was priced correctly so that it wasn’t overly high so that the customer would not accept the quote, but was high enough for IAC to be able to remain profitable and keep in business.

On completion of the project, the customer contacted IAC after a week of running material through the line to provide feedback of the results of the project. TATA clarified that the project had improved product quality as less pitting was occurring, and therefore less maintenance of the rollers was needed. The customer also informed IAC that the operators were now carrying out the cleaning mode rather than the engineers (as done previously), as the process is now safe and easy to implement.

TATA did report of a change they would like carried out that would vary from the FDS. This change was to eliminate the need for the stop and start buttons on the control desk. The change was carried out promptly, in a way that worked best for the system. This additional modification did come at an incurred cost, as the customer understood that the original FDS agreed was met.

As with any project, improvements can be made. An improvement that could be made if a similar project was to be carried out would be that equipment will be correctly utilised to reduce the chances of anything breaking. This improvement may cause slightly longer commissioning times, but could potentially save time in the long run by eliminating the risk of anything breaking, like the isolator did in this project. Furthermore, this small adjustment in commissioning could cut possible costs of buying new equipment, and also give the customer the finished project faster than if a new part was required.

Another improvement that could be made for future projects would be that the customer should sign off on the plan at the start of a project, to ensure that the schedule is finalised and shouldn’t be amended. This would eliminate alterations to schedules and therefore would no longer have any possible effect to other jobs that may have been planned with other customers.

Overall, the project was a success and the customer was more than happy with the work carried out. On the completion of the project, the customers satisfaction lead to a new project asked to be quoted by IAC for another part of plant.

# Reflection

This project has offered many opportunities to learn and grow professionally. It has given lots of exposure to how projects in heavy industry are carried out and what aspects are deemed essential. Additionally, this project has deepened my knowledge with heavy industry processes and equipment and given me a core understanding of how these plants operate, making it easier to understand how to approach future projects in this industry.

Carrying out this project has put my time management to the test. The rearranged schedules taught me to carefully steer through what jobs are critical to carry out first and what jobs have more flexibility in the time frame.

This project has also helped me to improve my programming knowledge. Although I was not unfamiliar with the Siemens Simatic manager software prior to the project, it has forced me to learn the intricate ways that can be used to navigate the software. Learning all the different methods and details with the software has advanced my skills for any future encounters with this software, making me more efficient at my job.

Carrying out this project has given me confidence in my own abilities and the confidence to put myself forward for more projects of a similar nature. It has shown me that I am capable of taking an idea given by a customer and turning it into a solution that they wanted. The project allowed me to face and overcome challenges, and from these challenges, learn how to improve in the future to ensure the same issues will not be faced.

# 10. 	References and Bibliography

[1] TATA steel. (2015). Tata steel opens Europe’s most powerful decoiler [online] available from:https://www.tatasteel.com/media/newsroom/press-releases/india/2015/tata-steel-opens-europes-most-powerful-decoiler/ - accessed on 17/10/24

[2] K V, Akpanyung and R T, Loto. Pitting corrosion evaluation: A Review. (2019) [online] available from: https://iopscience.iop.org/article/10.1088/1742-6596/1378/2/022088/pdf - accessed on 18/02/25

[3] G T, Burstein and S P, Vines. Repetitive nucleation of corrosion pits on stainless steel and the effects of surface roughness. (2001). [online] available from: https://citeseerx.ist.psu.edu/document?repid=rep1&type=pdf&doi=8c4dc0396e64722c194a04f05528f4eb11d66348 – accessed on 18/2/25

[4] S. Way, “Pitting due to rolling contact”, J. Appl. Mech 2, A49–A58. (1935)

[5] S Way. “Pitting and flaking in rolling contact”. Journal of Applied Mechanics 7(4), A-149. (1940)

[6] O P Datsyshyn. V V, Panasyuk. A Tu, Glazov. Modelling of fatigue contact damages formation in rolling bodies and assessment of their lifetime. (2010). [online] available from: Modelling of fatigue contact damages formation in rolling bodies and assessment of their lifetime - ScienceDirect – accessed on 18/2/25

[7] D&D coatings. (2025) [online] available from: What is Pitting Corrosion? How to detect and treat pitting corrosion?- accessed on 23/02/25

[8] Roberge, P. R. (2018). Handbook of Corrosion Engineering. McGraw-Hill Education.

[9] J P, Birat. Steel cleanliness and environment metallurgy. (2016). [online] available from: https://www.metallurgical-research.org/articles/metal/pdf/2016/02/metal150091.pdf- accessed on 19/02/25

[10] E. G. Kulikova and S. Y. Levenson. “Operation of Vibrating Dryer with Elastic Transporter for Granular Geomaterials”. Journal of mining science, vol. 57, no. 3, pp. 459–468. (2021)

[11] Hywell. A comprehensive guide to vibrating fluidized flatbed dryers. (2023) [online] available from: https://www.hywellco.com/A-Comprehensive-Guide-to-Vibrating-Fluidized-Bed-Dryers-id41576637.html - accessed on 24/10/24

[12] Witte. Vibrating fluid bed dryers replace oven air belt dryers for sanitary processing of nuts, fruits and cerials. (2025). [online] available from: https://www.witte.com/vibrating-fluid-bed-dryers-replace-open-air-belt-dryers-for-sanitary-processing-of-nuts-fruits-cereals/ - accessed on

[13] M. Zea, "Maximizing Oven Lifespan, Efficiency and Performance," Process Heating, vol. 30, (7), pp. 22-26. (2023).

[14] Infrared heating technologies. Electric convection oven for heat-setting fuel-lines. (2024). [online] available from:  https://www.infraredheating.com/electric-convection-oven-for-heat-setting-fuel-lines/ - accessed on

[15] Anon, Longer Super Air Knives. Rockaway: Advantage Business Media. (2015) [online] available from: https://www.proquest.com/trade-journals/longer-super-air-knives/docview/1726272134/se-2. -accessed on 25/10/24

[16] Anonymous. “Air knife strip tests completed successfully at BS Llanwern”. Steel Times, vol. 224, (10), pp. 367. (1996) Available from: https://www.proquest.com/docview/1357119040?sourcetype=Trade%20Journals. – accessed on 24/10/24

[17] PF Products finshing. Air knives achieves efficent cleaning, drying and cooling. (2024). [online] available from: https://www.pfonline.com/articles/air-knives-achieves-efficient-cleaning-drying-and-cooling - accessed on

[18] E. G. Kulikova and S. Y. Levenson. “Operation of Vibrating Dryer with Elastic Transporter for Granular Geomaterials”. Journal of mining science, vol. 57, no. 3, pp. 459–468. (2021)

[19] Hywell. A comprehensive guide to vibrating fluidized flatbed dryers. (2023) [online] available from: https://www.hywellco.com/A-Comprehensive-Guide-to-Vibrating-Fluidized-Bed-Dryers-id41576637.html - accessed on

[20] Solvair. PX2000 series air blower. (2025). [online] available from: https://solvair.co.uk/air-knife-blowers/px2000/ - accessed on 15/3/25

[21] invertek drives. Optidrive E3. (2025). [online] available from: https://www.invertekdrives.com/variable-frequency-drives/optidrive-e3/features - accessed on 15/3/25

[22] Sick sensor intelligence. IME18-12NPSZW2S. (2025). [online] available from:  https://www.sick.com/sk/en/catalog/products/detection-sensors/inductive-proximity-sensors/ime/ime18-12npszw2s/p/p228583 - accessed on 15/3/25

[23] RS PRO red flashing beacon. (2025). [online] available from: https://uk.rs-online.com/web/p/beacons/2399240?searchId=44b264dd-ffa3-4a57-b41f-fd5a10862241&gb=s – accessed on 15/3/25

[24] Siemens. PROFisafe safety components. (2024). [online] available from: https://mall.industry.siemens.com/mall/en/WW/Catalog/Products/10164644?activeTab=productinformation – accessed on 12/4/25

[25] International elecrotechnical commission. IEC 62061. (2021). [online] available from: https://webstore.iec.ch/en/publication/59927 - accessed on 12/4/25

[26] international standards organisation. ISO 13849-1:2023. (2023). [online] available from: https://www.iso.org/standard/73481.html - accessed on 12/4/25

[27] RS. ABB 3P Pole Base Mounting Isolator Switch. (2025). [online] available from: https://uk.rs-online.com/web/p/isolator-switches/6867278?gb=s – accessed on 15/3/25

[28] M. Stefanoni, U. Angst, and B. Elsener. “Local electrochemistry of reinforcement steel – Distribution of open circuit and pitting potentials on steels with different surface condition”. Corrosion science, vol. 98, pp. 610–618. (2015)

[29] O. P. Datsyshyn, V. V. Panasyuk, and A. Y. Glazov. “Modelling of fatigue contact damages formation in rolling bodies and assessment of their lifetime". Wear, vol. 271, no. 1, pp. 186–194. (2011).

[30] J. E. Johansson, M. T. Devlin, and B. Prakash. “Lubricant additives for improved pitting performance through a reduction of thin-film friction”. Tribology international, vol. 80, pp. 122–130. (2014)

# Appendices

## Appendix 1- Quote and FDS for Cleaning modes

## Appendix 2 – Quote and FDS for air knife

## Appendix 3 – IA Work instructions (IAC E100)

## Appendix 4 – TATA permits to work

## Appendix 5 – Job method statement and risk assessment

## Appendix 6- Excell document containing  I/O

## Appendix 7- Modified pages of drawings

## Appendix 8 – FAT document

## Appendix 9 – Program blocks for cleaning mode and blower control functions