Applied Computing

Wales Institute of Science and Art (WISA)

# Advanced Networking (ACCA5024)

Lab 8: Static Routing

n.kaliyamurthy@uwtsd.ac.uk                                                                                                                            1

Design the topology and execute basic configuration as stated below

Refer to the above topology for addressing information.

Tasks:

- Set Up the Topology and Initialize Devices

- Configure Devices and Verify Connectivity

Assign static IPv4 information to the PC interfaces.

Configure basic router settings. (Refer to our previous labs. Basic Configurations includes hostname, passwords, line configurations, device security, domain lookups, banners)

Configuring your network with complete information is highly recommended.

Verify network connectivity.

Cisco123

Ciscocon

Ciscovtyen

- Configure Static, Default and Loopback addresses for all the routers Configure IPv4 Address for all the router interfaces.

R1 interface Serial0/1/0 ip address 10.0.0.1 255.255.255.252 clock rate 128000 interface GigabitEthernet0/0 ip address 192.168.10.1 255.255.255.0 R2 interface Serial0/1/0 ip address 10.0.0.2 255.255.255.252 clock rate 2000000 interface GigabitEthernet0/0 ip address 192.168.20.1 255.255.255.0 interface Serial0/1/1 ip address 20.0.0.1 255.255.255.252 clock rate 128000 R3 interface GigabitEthernet0/0 ip address 192.168.30.1 255.255.255.0 interface Serial0/1/1 ip address 20.0.0.2 255.255.255.252

Configure loopback address (Lo1) as 50.50.50.50/32 at Router R2.

R2(config)#int loopback 1

R2(config-if)#ip address 50.50.50.50 255.255.255.255

On R1, R2 and R3 configure IPv4 static route to connect other networks. Do not include Lo1 network.

R1 R1(config)#ip route 0.0.0.0 0.0.0.0 10.0.0.2 R1(config)#ip route 192.168.20.0 255.255.255.0 10.0.0.2 R1(config)#ip route 192.168.30.0 255.255.255.0 10.0.0.2 R2 R2(config)#ip route 192.168.30.0 255.255.255.0 20.0.0.2 R2(config)#ip route 192.168.10.0 255.255.255.0 10.0.0.1 R3 R3(config)#ip route 192.168.10.0 255.255.255.0 20.0.0.1 R3(config)#ip route 192.168.20.0 255.255.255.0 20.0.0.1 R3(config)#ip route 0.0.0.0 0.0.0.0 20.0.0.1

Configure default static route to reach Lo1 network using next-hop ip address at R1

R1(config)#ip route 50.50.50.50 255.255.255.255 10.0.0.2

R3(config)#ip route 0.0.0.0 0.0.0.0 20.0.0.1

Configure default static route to reach Lo1 network using exit interface at R3

Interpret the output from the routing table.

R1 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/1/0 L 10.0.0.1/32 is directly connected, Serial0/1/0 192.168.10.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.10.0/24 is directly connected, GigabitEthernet0/0 L 192.168.10.1/32 is directly connected, GigabitEthernet0/0 S 192.168.20.0/24 [1/0] via 10.0.0.2 S* 0.0.0.0/0 [1/0] via 10.0.0.2 R2 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 10.0.0.0/30 is directly connected, Serial0/1/0 L 10.0.0.2/32 is directly connected, Serial0/1/0 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/1/1 L 20.0.0.1/32 is directly connected, Serial0/1/1 50.0.0.0/32 is subnetted, 1 subnets C 50.50.50.50/32 is directly connected, Loopback1 S 192.168.10.0/24 [1/0] via 10.0.0.1 192.168.20.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.20.0/24 is directly connected, GigabitEthernet0/0 L 192.168.20.1/32 is directly connected, GigabitEthernet0/0 S 192.168.30.0/24 [1/0] via 20.0.0.2 R3 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks C 20.0.0.0/30 is directly connected, Serial0/1/1 L 20.0.0.2/32 is directly connected, Serial0/1/1 S 192.168.10.0/24 [1/0] via 20.0.0.1 S 192.168.20.0/24 [1/0] via 20.0.0.1 192.168.30.0/24 is variably subnetted, 2 subnets, 2 masks C 192.168.30.0/24 is directly connected, GigabitEthernet0/0 L 192.168.30.1/32 is directly connected, GigabitEthernet0/0 S* 0.0.0.0/0 [1/0] via 20.0.0.1

Verify the status of the interfaces.

R1 GigabitEthernet0/0 is up, line protocol is up (connected) GigabitEthernet0/1 is administratively down, line protocol is down (disabled) GigabitEthernet0/2 is administratively down, line protocol is down (disabled) Serial0/0/0 is administratively down, line protocol is down (disabled) Serial0/1/0 is up, line protocol is up (connected) Serial0/1/1 is administratively down, line protocol is down (disabled) Serial0/2/0 is administratively down, line protocol is down (disabled) Serial0/2/1 is administratively down, line protocol is down (disabled) Serial0/3/0 is administratively down, line protocol is down (disabled) Serial0/3/1 is administratively down, line protocol is down (disabled) R2 GigabitEthernet0/0 is up, line protocol is up (connected) GigabitEthernet0/1 is administratively down, line protocol is down (disabled) GigabitEthernet0/2 is administratively down, line protocol is down (disabled) Serial0/0/0 is administratively down, line protocol is down (disabled) Serial0/0/1 is administratively down, line protocol is down (disabled) Serial0/1/0 is up, line protocol is up (connected) Serial0/1/1 is up, line protocol is up (connected) Serial0/2/0 is administratively down, line protocol is down (disabled) Serial0/2/1 is administratively down, line protocol is down (disabled) Serial0/3/0 is administratively down, line protocol is down (disabled) Serial0/3/1 is administratively down, line protocol is down (disabled) R3 GigabitEthernet0/0 is up, line protocol is up (connected) GigabitEthernet0/1 is administratively down, line protocol is down (disabled) GigabitEthernet0/2 is administratively down, line protocol is down (disabled) Serial0/0/0 is administratively down, line protocol is down (disabled) Serial0/0/1 is administratively down, line protocol is down (disabled) Serial0/1/0 is administratively down, line protocol is down (disabled) Serial0/1/1 is up, line protocol is up (connected) Serial0/2/0 is administratively down, line protocol is down (disabled) Serial0/2/1 is administratively down, line protocol is down (disabled) Serial0/3/0 is administratively down, line protocol is down (disabled) Serial0/3/1 is administratively down, line protocol is down (disabled)

Verify the status of the network connectivity.

Ping statistics for 192.168.10.11: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 1ms, Maximum = 16ms, Average = 7ms Ping statistics for 192.168.30.11: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 1ms, Maximum = 17ms, Average = 5ms

Record your routers running configurations here!!!  Do it separately for R1, R2 and R3

R1

R1#show running-config

Building configuration...

Current configuration : 1614 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R1

!

!

!

enable secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524MLCL-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 192.168.10.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

ip address 10.0.0.1 255.255.255.252

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router rip

!

ip classless

ip route 192.168.20.0 255.255.255.0 10.0.0.2

ip route 0.0.0.0 0.0.0.0 10.0.0.2

!

ip flow-export version 9

!

!

!

banner motd ^C

No Unauthorsied Access ^C

!

!

!

!

!

line con 0

password 7 0802455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0802455D0A1613030B

logging synchronous

login

transport input telnet

!

!

!

end

R2

R2# show running-config

Building configuration...

Current configuration : 1696 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R2

!

!

!

enable secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524AMMI-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 50.50.50.50 255.255.255.255

!

interface GigabitEthernet0/0

ip address 192.168.20.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

ip address 10.0.0.2 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

ip address 20.0.0.1 255.255.255.252

clock rate 128000

!

interface Serial0/2/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.10.0 255.255.255.0 10.0.0.1

ip route 192.168.30.0 255.255.255.0 20.0.0.2

!

ip flow-export version 9

!

!

!

banner motd ^C

Warning ^C

!

!

!

!

!

line con 0

password 7 0802455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0802455D0A1613030B

logging synchronous

login local

transport input telnet

!

!

!

end

R2#

R2 con0 is now available

Press RETURN to get started.

Warning

User Access Verification

Password:

R2>en

Password:

R2# show runn

R2# show running-config

Building configuration...

Current configuration : 1696 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R2

!

!

!

enable secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524AMMI-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Loopback1

ip address 50.50.50.50 255.255.255.255

!

interface GigabitEthernet0/0

ip address 192.168.20.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

ip address 10.0.0.2 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

ip address 20.0.0.1 255.255.255.252

clock rate 128000

!

interface Serial0/2/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.10.0 255.255.255.0 10.0.0.1

ip route 192.168.30.0 255.255.255.0 20.0.0.2

!

ip flow-export version 9

!

!

!

banner motd ^C

Warning ^C

!

!

!

!

!

line con 0

password 7 0802455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0802455D0A1613030B

logging synchronous

login local

transport input telnet

!

!

!

end

R3

R3#show running-config

Building configuration...

Current configuration : 1638 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 7

!

hostname R3

!

!

!

enable secret 5 $1$mERr$RldxcCZEZsTFTETUyRaA50

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX15245X98-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 192.168.30.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

ip address 20.0.0.2 255.255.255.252

!

interface Serial0/2/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.20.0 255.255.255.0 20.0.0.1

ip route 192.168.10.0 255.255.255.0 20.0.0.1

ip route 0.0.0.0 0.0.0.0 20.0.0.1

!

ip flow-export version 9

!

!

!

banner motd ^C

Warning ^C

!

!

!

!

!

line con 0

password 7 0802455D0A1606181C

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0802455D0A1613030B

logging synchronous

login local

transport input telnet

!

!

!

end