HND Civil Engineering and Environmental Management

Contract Practise and Procedures

Assignment 1

A report focusing on:

Choice of construction procurement methods and contractual arrangements

Module Code – M5X01878

Lecturer: Lara Hopkinson

Report By: Thomas Hinder (1701670)

Contents

1.0 Introduction.…………….………………………………….……….3

2.0 Case Study 1………………………………….……....…….……..3

3.0 Case Study 2…………………………………………………….…4

4.0 Case Study 3…………………………………………………….…5

5.0 Case Study 4…………………………………………………….…6

Conclusion………………………………………………………………6

1.0 Introduction

Procurement is a term that describes the activities undertaken by a client or employer who is seeking to achieve the development or refurbishment of a building. It’s a mechanism that provides an answer to the question: "how do I get my project built?"

On most projects, clients would start the process of procurement by devising a project strategy. The strategy entails weighing up the benefits, risks and budget constraints of a project to see what the foremost acceptable procurement method is, and what written agreement arrangements are going to be needed.

With each project, the client’s concerns concentrate on time, value and quality (or performance) in relation to both the design and construction of the building. The client’s policies, resources, organisational structure, and preferred contractual arrangements will all need to be taken into account in selecting the right procurement method for their project.

Understanding risk is essential, as although every procurement method follows a well-established set of rules and procedures, there are risks related to selecting any specific route. Successful procurement depends on all parties involved within the project complying with their respective obligations, and identifying and managing with risk appropriately from the outset.

2.0 Case Study 1

The construction of a new doctor’s surgery to accommodate four GPs, three nurses and two dentists. The new surgery is required to provide accommodation following the amalgamation of three different national health practices. It will be funded by the local health authority.

The procurement most suited for the construction of a new doctors surgery would be NEC3 Engineering and Construction Contract Option A (priced contract with activity schedule)

Option A is a priced contract with an activity schedule where the risk of carrying out the work at the agreed prices is largely borne by the contractor. This document contains all the clauses, the shorter schedule of cost components and contract data, relevant to an option A contract.

Thomas Telford (01/04/2013). NEC3: Engineering and Construction Contract Option A Priced contract with activity schedule. Retrieved from

https://www.neccontract.com/NEC3-Products/NEC3-Contracts/NEC3-Engineering-Construction-Contract/NEC3-Engineering-and-Construction-Contract-ECC-/NEC3-Engineering-and-Construction-Contract-Option

NEC was first published in 1993 as the New Engineering Contract. It is a set of construction contracts trying to promote partnering and collaboration between the contractor and client.

The Engineering and Construction Contract (ECC) is the most often used, and can be adopted on projects such as infrastructure, buildings, highways and process plants. It is used for the appointment of a contractor for engineering and construction work, including and level of design responsibility.

Designing Buildings (23/08/2018). NEC Option A: Priced contract with activity schedule. Retrieved from

https://www.designingbuildings.co.uk/wiki/NEC_Option_A:_Priced_contract_with_activity_schedule

3.0 Case Study 2

A new Tesco superstore for the town of Skewen it is to have a total floor area of 3400 m2 of which 1800 m2 will be sales area. Tesco wish the store to comply with their corporate image and to be open by September of 2019

Time and quality is very important in this Build that’s why I have chosen JCT – Design and Build to be the most suitable.

This combines design and construction into one contract accelerating project delivery. Design-build requires less administration can reduce project duration and cost by sharing risk with the contract in a community.

The brand new Design Build and Operate Contract allows Clients to procure a more integrated whole-life delivery solution. The development of this contract reflects the increasing demand for contracts extending into the operational phase.

Thomas Telford (22/06/2017). NEC4: Design Build and Operate Contract Bundle. Retrieved from https://www.neccontract.com/NEC4-Products/NEC4-Contracts/NEC4-Design-Build-and-Operate-Contract/NEC4-Design-Build-and-Operate-Contract-Bundle-Bu

4.0 Case Study 3

The construction of a new boutique hotel for Hilton Hotels in Swansea Marina. The project is to be privately financed by a merchant bank which has set a firm cost limit for the project.

In this situation a JCT – Lump sum contract would be best as the there has been a firm cost limit that has been set.

In lump sum construction contract contractor bids one fixed price for all activities within the project scope. Under a lump sum contract, a single ‘lump sum’ price for all the works is agreed before the works begin. This method is particularly used for big construction projects and is a conventional but most popular types of construction contract.

lump sum construction contract is considered as the most effective means of reducing construction price and is useful when projects and its activities are well defined.

This is generally appropriate where the project is well defined, when tenders are sought, and significant changes to requirements are unlikely. This means that the contractor is able to accurately price the works they are being asked to carry out.

The Constructor. Lump Sum Construction Contract – Its Advantages and Disadvantages. Retrieved from https://theconstructor.org/construction/lump-sum-construction-contract-advantages-disadvantages/14956/

Designing Buildings ( 05/11/18). Lump sum contract. Retrieved from https://www.designingbuildings.co.uk/wiki/Lump_sum_contract

5.0 Case study 4

Upgrading a small community Theatre in Merthyr Tydfil, the Theatre is managed by a group of volunteers who control the Theatre through a company limited by guarantee. This means that they might be liable for any losses the Theatre makes. The extent of the works is to refurbish the floor finishes, provided disabled toilet facilities and install a new kitchen that meets current hygiene standards.

JCT - Minor Works Building Contract would be the most suitable contact and procurement rout as the project is small and relatively straight-forward.

The JCT Minor Works Building Contract is more straight forward to administer than many other forms of contract, however, the parties will still need to be very clear of their obligations and liabilities and the risks that they are accepting. Generally the client’s architect or contract administrator will administer the contract.

Designing Buildings (16/10/2018). Minor works. Retrieved from https://www.designingbuildings.co.uk/wiki/Minor_works

There is a contract between the employer and the contractor with standard contract conditions that include information such as the contract period and contract sum.

The works are set out in the contract documents which typically include a specification and sometimes drawings.

John Wade. What is The JCT Minor Works Building Contract?. Retrieved from https://www.grumittwade.com/what-is-jct-minor-works-building-contract/

6.0 Conclusion

No procurement route offers a solution to all issues: it is always a compromise. Careful consideration must be given when selecting which procurement method to use for a construction project. Each individual project will have different factors which may or may not suit a particular procurement method.