# Built Environment Programmes – Level 4. BSc (Hons) Construction Management; BSc (Hons) Quantity Surveying; BSc (Hons) Building Surveying; BSc (Hons) Civil Engineering. HNC and HND pathways.

# Module:			Managing Health, Safety and Welfare in Construction		Module Level (4, 5, 6): 4

# Module code:		ACBA4003

# Lecturer:			Dr Deborah Hughes

# Internal Verifier:	Allan Nantel

# Assignment Title:	Managing Health and Safety in a project

# Assignment No:		2 of 2

# Hand Out Date:		Week Commencing 6/10/20

# Submission deadline:	24/11/20

### Table Of Contents

#### Executive Summary							3

#### Terms of reference							3

#### Procedure								3

#### Findings								4

#### 4.1. Construction Phase Plan				4

#### 4.2. Traffic Management Plan				21

#### 4.3. Sequence of work					26

#### 4.4. Method Statement					27

#### 4.5. Risk Assessments					32

#### Conclusion								35

#### 5.1. History Of CDM						35

#### 5.2. Roles within CDM					35

#### 5.3. Acts & Regulations					38

#### Bibliography							3

### Executive Summary

This report aims to: ‘Interpret the application of Construction Design and Management (CDM) regulations and the wider Health and Safety legislation whilst working with clients, contractors and other professionals.

Determine how Health, Safety and Welfare legislation influences working practices in the construction industry, how it is managed on site and the potential impacts that this might have.’

### Terms of reference

  - The principal focus are:

    - ?

    - ?

    - ?

  - This report was limited in access to resources and reading due to the SARS CoV2 Pandemic.

### Procedure

This section identifies the resources used.

  - The research included secondary sources on Health and safety within the construction industry including books, lecture notes & websites.

### Findings

### Construction Phase Plan

| Construction Phase Plan |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract Title: |  | Caretakers House Kings Rd, Swansea, SA1 8AS. 2020-2021 |  |  |  |  |  |  |  |
| Location / Address: |  | Kings Rd, Swansea, SA1 8AS |  |  |  |  |  |  |  |
| Project Details |  | Method Statements - See Attached |  |  |  |  | Risk Assessments - See Attached |  |  |
| Site Locations |  | As per specification |  |  |  |  |  |  |  |
| Key Dates |  | Start Date: |  | 7th December 2020 |  |  |  |  |  |
|  |  | Finish Date: |  | 9th July 2021 |  |  |  |  |  |
|  |  | Contract Duration: |  | 34 weeks |  |  |  |  |  |
| Prepared by: |  | M Robinson |  |  |  |  |  |  |  |
| Site Manager: |  | Joey Tribbiani |  |  |  |  |  |  |  |
| Accepted By: (Named Officer & Date) |  | xxx |  |  |  |  |  |  |  |
| Client: |  | Swansea University |  |  |  |  |  |  |  |
| Accepted By: (Named Officer & Date) |  | xxx |  |  |  |  |  |  |  |
| Document Control Sheet: |  |  |  |  |  |  |  |  |  |
| Version | Date |  | Prepared by |  |  | Checked by |  |  | Checked by |
| 0.1 | 13th October 2020 |  | M Robinson |  |  |  |  |  |  |
| Description of Project |  |  |  |  |  |  |  |  |  |
| General overview |  |  |  |  |  |  |  |  |  |
| Scheme Description: Caretakers House. New build Single story detached domestic building with loft room. The proposed foundations are concrete strip to a depth of 1m with standard cavity wall construction; block and block with partial fill cavity insulation and a smooth render externally. Plinths, soldier courses over all openings and porch pillars are to be natural sandstone. Works shall consist of (not limited to) as outlined within the Architectural, Mechanical and Electrical Specifications: Enabling works shall consist of (not limited to) as outlined within the Architectural Specification: |  |  |  |  |  |  |  |  |  |

| Project Team Details |  |  |  |
| --- | --- | --- | --- |
| Client Representative: | Chandler Bing chandler.bing@swansea.gov.uk Tel. 07000000000 |  |  |
|  | Swansea Council, Building Services, Heol Y Gors Depot, Swansea, SA5 8LD |  |  |
| Principal Designer: | Mark Robinson mark.robinson@bumblebeedesign.co.uk Tel No.01792 555555 |  |  |
|  | Bumblebee Designs The Hive Swansea SA1 BZZ |  |  |
| Designer: | Mark Robinson mark.robinson@bumblebeedesign.co.uk Tel No.01792 555555 |  |  |
|  | Bumblebee Designs The Hive Swansea SA1 BZZ |  |  |
| Principal Contractor: | Ross Geller ross.geller@swansea.gov.uk Tel No. 07000000000 |  |  |
|  | Swansea Council, Building Services, Heol Y Gors Depot, Swansea, SA5 8LD |  |  |
| Project Manager: | Rachel Green rachel.green@swansea.gov.uk Tel No. 07000000000 |  |  |
|  | Swansea Council, Building Services, Heol Y Gors Depot, Cwmbwrla, Swansea, SA5 8LD |  |  |
| Site Manager: | Joey Tribbiani – joey.tribianai@swansea.gov.uk Tel No. 07000000000 |  |  |
|  | Swansea Council, Building Services, Heol Y Gors Depot, Swansea, SA5 8LD |  |  |
| Safety Adviser: | Swansea Council Corporate Health, Safety & Wellbeing Room 254, The Guildhall Swansea, SA1 4PE | Tel No | 01792 000000 |
|  |  | E Mail | healthandsafety@swansea.gov.uk |

| General Contacts List |  |  |  |
| --- | --- | --- | --- |
| Utilities Providers and Sub Contractors |  |  |  |
| Utilities Providers | Western Power Distribution Newport Rd Cardiff CF3 5WW | Tel No: | Emergency tel 0800 052 0400 |
|  |  | EMail: |  |
| Utilities Providers | Dwr Cymru | Tel No: | Emergency tel 0800 052 0130 |
|  |  | EMail: |  |
| Utilities Providers | Sewerage | Tel No: | Emergency tel 0800 052 0130 |
|  |  | EMail: |  |
| Utilities Providers | Broadband/TV | Tel No: | Emergency tel 0800 052 0400 |
|  |  | EMail: |  |
| Sub-Contractor (Groundworks) | NT Groundworks Ltd 42 Ffynnon Dawel, Neath, SA10 8EQ | Tel No: | 01639 642746 |
|  |  | EMail: | info@ntgargroundworks.co.uk |
| Sub Contractor (Timber Frame) | Treharne Homes Ltd Cysgod Y Cwm, Mynachlog Road, Pontyberem, Llanelli, Carmarthenshire SA15 5EL | Tel No: | Mob: 07971437118 |
|  |  | EMail: | delfrig@treharnehomes.com |
| Sub Contractor (Bricklaying) | CMT Bricklayer, 30 Cherry Crescent, Penllergaer, Swansea SA4 9FG Chris Thomas | Tel No: | Mob: 07970425481 |
|  |  | EMail: | Christopher.cmt@byinternet.com |
| Sub Contractor (Windows) | Solar Windows Ltd. Unit 15b, Greenway, Bedwas House Industrial Estate, Bedwas, Caerphilly, Mid Glamorgan, CF83 8DW Anthony Evans | Tel No: | Tel: 02920858957 |
|  |  | EMail: | robert@solarwindows.co.uk |
| Sub Contractor (Decorating) | C J Else St Peter’s Terrace, Swansea, SA2 0FW Jacka Dave | Tel No: | Tel: 01792 587215 |
|  |  | E Mail: | Cj.else@btconnect.com |
| Sub Contractor (Roofing) | Building Services Heol Y Gors, Cwmbwrla, Swansea, SA5 8LD Darren Thomas | Tel No: | 07796275548 |
|  |  | E Mail: | Darren.thomas@swansea.gov.uk |
| Sub Contractor (Wall Tiling) | Natural Stone & Tile 19 Pontardawe Road, Clydach, SA6 5NS Johnathan | Tel No: | Mob: 07794077643 |
|  |  | E Mail: | jono@naturalstoneandtilecompany.co.uk |
| Sub Contractor (Flooring) | Carpets N Carpets 117 Bute Street, Treochy, Rhondda, Cynon Taff Rachel Morris | Tel No: | Mob: 01443772349 |
|  |  | E Mail: | carpetsncarpets@hotmail.com |
| Sub Contractor | TBC | Tel No: | Tel: Mob: |
|  |  | E Mail: |  |
| Sub Contractor | TBC | Tel No: | Tel: Mob: |
|  |  | E Mail: |  |
| Summary of project Information |  |  |  |
| Project Name | Caretakers House |  |  |
| Project Addresses | Kings Rd, Swansea, SA1 8AS. 2020-2021 |  |  |
| Description and Scope of Works | The project involves construction work, notifiable to the Health & Safety Executive under Regulation 2, on Kings Road, in the SA1 district of Swansea. The address of the property which will comprise all the permanent and temporary works are: - as identified within the construction drawing. New development of 1No. Detached Property for use as a caretakers house. Enabling works shall consist of (not limited to) as outlined within the Architectural Specification: |  |  |
| Outline Method Statement(s) | Appended in section 2 of site Health and Safety file. |  |  |
| Existing Environment | All relevant information will be included in the pre-construction information and design drawings |  |  |
| Previous Use of Land | Industrial (pre1980) and Commercial (pre 2000) |  |  |
| Soil Investigation | Information from the desktop studies will be included in the pre-construction information and a hard copy will appear in the appended section 12 of the site health and Safety file. |  |  |
| Surrounding Areas | Bounded by Offices & Student accommodation to the east and the river Tawe to the west. |  |  |
| Existing Services | Existing service drawings will be available for references during the construction process. |  |  |
| Existing Structures | N/A |  |  |
| Asbestos Reports | Geotechnical Investigation are being under taken on the site. Should at any time asbestos containing materials is suspected, works are to be ceased, the area made safe to limit potential exposure and the asbestos team consulted |  |  |
| Ground Conditions Site Investigations | To be carried out: Ecological survey Ground investigation report |  |  |
| Existing Traffic Systems | Access for Kings road site, will be from Fabian way to the north. Adjacent developments. During the works there will be another large development being carried out on land adjacent to the site. Construction of the new Car park to the north is programmed to be carried out at around the same time as the caretakers house scheme. This Scheme is also fronting onto kings road and as such will create increased traffic within the area. This will need to be considered when programming site activities and the traffic management plan and how possible future interaction and liaison between both sites will be managed. Vehicle, plant movement and deliveries are to be planned to limit congestion within the vicinity with consideration to planning conditions and be reflected within the Traffic plan. |  |  |
| Management of the Work |  |  |  |
| Health & Safety Aims: |  |  |  |
| The aim of this plan is to: - Deal with the risks and hazards identified within the pre- construction information plan and any other risks, identified associated with the work and to set out the arrangements to ensure safe systems of work are implemented which:- - Eliminate the risk or hazard. - Where elimination is not possible, reduce and control the risk or hazard to an acceptable level. - Set out clear roles & responsibilities within the project team to ensure compliance with CDM 2015 and the Clients brief for the duration of the Construction Phase. - The project to be completed so far as is reasonably practical without placing people’s health, safety & welfare at risk. |  |  |  |

Site Rules & Induction:

The site rules and induction are set out in this plan and included in the site safety file. Site inductions and tool box talks will be carried out by the site supervisor who will give an induction/toolbox talk to everyone (including sub-contractors and site visitors) on site at the beginning of their initial visit. The talks will cover all aspects of and requirements for entry onto the site and will cover topics such as: - CPP - Site safety. - Site rules. - Welfare arrangements - Requirements for entry onto the site and its confines & no go areas. - Emergency procedures - Responsible to site supervisor at all times All persons entering the site will be required to sign off on their understanding and acceptance of the induction and site rules. The rules will be clearly displayed on site. Tool Box Talks- - Covid-19 - First aid facilities and arrangements - Emergency evacuation procedures - Location of muster points - Any safe systems of work for the site - The wearing of PPE - Accident reporting - Any identified risks or hazards on site. (A selection of information on toolbox talks is included in section 1 of the Health & Safety file, this is not a definitive list and is only included as an aide for site inductions and talks)

Arrangements for Cooperation & Coordination

Engaging Contractors:

A Pre commencement meeting will be held with all contractors prior to their starting on site. As part of the agenda discussions will be held on scope of works, programme requirements, hazard identification and agreed safe systems of work, roles and responsibilities and the coordination of any other works activities. All Contractors will provide their Risk Assessments and written safe systems of work to the Principal Contractor for approval. The Contractors operatives will be inducted prior to commencing on site. Daily meetings will be undertaken between the Principal Contractors’ and the Contractors’ representative where any Health and Safety issues will be discussed. The Principal Contractors’ supervisor will monitor the health and safety Standards of all Contractors to ensure effective control measures remain in place.

Engaging Workers:

Methods for communicating safety information on this project will be via safety meetings/ briefings, structured tool box talks, risk assessments, method statements, posters, safety signs and permits to work where applicable. Also copies of the company safety policy and manual are issued to all employees. Toolbox / site induction talks will be given to all operatives on site, at the beginning of their first day. Visitors will also be given a safety talk. The talk will highlight the dangers present, emergency procedures and requirements for entry onto the site. These talks will be issued to every new arrival to site and amended and updated as required. Swansea Council (BS) to ensure that all employees/contractors are made aware of any potential challenging situations or environments. i.e Two person visits to a property. This will be carried out at the start of each working day, through verbal communication from the site manager/site supervisor. New Apprentices/trainee/Work experience workers with be supervised throughout their learning period, until the site manager/supervisor deems that person competent/confident to carry out tasks by him/herself. Health and Safety issues will be addressed during progress meetings held regularly on site between the site supervisor and the workforce. The outcomes of which will be recorded in the minutes of the meeting.

Coordination between other Team Members:

The appointed contract administrator/lead designer shall be the focal point for communications between interested parties. All communications regarding health and safety issues shall be through the contract administrator/lead designer in the first instance. The principal contractor will implement regular liaison meetings with the management team and report on progress and review all relevant health and safety issues. Regular meetings will be held throughout the construction phase to review and determine health and safety issues at intervals agreed with the contract administrator/lead designer. Health and safety will be high on the agenda and the principal contractor will provide a report on performance noting any incidents, accidents and other relevant matters. Design development issues or changes throughout the works, including during the construction stage, shall be communicated to all duty holders as set out in this plan. Planning and other preparation for construction where relevant to health and safety will be communicated to aid the flow of health and safety information between clients, designers and contractors. All details for contacts are as supplied in this document. The site supervisor or his nominated representative will make themselves known to and liaise frequently with the premise manager to discuss and organise any work that will have an impact on the premises, with the aim of fitting in with the day to day running of the premises.

| MANAGEMENT TEAM ORGANISATION & RESPONSIBILITIES |  |
| --- | --- |
| Management Team Function The Swansea Council (BS) have a wealth of experience within The Construction and building maintenance industry covering all disciplines which coupled with the engagement of specialist sub-contractors will be deployed in order to achieve the Project goals. The team function will:- - Integrate with the Consultants and sub-contractors to achieve the best solutions for the Project. - Work as one with the Client and site Management Team to achieve the objectives and safety goals. - Develop a team philosophy with Contractors so that there is a mutual understanding as to what must be achieved to meet the project expectations. - Employ our depth of management knowledge and previous experiences of undertaking construction projects to achieve the best outcome possible. |  |
| Project Organisation and Personnel |  |
| Chandler Bing (Client Representative) Tel: 01792 000000 Mob 07000000000 Mark Robinson (Principle Designer) Tel: 01792 000000 Mob 07000000000 Ross Geller (Principle Contractor) Tel: 01792 000000 Mob 07000000000 Rachel Green (Project Manager) Tel: 01792 000000 Mob 07000000000 Joey Tribbianai (Site Manager) Tel: 01792 000000 Mob 07000000000 |  |
| Arrangements for Cooperation & Coordination |  |
| Welfare: |  |
| Welfare Facilities will be provided throughout the construction phase. A self- contained Oasis container will be provided to comply with schedule 2 of the CDM regs 2015 and HSE's guidance leaflet CIS 59. The container will provide toilet and canteen facilities and will have its own dedicated generator to provide power lighting and heating to the facility. The site manager will inspect the welfare facilities on site weekly for cleanliness. Smoking is prohibited within all areas of site. Employees, contractors and authorised visitors to site will have access to adequate toilet and washing facilities provided. A washbasin large enough to allow people to wash their faces, hands and forearms are provided with a supply of clean hot and cold running water and located close to the toilets. Soap and paper towels/dryers are provided. Showers are provided at Heol Y Gors Depot. The welfare room will be provided/premises kitchen and seating facilities located within the building and will be made available for preparing and consuming refreshments. The rest facilities will be maintained at an appropriate temperature and provided with tables and chairs (with backs), a kettle and a microwave oven. Portable cold drinking water and cups are available. |  |
| CORONAVIRUS (COVID-19) Whilst the risk of contracting Coronavirus remains low, good hygiene is of vital importance to prevent ill health in general, below you will find advice on handwashing and a link to inform staff on effective sanitisation. These are general principles you can follow to help prevent the spread of respiratory viruses such as Novel Coronavirus. These include: • Washing your hands often with soap and water for at least 20 seconds. Use an alcohol-based hand sanitiser that contains at least 60% alcohol if soap and hot water is not available • Avoid touching your faces, eyes, nose and mouth with unwashed hands • Avoid close contact with people who are unwell • If you feel unwell, stay at home and do not attend work • Use a tissue to cover your cough or sneeze then throw the tissue into a bin, and then wash your hands immediately Clean and disinfect frequently touched objects and surfaces in the working environment First Aid:. The site supervisor and the supervisory cover are the identified qualified first aiders / appointed person on site. If they are not available to provide cover to the site, then another person will be identified and their details conveyed to all site personnel. There will be an appointed first aider/appointed person on site at all times. The contact details of the appointed person will be clearly displayed on the first aid poster on display within the site welfare facility. The first aid box / equipment will be located in the site office and be accessible at all times. Suggested contents list, HSE guidance L74 First aid at work. The Health and Safety (First-Aid) Regulations 1981 Guidance on Regulations. |
| Emergency Arrangements: |  |
| The nearest emergency facilities are at Morriston hospital: Heol Maes Eglwys, Morriston, Cwmrhydyceirw, Swansea SA6 6NL Minor injuries clinics can be found at singleton hospital: Sketty Ln, Sketty, Swansea SA2 8QA An evacuation strategy has been drawn up for the site (copy on display within the site welfare facility and a hard copy appended in section 12 of the site health and safety file). All operatives, contractors and visitors are to be made aware of the evacuation measures during their initial site induction. Fire routes and exits will be maintained in a safe manner. A person will be appointed to instigate the emergency plan, the details of whom will be clearly identified on the fire marshal poster displayed within the site welfare facility. Site operatives and visitors will in the event of an emergency evacuation; assemble within the pre-determined muster point identified on the site constraints/layout drawing, where the site supervisor or nominated person will check against the site visitor register that everyone on site has been evacuated. All escape routes will be kept clear of obstructions at all times and provided with adequate signage. Signs will be clearly visible and securely fixed. EMERGENCY ARRANGEMENTS - The emergency arrangements for this site will be explained to inductees as part of the site induction, and details posted on the Site Safety Notice board. - This site will have a dedicated, first aider (site manager/site supervisor) - A site Emergency can be one or more of the following: fire, accident, bomb threat or any other incident deemed an emergency by management. - In the event of a fire emergency all operatives are required to proceed immediately to the Fire Assembly Point and remain there until they have been accounted for and given permission to leave. No one is allowed to stop to gather tools or leave the site without notice. Any visitors to site will be accompanied at all times and informed of the emergency procedure. - As fire arrangements are subject to change as construction progresses, The Site Manager will have a duty to bring this to the attention of site operatives and Contractors. - In the event of any of the emergency services being required to attend site, the Site Manager will make himself known to the emergency services on fire escape directions and details of any flammable chemicals stored on site. - Site-specific Emergency arrangements are as displayed within the site welfare cabin and as appended in section 12. Accident Reporting as required by RIDDOR 2013 - All accidents on site must be reported even if no medical attention is given; to include visitors to the site and delivery drivers etc. The Site Manager shall use the Accident Reporting Procedure accordingly, advising the City & County of Swansea Health Safety and Wellbeing department where appropriate so that we are compliant with RIDDOR 2013. The Site Manager will oversee all entries made in the Site Accident Log which will be kept in the site office. - Following an accident or dangerous occurrence a complete investigation will then be carried out by Swansea Council Health Safety and Wellbeing department which would be conducted in accordance with the procedures highlighted in RIDDOR 2013. The HSE will be informed using the appropriate form appended in section 15. |
| Fire Safety Plan: |
| Fire Plan Principles Site specific fire arrangements will be briefed to all site operatives through the initial site induction. The site shall have a designated Fire marshal (refer to information poster within site welfare facilities) Fire Prevention Procedures It is our policy to carry out works so as to eliminate risk from fire and, to this end; we have adopted the Code of Practice laid down in HSE guidance "Fire Prevention on Construction Sites". - Materials and rubbish stored on site will be kept to a minimum. - Good housekeeping. Rubbish will be cleared from the site on a regular basis and site working and access areas kept clean. All rubbish will be stored a suitable distance away from flammable structures and materials. - Where flammable material or LPG must be kept on site, they will be stored in fireproof cages/containers away from fire exit routes and potential fire hazards. - Site accommodation will be of a half-hour fire resisting construction and Class 1 spread of flame, and protection materials shall generally meet 'Class 1 spread of flame standard'. - A "No Smoking" rule will be imposed in all working areas, storage areas or wherever there are fire hazards. - Where the works involve a naked flame. In the event of a fire the person discovering the fire shall raise the alarm and immediately report the situation to the Site Manager. - A person may attack the fire with the appliances provided only if they are trained to do so and as long as there is no risk to themselves or others. - If the fire cannot be safely tackled then all persons shall immediately leave the site by the nearest egress and report to the person in charge of the assembly point. The person in charge shall make a record of all those who have vacated the site and ensure that the emergency services have been called. |
| Fire Prevention Procedures The overall responsibility for the prevention of fire rests with the Swansea Council (BS).All the Authorities sites are no-smoking sites, except for the designated area and no burning is allowed. The Design Team and Client will, where possible, specify materials of a non-flammable nature. The Swansea Council (BS) is responsible for ensuring adherence to the Project fire plan and procedure, and will: - Provide and maintain in good working order, suitable and sufficient fire fighting - Equipment and establish safe and adequate fire assembly points. - Maintain means of safe access and egress in all areas at all times. - Update the fire plans as the Project progresses and inform site operatives accordingly. - Provide secure storage for flammable/highly flammable liquid and compressed gas. - Secure all ground floor openings to buildings under construction, to prevent access to trespass or vandalism. - Render all ladders inoperable to scaffolds at ground level out of working hours. - Provide out of hours site security and lighting to the site. The Contractor/sub contractor/operative is responsible for: - Complying with all relevant statutory requirements and directives relating to fire prevention as laid down by the Swansea Council (BS). - Using, where possible, materials of a non-flammable nature for temporary works, packaging etc. - Ensuring that safe systems of work are carried out on site - Ensure all COSHH materials brought onto site, to include data sheets. Means of Escape All works must be designed and erected so that a safe route is always available to a final exit (place of safety). These routes must be clearly identified, with a maximum travel distance in accordance with the relevant standards. Fire escape routes are to be kept unobstructed and, if necessary, illuminated at all times. When work necessitates the use of naked flames, for example in voids and other similar locations having a high probability of fire, a portable fire extinguisher is to be readily available. Fire Fighting The Swansea Council (BS) will establish a site procedure for dealing with a fire emergency that takes into account the hazards involved and any special features in the area. The procedure includes the following points: Sounding a warning alarm. - Dealing with a fire. - Closing down of operations. - Evacuation of the area. - Assembly at marshalling points and carrying out of roll calls. - Locating missing persons. The Swansea Council (BS) will arrange for adequate numbers of hand extinguishers of a suitable type to be available and maintained within the permanent works as they develop until such time as they are no longer required. All firefighting equipment will be properly maintained, examined and tested in accordance with the manufacturer's instructions, and shall conform to the requisite British Standard. Fire Emergency Protocol - In the event of the discovery of any fire, the fire alarm will be activated by the discoverer of the fire. (Shouting fire, fire. Activation of alarm bell/air horn). - All personnel within audible distance of the alarm will immediately evacuate the area and report to the assembly/muster point. - The site office must be notified immediately by the quickest practical means by the discoverer of the fire. - The site manager will establish where possible:- - The exact location of the fire. - The extent and nature of the fire. - Whether there are any casualties. - The evacuation status. - The site manager will immediately:- - Activate the fire Alarm (if not already activated) - Contact the appropriate emergency services requesting assistance. Advise CLIENT. - Ensure all areas are quickly evacuated and site personnel are at assembly points. - WITHOUT TAKING ANY PERSONAL RISKS, and if reasonably safe to do so, an attempt may be made to extinguish the fire by using the nearest appropriate extinguishers. - All Contractors' Supervisors will report to the site manager at the assembly point and verify that all personnel under his control are accounted for as a result of the evacuation. - The site manager will arrange for the Emergency Services to be met at the Main Gate and guide them to the incident. - Upon arrival of the Emergency Services, the Site Manager will brief the Officer in Charge, and provide such assistance as may be requested. - Upon hearing the alarm being raised:- - All machinery, vehicles etc. will be switched off. - Any machinery blocking access routes will be moved to facilitate access by the emergency services. - All personnel will proceed to the appropriate fire assembly point where the supervisor will confirm the attendance of their operatives. - No personnel will return to work until the "all clear" has been relayed from the Site Manager/Site Fire marshal to all relevant personnel. |  |
| Display of Statutory Notices Statutory notices will be displayed on site in accordance with current legislation and include: - Health & Safety at Work law Poster (which include the Safety Representative details for The Principal Contractor) - Employers Liability Insurance - Emergency arrangements (First Aider(s), Fire Assembly Point, , Contact numbers for Hospital/Police/Fire Brigade) - Fire Evacuation Arrangements and Site Plan detailing fire points etc. - Site Registers and Documentation The following will be available on site: - Accident Log - Record of inspections required under the Lifting Operations & Lifting Equipment Regulations 1998. - Waste transfer notes. - COSHH hazard data sheets, risk assessment. - Risk assessments. - Method Statements. - Emergency procedures. - Record of toolbox talks. - Record of Site Inductions. The following will be available within the Heol Y Gors depot plant maintenance department. - Electrical equipment registers, - Maintenance checks and records. - Plant inspection records. |  |
| Site Establishment Plan: |  |
| Prior to commencement on site a site establishment plan will be forwarded to the Client which identifies, where applicable • site security arrangements. • location of site compound including offices, storage and welfare facilities. • access/egress and muster points. |  |

| Risks associated with this Project |  |  |  |  |
| --- | --- | --- | --- | --- |
| Risks |  |  | Comments |  |
| Significant Construction Risks | Hazard |  | Control | Owner |
| Existing use of site | Protection of others from site activities |  | The site is a fully functional housing development. As such care will be taken with regard to maintaining the safety of residents from all site activities and movement of site vehicles. All plant and materials will be kept within the storage containers within the site compound whilst not needed. No plant or materials will be left in areas accessible to residents. All vehicular movement around the site will observe the highway code and be restricted to 5MPH within the confines of the compound. There will be no reversing of vehicles on site or within the compound without the use of a banksman. | BS |
| Asbestos | Asbestosis |  | All BS operatives are asbestos awareness trained. Should any work be carried out outside the scope of the survey there is a potential that ACM may be encountered. If unknown materials are encountered then work is to immediately stop and be brought to the site supervisor’s attention, who will contact the Asbestos removal department for the right course of action. For any queries regarding asbestos please refer to the Asbestos removal office at Heol Y Gors depot, Cwmbwrla, Swansea Tel 01792 000000 or on the Authorities intranet website (click on Departments/corporate building services/Asbestos surveys, identify premise then click on the highlighted yes). | BS |
| Maintaining access to property during works | Protection of residents during works |  | All paths to properties will be kept clear and a safe means of access maintained. All doorways leading beneath scaffolding will be protected with boarded canopies/fans above, to prevent materials, plant or debris falling onto anyone below. | BS |
| Existing Services | Electrocution |  | On houses where scaffolding is erected, the site supervisor/responsible person will ensure that any overhead power cables are adequately sheathed prior to erection to prevent any danger of scaffold poles touching or electrical arcing occurring (refer to 5.(c) of the BS method statement for procedures on working near incoming electrical services). | BS |
| Works at height | Falls of men and materials |  | All work which includes working at height will be carried out from scaffolds designed to suit the work in hand and compliant to the NASC standard TG20.13 | BS |
| Manual Handling | Musculo Skeletal injury |  | Deliveries will be organised to eliminate or reduce manual handling where possible by the use of mechanical offloading directly to areas where they will be used. Where this is not possible then a team lifting methods will be employed. It is not anticipated that materials used on this project will exceed the 20kg weight limit for material handling. All operatives have received training in manual handling techniques. | BS |
| Dust | Respiratory problems |  | Use of dust suppression devices fitted to dust generating tools, Relevant PPE supplied and worn, adequate ventilation supplied. | BS |
| Significant Hazardous Specified Materials | Risk | Control |  |  |
| Asbestos | Asbestosis | All removal of Licensed Asbestos will be carried out by a specialist contractor. Removal and disposal of Non licensed Asbestos will be carried out by our own site operatives under controlled conditions as laid out by CAR 2012. |  |  |
| Concrete tiles | Dust from cutting tiles | Dust suppression devices fitted to cutting machinery, or wet cut where possible. Respiratory PPE worn by all within the vicinity of the work. All cutting of tiles to be carried out in the open. |  |  |

| Monitoring |  |
| --- | --- |
| The Site Manager will actively monitor health and safety performance on site. He will carry out daily inspections of the works and will highlight to the Contractor concerned, any breaches of the regulations or bad practice that may be a danger to themselves or others working in the area. Swansea council’s Health Safety and Wellbeing department will make regular visits to the site and written reports will be issued to the Site manager and all relevant duty holders (If required). All non-conformances will be followed up by a re-visit. The Client representative will make periodic safety tours and advise the Site Manager of any corrective action needed. Monitoring of site Health and Safety will also be discussed at the site Contractors Liaison Meetings where related subjects will be raised, discussed and actioned. All Contractors, Consultants and visitors have a responsibility to comply with the Site Specific rules and statutory regulations. Ignorance is no defence. |  |
| Consulting and coordinating the views of people |  |
| SITE RULES The site rules will be notified through the Construction Phase Health and Safety Plan, reinforced through site inductions and clearly displayed at all times. Site Rules can be found in Appendix 12 of this Site health and safety plan file. |  |
| Additional Information |  |
| Duty of Care - Issues to be Immediately Raised and Communicated | Where issues arise which give rise to an immediate risk to life, serious injury or other danger, that person shall immediately bring to the attention of all parties details of the incident. This includes the Project Manager, Client Representative, Contract Administrator, Principal Contractor or any other person. |

### Traffic Management Plan

KINGS ROAD CARETAKERS HOUSE TRAFFIC MANAGEMENT PLAN

| Site Address: | Kings Rd, Swansea, SA1 8AS. |
| --- | --- |

Created by: 					Mark Robinson

Date created: 				03th November 2020

Checked/verified (Site manager):		Joey Tribbiani

Checked/verified (Project manager):	Rachel Green

| Principal Contactor: | Ross Geller Building Services Swansea Council Heol Y Gors Swansea SA5 8LD |  |
| --- | --- | --- |
| Contracts Manager | Monica Geller | Tel: 07000000000 Email:monica.geller@swansea.gov.uk |
| Site Manager | Joey Tribbiani | Tel: 07000000000 Email:joey.tribbiani@swansea.gov.uk |
| Infrastructure Contractor | Construction Section Highways & Transportation Swansea Council Players Industrial Estate Clydach Swansea SA6 5BJ |  |
| Contracts Manager | Phoebe Buffay | Tel: 07000000000 Email: phoebe.buffay@swansea.gov.uk |

Description of Work

New build Single story detached domestic building with loft room.

Existing Site Conditions

Kings road joins onto Faban way (dual lane road) which is a 30mph zone entering Swansea city from the East, it is one of the two main routes leading into the city and is the location of many large office blocks and university buildings.

Kings road has traffic that leads to office blocks, housing estates, industrial buildings, university buildings and resteraunts. The single carriageway road has a pedestrian footpath both sides and a speed limit of 30mph. Behind the site is the river Tawe just before in enters Swansea docks. Adjacent to the proposed site and programmed to start at the same time is a multi-storey car park.

Work Programme

The works are programmed to commence 07th December 2020 with a completion date of 09th July 2021). A full programme is currently being developed.

Whereas the majority of works will be contained within the site boundary, the following activities have been identified as impacting on kings road.

Site access/egress.

Lifting operations.

Junction remodelling.

Installation of new service connections.

Resurfacing of any damage to footpath and carriageway.

Hours of operation are:

08.00 – 17.00 Monday to Friday

08.00 to 13.00 Saturdays.

Works may take place outside of these hours upon prior agreement with the Client.

General contacts list

Ross Geller 	(Principal Contractor)   		mob 07000000000

Rachel Green 	(Project Manager)     		mob 07000000000

Joey Tribbiani   	(Site Manager)       		mob 07000000000

Mark Robinson	(Lead Designer)      		mob 07000000000

Chandler Bing 	(Client Representative)    	mob 07000000000

Routing of site traffic

Site access/egress and containment will be set in line with The HSEs guidance document HSG151 Protecting the public ‘Your next move’ and in accordance with the requirements of Chapter 8 of the Traffic Signs Manual, and the recommendations laid down in Safety at Street works and Roadworks  - A code of Practice.

Site Compound access point

The site compound will be accessed from kings road, Deliveries will be required to use a one way system in order to manage the traffic (see traffic management drawing). The compound site entrance will be checked for all the visibility requirements prior to the site set up works. Space on the site is limited so deliveries will be staggered and timetabled in order to maintain flow of traffic. The gates to the construction site will be monitored constantly and controlled by use of a banksman.

This access will be used for all daily traffic (contractors, subcontractors, operatives, deliveries and visitors) to site. Contractor vans will need to be parked in Swansea university carpark nearby when not dropping on materials. All traffic to site will have to sign in and out of the site upon arrival and departure.

Space for turning inside the compound will be limited, HGV’s will be required to supply their deliveries from the curb. At this time a banksman will be on hand to liaise with the driver. Vans and smaller vehicles will be able to access the site compound where there will be a turning area to the rear for vehicles to safely turn around prior to exiting the compound. Directional/warning signs will be displayed informing vehicles of requirements for entering and leaving the compound site entrance. There will be a lead in point to the compound to enable deliveries to pull into an area off the main road, whilst waiting to gain entry into the compound area.

Contractor, Sub-contractor and visitor parking has been negotiated with the university to allow parking in their main car park to the south, drop offs & deliveries will be allowed at the site but long term parking will be at the university car park. There is also an area dedicated to the site waste management facility and to material storage. The site plant and the machinery used by the ground works contractor must comply with the traffic plan. However, any operations affecting the pre-planned traffic management should be considered by the Site manager and ground works contractor.

Deliveries will be timetabled to avoid multiple deliveries at the same time and not during peak traffic times.

Traffic routes/pedestrian segregation within the compound and construction site

Traffic movement and pedestrian segregation will be as dictated on the site traffic management plan drawing. This will be updated as circumstances dictate on site. During the site set up and ground clearance there will be between 5 to 10 vehicle movements each day As the construction work increases then the volume of traffic will increase to 5/10 vehicles, all new traffic movement will be monitored on a daily basis.

During the initial site groundworks and setting out removal of spoil from site will be carried out thus. All site excavation plant will be kept within the construction site confines and will not access the main road unless wheels have been cleaned beforehand. Lorries used for the removal of all spoil will enter the site compound to be loaded up by the excavation machinery, they will be required to revese into the compound guided by a banksman. The traffic route within the compound will be temporarily stoned up to ensure that the vehicles entering and leaving the compound will remain clean and prevent contamination of the highway with mud and clay.

There will be a designated wheel washing area set up (as identified on the site traffic management drawing) and each lorry before leaving the site will stop and wash off any excess mud/slit/dirt, with a jet power washing system. A road sweeper will also be used each day to maintain that all main roads around the construction site will remain clean from mud and silt.

Once all initial setting out and groundworks are complete. The roadway within the construction site will be temporarily stoned up to provide a through route through the compound and site this will avoid where practical the need for unnecessary manoeuvring of vehicles and the retain segregation of pedestrians and vehicles.  Refer to site traffic management plan drawing.

This plan will be updated as the project develops and site circumstances change.

Offsite Chapter 8 arrangements

Temporary information signs indicating site access and egress points will be erected on semi-permanent poles outside the site boundary to inform the general public of vehicle movement activities in these areas.

Semi-permanent pedestrian crossovers have been constructed outside the scope of any highway works to accommodate pedestrian diversions when footpath adjacent to the site requires closure. These crossovers will be suitable for wheelchair/mobility scooter access.

Any activity impacting on kings road will be carried out in accordance with the requirements of Chapter 8 of the Traffic Signs Manual, and the recommendations laid down in Safety at Street works and Roadworks - A code of Practice. This will include

Pedestrian diversions during periods of footpath closure.

Passive give and take arrangements when conditions allow.

Stop go boards for short duration works.

Temporary Traffic Lights for all other works requiring lane closures

A risk assessment will be carried out to determine the appropriate control measures prior to carrying out the works.

Details of the chapter 8 arrangements will be forwarded to the Streetworks Management Officer for inclusion onto the Swansea Roadwatch report. These arrangements shall set out the type of works being undertaken, the chapter 8 requirements and duration of works. Minor works shall be notified at least 3 days prior to commencement and standard works 7 days.

Maintenance of Public Highway

The public highway shall be maintained in a clean state at all times. A road sweeper will be used each day to maintain that all roads around the construction site will remain clean from mud and silt. In periods of dry weather a water bowser will be utilised to supress the dust from the site. The site supervisor will inspect the highways daily to make sure that there is no debris on the roads and pathways. All on site traffic shall stick to the defined stoned up traffic routes so as to prevent mud and detritus from being carried onto the public highway.

All temporary highway construction works signage shall be checked on a daily basis by the site supervisor. An out of hour’s telephone number shall be displayed at the site entrance for public use.

Utility Providers

The designs for service provision (Gas, Water, Electric, BT etc) have been agreed with relevant Utility Providers. A programme for on-site installation will be agreed. Services which cross the highway will run through ducts to be installed prior to commencement on site. Joint pits shall be opened up the day before connection dates so as to minimise disruption to the highway/pedestrians.

This traffic management plan will be reviewed and amended during the construction programme to ensure a safe traffic management system is in place at all times.

It is important to review the stages of site development and property occupation in a methodical sequence to incorporate a suitable and effective system. Regular coordination between the site team during construction should take place to provide detail information to occupiers and the site visitors.

The below information is provided in support of discharging Pre-commencement planning condition point 3.

1. The parking of vehicles of site operatives and visitors.

All site operative and visitor parking will be within the allocated parking spaces within Swansea university’s car park. Please refer to the attached plan for exact locations.

2. Loading and unloading of plant and materials.

Access into the site will be from Kings Road via the one-way system. All deliveries of plant, materials or other items will be co-ordinated and managed beforehand and under the control of a competent Building Services Banksman. Vehicles will access the fenced off area where a dedicated laying down zone will be established.

3. Storage of plant and materials used in constructing the development.

All plant and materials will be stored within the fenced off and secured site storage compound. Any specific higher risk materials will be stored within lockable containers. No plant or materials will be stored on the public highway.

4. The erection and maintenance of security hoarding including decorative displays and facilities for public viewing where appropriate.

Temporary “Herras” type fencing will be used to establish the site, replaced by solid wooden hoarding as the site set-up proceeds. All decorative displays and public information or viewing areas will be agreed with the Project Manager and Principal Contractor, taking into consideration the locality of external public highway.

5. Wheel washing facilities.

An on-site wheel washing area will be established within the site compound area as identified in the site set-up procedures. All run-off will not be allowed to discharge onto any public highway or water course, by the use of bunding and localised drainage with relevant interceptor tanks where required.

6. Measures to control the emission of dust and dirt during demolition and construction.

Damping down by use of localised water suppression will be utilised where there is a risk of airborne dust being generated as part of the works. A bowser and sprinkler system will be used to manage any haul routes wot control the risk of emissions during vehicular movement on site.

7. A scheme for recycling/disposing of waste resulting from demolition and construction works

A dedicated on-site waste management and storage station will be set-up as per the attached plan. All waste material generated will be sorted and segregated at site, where relevant, and all removal will be undertaken by licensed waste carriers where required.

All of the above, will be actively reviewed by the relevant competent person at site updated, amended and changed as the project evolves.

### Sequence of work

Sequence of the work:

Sequence of works to be carried out in conjunction with works programme and scope of works

1. Erection of secure works boundary fence.

2. Site strip and compound groundworks.

3.  Set up compound and site storage/welfare facility.

4. Site infrastructure and ground works (working to programme of works)

5. Service connections into site (service utilities providers)

6. Insulated foundations/bases (working to programme of works)

7. Erect scaffolding around building (working to an agreed programme of works)

8. blockwork walls constructed (working to an agreed programme of works)

9. Carry out roof works as identified in schedule of work (working to an agreed programme of works)

10. External finishes

11. Internal services and finishes

12. Clear away site and remove all temporary welfare and storage facilities, upon completion of contract.

13. External landscaping of property

14. Handover property.

### Method Statement

Method Statement

Caretakers House 20/21

Site address:

Kings Rd,

Swansea,

SA1 8AS

Date of works:

7th December 2020 - 9th July 2021

Description of works

New build Single story detached domestic building with loft room.

The proposed foundations are concrete strip to a depth of 1m with standard cavity wall construction; block and block with partial fill cavity insulation and a smooth render externally. Plinths, soldier courses over all openings and porch pillars are to be natural sandstone.

Timing

The works are programmed to start on  9th December 2020

Location

The project involves construction work, notifiable to the Health & Safety
Executive under Regulation 2, on Kings Road, in the SA1 district of Swansea.

Enabling works shall consist of (not limited to) as outlined within the Architectural Specification:

Access

Construction of access routes

All works will be in accordance with reports commissioned to date.

Duration

The works are scheduled to take place over a 34 week programme (based on information available at the time prior to site commencement).

Details of waste control and removal

Please refer to Site Waste Management Plan appended to the Construction Phase Health and Safety Plan.

| Project Manager | Rachel Green Swansea Council, Building Services, Heol Y Gors Depot, Cwmbwrla, Swansea, SA5 8LD | Tel: 07000000000 Mob: 07000000000 Email: rachel.green@swansea.gov.uk |
| --- | --- | --- |
| Site Manager | Joey Tribbiani Swansea Council Building Services Heol Y Gors Swansea SA5 8LD | Tel: 07000000000 Mob: 07000000000 Email: joey.tribianai@swansea.gov.uk |
| Site Foreman | Jules Holland Swansea Council Building Services Heol Y Gors Swansea SA5 8LD | Tel: 07000000000 Mob: 07000000000 Email: jules.holland@swansea.gov.uk |

Supervision

Working at Heights

The site supervisor will prepare a safety method statement and carry out a risk assessment before work at height begins. The site supervisor will be responsible for the supervision of all work at height on site and will be responsible for checking that risks associated with work at height are adequately controlled.

All BS employees on site have attended mandatory half day ‘Work at Height’ training. All apprentices will be closely supervised by their mentor/site supervisor.

Inspection of work equipment

Ladders and stepladders will be inspected prior to each use to identify any obvious visual defects and to ensure the ladder is safe to use. Where faults are identified the equipment will be taken out of use and removed form site for repair or safe disposal. The supervisor will inspect the working platforms/mobile working platforms work equipment every 7 days and record and keep his findings. The supervisor will also inspect working platforms/mobile working platforms work equipment each time that the equipment has been exposed to conditions causing deterioration likely to result in dangerous situations which are liable to jeopardise the safety of the work equipment, to detect any deterioration.

Independent scaffolding

Independent scaffolding will be provided for general access and will be designed to suit the work in hand. The system will be subject to inspections using a 'scaftag' management system that will be used by the scaffolding supplier/contractor for the first week and handed over to the site supervisor for further 7 day inspections. The Authority expects all scaffolding contractors to work to standards for scaffolding erection defined in the NASC guidance note.

The Scafftag system is used to monitor and control the use of independent scaffolding, by means of pre stated aims and rules.

These rules are:-

- When in use the scaffold must be inspected at least once every seven calendar days, by a competent and qualified person

2. 	Every time the scaffold is inspected the Scafftag must be signed in indelible ink

3. 	The Scafftag must be placed in its holder which is attached to the scaffolding at its point 	of 	access prior to anyone using the scaffold.

4. 	When the scaffold is not in use the Scafftag must be removed from its holder and kept in a safe 	place on site.

5. 	No one is to use the scaffold without the Scafftag being in place, or if it has not been signed within 	the last seven days.

6. 	The scaffold must only be altered or amended by a qualified scaffolder. Each time this happens the 	scaffold must be checked and the Scafftag signed to confirm it is safe for use.

7. 	Where required by current legislation roof edge protection will be installed as part of any 	scaffold system erected to carry out works to the property.

Protection from falls

Scaffolding

Where scaffolding works will be required as part of the project then the following operational facilities must be in place with regard to the use of scaffold :-

- All ladders to be suitable for purpose and fixed, enabling side on 	access and exit off 	scaffold.

- A Sprung access gate will be present at each working platform level, on all scaffolds with external ladders.

- Any fixed ladders to be rendered inoperable when scaffold not in use. Base plates to be banded and level (if site allows).

- Scafftag system to continue. Weekly scaffold inspection forms to be completed and a record kept on site, same form to be included in this CDM site plan.

- Air bags will be used within the buildings in areas where operatives are working above and at risk from falls at height.

Falling materials

Working beneath scaffolds will not be permitted when there is work being carried out overhead.

Scaffold platforms are to be kept clean at all times, the storage of tools, materials will not be permitted on scaffolds, and all debris is to be removed as it arises.

Mandatory PPE

Gloves

Gloves must be worn where operatives are likely to come into contact or work with materials or substances injurious to their health.

Safety footwear

Safety footwear must be worn by everyone who enters on site, at all time, and without exception

Safety headgear

The authority states that safety helmets must be worn by everyone, who enters on site at all times and without exception.

Ear defenders

Ear defenders must be worn at all times when carrying out works that generate noise i.e. impact drilling into masonry, cutting chases by mechanical means etc. Especially when working in close proximity or in enclosed or restricted spaces.

Safety Goggles

Safety goggles must be worn at all times when carrying out works where there is a possibility of dust or debris occurring, to protect the eyes i.e. drilling, grinding etc.

Face masks

Where necessary a face masks will be supplied to the operatives. The face mask will be of a type suitable to the work in hand. (Site supervisor is to identify the type of mask required from an identification chart in the stores at Heol Y Gors depot).

Sun Protection

Care should be taken to protect the skin from the effects of the sun when working outdoors. Sunblock can be obtained from the Stores at Heol y Gors.

High Visibility Vest

High visibility vest will be worn by all operatives and site visitors when loading and unloading vehicles off kerbside and in areas where moving vehicles and machinery are in operation.

CORONAVIRUS (COVID-19)

Whilst the risk of contracting Coronavirus remains low, good hygiene is of vital importance to prevent ill health in general, below you will find advice on handwashing and a link to inform staff on effective sanitisation.

These are general principles you can follow to help prevent the spread of respiratory viruses such as Novel Coronavirus. These include:

- Washing your hands often with soap and water for at least 20 seconds. Use an alcohol-based hand sanitiser that contains at least 60% alcohol if soap and hot water is not  available

- Avoid touching your faces, eyes, nose and mouth with unwashed hands

- Avoid close contact with people who are unwell

- If you feel unwell, stay at home and do not attend work

- Use a tissue to cover your cough or sneeze then throw the tissue into a bin, and then wash your hands immediately

- Clean and disinfect frequently touched objects and surfaces in the working environment.

### 4.5	Risk Assessments

| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Working near river bank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | x | Employees |  | x | Contractor |  |  |  | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Risk of falling into water |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 2 | Bank Collapse |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 12 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Ensure heras fencing is put up 1m from bank edge and fix securely in order to protect it from falling into river and causing an environmental hazard. If work is required beyond fencing then harnesses will be required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Signage put up to warn workers of risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Tool box talks can highlight the hazard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Ensure machinery is not used close to bank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Do not store materials or add any additional weight near the bank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Risk of falling into water |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 4 |  |  |  |
| 2 | Bank Collapse |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 4 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: Mark Robinson Position: Principle Designer |  |  |  |  |  |  | Signed: Mark Robinson |  |  |  |  |  |  |  |  |  | Date of Completion: 30/10/2020 |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |
| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| Public Footpath Safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 02 |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Employees |  |  | Contractor |  |  | x | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Risk of traffic incident |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 5 |  |  |  |  | 10 |  |  |  |
| 2 | Risk of slipping on material from site |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 3 |  |  |  |  | 9 |  |  |  |
| 3 | Risk of dust inhalation from site works |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 4 | Risk of excessive noise for pedestrians |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Banksman controls all vehicles when they arrive on site & delivery times for HGV’s are to be outside of ‘rush hour’ times |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Vehicles cleaned before existing site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | State of paths to be monitored and cleaned as required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Water used to reduce dust created |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Sound reducing fencing to be used on walkway |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Alternative materials and/or materials cut off site to reduce dust & noise levels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Risk of traffic incident |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 5 |  |  |  |  | 5 |  |  |  |
| 2 | Risk of slipping on material from site |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 3 | Risk of dust inhalation from site works |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 4 | Risk of excessive noise for pedestrians |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: Mark Robinson Position: Principle Designer |  |  |  |  |  |  | Signed: Mark Robinson |  |  |  |  |  |  |  |  |  | Date of Completion: 30/10/2020 |  |  |  |  |  |  |  |  |
| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| Cutting particle boards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 03 |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | x | Employees |  | x | Contractor |  |  |  | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Dust Inhalation |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  | 16 |  |  |  |
| 2 | Noise outside guidelines |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  | 16 |  |  |  |
| 3 | HAV outside guidelines |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  | 16 |  |  |  |
| 4 | Eye Irritation |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 3 |  |  |  |  | 12 |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Wearing of correct PPE: Face fit respirators, Ear protection & Eye protection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Using tools that have a low vibration & noise value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Toolbox talks to highlight hazards and teach about using tools in a way to reduce risks such as reduced vibration and importance of keeping warm to aid good circulation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Rotate Gangs or teams of workers to they are not exposed to the same risk for an extended period of time. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Use pre-cut boards where possible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Dust Inhalation |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 4 |  |  |  |
| 2 | Noise outside guidelines |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 4 |  |  |  |
| 3 | HAV outside guidelines |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 4 | Eye Irritation |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 4 |  |  |  |
| Name: Mark Robinson Position: Principle Designer |  |  |  |  |  |  | Signed: Mark Robinson |  |  |  |  |  |  |  |  |  | Date of Completion: 30/10/2020 |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |

### Conclusion

  - History of CDM (2007 & 2015)

The regulations set out in 2007 and then revised and updated in 2015 set out clear responsibilities for each person involved in a project in term of health and safety risk, its management and its risk mitigation. It gives clarity for all parties at times when the burden for health and safety management is not clear.

(HSE, 2015)

The aim for the CDM regulations are to:

- Sensibly plan the work so the risks involved are managed from start to finish.

- Have the right people for the right job at the right time.

- Cooperate and coordinate your work with others.

- Have the right information about the risks and how they are being managed.

- Communicate this information effectively to those who need to know.

- Consult and engage with workers about the risks and how they are being managed.

(HSE, 2015)

  - Roles Within CDM

Client (Commercial)

Commercial clients carry out works as part of a business, they influence the running of the project including its health and safety risks and determine key factors such as time and money. (HSE, 2015)

The role of the client under CDM is to:

- Make suitable arrangements for managing their project, enabling those carrying it out to manage health and safety risks in a proportionate way.

These arrangements  include:

  - appointing the contractors and designers to the project (including the principal designer and principal contractor on projects involving more than one contractor) while making sure they have the skills, knowledge, experience and organisational capability

  - allowing sufficient time and resources for each stage of the project

  - making sure that any principal designer and principal contractor appointed carry out their duties in managing the project

  - making sure suitable welfare facilities are provided for the duration of the construction work

- Maintain and review the management arrangements for the duration of the project.

- Provide pre-construction information to every designer and contractor either bidding for the work or already appointed to the project.

- Ensure that the principal contractor or contractor (for single contractor projects) prepares a construction phase plan before that phase begins.

- Ensure that the principal designer prepares a health and safety file for the project and that it is revised as necessary and made available to anyone who needs it for subsequent work at the site

For notifiable projects (where planned construction work will last longer than 30 working days and 	involves more than 20 workers at any one time; or where the work exceeds 500 individual worker 	days), commercial clients must:

- Notify HSE in writing with details of the project

- Ensure a copy of the notification is displayed in the construction site office

(HSE, 2015)

Designer

The designers construction background can be based in a number of roles, the designer can be an architect, quantity surveyor, engineer along with a number of other roles, They can be the person performing the role or organising for it to be completed, they can even be principle contractors or tradespeople in certain circumstances. This role will ether include performing the design duties or arranging for them to be done, these duties include drawings, specification, design details and bills of quantity. (HSE, 2015)

The role of the designer under CDM is to:

- Make sure the client is aware of the client duties under CDM 2015 before starting any design work when preparing or modifying designs.

- Take account of any pre-construction information provided by the client (and principal designer, if one is involved).

- Eliminate foreseeable health and safety risks to anyone affected by the project (if possible).

- Take steps to reduce or control any risks that cannot be eliminated.

- Provide design information to:

- The principal designer (if involved), for inclusion in the pre-construction information and the health and safety file.

- The client and principal contractor (or the contractor for single contractor projects) to help them comply with their duties, such as ensuring a construction phase plan is prepared.

- Communicate, cooperate and coordinate with:

- Any other designers (including the principal designer) so that all designs are compatible and ensure health and safety, both during the project and beyond.

- All contractors (including the principal contractor), to take account of their knowledge and experience of building designs.

(HSE, 2015)

Principle Designer

In projects that include more than one contractor the client will appoint a principle designer to oversee the project. In addition to the designers standard duties the principle designer must coordinate the contractors & also take responsibility for any health and safety issues and insure they are properly considered during design & development. (HSE, 2015)

The role of the principle designer under CDM is to:

- Plan, manage, monitor and coordinate health and safety in the pre-construction phase. In doing so they must take account of relevant information (such as an existing health and safety file) that might affect design work carried out both before and after the construction phase has started.

- Help and advise the client in bringing together pre-construction information, and provide the information designers and contractors need to carry out their duties.

- Work with any other designers on the project to eliminate foreseeable health and safety risks to anyone affected by the work and, where that is not possible, take steps to reduce or control those risks.

- Ensure that everyone involved in the pre-construction phase communicates and cooperates, coordinating their work wherever required.

- Liaise with the principal contractor, keeping them informed of any risks that need to be controlled during the construction phase.

(HSE, 2015)

Contractor

The contractor will employ, coordinate and manage construction workers, this may also include a list of subcontractors that will do the work on behalf of the contractor. They must be capable of managing the workforce and organising the programme of works for the project, this includes having the capacity to work on a project of the required scale. (HSE, 2015)

The role of the contractor under CDM is to:

- Make sure the client is aware of the client duties under CDM 2015 before any work starts.

- Plan, manage and monitor all work carried out by themselves and their workers, taking into account the risks to anyone who might be affected by it (including members of the public) and the measures needed to protect them.

- Check that all workers they employ or appoint have the skills, knowledge, training and experience to carry out the work, or are in the process of obtaining them.

- Make sure that all workers under their control have a suitable, site-specific induction, unless this has already been provided by the principal contractor.

- Provide appropriate  supervision, information and instructions to workers under their control.

- Ensure they do not start work on site unless reasonable steps have been taken to prevent unauthorised access.

- Ensure suitable welfare facilities are provided from the start for workers under their control, and maintain them throughout the work.

- In addition to the above responsibilities, contractors working on projects involving more than one contractor must;

- Coordinate their work with the work of others in the project team.

- Comply with directions given by the principal designer or principal contractor.

- Comply with parts of the construction phase plan (PDF)- Portable Document Format relevant to their work.

(HSE, 2015)

Principle Contractor

A principle contractor will be appointed on schemes where more than one contractor are required, they will liaise with other contractors the organise work and will take responsibility for health and safety risks during construction. (HSE, 2015)

The role of the principle contractor under CDM is to:

- Plan, manage, monitor and coordinate the entire construction phase.

- Take account of the health and safety risks to everyone affected by the work (including members of the public), in planning and managing the measures needed to control them.

- Liaise with the client and principal designer for the duration of the project to ensure that all risks are effectively managed.

- Prepare a written construction phase plan (PDF)- Portable Document Format before the construction phase begins, implement, and then regularly review and revise it to make sure it remains fit for purpose.

- Have ongoing arrangements in place for managing health and safety throughout the construction phase.

- Consult and engage with workers about their health, safety and welfare.

- Ensure suitable welfare facilities are provided from the start and maintained throughout the construction phase.

- Check that  anyone they appoint has the skills, knowledge, experience and, where relevant, the organisational capability to carry out their work safely and without risk to health.

- Ensure all workers have site-specific inductions, and any further information and training they need.

- Take steps to prevent unauthorised access to the site.

- Liaise with the principal designer to share any information relevant to the planning, management, monitoring and coordination of the pre-construction phase.

(HSE, 2015)

Worker

The term ‘worker’ on a construction site covers a vast array of different specialised individuals it covers bricklayers, plasterers, plumbers and their supervisors and will make up the majority of the personnel involved within a project. (HSE, 2015)

The role of the worker under CDM is to:

- Only carry out construction work if they have the relevant skills, knowledge, training and experience - or they are provided with the training and supervision that enables them to do it safely and without risk to health.

- Make themselves aware of the health and safety risks involved in work on every site and the way those risks are managed.

- Always follow site rules and procedures.

- Cooperate with other dutyholders, such as the contractor in control of their work and the principal contractor (who controls the overall project when there is more than one contractor) .

- Report any risks they find to whoever controls the work on site, whether the risks affect their own health and safety or anyone else, including other workers and members of the public.

(HSE, 2015)

5.3	Acts & Regulations

Health and Safety at Work Act 1974

This is the primary most largely known and used piece of legislation; it sets out the expected health and safety duties and responsibilities for all persons.

Specifically it details:

- Employers have towards employees and members of the public.

- Employees have to themselves and to each other.

- Certain self-employed have towards themselves and others.

(HSE, 1974)

Work at Height Regulations 2005

The regulations set forth within this document aim to prevent injury and death caused by falling from height. It sets forth the responsibilities for the party’s involved and what procedures and plans should be in place to mitigate any risk.

(HSE, 2005)

Personal Protective Equipment (Enforcement) Regulations 2018

This regulation is revised regularly  in order to keep up with industry and cultural changes, the most recent update was in order to take into account the SARS CoV2 Pandemic. The document sets forth what protective equipment should be worn for the industry and who is responsible for its purchase and its maintenance.

(HSE, 2018)

### 6.0	Bibliography

- CDM 2015October 30, 2020-last update. Available: https://www.designingbuildings.co.uk/wiki/CDM_2015 [November 05, 2020].

- Commercial clients: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/commercial-clients.htm [November 07, 2020].

- The Construction (Design and Management) Regulations 2015. Available: https://www.hse.gov.uk/construction/cdm/2015/index.htm [November 05, 2020].

- Construction workers: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/workers.htm [November 02, 2020].

- Contractors: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/contractors.htm [November 07, 2020].

- Designers: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/designers.htm [November 07, 2020].

- Health and Safety at Work etc Act 1974. Available: https://www.hse.gov.uk/legislation/hswa.htm [November 05, 2020].

- Principal contractors: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/principal-contractors.htm [November 07, 2020].

- Principal designers: roles and responsibilities. Available: https://www.hse.gov.uk/construction/cdm/2015/principal-designers.htm [November 07, 2020].

- Risk at Work - Personal protective equipment (PPE). Available: https://www.hse.gov.uk/toolbox/ppe.htm [November 05, 2020].

- Work at Height Key messages. Available: https://www.hse.gov.uk/work-at-height/key-messages.htm [November 05, 2020].