CONTROL VOLUMES

STEADY FLOW SYSTEMS

SOLUTION TO EXERCISE 1:

q = 25 kJ/kg

P1 = 500kN/m2

c1 = 250 m/s

u1 = 2000 kJ/kg

v1 = 0.4 m3/kg

Control volume

m=3 kg/s

P2 = 100kN/m2

c2 = 125 m/s

u2 = 1250 kJ/kg

v2 = 1.3 m3/kg

The steady-flow energy equation for this system is:

u1+P1v1+c122+q=u2+P2v2+c222+w

From this:

w=u1-u2+P1v1-P2v2+c122-c222+q

w=2000-1250×103+500×0.4-100×1.3×103+25022-12522-25×103

w=818.4 kJ/kg

This is positive and hence is work done by the system.

The substance flows at the rate of 3 kg/s

power output from the system = 818.4 kJ/kg  3 kg/s

= 2455.3 kJ/s

= 2455.3 kW      (since 1 J/s = 1 W)

SOLUTION TO EXERCISE 2:

c1 = 80 m/s

h1 = 2900 kJ/kg

v1 = 0.9 m3/kg

Wout

q = 40 kJ/kg

h2 = 2000 kJ/kg

v2 = 1.5 m3/kg

A2 = 0.025 m2

Turbine

m=2 kg/s

- At inlet:

m=A1c1v1

A1=m v1c1

A1=2×0.980=0.0225 m2

- At exit:

m=A2c2v2

c2=m v2A2

c2=2×1.50.025=120 m/s

- The steady-flow energy equation for this system is

h1+c122+q=h2+c222+w

w=h1-h2+c122-c222+q

w=2900-2000×103+8022-12022-40×103

w=856 kJ/kg

Therefore the power developed = 856 kJ/kg  2 kg/s

= 1712 kJ/s

= 1712 kW

SOLUTION TO EXERCISE 3:

60°C

h1=251.18kJkg

m1

Mixing

chamber

45°C

h2=42.022kJkg

m2

h3=188.44kJkg

m3

10°C

The mass and energy balances for this steady-flow system can be expressed in the rate form as follows:

- Mass balance:

inm=outm

∴     m1+m2=m3

- Energy balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

∴    0=outm.h-inm.h               since  Q≅0, W≅0, ke≅pe≅0

∴    0=m3h3-m1h1+m2h2

∴    m1h1+m2h2=m3h3

∴    m1h1+m2h2=m1+m2h3        since   m1+m2=m3

Dividing this equation by m2 yields

m1m2h1+m2m2h2=m1+m2m2h3

∴      m1m2h1+h2=m1m2+1h3

∴      m1m2h1-h3=h3-h2

∴      m1m2=h3-h2h1-h3

∴      m1m2=188.44-42.022251.18-188.44=146.41862.74

∴      m1m2=2.33

Note that the mass flow rate of the hot water must 2.33 times the mass flow rate of the cold water for the mixture to leave at 45°C.