EXAMINATIONS 2013/2014		Semester 2

AHOLIADAU  2013/2014

FACULTY OF: APPLIED DESIGN AND ENGINEERING SCHOOL OF APPLIED COMPUTING

| NAME OF COURSE/PROGRAMME: | BSc (Hons) Computer Networks HND Computer Networks |
| --- | --- |
| YEAR OR LEVEL: | 4 |
| NAME OF MODULE/UNIT/EXAM PAPER: | IP Internetworking |
| MODULE NO: | M1X8609 M5X8914 |
| DATE OF EXAMINATION: |  |
| EXAMINATION TIMES: |  |
| START TIME: |  |
| FINISH TIME: |  |
| TIME ALLOWED: 2 hours |  |

INSTRUCTIONS TO CANDIDATES:

Answer three out of the four questions. Each question carries equal marks.

Candidates are not permitted to bring calculators, dictionaries; text books/case studies or any other reference materials into the exam room.

QUESTION  ONE

- What is the purpose of dynamic routing protocols?					   (5 marks)

- Compare RIPv1 and RIPv2 							   	   (10 marks)

- Compare classful and classless routing protocols 					   (10 marks)

QUESTION TWO

- Design the IP addressing scheme for the network shown in figure 1. The network address is 172.16.1.0/24. Write subnet address, subnet mask, first IP address, last IP address and broadcast address for each subnet. 				   			    (25 marks)

Figure 1

QUESTION THREE

(a) What is the solution implemented in OSPF to LSA flooding issue?  	     	(9 marks)

- In OSPF protocol, what are the criteria for getting elected as DR/BDR?		(6 marks)

- Explain OSPF packet types. 								(10 marks)

.

QUESTION FOUR

- EIGRP uses five parameters to calculate metric.

  - List them all.

  - Write down the default values for all the parameters based on a Serial link.

  - Of the five which parameters are used as default? 				   (7 marks)

- In reference with EIGRP, explain the following

  - Successor

  - Feasible successor

  - Feasible distance

  - Reported distance 							    	   (10 marks)

- Refer to the figure 3, router R1 is not able to reach 172.16.1.0, 172.16.5.0, and 172.16.4.0 networks. Suggest a solution to solve the connectivity issue and write the steps.	      (8 marks)

Figure 2