M4X01974 TECHNOLOGY AND ENVIRONMENT 1

EXAMINATION PAPER - MAY 2020

ATTEMPT ALL QUESTIONS – 20 MARKS FOR EACH QUESTION

Note: Equipment required for the examination to include – pen, pencil, ruler and a calculator.

Question 1 SUSTAINABILITY, ENERGY AND ENVIRONMENT

- Sustainability is often referred to as comprising the three pillars social, economic and environment. What is your understanding of the term ‘sustainability’ in terms of architecture? (8 Marks available)

- ENERGY AND ENVIRONMENT – Name three advantages and three disadvantages of solar energy technologies? (6 Marks available)

- Name three advantages and three disadvantages of nuclear power? (6 Marks available)

Question 2 WALL CONSTRUCTIONS

- Describe the design performance factors with reference to common methods of constructing external walls in domestic construction in the UK?

(8 Marks)

- Describe ways of improving the thermal performance of wall constructions and identify potential areas of concern for the designer.

- Marks)

- What are the advantages/disadvantages of using timber in external wall construction?

(6 Marks)

Question 3  WINDOWS

- Draw/ sketch at a scale of 1:5 sections through the head, jambs and sill of a casement timber window showing how it is constructed and assembled within a masonry wall which has a 100mm brick outer leaf, 100mm cavity and a 100mm block inner leaf.

(10 Marks)

- What is meant by the following terms? Illustrate your answers where applicable with sketches. Mullion; transom; casement; opening light; metal lintel, cavity closer; sill; sub-sill.

(10 Marks)

Question 4 STAIRS

- Stairs make it possible to pass from one level to another safely. Stairs, ladders and ramps shall offer safety to users moving between levels of the building. Sketch and describe 3 types of stairs and their key features.

(10 Marks)

- There are several types of stairs and the simplest is a straight flight consisting of a straight run of parallel steps. A new house due for construction has a ground floor to first floor level overall height of 2600mm. Provide sketch drawings in plan and section to describe the key requirements for the stairs to ensure compliance with Part K of the building regulations.

(10 Marks)

Question 5 ROOF

Sketch/ draw a section drawing through a pitched roof for a domestic dwelling to explain the differences between roof structures showing the component parts of each and describe their function/ merits.

- A cut roof:

(4 Marks)

- A trussed Rafter Roof:

(4 Marks)

- What are the advantages and disadvantages of using these forms of roof construction?

(8 Marks)

- What design performance factors/functions should be considered in designing a roof?

(4 Marks)

Diwedd y Papur/ End of Paper