Design an N=7 finite impulse response (FIR) filter (use the frequency transformation method) that is band pass between  Hz and  Hz and has a sampling frequency of  Hz.  Use a Hamming window (equation 1) in your design.

Equation 1

Step 1:

Draw a sketch of the analogue filter response

Magnitude

1

Frequency (Hz)

4500Hz

2500Hz

Step 2:

Convert any frequencies to normalised frequency

υl=flfs=250010000=0.25

υu=fufs=450010000=0.45

Step 3:

Redraw the sketch using normalised frequencies

Magnitude

1

Normalised Frequency

-0.45

-0.5

0.5

-0.25

0

0.45

0.25

Step 4:

Refer to Frequency Transformation Method Tables and insert values for filter design

l

0

1

u

-u

-0

-l

Calculate the Low Pass Prototype response:

Calculate the centre frequency:

Calculate Alpha

Step 5:

Using data from step 4 draw a sketch of the prototype low pass filter

Magnitude

1

B=0.2

Normalised Frequency

0.1

-0.1

-0.5

0.5

0

Determine the filters total bandwidth B from the rectangle width

Step 6

Calculate the set of filter values for the low pass prototype

Substitute calculated values:

| k |  |
| --- | --- |
| 0 | 0.1009 |
| 1 | 0.1514 |
| 2 | 0.1871 |
| 3 | 0.2 |
| 4 | 0.1871 |
| 5 | 0.1514 |
| 6 | 0.1009 |

Step 7

Calculate the transformation equation values and apply to the low pass prototype

| k |  |  |  |
| --- | --- | --- | --- |
| 0 | 0.1009 | 1.9021 | 0.1919 |
| 1 | 0.1514 | -0.6180 | -0.0935 |
| 2 | 0.1871 | -1.1756 | -0.2199 |
| 3 | 0.2 | 2.0000 | 0.4000 |
| 4 | 0.1871 | -1.1756 | -0.2199 |
| 5 | 0.1514 | -0.6180 | -0.0935 |
| 6 | 0.1009 | 1.9021 | 0.1919 |

Step 8

Calculate the window coefficients and apply to the filter by multiplying the window coefficient column with the filter column.

| k |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.1009 | 1.9021 | 0.1919 | 0.1256 | 0.0241 |
| 1 | 0.1514 | -0.6180 | -0.0935 | 0.4376 | -0.0409 |
| 2 | 0.1871 | -1.1756 | -0.2199 | 0.8268 | -0.1819 |
| 3 | 0.2 | 2.0000 | 0.4000 | 1.0000 | 0.4000 |
| 4 | 0.1871 | -1.1756 | -0.2199 | 0.8268 | -0.1819 |
| 5 | 0.1514 | -0.6180 | -0.0935 | 0.4376 | -0.0409 |
| 6 | 0.1009 | 1.9021 | 0.1919 | 0.1256 | 0.0241 |

The design is now complete!