Formula Sheet

ACDC-5009 - Electromagnetic Radiation

| Speed of light C=3×108m/s C = fλ | Unit Vector aOA= OAOA = 1 |  | Critical Angle ϕc=Sin-1n2n1 |
| --- | --- | --- | --- |
| Refractive index n=cv | Numerical Aperture NA=n12-n22 |  | Acceptance Angle ϕa=Sin-1NAn0 |
| Time taken for an echo ∆t= 2Rc | Velocity of an object V=λfdp2 Where, fdp=fr-ft |  | The cutoff frequency fc=c2ma2+nb2 |
| The cutoff wavelength λc=cfc | Radar Range Equation Pr=PtGtGrλ2σ(4π)3R4L |  | Force between two parallel current carrying conductors F=μI1I2l2πd |
| Length of antenna L=λ2 L=λ10 L= λ | Absolute permittivity ε=ε0εr ε0= Permeability of free space (vacuum) ε0=8.854×10-12F/m |  | Absolute Permeability μ=μ0μr μ0= Permeability of free space (vacuum) μ0=4π×10-7H/m |
| Self-inductance of solenoid L=μN2A | Inductance of Coaxial cable Ld=μ2πlnba |  | Magnitude of force F=Q1Q24πε0d2 |
| Force in Vector Form F=Q1Q24πε0d2a12 | Electric Field intensity spherical and cartesian Coordinate system E= Q14πε0d1p2a1p |  | Electric Flux Density (D) D=εE |
| Electric Flux density due to line charge D= ρl 2πdad | Electric Flux density due to Surface charge D= ρS 2ad |  | Electric Flux density due to Volume charge D=Volρv dV4πd2ad |
| Line charge Density ρL=Total ChargeTotal Length | Surface charge Density ρS=Total ChargeTotal surface area |  | Volume charge Density ρv=Total Charge Total Volume |
| Line charge Q=LρL dl | Surface charge Q=sρS dS |  | Volume charge Q=Volρv dV |
| Electric Field intensity due to line charge EL=LρL dl4πε0d2ad | Electric Field intensity due to surface charge ES=Sρs dS4πε0d2ad |  | Electric Field intensity due to volume charge EV=VoLρv dV4πε0d2ad |
| Dot Product A.B=ABcosθAB | Angle between the vectors θAB=cos-1A.BAB |  | Parallel vectors A.B=AB |
| Relation between Cartesian and Cylindrical System r=x2+y2 ϕ=tan-1(yx) z=z | Relation between Cartesian and spherical System r=x2+y2+z2 ϴ=cos-1(zx2+y2+z2) ϕ=tan-1(yx) |  | Rectangular to Polar Coordinates r=X2+Y2 and ϕ=tan-1(YX) Polar Representation= r∠ϕ=X2+Y2 ∠tan-1(YX) |
| Cross Product A×B=ABsinθABan | Perpendicular ax×ay=az ay×az=ax az×ax=ay |  | Cross Product of cartesian System A×B=axayazAxAyAzBxByBz |
| Addition of two vectors A and B C=A+B A=Ax ,Ay ,Az and B=(Bx ,By ,Bz) C=Ax+Bxax+Ay+Byay+Az+Bzaz |  | Subtraction of two vectors A and B D=A+(-B) A=Ax ,Ay ,Az and B=(Bx ,By ,Bz) D=Ax-Bxax+Ay-Byay+Az-Bzaz |  |
| Position and Distance Vector Consider a point P(x1,y1, z1) in the cartesian co-ordinate system. Three components of this position vector rOP=X1ax+Y1ay+Z1az The magnitude of this vector is rOP=(x1)2+(y1)2+(z1)2 |  |  |  |
| For two points in the cartesian coordinate system P and Q, with coordinates, P(x1,y1, z1) Q(x2,y2, z2) The individual position vector of the points are P=x1ax+y1ay+z1az Q=x2ax+y2ay+z2az The magnitude of this vectors are PQ =Q-P= [x2-x1]ax+[y2-y1]ay+[z1-z2]az PQ=(x2-x1)2+(y2-y1)2+(z2-z1)2 Unit Vector: aPQ=unit Vector along PQ=PQPQ |  |  |  |
| Differential element in Cartesian Co-ordinates Differential Cartesian Co-ordinates, (dx, dy, dz) The differential Vector length is: dl=dxax+dyay+dzaz The distance of P'from P is given by magnitude of the differential vector length dl=(dx)2+(dy)2+(dz)2 |  |  |  |
| Cylindrical co-ordinate Cylindrical Coordinate have three coordinates (r, ϕ, Z) The vector of point P can be represented as. : P=Prar+Pϕaϕ+PZaZ |  |  |  |

Integration Formula

dx=x+c

adx=ax+c

xndx=xn+1n+1+c

sinxdx=-cosx+c

cosxdx=sinx+c

sec2dx=tanx+c

exdx=ex+c

sin (n x)dx=-cos (nx)n+c

cos (n x)dx=sin⁡(nx)n+c

udv=uv-vdu

Differentiation formula

d xn=nxn-1dx