Revision 2

Q1

- For the functions the F1 (A, B, C)=∑m  (4,5,6,7)   and F2 (A, B, C)=∑m  (1, 2, 3).

Answer the following.

  - Illustrate the implementation of functions F1 and F2  using a single decoder.

  - Present the truth tables and the fundamental Boolean expressions for both F1 and F2.

  - Design F1 and F2 using Programmable Logic Array (PLA).

Q2

For the functions  w(A, B, C, D) = ∑m(2, 12, 13) and z(A, B, C, D) = ∑m(1, 2, 8, 12, 13)

Answer the following:

- Illustrate the implementation of functions 𝑤 and 𝑧 using a single decoder.

- Present the truth tables and the fundamental Boolean expressions for both w and z.

- Derive and show the simplified Boolean logic expressions for w and z.

- Design w and z using Programmable Array logic (PAL).

Q3

- Design the following function using PAL.

- f1= x1x2x3+x1x2x3 and

- f2= x1x2+ x1x2x3

Q4

- For a  3- input LUT of an FPGA Logic block and  determine the bits to be stored in its storage cell by using   the function  f= ABC+ABC+ABC+ABC  to realize by answering the following

  - Truth Table

  - Design a  3- input LUT of an FPGA by assuming the value of inputs to be 001 respectively.

Q5

- For a  4- input LUT of an FPGA Logic block and  determine the bits to be stored in its storage cell by using   the function  f=ABCD+ABCD+ABCD+ABCD  to realize by answering the following

- Truth Table

- With the help of the above truth table, design a  4- input LUT.

- Also, explain its operation in detail by assuming the values of inputs to be 0010.

Q6

Program FPGA for the functions  f= f1+ f2  by considering  f1= x1x2 andf2= x2x3

Q7

- Write short notes on Programming FPGA

- Explain the operation of 3 input LUTs

Q8

- Program FPGA for the functions  f= f1+ f2  by considering  f1= x2x3 andf2= x1x3

Q9

Differentiate Programmable Array Logic and Programmable Logic Array.