Combustion Emissions, Calculations and Mapping for Efficiency

for

Malcolm McDonald

11/01/2018

Alexander Thaddaeus Przibylla (P144999)			        Module: M3X8824

Table of Content

1. Introduction & Objectives	3

2. Results	3

2.1. NOx model for FER from Φ=0,6 to Φ=1 without EGR	3

2.2. NOx model for FER of Φ=0,97 and with EGR of 1% to 30%	9

3. Analysis	15

3.1. NOx model for FER from Φ=0,6 to Φ=1 without EGR	15

3.1.1. Calculating adiabatic flame temperatures	15

3.1.2. Calculating the temperature in the burned zone	16

3.1.3. Calculating the NOx concentration based on the reaction rate using the “Zeldovich Mechanism”	17

3.2. NOx model for FER of Φ=0,97 and with EGR of 1% to 30%	19

3.2.1. Calculating adiabatic flame temperatures	19

3.2.2. Calculating the temperature in the burned zone	19

3.2.3. Calculating the NOx concentration based on the reaction rate using the “Zeldovich Mechanism”	19

3.3. Discussion of Assumptions and Results	20

4. Conclusion	20

5. Bibliography	21

# 1. Introduction & Objectives

This assignment is titled “Combustion Emissions, Calculations and Mapping for Efficiency” and is based on the calculation and analysis of the formation of NOx inside an internal combustion engine. For this purpose, a model was developed in Excel to determine the concentration levels of NOx for a naturally aspirated four-cylinder engine of 1600 ccs capacity, fuelled with isooctane and running at an engine speed of 1800 RPM on the lean side of the stoichiometric air/fuel-ratio. The conditions under which it is theoretically run are supposed to represent a typical load situation during the “New European Drive Cycle” (NEDC), which is used to determine the compliance of car engines with the laws and restrictions on the legal emissions for cars. Therefore, the Excel model is based on a pressure rise inside the cylinders from 10 bar at the start of combustion to 50 bar at 20 degrees of crank angle after the start of combustion, with a linear increase in cylinder pressure of 2 bar per degree of crank angle, assuming a volumetric efficiency of 40 %, the pressure at inlet valve closing to be of 1 bar, the initial temperature of the air and fuel mixture to be 20 °C, a polytropic index of 1,2 for the burned zone, that the volume of the air and fuel mixture equals the cylinder head volume of 40 ccs and that the concentrations of N2 and O2 can be considered constant during combustion. The model was developed for fuel equivalence ratios from Φ=0,6 to Φ=0,9 in steps of 0,05 and steps of 0,01 from Φ=0,9 to Φ=1, where Φ=0,1 describes the stoichiometric ratio. In addition, the same model was developed for the effect of “Exhaust Gas Recirculation” (EGR) on the NOx concentration levels, if 1% to 30% of EGR are used.

# 2. Results

# 2.1. NOx model for FER from Φ=0,6 to Φ=1 without EGR

Excerpt of copy from NASA tables for the specific heats and enthalpy of CO2, H2O, N2 and O2 at temperatures from 200 to 5000 K:

(Przibylla, 2017)

Graphs of enthalpy of CO2, H2O, N2 and O2 at temperatures from 200 to 5000 K based on the copy from NASA tables, including the polynomial equations for each graph given by Excel:

(Przibylla, 2017)

(Przibylla, 2017)

Excerpt of the table for enthalpy required from CO2, H2O, N2 and O2 for temperatures from 298 to 3000 K based on the polynomial equations and excerpt of the table for total enthalpy released from Φ=0,6 to Φ=1:

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

# 2.2. NOx model for FER of Φ=0,97 and with EGR of 1% to 30%

Calculated products for any FER based on a drop-down list:

(Przibylla, 2017)

Excerpt of the table for enthalpy required from CO2, H2O, N2 and O2 for temperatures from 298 to 3000 K based on the polynomial equations and excerpt of the table for total enthalpy released from Φ=0,97:

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

(Przibylla, 2017)

# 3. Analysis

# 3.1. NOx model for FER from Φ=0,6 to Φ=1 without EGR

# 3.1.1. Calculating adiabatic flame temperatures

To be able to calculate the adiabatic flame temperatures NASA tables for the specific heats and enthalpy of CO2, H2O, N2 and O2 at temperatures from 200 to 5000 K have been provided (Turn, 2006). These tables are based on results acquired through experiments aimed at determining the enthalpy of formation of the molecules, which are done at constant atmospheric pressure of 1 bar and a temperature of 298,15 K, and measuring the enthalpy required to get from this datum zero to the different temperatures in K (Haywood, 1988). With the help of polynomial equations given for the graph of each molecule by Excel, more accurate tables with smaller temperature steps can be created. If it is assumed that all the heat released goes to warming the products of the combustion and that the pressure is constant during this process, which therefore means that the enthalpy is the same as the change in heat, the adiabatic flame temperatures for the different fuel equivalence ratios can now be calculated from the stoichiometric equation as follows.

Reactants						Products

Φ*C8H18+12,5*O2+3,76*N2 	→	 a*CO2+b*H2O+c*N2+d*O2

For Φ=0,9:

0,9*C8H18+12,5*O2+3,76*N2          	→          7,2*CO2+8,1*H2O+47*N2+d*O2

Determining the amount of free O2:

On LHS:						On RHS:

Number of O-atoms=25				2a+b=2*7,2+8,1=22,5

25=22,5+2d
d=1,25

Calculating the net enthalpy released using values of enthalpy from (Haywood, 1988):

On LHS:

Enthalpy of formation of reactants required=0,9*224=201,6 MJ

On RHS:

Enthalpy of formation of products released=7,2*393+8,1*242=4789,8 MJ

Net Enthalpy (released)=4789,8-201,6=4588,2 MJ

Using the improved NASA tables and a method of interpolation to determine the corresponding adiabatic flame temperature with the “Index” and “Match” function in Excel:

T-22402241-2240=4588,2-4585,964588,58-4585,96

T=2240,856 K

# 3.1.2. Calculating the temperature in the burned zone

A formula for the temperature in the burned zone at each degree of crank angle during the pressure rise inside the cylinder can be derived from assuming a polytropic compression and using the ideal gas law as follows.

P*Vn=constant

P1*V1n=P2*V2n

But P*V=m*R*T and for the burned zone V=m*R*TP

Hence P1*m*R*T1P1n =P2*m*R*T2P2n

P1*T1nP1n=P2*T2nP2n

T1nP1n-1=T2nP2n-1

T2n=T1n*P2n-1P1n-1

T2n=T1n*P2P1n-1

T2=T1*P2P1n-1n

The temperature in the burned zone can now be calculated for the pressure rise with every degree of crank angle beginning with the start of combustion and the adiabatic flame temperature previously determined, assuming a polytropic index of n=1,2.

# 3.1.3. Calculating the NOx concentration based on the reaction rate using the “Zeldovich Mechanism”

Since the production of NOx in the burned zone is controlled by the reaction between N2-molecules and O-atoms, where the controlling equation is described by N2+O ←→ NO+N, the temperatures allow the reaction rate to be calculated using chemical kinetics and the “Zeldovich Model” (McDonald, 2017).

d[NO]dt=4,17*1013*N2*O212*exp-67837T

Where exp-67837T=Reaction Rate
from Boltzman Probability=exp-Activation EnergyBoltzman Constant*Absolute Temperature=exp-EAKB*T , which is the probability of the molecules having enough energy to react based on their kinetic energy that is proportional to the absolute temperature, but differs with the structure of the molecules, as the more complex molecules are slower and the simpler ones faster due to less degrees of freedom to move, rotate and vibrate, with increasing temperature making it more likely for N-atoms of N2-molecules to split up and react with the free O-atoms, because the vibration within the molecules increases and they become more active as a result of the energy absorbed (McDonald, 2017).

And N2 and O212= Concentration of N2 and O2 
with the concentration of O -atoms being proportional to O212, because there is always an equilibrium between the O2 concentration and the O -atom concentration in the reaction described by the controlling equation O2 ←→ O+O, resulting in:

KO2=O*[O][O2]=O²[O2]

O2=KO2*O2

O=KO2*O212

Therefore O∝O212

To be able to calculate the concentrations of N2 and O2 and ultimately NO, some multiplying constant needs to be determined that accounts for the actual amount of kmols inside the cylinder during combustion, using the ideal gas law and assuming 40% volumetric efficiency.

From P*V=n*Ru*T

With 
P=1 bar=1*105Nm2
V=0,4*400=160 ccs=160*10-6 m3

T=20 °C=293 K

Ru=8314 J/kmolK

n=P*VRu*T=1*105*160*10-68314*293=6,57*10-6

And the Multiplying Constant= nNumber of kmols in the Stoichiometric Equation

For Φ=0,9:

Multiplying Constant= 6,57*10-60,9+12,5*(1+3,76)=6,57*10-660,4=1,1*10-7

The concentration of N2, O2  and therefore NO can now be calculated as follows.

N2=kmols N2 Burned Volume=Multiplying Constant*N2 in Equation40*10-6

O2=kmols O2 Burned Volume=Multiplying Constant*O2 in Equation40*10-6

Kmols of NOx made per Degree of Crank Angle
=dNOdt*Time for Crankshaft to turn through 1 Degree of Crank Angle in Seconds

=4,17*1013*N2*O212*exp-67837T*1360*60RPM

=4,17*1013*N2*O212*exp-67837T*16*RPM

Total kmols NOx made in Combustion per Cylinder

=dNOdt*16*RPM*Burned Volume=dNOdt*16*RPM*40*10-6

Total NOx Parts per Million=Total kmols NO made in CombustionTotal kmols of Products in the Exhaust*1*106

# 3.2. NOx model for FER of Φ=0,97 and with EGR of 1% to 30%

Due to the highest level of NOx concentration and formation occurring at an FER of Φ=0,97, the effect of adding EGR is calculated and assessed for this FER and the same volumetric efficiency, where the EGR percentage is described by:

EGR %=Residual Gas Fraction=Mass of Exhaust Gas returned to CylinderMass of Air and Fuel in Cylinder

# 3.2.1. Calculating adiabatic flame temperatures

The adiabatic flame temperatures are calculated using the same method as without EGR, however the amount of products has gone up by EGR %, whilst the net enthalpy and therefore heat released during combustion remains the same as without adding EGR due to the extra enthalpy cancelling on each side of the stoichiometric equation, which results in lower flame temperatures, as more enthalpy is required to warm the products (McDonald, 2017).

Reactants

Φ*C8H18+12,5*O2+3,76*N2    +EGR%*(a*CO2+b*H2O+c*N2+d*O2)

→

Products

a*CO2+b*H2O+c*N2+d*O2      +EGR%*(a*CO2+b*H2O+c*N2+d*O2)

# 3.2.2. Calculating the temperature in the burned zone

The temperature in the burned zone is calculated using the same method as without EGR.

# 3.2.3. Calculating the NOx concentration based on the reaction rate using the “Zeldovich Mechanism”

The NOx concentration levels are calculated using the same method as without EGR, however the multiplying constant that accounts for the actual amount of kmols inside the cylinder during combustion changes with adding EGR.

For Φ=0,97 and EGR of 10%:

Multiplying Constant= 6,57*10-60,97+12,5*1+3,76+0,1*7,76+8,73+47+0,375

=6,57*10-666,7865=9,8*10-8

# 3.3. Discussion of Assumptions and Results

The emissions of NO and NO2 are usually grouped together as NOx, with NO being the more dominant inside combustion engines, where the primary source of NO is the oxidation of atmospheric nitrogen, however the oxidation of nitrogen-containing fuel compounds can be an additional source, if they contain significant amounts of N2 (Haywood, 1988). The kinetic route of NO formation in combustion is the attack of O-atoms from the H2-O2 radical pool or its dissociation from O2 on N2-molecules starting the simple chain shown by the reactions N2+O ←→ NO+N and N+O2 ←→ NO+O (Glassman & Yetter, 2008). The formation of thermal NOx is strongly dependent on the combustion temperature, with it being part of the exponential term in the equation of the Zeldovich model, and less dependent on the oxygen concentration, which gets proven by the NOx model developed showing an increase of NOx concentrations levels with increasing flame temperatures and therefore temperatures in the burned zone, rather than with an increase of free oxygen from leaner fuel mixtures, i.e. lower fuel equivalence ratios (Glassman & Yetter, 2008). Since the combustion in engines occurs at very high pressures, the flame reaction zone is extremely thin, whereas the surrounding cylinder head volume in which the combustion takes place is very large, which gives the local burning almost unlimited space to expand into and therefore allows the assumption of adiabatic constant-pressure combustion of the air-fuel mixture in a constant volume (Haywood, 1988). In addition, the NOx model is based on the assumption of a homogenous air-fuel mixture inside the cylinder, which is nearly impossible, as the fuel rarely has enough time for sufficient mixing with the air before combustion, especially in engines working with direct-injection of the fuel after the exhaust valve has been closed to stop hydrocarbons from escaping through the exhaust (McDonald, 2017). In reality, instead of a homogenous mixture throughout, the gases inside the cylinder form richer and leaner patches, which results in the hot, richer patches being fed free O2 from neighbouring, leaner patches and ultimately causes the formation of NOx even with an FER of Φ=1, i.e. a stoichiometric air-fuel ratio, that the developed NOx model cannot account for and means a slight offset of the graphs for NOx concentration and NOx parts per million towards the lean side of stoichiometric with a much steeper drop than realistically achievable (McDonald, 2017). Due to the temperature in the burned zone having the largest effect on the formation of NOx, recycling exhaust gases from previous cycles by adding them to the air that is being sucked into the cylinder through the intake and therefore increasing the heat capacity of the cylinder charge per unit mass of fuel, hence reducing the adiabatic flame temperature and maximum temperature in the burned zone, exhaust gas recirculation is an effective way of controlling the NOx emissions of an engine and a vastly used measure of the automotive industry to ensure the compliance of their engines with latest emissions regulations, even though EGR effects the combustion negatively by slowing down the speed of flame propagation and is limited to around 30% at low load, before the engine would start to misfire (Haywood, 1988) (McDonald, 2017). The NOx model developed shows the significant impact of adding EGR on the rate of formation and level of concentration of NOx.

# 4. Conclusion

The mechanisms responsible for the formation of NOx have been described and the calculations for developing a basic NOx model with and without the effect of adding EGR have been explained. Furthermore, the NOx model developed is presented in detail and the results from the model built in Excel, as well as the assumptions it is based on, have been discussed.

# 5. Bibliography

Glassman, I. & Yetter, R. A., 2008. Combustion. 4th ed. London, Great Britain: Elsevier.

Haywood, J. B., 1988. Internal Combustion Engine Fundamentals. USA: McGraw-Hill Inc..

McDonald, M., 2017. Powertrain & Sustainability Lecture - Competition Engine Design Module. Swansea, Great Britain: s.n.

Przibylla, A., 2017. CombustionEmissionsPrzibylla - Excel Workbook. Swansea, Great Britain: UWTSD.

Turn, S. R., 2006. Thermodynamics: Concepts and Applications. 1st ed. New Yorck, USA: Cambridge University Press.