Lab – Understanding Ethernet Cable Standards

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.10.1 | 255.255.255.0 | N/A |
| PC-B | NIC | 192.168.10.2 | 255.255.255.0 | N/A |

Objectives

Part 1: Analyze Ethernet Cabling Standards and Pinouts

Part 2: Build an Ethernet Crossover Cable

Part 3: Test an Ethernet Crossover Cable

Background / Scenario

In this lab, you will build and terminate an Ethernet crossover cable and test it by connecting two PCs together and pinging between them. You will first analyze the Telecommunications Industry Association/Electronic Industries Association (TIA/EIA) 568-A and 568-B standards and how they apply to Ethernet cables. You will then construct an Ethernet crossover cable and test it. Finally, you will use the cable you just constructed to connect two PCs together and test it by pinging between them.

Note: With autosensing capabilities available on many devices, such as the Cisco 1941 Integrated Services Router (ISR) switch, you may see straight-through cables connecting like devices.

Required Resources

One length of cable, either Category 5 or 5e. Cable length should be 0.6 to 0.9m (2 to 3 ft.)

2 RJ-45 connectors

RJ-45 crimping tool

Wire cutter

Wire stripper

Ethernet cable tester (optional)

2 PCs (Windows 7 or 8)

Analyze Ethernet Cabling Standards and Pinouts

The TIA/EIA has specified unshielded twisted pair (UTP) cabling standards for use in LAN cabling environments. TIA/EIA 568-A and 568-B stipulates the commercial cabling standards for LAN installations; these are the standards most commonly used in LAN cabling for organizations and they determine which colour wire is used on each pin.

With a crossover cable, the second and third pairs on the RJ-45 connector at one end of the cable are reversed at the other end, which reverses the send and receive pairs. The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. Crossover cables are normally used to connect hubs to hubs or switches to switches, but they can also be used to directly connect two hosts to create a simple network.

Reflection:

Write a research report (Max word count 600) on Cat 5e and 6 Ethernet cable standards.

Introduction

The natural evolution from other previous categories of wiring, such as coaxial for a token ring network, Category 5e and later 6 have become the industry standard for ethernet cabling.

“The introduction of twisted pair wiring into standard Ethernet networking ushered in a new age of network connectivity” [1]

This is because of higher transmit speeds, with reliability, cable flexibility, and lower production costs. The cabling was also easy to customise and tailor using simple tools.

The Standard

The Cat 5 and 6 cabling standards are published and now maintained by the Telecommunications Industry Association. The “Engineering Committee TR-42 develops and maintains voluntary telecommunications standards for telecommunications cabling infrastructure” [2]. There have been a number revisions over the years as the standards are refined.

Cabling Structure

A Single Cat 5e Ethernet cable contains 8 copper wires that are individually sheathed, the sheathing is colour coded to allow for easy identification. To avoid the cable receiving and giving off interference, the 8 individual copper wires are arranged in twisted pairs i.e. 4 sets of two twisted copper wires.

“Twisted-pair cables are each pair of wire is twisted around one another because help reduce crosstalk and noise susceptibility. The twists create coupling among the two wires in the pair such that opposing electromagnetic fields are canceled.” [3]

The TIA standard also specifies the arrangement of the wires at the connecting ends. This arrangement at the terminating ends of the inner wires is commonly known as straight-through type or cross-over type. This is described in the colour coding standard “T-568-A” and “T-568-B”. The difference between these two standards is how the order of the inner copper wires are terminated. A cable terminated both ends with the T568-A pattern is a straight-through, a cable terminated with the T-568-A on one end and T568-B on the other is a cross-over cable [1].

The Category 5e cabling supports 100Mhz networks, this allows the data bandwidth to be from 100MB up to 1Gigabyte. The rating of the network cabling is usually described in MHz as encoding techniques can produce higher data throughput results as datagrams can contain additional information [1]

Category 5e and 6 are typically terminated each end using the standard RJ45 (Registered Jack type 45), connector.

On visual inspection, the Category 5 and 6 cables are essentially the same physical structure. However, the difference between the two cables is that the category 6 cable has a higher MHz bandwidth and can achieve throughput of 10GB with its lower interference capabilities.

A key benefit of both Cat 5 and 6 cables is that they are backwards compatible and so will carry signals in older networks.

The maximum run and the specification for Cat 5e and 6 is 100metres. This is enough for most common uses such as within a building. Longer distances would require an intermediate repeater or router to maintain the signal strength and reliability.

Where are these cables used?

These connectors types are now commonplace and can be found anywhere across household goods and smart devices such as TVs and home broadband routers.

Straight-through and cross over uses: Straight-through cables are used for connections between a network device such as a PC or Printers where they connect to a switch or “Hub” device. A cross-over cable would be used for directly connecting two devices e.g. PC to a PC to allow direct communication.

With Cat5e allowing up to 1gb transmission rate they are most commonly used around home and office use. The cables are also

The Future

There are additional standards for Cat 7 and 8, which again maintain the same arrangement of twisted pairs and RJ45 connections, but each increment in the standard essentially describes the ability for the cable to provide higher throughput. With the current high through-put it seems that Cat5e and 6 will be around for many years to come.

# References

| [1] | J. Trulove, "Lan Wiring," in LAN Wiring : An Illustrated Network Cabling Guide. 2nd ed., New York, McGraw-Hill, 2000, pp. 118-122. |
| --- | --- |
| [2] | T. I. Association, "standards-committees/tr-42," [Online]. Available: https://tiaonline.org. [Accessed 4 12 2019]. |
| [3] | H. A. N. S. S. A. A. S. J. A. M. A. Z. a. M. Z. Y. Munir, " "Fundamental Study and Analysis of CAT5e Cable for NEXT and ELFEXT."," 2012 4th International Conference on Intelligent and Advanced Systems (ICIAS2012), pp. 217-21, 2012. |

Analyze diagrams and tables for the TIA/EIA 568-A standard Ethernet cable.

The following table and diagrams display the color scheme and pinouts, as well as the function of the four pairs of wires used for the 568-A standard.

Note: In LAN installations using 100Base-T (100 Mb/s), only two pairs out of the four are used.

568-A 10/100/1000Base-TX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Green | Transmit | BI_DA+ |
| 2 | 2 | Green | Transmit | BI_DA- |
| 3 | 3 | White/Orange | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Orange | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

The following diagrams display how the wire color and pinouts align with an RJ-45 jack for the 568-A standard.

Analyze diagrams and tables for the TIA/EIA 568-B standard Ethernet cable.

The following table and diagram display the color scheme and pinouts for the 568-B standard.

568-B 10/100/1000-BaseTX Ethernet

| Pin Number | Pair Number | Wire Color | 10Base-T Signal 100Base-TX Signal | 1000Base-T Signal |
| --- | --- | --- | --- | --- |
| 1 | 2 | White/Orange | Transmit | BI_DA+ |
| 2 | 2 | Orange | Transmit | BI_DA- |
| 3 | 3 | White/Green | Receive | BI_DB+ |
| 4 | 1 | Blue | Not Used | BI_DC+ |
| 5 | 1 | White/Blue | Not Used | BI_DC- |
| 6 | 3 | Green | Receive | BI_DB- |
| 7 | 4 | White/Brown | Not Used | BI_DD+ |
| 8 | 4 | Brown | Not Used | BI_DD- |

Build an Ethernet Crossover Cable

A crossover cable has the second and third pairs on the RJ-45 connector at one end, reversed at the other end (refer to the table in Part 1, Step 2). The cable pinouts are the 568-A standard on one end and the 568-B standard on the other end. The two following diagrams illustrate this concept.

Build and terminate a TIA/EIA 568-A cable end.

Determine the cable length required. (Your instructor will let you know the cable length you should make.)

Note: If you were making a cable in a production environment, the general guideline is to add another 12 in. (30.48 cm) to the length.

Cut off a piece of cable to the desired length and using your wire stripper, remove 5.08 cm (2 in.) of the cable jacket from both ends.

Hold the four pairs of twisted cables tightly where the jacket was cut away. Reorganize the cable pairs into the order of the 568-A wiring standard. Refer to the diagrams, if necessary. Take as much care as possible to maintain the twists in the cable; this provides noise cancellation.

Flatten, straighten, and line up the wires using your thumb and forefinger.

Ensure that the cable wires are still in the correct order for the 568-A standard. Using your wire cutters, trim the four pairs in a straight line to within 1.25 to 1.9 cm (1/2 to 3/4 in.).

Place an RJ-45 connector on the end of your cable, with the prong on the underside pointing downward. Firmly insert the wires into the RJ-45 connector. All wires should be seen at the end of the connector in their proper positions. If the wires are not extending to the end of the connector, take the cable out, rearrange the wires as necessary, and reinsert the wires back into the RJ-45 connector.

If everything is correct, insert the RJ-45 connector with cable into the crimper. Crimp down hard enough to force the contacts on the RJ-45 connector through the insulation on the wires, thus completing the conducting path. See the following diagram for an example.

Build and terminate a TIA/EIA 568-B cable end.

Repeat steps 1a to 1g using the 568-B color wiring scheme for the other end.

Test an Ethernet Crossover Cable

Test the cable.

Many cable testers will test for length and mapping of wires. If the cable tester has a wire map feature, it verifies which pins on one end of the cable are connected to which pins on the other end.

If your instructor has a cable tester, test the crossover cable for functionality. If it fails, check with your instructor first as to whether you should re-cable the ends and re-test.

Connect two PCs together via NICs using your Ethernet crossover cable.

Working with a lab partner, set your PC to one of the IP addresses shown in the Addressing Table (see page 1). For example, if your PC is PC-A, your IP address should be set to 192.168.10.1 with a 24-bit subnet mask. You partner’s IP address should be 192.168.10.2. The default gateway address can be left empty.

Using the crossover cable you made, connect the two PCs together via their NICs.

On the PC-A command prompt, ping the PC-B IP address.

Note: The Windows firewall may have to be temporarily disabled for pings to be successful. If the firewall is disabled, make sure you re-enable it at the conclusion of this lab.

Repeat the process and ping from PC-B to PC-A.

Assuming IP addressing and firewall are not issues, your pings should be successful if the cables were properly made.

Reflection

Which part of making cables did you find the most difficult?

The difficult part is ensuring the correct ordering of the wires following the straight through or crossover pattern. The individual copper wires can be quite difficult to align ready for placing in to the RJ-45 connector, the use of a tool may help this stage. _____________________________________________________________________________________

Why do you have to learn how to make a cable if you can easily buy pre-made cables?

Learning how to make a cable helps aid in the understanding of how they work. This includes the knowledge regarding shielding, wire layout, the twisted nature and pairing of the individual copper wires. In addition there may also be specialist cabling requirements for example within a data centre, so the understanding of the architecture of the cable can be beneficial in understanding its capabilities and limitations._______________________________________________________________