Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

The two means two login attempts.

What does the block-for 30 mean in the above command?

It means block login attempts for 30 seconds.

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh not a valid command

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

Telnet doesn’t work because the transport Input SSH command sets the input to SSH.

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection?

Yes

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

The session gets terminated.

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed?

My banner was displayed- Unauthorized access is prohibited.

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

I get taken straight into exec mode but don’t get prompted for a password. I wouldn’t get disconnected regardless because the disconnect only applies to the SSH connection and not the commands on the router.

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

Yes I am able to connect and log in to the switch

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

Protocol Version 1

Protocol Version 2

How would you provide multiple users, each with their own username, access to a network device?

To provide multiple users on the device I would use the username command to create and configure users individually.

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

To successfully compare various Encryption standards, we must first have a sound knowledge of what  Encryption is. Internet Society states that “Encryption is the process of scrambling or enciphering data so it can be read only by someone with the means to return it to its original state”. [1]. The main purpose of Encryption is to offer security. Encryption effects a great amount of people although not everyone is aware of what it actually is. Encryption can offer protection whilst carrying out day to day activitys. These activities can include shopping online, using devices or just simply browsing the web. Encryption can help protect your data without evening using the internet. Internet Society states that “Companies use encryption to protect their sensitive information, like customer information, trade secrets, and financial records.” [1]. This again highlights how much of an integral part Encryption plays in our security. Now that we have an understanding what Encryption means, we can start to compare the various standards. I will compare a selection of standards to show how the compare and differ with each other.

There are many different Encryption standards available. These standards differ depending on the individual’s needs. The first standard I will discuss is AES. AES stands for Advanced Encryption Standard. This is one of the most secure standards. AES is a symmetric key encryption meaning that you must share the key with someone for them to be able to access the data. This would be suitable to use to protect classified information.  This method uses a block cipher, which encrypts data one fixed-size block at a time, unlike other types of encryption, such as stream ciphers, which encrypt data bit by bit. The disadvantage of this particular encryption Is that the key has to be shared with a different user in order to access the encrypted data. This in itself imposes a security risk due to the format in which the key could be shared. Top Ten Reviews states that “AES is comprised of AES-128, AES-192 and AES-256. The key bit you choose encrypts and decrypts blocks in 128 bits, 192 bits and so on. There are different rounds for each bit key. A round is the process of turning plaintext into cipher text. For 128-bit, there are 10 rounds; 192-bit has 12 rounds; and 256-bit has 14 rounds.” [2].  Now that we know what AES standards entail, we can compare it against other standards.

The second standard I would like to discuss is 3DES. 3DES stands for Triple Date Encryption standard. It encrypts your data three times and uses a block cipher like the AES. The disadvantage of 3DES is Because it encrypts your data three times, it is much slower to use compared to the other standards reviewed. The obvious advantage of 3des is that it encrypts your data 3 times. This standard is quite dated now and Because it uses shorter block length, it is easier to decrypt and leak the data.

The third standard I would like to review is Twofish. Towfish is an involved version of Blowfish. Top Ten Reviews states that “Similar to AES, it implements rounds of encryption to turn plaintext into cipher text. However, the number of rounds doesn’t vary as with AES; no matter the key size, there are always 16 rounds.” [2].  This is interesting to see how it compares with the AES. Twofish can be used without the restriction of the other standards as it is unpatented and license free. Which means that it can offer flexibility.

The fourth and final standard to address is RSA. RSA is named after Ron Rivest, Adi Shamir and Len Adelman. RSA offers good security as there are two keys needed to encrypt and decrypt. The two keys are made up of a public key and a private key. RSA is similar to 3DES as the encryption method is slower when compared to others. This is because of the long key size. However, the length of the key size does offer substantially more security.

To Conclude, there are many different Encryption standards that offer different benefits. By comparing them we have been able to assess their pro and cons and how they stack up against one another. By becoming aware of which ones offer the most security, we can hopefully keep data and personal information confidential. By doing this, the selected Encryption standard will work to it’s full potential and match users expectations.

# References

| [1] | "https://www.internetsociety.org/encryption/what-is-encryption/?gclid=CjwKCAiAxMLvBRBNEiwAKhr-nHEVwR7cQBUmjaUOaQ1kSZoKcQqXdhJUTVDMYx7pehjknoO6HduVFxoCEcUQAvD_BwE," [Online]. [Accessed December 2019]. |
| --- | --- |
| [2] | o. T. R. contributor, "https://www.toptenreviews.com/secure-encryption-methods," 2017. [Online]. [Accessed 2019]. |