R2

R2#sh run

Building configuration...

Current configuration : 1379 bytes

!

! Last configuration change at 13:58:09 UTC Thu Dec 12 2019

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R2

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

cts logging verbose

!

!

license udi pid CISCO1941/K9 sn FCZ2125B0F2

!

!

!

redundancy

!

!

!

!

!

!

interface Loopback60

ip address 172.16.60.1 255.255.255.0

!

interface Loopback61

ip address 172.16.61.1 255.255.255.0

!

interface Loopback62

ip address 172.16.62.1 255.255.255.0

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 192.168.15.1 255.255.255.0

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

bandwidth 512

ip address 1.1.1.1 255.255.255.252

clock rate 2000000

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

!

!

!

!

control-plane

!

!

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

End

R2#SH IP INT BRIEF

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES unset  administratively down down

GigabitEthernet0/0         192.168.15.1    YES manual administratively down down

GigabitEthernet0/1         unassigned      YES unset  administratively down down

Serial0/0/0                1.1.1.1         YES manual up                    up

Serial0/0/1                unassigned      YES unset  administratively down down

Loopback60                 172.16.60.1     YES manual up                    up

Loopback61                 172.16.61.1     YES manual up                    up

Loopback62                 172.16.62.1     YES manual up                    up

R2#SH IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override

Gateway of last resort is not set

1.0.0.0/8 is variably subnetted, 2 subnets, 2 masks

C        1.1.1.0/30 is directly connected, Serial0/0/0

L        1.1.1.1/32 is directly connected, Serial0/0/0

172.16.0.0/16 is variably subnetted, 6 subnets, 2 masks

C        172.16.60.0/24 is directly connected, Loopback60

L        172.16.60.1/32 is directly connected, Loopback60

C        172.16.61.0/24 is directly connected, Loopback61

L        172.16.61.1/32 is directly connected, Loopback61

C        172.16.62.0/24 is directly connected, Loopback62

L        172.16.62.1/32 is directly connected, Loopback62

R2#SH VTP STATUS

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 20

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xBF 0x86 0x94 0x45 0xFC 0xDF 0xB5 0x70

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 192.168.15.1 on interface Gi0/0 (first interface found)