Report on Lego Mindstorms Robot’s Course

Name: Tanisha Bath

Student Number: 1905383

Course: Electrical and Electronic Engineering (Foundation Entry)

Word Count: 2,888 words

Abstract

In this report, I will be showing the different ways of programming a Lego Mindstorms robot to follow a set course and show a flowchart on how I would program the Lego Mindstorms robot to maneuverer through the course.

Contents

1. 0 Introduction

2. 0 Following a Line

2. 1. a Detecting the Block

2. 1. a1 Ultrasonic Sensor

2. 1. a2 Touch Sensor

2. 1. b Moving around the block

2. 1. b1 Sweep the block out of the way

2. 1. b2 Climbs over the block

2. 1. b3 Drives around the block

3.0 Navigating the Maze

3. 1 Two Ultrasonic sensors (following the left - hand wall)

3. 2 One of each sensor (following the left - hand wall)

3. 3 One Ultrasonic sensor

3. 4 One Touch sensor

4. 0 Finishing the Maze

5. 0 Flowchart

6. 0 Bibliography

1. 0 Introduction

Within this report, I will be discussing the different ways you could program a Lego Mindstorms robot to complete a course. This course will be set up in a set way which consists of following a line with a block on it that the Lego Mindstorms robot must manoeuvre around (said block is a light wooden object and should avoid being touched as to not move it), completing a maze and indicating that it has reached the finishing point in a way such as playing a little tune or signal at the end. I will show a flowchart / pseudocode to show the logic of how the Lego Mindstorms robot will negotiate the course in the way I think would be best.

There should be a set amount of time between the line following and before the start of the maze to edit the build and / or program of the Lego Mindstorms robot.

2. 0 Following a Line

The first task of the course is to follow a line, this could be achieved by including a Colour Sensor pointing towards the ground. The colour sensor is capable of distinguishing between 8 different colours, is able to do this by emitting a red light which measures the intensity of the light that is reflected back into the colour sensor that then sends signals to the intelligent brick that the Lego Mindstorms robot then uses to detect and differentiate between colours allowing it to follow the line on the floor.

2. 1 Avoiding the Block

To begin the course the Lego Mindstorms robot must follow the coloured line that is on the floor while following the line the Lego Mindstorms robot must be able to detect a block that is in its path, it could detect the block by using the Ultrasonic Sensor or a Touch Sensor.

2. 1. a Detecting the Block

2. 1. a1 Ultrasonic Sensor

The Ultrasonic Sensor works by generating sound waves that bounce off objects and reads their echo to calculate how far the object is like a sonar it can also start a program by listening for sound waves.

The pros to this device being used are that once it is the desired distance from the block it will stop which gives the Lego Mindstorms robot space to move after.

There are also cons to this device where it could possibly stop too soon because it heard other sounds around it or it could be a little faulty and not read any sounds and be unable to detect the block in its path.

2. 1. a2 Touch Sensor

The Touch Sensor’s programming is rather simple but precise, this device could be programmed to stop the Lego Mindstorms robot when the button is pressed once it runs into the block.

The pros to this device being used are that it is very precise and would not get stopped accidentally unless someone walked over the course.

There are also cons to this device such as it needs to touch the block in order to stop which in turn could (most likely) move the block, and even if it doesn’t move the block it won't give the Lego Mindstorms robot much space to move after.

Once the block has been detected it the Lego Mindstorms robot has stopped it must then manoeuvre around the block, there are a few ways that to Lego Mindstorms robot could accomplish this;

2. 1. b Moving around the block

2. 1. b1 Sweep the block out of the way

It is possible that the Lego Mindstorms robot could just ram the block or sweep it out of the way and then continue forward till it finds the coloured line again.

2. 1. b2 Climbs over the block

If there are enough materials to build a mechanical system that would allow the Lego Mindstorms robot to climb, then it could be programmed to climb over the block.

2. 1. b3 Drives around the block

There is also the option to program the Lego Mindstorms robot to drive around to the other side of the block and back onto the coloured line.

Once the Lego Mindstorms robot has moved around to the other side of the block and found the coloured line again it should continue to follow it through the loop and up until it reaches the maze and should stop.

(there should be a set amount of time here between the different stages in the course to change the build and / or program of the Lego Mindstorms robot)

3. 0 Navigating the Maze

After completing the first half of the course that is following the coloured line the Lego Mindstorms robot must then navigate the maze to the right.  Here are several different methods that could be used to navigate to the end of the maze;

3. 1 Two Ultrasonic sensors (following the left - hand wall)

For the maze above the Lego Mindstorms robot could have one sensor that is attached to the side of your Lego Mindstorms robot to move forward when it detects an object next to the Lego Mindstorms robot and turn left when there is nothing there, then program the other sensor on the front of the Lego Mindstorms robot to turn right when it detects and object in front of the Lego Mindstorms robot.

3. 2 One of each sensor (following the left - hand wall)

For the maze above the Lego Mindstorms robot could have the ultrasonic sensor that is attached to the side of the Lego Mindstorms robot to move forward when it detects an object next to the Lego Mindstorms robot and turn left when there is nothing there, then program the touch sensor on the front of the Lego Mindstorms robot to turn right when it detects an object by pushing the button into the wall.

3. 3 One Ultrasonic Sensor

For the maze above the Lego Mindstorms robot could have an ultrasonic sensor attached to the front and have it turn left every time it detects an object (if it turned right the Lego Mindstorms robot would get trapped in a loop on the right - hand side of the maze).

3. 4 One Touch Sensor

For the maze above the Lego Mindstorms robot could have a touch sensor attached to the front and have it turn left every time it detects an object by pushing the button into the wall (if the Lego Mindstorms robot turned right it would get trapped in a loop on the right - hand side of the maze).

4. 0 Finishing the Maze

Once the Lego Mindstorms robot gets to the end of the maze it needs a way to differentiate between the maze wall and the finishing (which is a red block) for this I suggest using a colour sensor pointing forwards, this should be programmed to, once it sees the red block, stop and play a little tune once to indicate that it has finished the course.

5. 0 Flowchart

The flowchart below is how I would personally program my Lego Mindstorms robot by deciding between the decisions explained above and what ones I believe would work best.

The flowchart / pseudocode below starts I have calibrated the colour sensor to notice the difference between colours so it knows what colour to follow when the maze starts and when to stop moving so it can play its little tune / signal to finish and state that it is at the course.

On my Lego Mindstorms robot is a colour sensor facing the floor (not touching the floor but just hovering above it) and an ultrasonic sensor at the front of the Lego Mindstorms robot facing forward.

Follow coloured line (using colour sensor) > > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees right

> > > > Drive forward 30cm > > > >Turn Lego Mindstorms robot 90 degrees left

> > > > Drive forward 40cm > > > > Turn Lego Mindstorms robot 90 degrees left

> > > > Drive forward 30cm > > > > Reconnect with coloured line (using colour sensor) > > > > Follow coloured line (using colour sensor) > > > > Break at green line at start of maze (using colour sensor) > > > > Stop program

This is where I will be given a set amount of time to make and changes to the build and or program of the Lego Mindstorms robot, in this time I would move the colour sensor from the bottom of the Lego Mindstorms robot and place it on the front of the Lego Mindstorms robot (a little further forward than the ultrasonic sensor).

(This flowchart is assuming that there is enough room in the maze to stop 10cm away)

Drive forward > > > > Break 10cm away from block (using ultrasonic sensor)

> > > >Turn Lego Mindstorms robot 90 degrees left > > > > Drive forward > > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees left > > > > Drive forward > > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees left > > > > Drive forward > > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees left > > > > Drive forward

> > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees left > > > > Drive forward > > > > Break 10cm away from block (using ultrasonic sensor) > > > > Turn Lego Mindstorms robot 90 degrees left > > > > Break once colour sensor detects change in colour (from black maze wall too red finish block) > > > > Bark like dog (to indicate that robot has finished the course) > > > > Stop program

6. 0 Bibliography

Lego Mindstorms Robot’s Course Image

S. Maw 2020, GA36a-M4X01506 - Foundation Year Integrating Project ALTERNATIVE ASSIGNMENT moodle.uwtsd.ac.uk.  (n.d.). UWTSD Moodle 2019/20: Log in to the site. [online] Available at: https://moodle.uwtsd.ac.uk/course/view.php?id=3238 [Accessed 6 May 2020].

‌

Colour Sensor Information

Lego (n.d.). EV3 Colour Sensor 45506 | MINDSTORMS® | Buy online at the Official LEGO® Shop GB. [online] www.lego.com. Available at: https://www.lego.com/en-gb/product/ev3colorsensor45506ef_id=CjwKCAjwwMn1BRAUEiwAZ_jnEnWohAkomZWJz9L0P9fUMxt3QfNmuv28lmFnVMpr7jJAfz-ccO9PQxoCxqoQAvD_BwE:G:s&s_kwcid=AL [Accessed 6 May 2020]

‌

Ultrasonic Sensor Information

Lego (n.d.). EV3 Ultrasonic Sensor 45504 | MINDSTORMS® | Buy online at the Official LEGO® Shop GB. [online] www.lego.com. Available at: https://www.lego.com/en-gb/product/ev3-ultrasonic-sensor-45504 [Accessed 6 May 2020].

‌

Touch Sensor Information

Lego (n.d.). EV3 Touch Sensor 45507 | MINDSTORMS® | Buy online at the Official LEGO® Shop GB. [online] www.lego.com. Available at: https://www.lego.com/en-gb/product/ev3-touch-sensor-45507 [Accessed 6 May 2020b].