Housing Development.

Cost Studies of Buildings.

Assignment 1.

Introduction.

I will be producing a cost plan for the client: Knights garden housing development, for the address of (unknown). As part of the new development, a group of 4 buildings, 6 semi-detached homes and 3 terraced houses are being built. The drawings attached were produced by an architect firm called BAM Architect Ltd.

Plans proposed the construction will be situated on a possible green field site. From what can be taken from the drawings attached, a ground survey may possibly need to be undertaken. The exterior grounds of the site will be finished with hard and soft landscaping.

Quality levels of construction, internal fitting and finish will be presumed to be medium to high (average to higher quality on completion) in accordance with BCIS. (BCIS) I have made this presumption due to the design quality of the buildings.

Costs and information for the estimated build cost has been taken from BCIS and calculations for inflation have not been factored into the estimate, however the most recent data has been used to form this estimated cost plan.

Costs and information used for the formulation of the elemental cost structure has been taken from the BCIS and other sources. Calculations for inflation have not been factored into the estimate, however up to date data has also been used in the elemental cost plan.

Assumptions.

To formulate the estimates and cost plan I have had to make some assumptions, in order to fill in the missing gaps which are present at this early stage of the project. These assumptions include the floor size of the buildings. This is due to the metric not being included on the designs to formulate the estimated build cost. Therefore, the average size has been used for the correct building type and the right amount of rooms, so that the size is as accurate as possible. The next assumption was made on the price per M² to build in the UK, as there is no set price available. This was calculated using BCIS website to look at past projects in the UK, in order to find out the cost per M² to build. This figure can be used with the average floor area of the buildings to work out the most accurate estimated build cost possible. Assumptions have also been made about the type of foundation being used on the project, due to there being no specified foundation type. The most viable foundation type will be used and will most likely be strip foundations. Assumptions will also be taken upon the quality of fittings and finishes e.g., kitchen and bathrooms. This assumption must be made as there are no specified details about these elements at this stage of the project, and a price is required for the elemental cost plan.

(self-build, Wickes).

Estimated build cost.

This is an initial estimate for the cost of construction, due to the floor size of the buildings according to the assumptions made. This estimate was comprised before the tendering process, and is a realistic representation of the cost of the construction stage of the project.

‘BCIS website’ was used to comprise this estimate using average sizes for similar types of buildings with comparable bedrooms. The price per M² of construction in the UK was used to produce this estimate.

| Type/Number | GIFA M² | Cost per M² | Cost per Unit | Total Cost |
| --- | --- | --- | --- | --- |
| Terrace 1Bed | 63.91 | £1,027 | £65,635.57 |  |
| x3 | 191.73 | £1,027 |  | £196,906.71 |
| Semi-detached 3Bed | 95.97 | £1,437 | £137,908.89 |  |
| x6 | 575.82 | £1,437 |  | £827,453.34 |
| Total | 767.55 | Total Construction Cost |  | £1,024,360.05 |

Information gathered from the (BCIS data base, DWH and national statistical database).

Schedule of accommodation.

Terrace house-

| Kitchen | Lounge | Dinning | W/C | Family Bathroom | En-suite | Double Beds | Single Bed | Cot Room |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-Combined |  | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| X3 Units |  |  |  |  |  |  |  |  |
| 3 |  | 0 | 3 | 3 | 0 | 3 | 0 | 3 |

Semi-detached-

| Kitchen | Dinning | Lounge | W/C | Family Bathroom | En-suite | Double Beds | Single Bed | Cot Room | Study |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-Combined |  | 1 | 1 | 1 | 2 | 3 | 0 | 0 | 1 |
| X6 Units |  |  |  |  |  |  |  |  |  |
| 6 |  | 6 | 6 | 6 | 12 | 18 | 0 | 0 | 6 |

Elemental cost plan.

The elemental cost plan will be split into 6 main categories-

- substructure

- superstructure

- internal finishes

- fittings, furnishes, equipment

- services

- external works

(BCIS 2014-2017)

These categories will be inserted into a spread sheet to be shown in tabular form. The table will show a complete break down of the elements and the costs.

Terraced house:

Information obtained from (BCIS database 2018 and contact solar panels).

Semi-detached:

Information gathered from (BCIS database, roofing superstore and contact solar panels).

External ground works for site:

Information gathered from (BCIS database).

Total cost of construction:

Contractor prelims, fees and contingency.

These costs have been kept separate for the elemental cost plan and GIFA cost due to the Contractor prelims, fees and contingency sums being formulated after the elemental cost plan.

| contractors’ prelims | 20% | £199,723.43 |
| --- | --- | --- |
| site set up and works |  |  |
| professional fees | 7% | £69,903.20 |
| architects, management costs, site supervision |  |  |
| contingency | 5% | £49,930.86 |
| total cost |  | £1,318,174.65 |

Information gathered from (BCIS database).

Additional costs.

Additional costs can be unforeseen but should be considered so that the project isn’t left without the funds to fix the issue. A contingency sum should be factored into the elemental cost plan to cover any of these unforeseen costs. Weather is the most common cause of additional costs. Poor weather conditions can slow down work or stop construction all together, while contractors will still need paying while not being able to do any work for the duration of the bad weather. On top of this, once the bad weather has stopped additional time may be needed to let the site dry. Thus, creating further delays which run up extra costs.

Other additional cost may be caused by changes in the plan, late delivery of materials and incorrect deliveries. It is essential to ensure that all contracts are in place before the start of the project, to avoid any additional costs being added to the contracts.

To avoid conflict between parties site surveys and infrastructure inspections should be carried out in plenty of time before the planned work is due to begin. The design and structure of the project should be clearly stated, so that all parties are clear on the job at hand and there is no confusion and miscommunication between parties.

To avoid unforeseen costs which may cause the project to stall, a contingency sum will be drawn up and put to one side to pay for anything if something may occur.

Here are some common things that happen on site which would be classed as unforeseen costs, and could be allocated contingency funds:

- break down of essential equipment.

- Corrections of variation from the architect’s designs/plans via site contractors.

- Curational last minute changes to the project.

- Any delays be it weather, breakdowns, construction or delivery permitted.

- Corrections of poor work.

Additional costs due to site conditions could include:

- ground testing and geological surveys.

- Contaminated ground and subsequent clean up costs of contaminated ground.

- Changes in ground works due to ground surveys. If piling or ring beams are needed to overcome the challenging ground conditions it could cost over £10,000 per home.

Word count with tables is 2225*.

Reference list.

David Wilson Homes. Average House Sizes in UK. Available at: https://www.dwh.co.uk/library/Average-UK-house-sizes/ Last accessed: 30th January 2019.

https://service.bcis.co.uk/BCISOnline/direct/H0075

office of national statistics. 2017. House prices: how much does one square metre cost in your area?

www.ons.gov.uk/peoplepoulationandcommunity/housing/articales/housepriceshowmuchdoessquaremetercostinyourarea/2017-10-11  Last Accessed 25th January 2019.

Build It. 2013. Types of House Foundation. Available at: https://www.self-build.co.uk/types-house-foundation/. Last accessed 28th January 2019.

Wickes. Bathroom Packages. 2019. Available at: https://www.wickes.co.uk/Products/Bathrooms/Bathroom-Suites/Bathroom-Packages/c/1059000. Last accessed 25th January 2019.

Wickes. 2019. Atlanta Shaker Range. Available at: https://www.wickes.co.uk/kitchen/contemporary-kitchens/atlanta-range. Last accessed 26th January 2019.

Wickes. 2019. Basins. Available at: https://www.wickes.co.uk/Products/Bathrooms/Basins/c/1000994.  Last accessed 30th January 2019.

Wickes. 2019. Toilets and accessories. Available at: https://www.wickes.co.uk/Products/Bathrooms/Toilets-and-Accessories/c/1000992. Last Accessed 28th January 2019.

Contact solar. https://www.contactsolarpanels.co.uk/?gclid=EAIaIQobChMIntH_lJqW4AIVGc53Ch0ubQLrEAAYAiAAEgIlBPD_BwE Last Accessed 28th January 2019.

Roofing Superstore. VELUX GGL CK04 2070 White Centre Pivot Window Laminated - 55cm x 98cm. Available at: https://www.roofingsuperstore.co.uk/product/velux-ggl-ck04-2070-white-centre-pivot-window-laminated-55cm-x-98cm-1.html?gclid=EAIaIQobChMIxPb19p6W4AIVB853Ch0quQbMEAYYASABEgLhYfD_BwE. Accessed 30th January 2019.