| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2026 | 2026 |

MODULE CODE: ACMA4031

Electrical Prinicples

TIME:  2 HOURS

Maximum Marks: 100

answer ALL the questions

Equipment or materials required – Calculator, formula sheet

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig1, Calculate the following

- Total resistance in the circuit			[4 Marks]

- Total current in the circuit.			[2 Marks]

- Current across each resistor			[8 Marks]

- The voltage across R7 resistor			[2 Marks]

- Power delivered by the battery			[2 Marks]

Fig: 1

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?							[2 Marks]

- Calculate the potential difference across the bulb if 2kJ of work is required to push a charge of 250C through the bulb. 							[2 Marks]

- What length of German silver wire, diameter 0.050 cm, is needed to make a 28 resistor, if the resistivity of German silver is 2.2×10-7  m?				[3 Marks]

Question 2 [25 Marks]

- Calculate the current flowing when a charge of 60C flows past a point over a period of 2 minutes.										[2 Marks]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)	[13 Marks]

Fig.2

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)		[4 Marks]

- Explain the following terms:

- Resistance									[2 Marks]

- Reactance									[2 Marks]

- Impedance									[2 Marks]

Question 3 [25 Marks]

- The alternating current is given by i=40 sin314t A. Determine the following

- Peak Value of current			[1 Mark]

- RMS Value of Current			[2 Marks]

- Frequency			[2 Marks]

- Time period			[1 Mark]

- An oscilloscope display shows the below voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

2V/div

10ms/div

Fig:3

- Estimate the period of the waveform.					[1 mark]

- From the estimated period, determine its frequency.			[2 marks]

- Determine its peak-peak amplitude					[2 marks]

- Determine the r.m.s. value of the waveform.				[2 marks]

- An RLC circuit is shown in figure 4 below. Calculate the following

- The total circuit impedance 						[4 Marks]

- Total current flowing in the circuit.					[2 Marks]

- Determine the voltage drop across each component in the circuits.	[6 Marks]

Fig: 4

Question 4 [25 Marks]

- State Maximum power transfer theorem.					[2 Marks]

- A transformer has 300 turns in its secondary coil and is used to step up a 12v AC voltage to 200V.

  - How many turns would there be in the primary coil.			[3 Marks]

  - Under full load conditions, the current demand in the secondary circuit is 2.5A.  Determine the Power output rating of this transformer.			[2 Marks]

  - Determine the maximum load resistance that this transformer could supply.												[2 Marks]

  - we assume that the transformer is 97.5% efficient, estimate the current flow in the primary circuit for the conditions present in part b).			[2 Marks]

- Describe the functions of the following components used in DC machines

- Field System								[2 Marks]

- Armature									[2 Marks]

- In the transient circuit, a Capacitor of Value 1μF and a resistor of 5.45MΩ are connected in series across a 220V power supply through a switch. Calculate the time by which the capacitor will be charged to 60 percentage of the supply voltage.		[10 Marks]