| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| February 2017 | 2017 |

MODULE CODE: M3X8859

Lean Six Sigma Manufacturing Strategies

TIME:  2 HOURS

answer Three quESTION out of four

Equipment or materials required – Calculators may be used and graph paper and average range SPC chart paper to be provided

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER three QUESTIONS

Question 1

Six-sigma is often referred to as a tool that significantly improves customer satisfaction and shareholder value by reducing variability in every aspect of our business.

Prepare a report that fully evaluates what activities would be carried out in either the ‘define’ or ‘measure’ stages of the DMAIC methodology.

(25 marks)

Question 2

The study of reliability of equipment and failure patterns is an essential part of process and equipment improvement plans.

- A leading manufacturer of centrifugal pumps has gathered the following data related to repairs required by model MFG over the past 12 years

| Year | Failure rate |
| --- | --- |
| 1 | 18,000 |
| 2 | 2,600 |
| 3 | 950 |
| 4 | 875 |
| 5 | 800 |
| 6 | 825 |
| 7 | 890 |
| 8 | 970 |
| 9 | 960 |
| 10 | 1750 |
| 11 | 28,000 |
| 12 | 60,000 |

Using the data above, construct a bathtub curve and clearly explain the significance of the main sections of the curve in relation to reliability engineering.

(12 marks)

- SHD Logistics is considering installing a new picking line, but requires data on the overall reliability of the system.  Using the data below calculate the overall reliability of the picking line.

R1 = 0.89

R2 =0.82

(13 marks)

R3 = 0.95

R4 = 0.95

R8 = 0.90

R6 = 0.98

R5 = 0.90

R7 = 0.98

Question 3

Efficient handling and movement of material is essential for effective process management.  Materials handling can represent a significant proportion of manufacturing costs and this needs to be managed cost effectively.

- Discuss three applications that automated guided vehicle systems can be used for in manufacturing.

(6 marks)

- Using the data below determine the number of vehicles required to meet the delivery schedule for this layout if a total of 64 del/hr are required.

Loading time at the load station = 0.60min and unloading time at the unload station = 0.50min.  The following performance characteristics are given: vehicle velocity 40m/min, availability 0.95, and traffic factor 0.85 and operator efficiency 0.75.

Unload Station

35m

75m

60m

35m

Load Station

Direction of movement

Figure 1 AGVs Layout

Determine the following

- Distances travelled loaded and unloaded.

(1 mark)

- The ideal delivery cycle time.

(3 marks)

- The number of vehicles required to meet the delivery schedule 64 del/hr.

(6 marks)

- Explain three ratios that can be used to evaluate the performance of materials handling function.

(9 marks)

Question 4

ERD Engineering extrudes plastic pipes for use in the construction and building sector.  Every hour a sample of five pipes is taken and the diameter measured.  For the past 10 hours the following data has been collected.

| Sample | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X1 | 25.83 | 25.47 | 24.83 | 25.87 | 25.89 | 26.20 | 25.83 | 25.91 | 26.20 | 25.10 |
| X2 | 26.01 | 25.71 | 25.01 | 25.78 | 26.20 | 26.35 | 25.21 | 25.20 | 26.20 | 25.12 |
| X3 | 25.25 | 25.63 | 25.71 | 25.64 | 26.01 | 26.70 | 25.38 | 25.18 | 26.71 | 25.15 |
| X4 | 25.31 | 25.87 | 25.41 | 25.60 | 26.95 | 26.12 | 25.47 | 25.61 | 26.83 | 25.18 |
| X5 | 25.41 | 25.24 | 25.32 | 25.70 | 25.32 | 26.28 | 25.48 | 25.33 | 26.94 | 25.28 |

- Using the data above construct a suitable control chart to demonstrate whether the process is in control or not and comment on your findings.

(15 marks)

- Explain the main objectives of statistical process control.

(6 marks)

- Discuss concisely why it is necessary to calculate Cp and CpK values when determining the capability of a process.

(4 marks)