Variations

Pro-Rata using my ‘4 step’ method:

Step 1 – deduct OHP

Step 2- break down L, P & M costs

Step 3 – adapt costs to suit new variation item

Step 4  – add back OHP

Step 1 – Ddt OHP

If OHP total 15% on a gross bill rate of £32.26 how could we work this back to a nett bill rate, i.e. ddt the OHP from the gross rate?

- £34.53 / 1.15 = £30.03

Note: 100% + 15% = 115% change to decimal of 1.15

Step 2 – breakdown LPM

Note: if unsure use SPONS for a rough % of LPM of bill rate

e.g. SPONS 2020 Page 320 - Blockwork

Total rate = £34.53/m2 (labour = 13.62 + materials = 20.91)

Labour = 13.62 / 34.53 x 100 = 39.44 say 40%

Materials = 20.91 / 34.53 x 100 = 60.55 say 60%

Therefore

40% of £30.03 = £12.01

60% of £30.03 = £ 18.02

Step 3 – adapt costs to suit variation

Variation to the bill rate was for a block of 140mm thick

Labour = £12.01 x (140/100) = £16.81

Materials = £18.02 x (140/100) = £25.23

New all-in-rate = 16.81 + 25.23 = £42.04

Step 4 – add back OHP

£42.04 + OHP @15% = £48.35/m2

Now lets use MS Excel to do the same calculation…….

Example 2

Bill item

38mm thick cement and sand (1:3) screed exceeding 600mm wide, level and laid to falls only, not exceeding 15º from horizontal to concrete with steel trowel finish - £32.26/m2

Variation Item

50mm thick cement and sand (1:3) screed exceeding 600mm wide, level and laid to falls only, not exceeding 15º from horizontal to concrete with steel trowel finish – cost per m2

(Note:  OH = 15%;  Profit = 5%)