University of Wales Trinity Saint David

Wales Institute of Science and Art

Engine performance    testing and analysis

Motorcycle technology

ACHA4003/4004/4012

lars milner

2404286

28/03/2025

# Contents

1. Introduction                                                                                                                                                        1

2. Methodology                                                                                                                                                      2

2.1 equipment

2.2 equations

3. Results                                                                                                                                                                 4

4. Analysis	6

5. Conclusion	7

6. Evaluation	8

7. References	9

introduction

The aim of this report is to gather data on an engine and display its characteristics graphically. Data can then be analysed to evaluate the performance of the engine and its suitability for its intended use. The procedure, equipment used and personal performance in the experiment are also evaluated to identify possible improvements that could be made to the investigation.

2. Methodology

2.1: equipment

A 2001 Honda VTECH 1.6L engine was connected by driveshaft directly from its crankshaft to a Schenk WS230 eddy current dynamometer. This means that the engine speed and dyno speed are equal. The engine runs with the help of auxiliaries such as a fuel supply board and exhaust gas extraction incorporating a fuel flow meter and lambda sensor. Other sensors monitoring the engine include coolant temperature, throttle position (controlled by computer via throttle actuator box) and speed (measured by a sensor on the dynamometer).

The engine spins a toothed rotor and coil inside the outer housing or ‘pendulum body.’ Variable current passed through the coil excites a magnetic field across the pendulum body and toothed rotor. As the teeth spin the constantly changing air gaps formed cause the magnetic poles to reverse back and forth which produces rotational force acting on the pendulum body. Current through the coil is controlled by the test stand controller and an increase in magnetic field strength increases rotational force exerted on the pendulum body and therefore more load on the inner rotor and engine.

figure. 1 Schenk ws230 dynamometer

(Schenk Pegasus, 2001)

The pendulum body is cradle mounted to support its own weight but allow it to spin freely from the electromagnetic force acting on it from the inner rotor. However, it is prevented from turning by a strain gauge sending a voltage reading to the computer which converts it into torque.

To calibrate the dynamometer arms of a known length are bolted to the pendulum body and a known weight is suspended from one end. Two of the same calibration arms are mounted on opposing sided so their moments cancel each other out. The torque from the weight pulls on the strain gauge so a known torque is represented by a voltage reading. This is repeated using different weights for greater accuracy during testing.

Selected data was gathered from a sweep test. In a sweep test the throttle is opened fully and the speed is gradually increased by decreasing load on the engine. A voltage reading from the strain gauge was logged every 50 rpm and converted into load in Newtons.

2.2 equations

T=mepLAn4π

T=dyno load×length of calibration arm

Pkw=2πNT60

capacity=2πr2×L×n

brake SFC=m'bP

brake thermal efficiency=bPSFC×CV

PsP=pstdpa×√tatstd

Pressure std= 1.01325pa

Temperature std= 273K    as according to DIN standard 70020

3. Results

4. Analysis

figure.2

Relationship between specific fuel consumption and engine speed.

Specific fuel consumption is a good indicator of an engine’s efficiency and generally decreases with an increase in engine load (Russo, Dassisti, Lawlor, Olabi, (2012). The graphs show engine speed at peak torque coinciding with the speed at which the engine is the least fuel efficient. The loss of efficiency here can be explained by the maximum volumetric efficiency and therefore higher pressure in the combustion chamber meaning increased pumping losses. Also, A component of the force through the connecting rod acts perpendicular the line of the cylinder against the cylinder wall. At peak torque this force is at its highest therefore so is friction and the force from the expansion of fuel must overcome the greater friction and chamber pressure.

figure.2 relationship between torque and engine speed.

Torque varies with engine speed because intake and exhaust valves have the right size and timing for the engine speed range where peak torque is desired. This investigation found peak torque to be around 2500rpm and at this speed the volumetric efficiency is at its highest, possibly as much as 120% (Robinson, 1994, pg.4). Below this speed, the valves open too early, close too late and stay open too long. This allows some of the fresh charge to be lost into the exhaust and some exhaust gas to remain in the combustion chamber. After a certain speed torque begins to drop because the gases in the combustion chamber and intake take time to accelerate and the time interval is too short and valve area too small for the gas flow needed for effective scavenge.

Figure. 3 relationship between brake power and engine speed.

Power is the rate of work done. If torque was constant and engine speed increased power would increase linearly and this is shown at medium engine speed where torque is relatively constant and power increases linearly as speed does. Torque begins to fall at 3800rpm but not enough for power to drop as the engine speed is still increasing. After 4800rpm torque begins to fall at a faster rate than engine speed increases and power begins to drop.

Figure. 4 Lambda against engine speed

5. Conclusion

The Honda 1.6 VTEC engine is suited to its purpose for use in a small car as the relatively flat torque curve means it is easy to operate without constantly changing gear. The consistent torque is likely to be a result of Honda’s VTEC (variable valve timing and lift electronic control) system.

“Commercial spark ignition engines can work with a brake thermal efficiency of about 30-36%” (Dahham, Rami Y, Wei, Haigiao, Pan, Jiaging, 2022). The Honda engine’s measured maximum brake thermal efficiency was 31% and minimum 21% which is reasonable for 30 years ago. The difference between this and the modern values quoted could be attributed to use of more thermally efficient materials such as aluminium alloys with high silicon content.

Torque should be between 135-145Nm depending on exact model of engine (motorreviewer, 2014). The investigation put the torque produced at a maximum of 103 Nm suggesting the engine is not running at its best. Figure 4 shows data collected from the lambda sensor which proves the engine is running consistently lean and this is likely the reason. Although a lean mixture will burn more completely and the vehicle will be more fuel efficient it will also produce higher nitrogen oxide emission levels (Al-Arkawazi, 2019). An engine fuelled by a high air fuel ratio mixture will also run hotter and could cause damage to an engine from uneven thermal loads, effectively changing clearances and causing excess friction. In its current state, the potential unreliability and harmful emissions make this engine unsuitable for its intended use.

6. Evaluation

An improvement that could be made to the experiment is to gather data from the dynamometer using a step test instead of a sweep test. A step test holds the engine at each RPM interval for a few seconds to give it time to settle and for the sensors to record more accurate data. The downside of this is it puts more strain on the engine and may increase its temperature above its normal operating temperature.

The data recorded may not be a reliable representation of the engines full potential as it was running lean. This was likely caused by the makeshift air filter fitted from a lawnmower and the exhaust system comprising of only the downpipes with no proper silencer.

References

- D Russo, M dassisti, V Lawlor, A.G Olabi  (7.2.2012)

state of the art of biofuels from pure plant oil

https://www.sciencedirect.com/science/article/pii/S1364032112001189

- John Robinson (1994)

Motorcycle tuning: four stroke

Second edition

Newnes

- Honda motor co LTD (1987)

Honda civic coupe CRX chassis maintenance and repair

- Schenk Pegasus (2001)

Technical specification eddy current dynamometer W series

- Dahham, Rami Y, Wei, Haigiao, Pan, Jiaying (2022)

Improving thermal efficiency of internal combustion engines: recent progress and remaining challenges.

Volume 15, issue 17

- Motorreviewer (2014)

Honda D16A (B, V, W, Y, Z) Engine Specs, Problems, Oil, Integra, Civic

- Shamil Ahmed Flamarz Al-Arkawazi (2019)

Analysing and predicting the relationship between air-fuel ratio, lambda and exhaust emissions percentages and values of gasoline fuelled vehicles using versatile and portable emissions measurement system tool

Volume 1, article number 1370