| Drawing Name/No. | CAM_Assign_Task_1 |
| --- | --- |
| Description | Aluminium Plate |
| Program No. | P1000 (main program) |
| Material | Aluminium |
| Program by: | Gareth Williams (1302811) |
| Date | 15th May 2020 |

Tooling and Setup Data T2 - Ǿ12mm End Mill – Number of teeth = 2 T5 - Ǿ4mm Twist Drill - Number of teeth = 2 T6 - Ǿ5mm Twist Drill - Number of teeth = 2 G54 – Datum Point = X 0.0, Y0,0 (Bottom left hand corner of part) Holding Method - I decided to clamp the part on the bed of the machine, raised by parallels so that I was able to drill all the way through the material without drilling into the bed. The location of the clamp was just below the P.C.D holes making sure that I had plenty of clearance for my drill bit. The reason I decided to clamp here was that I could carry out the whole machining operation in one go rather than having to move the job so that I could machine the profile around the outside of the part. I decided to drill the 3 off 4mm holes first using T5 (4mm Twist Drill) in the tooling list. To work out the spindle speed for that drill bit I used the following formula. Spindle Speed (RPM)=1000 x 120π x 4 = 9549 RPM I then worked out the feed rate using the following formula Feed Rate mm permin=Feed rate x Spindle Speed = 0.10 x 9549 = 1909 mm/min I then moved on to the 4 off 5mm holes on a 16mm P.C.D. I used a drilling cycle (G81). Finally I machined the outside profile. I created a canned cycle as this saved a lot of time. The program that I created to machine the profile (P2000 Sub Program1) did not start and finish at the same point. The reason for this is because If I had continued to machine in the X axis after the R10 radius at the bottom right hand corner of the job, the 12mm End Mill would have clashed with my clamping device.

| N | G | M | X | Y | Z | R | F | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | G21 | G90 | G54 | G40 |  |  |  | Metric dimensions, Absolute movements, Datum Point, Cutter radius compensation cancelled in case anything left over from last program. |
| 20 |  | M06 | T05 |  |  |  |  | Tool change, Tool holder position |
| 30 | G43 |  | H05 |  |  |  |  | Tool compensation On, Tool length |
| 40 |  | M03 | M08 |  |  |  | S9549 | Spindle rotation CW, Flood coolant ON, Spindle speed set |
| 50 | G00 |  |  |  | Z20.0 |  |  | Rapid Z axis |
| 60 | G98 |  |  |  |  |  |  | Tool to return to initial level after each canned cycle |
| 70 | G81 |  | X20.0 | Y10.0 | Z-10.0 | R2.0 | F1909 | Drilling Cycle, Z axis start position, Feed rate set |
| 80 |  |  |  | Y20.0 |  |  |  |  |
| 90 |  |  |  | Y30.0 |  |  |  |  |
| 100 | G99 |  |  |  |  |  |  | Tool to return to R point after each canned cycle |
| 110 | G80 |  |  |  |  |  |  | Canned cycle cancelled |
| 120 |  | M06 | T06 |  |  |  |  | Tool Change, Tool Holder position |
| 130 | G43 |  | H06 |  |  |  |  | Tool compensation On, Tool length, |
| 140 |  | M03 | M08 |  |  |  | S7639 | Spindle rotation CW, Flood coolant ON, Spindle speed set |
| 150 | G00 |  |  |  | Z20.0 |  |  | Rapid Z axis |
| 160 | G81 | G98 | X48.50 | Y23.0 | Z-10.0 | R2.0 | F840 L0 | Drilling cycle, Centre point of P.C.D |
| 170 | G70 | I8.0 | J0.0 | L4 |  |  | F840 | Fixed cycle, I = Radius of PCD, J = 3’oclock, L=Number of repetitions |
| 180 | G80 |  |  |  |  |  |  | Canned cycle cancelled |
| 190 | G54 |  |  |  |  |  |  | Datum point |
| 200 |  | M06 | T02 |  |  |  |  | Tool Change, Tool Holder position |
| 210 | G43 |  | H02 |  |  |  |  | Tool compensation On, Tool length, |
| 220 |  | M03 | M08 |  |  |  | S3183 | Spindle rotation CW, Flood coolant ON, Spindle speed set |
| 230 | G41 |  | D02 |  |  |  |  | D = Diameter of tool so knows to shift half to left |
| 240 | G00 |  | X0.0 | Y0.0 |  |  |  | Rapid to datum |
| 250 | G00 |  |  |  | Z2.0 |  |  | Rapid to Z 2.0 |
| 260 | G01 |  |  |  | Z-3.0 |  | F1082 | Feed into work to depth of 3.0mm |
| 270 |  | M98 | P2000 |  |  |  |  | Sub Program called |
| 280 | G01 |  |  |  | Z-6.0 |  |  | Feed into work to depth of 6.0mm |
| 290 |  | M98 | P2000 |  |  |  |  | Sub Program called |
| 300 | G01 |  |  |  | Z-9.0 |  |  | Feed into work to depth of 9.0mm |
| 310 | G80 | M05 | M09 | G40 |  |  |  | Cancel canned cycle, Spindle stop, Coolant off, Cutter radius comp - OFF |
| 320 | G28 | M02 |  |  |  |  |  | Return to reference point, End of program |

P2000 Sub Program 1

| N | G | M | X | Y | Z | R | F | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | G01 |  | X0.0 | Y24.0 |  |  |  | Linear movement |
| 20 | G01 |  | X18.0 | Y42.0 |  |  |  |  |
| 30 | G01 |  | X79.0 | Y42.0 |  |  |  |  |
| 40 | G03 |  | X85.0 | Y36.0 |  | R6.0 |  | Circular Movement CCW |
| 50 | G01 |  |  | Y10.0 |  |  |  |  |
| 60 | G02 |  | X75.0 | Y0.0 |  | R10.0 |  | Circular Movement CW |
| 70 | G01 |  | X65.0 | Y0.0 |  |  |  |  |
| 80 | G01 |  |  | Y-20.0 |  |  |  |  |
| 90 | G01 |  |  |  | Z50.0 |  |  | Move cutter well clear of clamp before next cycle starts so that It does not clash. |
| 100 | M99 |  |  |  |  |  |  | End of sub program |