University of Wales Trinity Saint David.
School of Computing.
Nicholas Jones, P149676.
21/05/2018
Micros, Peripherals and Interfacing (ACDA4000).
Lecturer, Gareth Jones.

Micros, Peripherals & Interfacing.

# Abstract.

MPLAB is an integrated development environment created by Microchip with the purpose to enable developers to create embedded systems by using microcontrollers, the software package houses the ability to write code, test the code with the built-in simulation tool and burn the code to a microcontroller.

A ready built live circuit board was provided, and the schematic was supplied, consulting with the schematic and the necessary datasheets a description of how the circuit works was written up then the circuit was reproduced with in Proteus simulation software, Proteus was used for testing code before burning to the microcontroller. Several tasks were asked to be conducted, theses tasks included correcting a program which contained purpose written error code, once this program was error free it was simulated in MPLAB which gave the ability to see the necessary registers and memory locations which could be analysed to get a feel for how the microcontroller in this case the PIC16F84 is likely to behave. The rest of the tasks asked for the PIC to be programmed in various ways as to enable the two input switches, the 7-segment display and the sounder to be utilised as per tasking.

This tasking has greatly increased knowledge and understanding with low level programming language known as assembly language, interfacing peripherals with microcontrollers, working with binary and hexadecimal number systems, reading datasheets and circuit diagrams.

Contents.

1	Introduction.	1

1.1	MPLAB Overview.	1

2	count.asm errors.	2

3	PIC16F84 assignment board breakdown.	4

3.1	Power supply stage.	4

3.2	Logical stage.	5

4	Details and code for the 7 tasks.	7

4.1	Table showing bit patterns needed for 7 segment display.	8

4.2	Default configuration for PIC16F84.	8

4.3	Task 1.	8

4.3.1	Thought process.	8

4.4	Task 2.	20

4.4.1	Thought process.	20

4.5	Task 3.	30

4.5.1	Thought process.	30

4.6	Task 4.	42

4.6.1	Thought process.	42

4.7	Task 5.	51

4.7.1	Thought process.	51

4.8	Task 6.	62

4.8.1	Thought process.	62

4.9	Task 7.	74

4.9.1	Thought process.	74

5	Conclusion.	84

5.1	Overall thoughts.	84

5.2	Future programming practices.	85

6	References.	86

# Introduction.

## MPLAB Overview.

MPLAB Integrated Development Environment (IDE) is a software package that has been created to enable engineers to develop applications for Microchip microcontrollers and digital signal controllers [1].

MPLAB is an excellent program to use when designing applications for embedded systems, the package is full of useful features to enable coding, simulation and burning to a microcontroller.

Opening the software presents the user with a graphical user interface which is full of tools that can be customised extensively to the user’s preference.

MPLAB gives the ability to write source code in a low level programming language known as assembly language, this then translates directly into machine code, or by using a higher level language such as ‘C’  and MPLAB ‘C’ compiler, ‘C’ a more natural language to use [2]. This assignment involves writing code in assembly language, there by this report will be based on the use of this language.

Within MPLAB is a project wizard which gives the ability to quickly setup a new project, utilising drop down boxes to select what PIC family and tool suite is desired, options on save locations and adding existing files to the new project.

To start writing source code it is as simple as creating a new file and then saving it with an assembler file extension (.asm), MPLAB creates this extension as default then it can be added to the source folder for to be used.

To test the source code then a debug tool is needed, MPLAB SIM is the debug tool used in this instance, this simulating tool displays the contents of program memory, random access memory (RAM), working registers and the register banks. Simulating the code this way is extremely useful as it gives the ability to step through the code line by line and display what is happening within the PIC’s registers.

When MPLAB assembles the code, several files are created. Files that may be of interest are, a listing file with an extension of .LST, this file lists information on memory, source code any errors and other information that may be useful for diagnostics purposes, a hex file which is the machine code that is burnt on to the microcontroller the file extension of .HEX is used here and an error file with a file extension of .ERR which displays any errors that have occurred during a build.

# count.asm errors.

Following the tutorial sheet, a new project was built and the count.asm file which contains deliberate errors was added. Due to these errors building (compiling) the project fails, to rectify these errors the first thing to do is go through the output window which contains the errors of the failed build. The output displays the following errors and warnings.

Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 30 : Symbol not previously defined (status)
Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 31 : Symbol not previously defined (status)
Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 33 : Symbol not previously defined (status)
Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 36 : Symbol not previously defined (status)
Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 57 : Symbol not previously defined (get_count)
Warning[202] E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 74 : Argument out of range.  Least significant bits used.
Error[113]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 75 : Symbol not previously defined (mCont)
Warning[207] E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 89 : Found label after column 1. (en)
Error[122]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 89 : Illegal opcode (d)
Error[129]   E:\2-BENG\YEAR-1\L4 - MICROS, PERIPHERALS AND INTERFACING (ACDA4000)\ASSIGNMENT\COUNT-ERROR\COUNT.ASM 99 : Expected (END)
Reading the first error it starts with the word Error and a code in square brackets, then the file name and location which is the count.asm file, then the line number which is 30 here, then a message of what the compiler thinks is wrong. Below is a breakdown and rectification of each error listed.

Error line 30: Symbol not previously defined (status). This indicates that a variable name status is used in the program and has not been defined (declared) at the beginning of the program. looking at the ‘cpu equates’ section in the program ‘Status’ has been defined, the obvious issue here is a capital S has been used at the beginning of program and a small s used on line 30. This error with the same spelling mistake also occurs on line 31, 33, 36, as these four lines spell ‘status’ the same way it is easier and quicker to correct this at the ‘cpu equates’ section of the program.

Error line 57: Symbol not previously defined (get_count). Here the ‘goto get_count’ instruction is a problem, looking through the code there is a label of similar name ‘get_counnt’ but has been spelt differently to rectify this it is best to change the label name ‘get_counnt’ to match as it reads better and may be called somewhere else in the program.

Warning line 74: Argument out or range. This is a warning message, warning messages on their own usually don’t cause a build to fail, but if these messages are not rectified then the program may not run as intended, the issue here is one to many f’s used in the hex number, changing from ‘0xfff’ to ‘0xff’ will cure this.

Error line 75: Symbol not previously defined (mCont). Looking back at the ‘cpu equates’ section this is a spelling mistake, correcting this to ‘mCount’ at line 75 is best as the same name is used elsewhere in the program.

Warning line 89: Found label after column 1 (en): Here the issue the space between ‘en’ and ‘d’. changing this line to ‘end’ will rectify this.

Error line 89: Illegal opcode (d): Rectifying the warning message above for this line will cure this error.

Error line 99: Expected (END): This will naturally be rectified now the error on line 89 has been fixed.

After rectifying the errors and warnings a rebuild of the program succeeded with no other errors.

Whilst reading through the code line by line it is noticed on line 27 the hex number has one to many zero’s, as this is zero then it should not cause any problems but it’s best practice to rectify to ensure no unintended issues arise whilst running the program.

# PIC16F84 assignment board breakdown.

The schematic for this assignment board can be split in to two stages, the power supply stage and the logical stage.

## Power supply stage.

Schematic for the power supply stage which has been created in Proteus:

Figure 1 Power supply schematic from Proteus.

This part of the circuit is designed to convert a higher supply voltage down to the 5 volts that is needed for the PIC16F84. The supply voltage used in this instance is from a DC power source which is connected at J1 where pin 2 is positive and pin 1 is negative, there is a single throw single pole switch (SW3) which is used as on / off power switch, the main component in this circuit is the LM7805 voltage regulator, the purpose of this regulator is to drop the supply voltage down to a constant 5 volts required by the PIC, consulting with the data sheet [3] for the LM7805 it is recommended that a minimum of 7 volts and a maximum of 25 volts is used as the input, it is also capable of supplying 1.5 Amps or more if provided with good heat sinking properties.

There are two capacitors connected at either positive side of the regulator, the datasheet explains that capacitor (C3) is,

“Required if the regulator is located far from the power supply filter”[3].

and capacitor (C4)

“Although no output capacitor is needed for stability, it does help transient response”[3].

See diagram below taking from data sheet.

[3]

Figure 2 Screen shot, regulator circuit.

The LED (D1) is used to indicate if the power to the PIC is on or off and the resistor (R13) is there to limit the current for the LED.

## Logical stage.

Schematic for the logical stage:

Figure 3 Logical stage schematic.

Pin layout for PIC 16F84 from datasheet:

[3]

Figure 4 PIC16F84 Pin configuration.

The main component in this stage of the circuit is the PIC16F84, this microcontroller when programmed is what does all the calculations and operations that enables inputs in this case from two switches and outputs to the 7-segment display and piezo buzzer to work as required. The data sheet [4] shows this PIC depending on configuration can have up to 13 inputs / outputs with 5 residing on port A (RA0 – 4) and 8 on port B (RB0 – 7). The speed at which the PIC operates known as clock rate is determined by the external crystal oscillator circuit, this part of the circuit consist of a 4MHz crystal oscillator connected across pins 15 and 16 and two capacitors in series with each of these pins and ground. This clock produces a continuous square wave which regulates the timing of instructions within the microcontroller. Not shown in the above schematic, to power up the PIC pin 5 Vss goes to ground (0V) and pin 14 Vdd goes to +5V from the voltage regulator. Pin 4 MCLR is the Master Clear Reset pin, in normal operation this pin needs to be pulled high hence the use of a 4.7K resistor in series between pin 4 and +5V, to activate the reset option then pin 4 would need to be pulled low by way of a switch or other external circuit.

Resetting the device this way points the program counter to the start of program execution or memory at address 0x00 [5]. This pin is not used in this instance so is constantly pulled high.

Pins 17 (RA0) and 18 (RA1) are used as inputs, these pins are pulled high through resistors in normal use and are brought to ground through the normally open button switches if a switch is pressed. The resistors are needed to stop a short circuit from happen between +5V and 0V when a switch is pressed, they can also limit current if a fault were to develop causing the PIC to sink or source excessive power through theses pins.

One output peripheral connected is a 7-segment common anode display, this display is made up of 7 LED’s, the common anode dictates that the 7 LED’s share the positive voltage, to illuminate the display zero volts needs to be applied to one of the other 7 pins, these 7 pins are connected to the PIC pins 6 (RB0) through to 12 (RB6), there is a 220Ω resistor in series with each pin to limit the current.

The sounder used in this circuit is a piezo buzzer, for this sounder to operate then some extra circuitry is used. Driving the sounder, pin 13 (RB7) is connected to the base of the transistor via a current limiting resistor, the electrical characteristics of this type of sounder means it requires a pulse width modulation (PWM) type signal to operate, consulting the data sheet[6] the sounder is effectively seen as a capacitor which needs to be charged and discharged rapidly to vibrate, to accomplish this a recommend value of 1KΩ resistor (R5) is placed in parallel with the sounder.

Screen shot from data sheet showing recommended circuit configuration for piezo buzzer.

[6]

Figure 5 Piezo buzzer circuit configuration.

# Details and code for the 7 tasks.

Several tasks have been asked where by either additional coding or modification of the code from the original error free count.asm file is needed, each task needs to show either a flowchart or pseudo-code and include the assembly language listing (.LST) created by MPLAB.

MPLAB simulation tool will be used along with Proteus simulation to test the program before flashing it to the PIC for live test.

Reading through the tasks its identified that certain numbers and letters will need to be displayed on the 7-segment display, to enable theses alphanumeric symbols to be shown correctly a certain bit pattern needs to be put onto port B of the PIC. To make coding quicker and easier a table with the corresponding bit pattern, hexadecimal number and decimal number has been created. This table will be referenced when creating code as needed.

## Table showing bit patterns needed for 7 segment display.

| 7 seg display |  |  |  |
| --- | --- | --- | --- |
| Display Output | Decimal | Hex | Binary |
| 0 | 64 | 40 | 0100 0000 |
| 1 | 61 | 79 | 0111 1001 |
| 2 | 36 | 24 | 0010 0100 |
| 3 | 48 | 30 | 0011 0000 |
| 4 | 25 | 19 | 0001 1001 |
| 5 | 18 | 12 | 0001 0010 |
| 6 | 2 | 02 | 0000 0010 |
| 7 | 120 | 78 | 0111 1000 |
| 8 | 0 | 00 | 0000 0000 |
| 9 | 24 | 18 | 0001 1000 |
| A | 8 | 08 | 0000 1000 |
| B | 0 | 00 | 0000 0000 |
| C | 70 | 46 | 0100 0110 |
| D | 64 | 40 | 0100 0000 |
| E | 6 | 06 | 0000 0110 |
| F | 14 | 0E | 0000 1110 |
| s | 18 | 12 | 0001 0010 |
| t | 7 | 07 | 0000 0111 |
| o | 64 | 40 | 0100 0000 |
| p | 12 | 0C | 0000 1100 |

## Default configuration for PIC16F84.

As there is a number of tasks asked for this PIC there will be repeated code with regards to initial configuration, this code when written includes the type of PIC used, the cpu equates section which is essentially a memory map and enabling port A as inputs with port B as outputs, this initial configuration will be referenced as header section when writing a flowchart or pseudo-code.

## Task 1.

Modify ‘count.asm’ to enable the green switch to speed up the display refresh and the black button to slow it down.

### Thought process.

The error free count.asm file when run on the PIC has a loop that increments the bit pattern on port B, starting at 0 and counting to 255 in steps of 1, within this loop a pause subroutine is called creating a delay before each display refresh. Thoughts are to create three delay subroutines in which each one will call the pause subroutine x number of times depending on the delay needed for display refresh rate. The original code that updates the display could be turned into a subroutine and called each time the display needs to be updated. A loop is needed to check the switch state of each switch and if none are pressed then call a delay of say 1 second before updating display. If a switch is pressed call a different delay to alter the refresh rate of the display.

#### Pseudo-code.

- Header code for PIC16F84

- Set count to 0

- Display loop

- put count into w then move to portB to update display

- Add 1 to then move to count

- Check for switch press

- If the black switch is pressed pause for less than 1 second to speed up display

- If the green switch is pressed pause for more than 1 second to slow down display

- Else pause for 1 second (default display refresh speed)

- Keep looping

#### Listing file output.

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-1.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 25th April 2018

00004 ; DESCRIPTION:  Alter display refresh speed.

00005 ;                               default refresh is approx 1 second

00006 ;                               holding green switch speeds up refresh by approx 600ms

00007 ;                               holding black switch slows down refresh to approx 2 seconds

00008 ;                               The pause subroutine takes apx. 0.2 sec.

00009 ;                               If pause is called 5 times = 1sec

00010

00011         list    p=16F84         ; select processor type

00012         radix   hex                     ; define radix as hexadecimal

00013         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00014         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00015 ;----------------------------------------------------------------------

00016 ;       cpu equates (memory map)

00000005            00017 portA   equ 0x05                ; port A address

00000006            00018 portB   equ     0x06            ; port B address

00000005            00019 trisa   equ 0x05                ; trisA address in registar bank 1

00000006            00020 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00021 status  equ 0x03                ; status register address

00000005            00022 RP0             equ 5                   ; bit 5 of status register

00000006            00023 RP1     equ     6                       ; bit 6 of status register

0000000C            00024 count   equ     0x0c            ; counter 0 to 255

0000000D            00025 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00026 mCount  equ     0x0e            ; pause routine outer loop counter

00027

00028 ;----------------------------------------------------------------------

00029

0000                  00030         org     0x00                    ; program start at this address

00031

0000                  00032 start

0000   1283           00033         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00034         bcf             status,RP1  ; unimplemented, maintain as 0

00035

0002   1683           00036         bsf             status,RP0      ; select bank 1

0003   3000           00037         movlw   0x00            ; load w with 0x00

0004   0086           00038         movwf   trisb           ; load it into trisb to make portb all outputs

0005   301F           00039         movlw   B'00011111' ; load w with Binary 00011111

0006   0085           00040         movwf   trisa           ; load it into trisa to make porta as inputs

0007   1283           00041         bcf             status,RP0      ; reselect bank 0

0008   0186           00042         clrf    portB           ; clear all lines low

0009   018C           00043         clrf    count           ; initialize counter to 0

00044

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000A                  00045 display

000A   080C           00046         movf    count, w        ; move count to w

000B   0086           00047         movwf   portB           ; move w to port B to update the display

000C   3E01           00048         addlw   1                       ; add 1 to w

000D   008C           00049         movwf   count           ; save w in count

00050

000E   1C05           00051         btfss   portA,0         ; check black switch skip if not pressed

000F   2826           00052         goto    delay2sec       ; call subroutine delay2sec (slowdown refresh)

0010   1C85           00053         btfss   portA,1         ; check green switch skip if not pressed

0011   281C           00054         goto    delay           ; call subroutine delay (speed up refresh)

0012   2820           00055         goto    delay1sec       ; call subroutine delay1sec (default refresh speed)

00056

00057 ;--------------------------------------------------------------------

00058 ;subroutine pause: lets waste some time....

00059 ;

00060 ;inputs: nothing

00061 ;outputs: nothing

00062 ;

00063 ;does: nothing but waste some time

00064 ;corrupts: register W

00065 ;

00066 ;description: two loops using "mCount" and "nCount" register files

00067 ;                         "mCount" is the outer loop counter (255 down to zero)

00068 ;                         "nCount" is the inner loop counter (255 down to zero)

00069 ;

00070 ;--------------------------------------------------------------------

0013                  00071 pause

0013   30FF           00072         movlw   0xff            ; set w = 255 decimal

0014   008E           00073         movwf   mCount          ; mCount = w: outer loop counter

0015                  00074 loadN

0015   30FF           00075         movlw   0xff            ; set w = 255 decimal

0016   008D           00076         movwf   nCount          ; nCount = w: inner loop counter

0017                  00077 decN

0017   0B8D           00078         decfsz  nCount, f       ; nCount--

0018   2817           00079         goto    decN            ; if nCount != 0 then repeat nCount--

00080

0019   0B8E           00081         decfsz  mCount, f       ; else decrement Count

001A   2815           00082         goto    loadN           ; if mCount != 0 then

00083                                                 ; reload nCount to 255 and decrement

001B   0008           00084         return                          ; else exit subroutine

00085

001C                  00086 delay                                   ; delay = 0.6 seconds

001C   2013           00087         call    pause           ; 1 pause = 200ms

001D   2013           00088         call    pause

001E   2013           00089         call    pause

001F   280A           00090         goto    display

00091

0020                  00092 delay1sec                               ; delay1sec = 1 second

0020   2013           00093         call    pause           ; 1 pause = 200ms

0021   2013           00094         call    pause

0022   2013           00095         call    pause

0023   2013           00096         call    pause

0024   2013           00097         call    pause

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

0025   280A           00098         goto    display

00099

0026                  00100 delay2sec                               ; delay2sec = 2 seconds

0026   2013           00101         call    pause           ; 1 pause = 200ms

0027   2013           00102         call    pause

0028   2013           00103         call    pause

0029   2013           00104         call    pause

002A   2013           00105         call    pause

002B   2013           00106         call    pause

002C   2013           00107         call    pause

002D   2013           00108         call    pause

002E   2013           00109         call    pause

002F   2013           00110         call    pause

0030   280A           00111         goto    display

00112

00113         end

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

count                             0000000C

decN                              00000017

delay                             0000001C

delay1sec                         00000020

delay2sec                         00000026

display                           0000000A

loadN                             00000015

mCount                            0000000E

MPASM  5.51                        TASK-1.ASM   5-12-2018  8:35:22         PAGE  6

SYMBOL TABLE

LABEL                             VALUE

nCount                            0000000D

pause                             00000013

portA                             00000005

portB                             00000006

start                             00000000

status                            00000003

trisa                             00000005

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX X---------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    49

Program Memory Words Free:   975

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 2.

Modify ‘count.asm’ to enable the display to show ‘stop’ at a readable pace.

### Thought process.

This task can easily be accomplished by putting the relevant bit pattern onto port B and using a delay subroutine which calls the pause subroutine x number times to slow down the refresh rate of the display, the code will then reside inside an infinite loop. Reference to the table in chapter 4.1 will assist when writing the program.

#### Pseudo-code.

- Header code for PIC16F84

- Load w register with one’s and then move to portB to switch off display

- Start of main loop

- Put binary value for letter ‘s’ into w register

- Move w register to portB to display the letter s

- Delay for 1 second

- Keep looping whilst iterating through each of the letters

#### Listing file output.

MPASM  5.51                        TASK-2.ASM   5-3-2018  11:30:25         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-2.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 25th April 2018

00004 ; DESCRIPTION:  Displays the word 'Stop' on seven segment display.

00005 ;                               The pause subroutine takes apx. 0.2 sec.

00006 ;                               Pause is called 5 times which = 1sec

00007

00008         list    p=16F84         ; select processor type

00009         radix   hex                     ; define radix as hexadecimal

00010         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00011         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00012 ;----------------------------------------------------------------------

00013 ;       cpu equates (memory map)

00000006            00014 portB   equ     0x06            ; port B address

00000006            00015 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00016 status  equ 0x03                ; status register address

00000005            00017 RP0             equ 5                   ; bit 5 of status register

00000006            00018 RP1     equ     6                       ; bit 6 of status register

0000000D            00019 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00020 mCount  equ     0x0e            ; pause routine outer loop counter

00021

00022 ;----------------------------------------------------------------------

00023

0000                  00024         org     0x00                    ; program start at this address

00025

0000                  00026 start

0000   1283           00027         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00028         bcf             status,RP1  ; unimplemented, maintain as 0

00029

0002   1683           00030         bsf             status,RP0      ; select bank 1

0003   3000           00031         movlw   0x00            ; load w with 0x00

0004   0086           00032         movwf   trisb           ; load w into trisb to make portb all outputs

0005   1283           00033         bcf             status,RP0      ; reselect bank 0

0006   30FF           00034         movlw   0xff            ; put all ones into w

0007   0086           00035         movwf   portB           ; load w into portB put lines high to switch off display

00036

0008                  00037 display

0008   3012           00038         movlw   0x12            ; load Letter 's' into w

0009   0086           00039         movwf   portB           ; move w to portB to update the display

000A   201E           00040         call    delay           ; pause

00041

000B   3007           00042         movlw   0x07            ; load Letter 't' into w

000C   0086           00043         movwf   portB           ; move w to portB to update the display

000D   201E           00044         call    delay           ; pause

MPASM  5.51                        TASK-2.ASM   5-3-2018  11:30:25         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00045

000E   3040           00046         movlw   0x40            ; load Letter 'o' into w

000F   0086           00047         movwf   portB           ; move w to portB to update the display

0010   201E           00048         call    delay           ; pause

00049

0011   300C           00050         movlw   0x0C            ; load Letter 'p' into w

0012   0086           00051         movwf   portB           ; move w to portB to update the display

0013   201E           00052         call    delay           ; pause

00053

0014   2808           00054         goto    display         ; keep looping

00055

00056 ;--------------------------------------------------------------------

00057 ;subroutine pause: lets waste some time....

00058 ;

00059 ;inputs: nothing

00060 ;outputs: nothing

00061 ;

00062 ;does: nothing but waste some time

00063 ;corrupts: register W

00064 ;

00065 ;description: two loops using "mCount" and "nCount" register files

00066 ;                         "mCount" is the outer loop counter (255 down to zero)

00067 ;                         "nCount" is the inner loop counter (255 down to zero)

00068 ;

00069 ;--------------------------------------------------------------------

0015                  00070 pause

0015   30FF           00071         movlw   0xff            ; set w = (0xff) 255 decimal

0016   008E           00072         movwf   mCount          ; mCount = w: outer loop counter

0017                  00073 loadN

0017   30FF           00074         movlw   0xff            ; set w = (0xff) 255 decimal

0018   008D           00075         movwf   nCount          ; nCount = w: inner loop counter

0019                  00076 decN

0019   0B8D           00077         decfsz  nCount, f       ; nCount--

001A   2819           00078         goto    decN            ; if nCount != 0 then repeat nCount--

00079

001B   0B8E           00080         decfsz  mCount, f       ; else decrement Count

001C   2817           00081         goto    loadN           ; if mCount != 0 then

00082                                                 ; reload nCount to 255 and decrement

001D   0008           00083         return                          ; else exit subroutine

00084

001E                  00085 delay                                   ; delay for 1 second

001E   2015           00086         call    pause           ; 1 pause = 200ms

001F   2015           00087         call    pause

0020   2015           00088         call    pause

0021   2015           00089         call    pause

0022   2015           00090         call    pause

0023   0008           00091         return

00092

00093         end

MPASM  5.51                        TASK-2.ASM   5-3-2018  11:30:25         PAGE  3

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-2.ASM   5-3-2018  11:30:25         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

decN                              00000019

delay                             0000001E

display                           00000008

loadN                             00000017

mCount                            0000000E

nCount                            0000000D

pause                             00000015

portB                             00000006

MPASM  5.51                        TASK-2.ASM   5-3-2018  11:30:25         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

start                             00000000

status                            00000003

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXX------------ ----------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    36

Program Memory Words Free:   988

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 3.

Modify count.asm so that the 7 segment display counts from 0 to F in hexadecimal.

### Thought process.

This task can be accomplished in the same way as task 2 the only difference is more bit patterns are needed to be put on to port B for the full range of 0 to F again using an infinite loop. Reference to table in chapter 4.1 will be used here.

#### Pseudo-code.

- Header code for PIC16F84

- Load w register with one’s and then move to portB to switch off display

- Start of main loop

- Put binary value for number ‘0’ into w register

- Move w register to portB to display the number ‘0’

- Delay for 1 second

- Keep looping whilst iterating through each of the numbers and letters

#### Listing file output.

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-3.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 25th April 2018

00004 ; DESCRIPTION:  Count from '0' to 'F' at approx 1 second intervals and display on a seven

00005 ;                               segment display.

00006 ;                               The pause subroutine takes apx. 0.2 sec.

00007 ;                               Pause is called 5 times which = 1sec

00008

00009         list    p=16F84         ; select processor type

00010         radix   hex                     ; define radix as hexadecimal

00011         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00012         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00013 ;----------------------------------------------------------------------

00014 ;       cpu equates (memory map)

00000006            00015 portB   equ     0x06            ; port B address

00000006            00016 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00017 status  equ 0x03                ; status register address

00000005            00018 RP0             equ 5                   ; bit 5 of status register

00000006            00019 RP1     equ     6                       ; bit 6 of status register

0000000D            00020 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00021 mCount  equ     0x0e            ; pause routine outer loop counter

00022

00023 ;----------------------------------------------------------------------

00024

0000                  00025         org     0x00                    ; program start at this address

00026

0000                  00027 start

0000   1283           00028         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00029         bcf             status,RP1  ; unimplemented, maintain as 0

00030

0002   1683           00031         bsf             status,RP0      ; select bank 1

0003   3000           00032         movlw   0x00            ; load w with 0x00

0004   0086           00033         movwf   trisb           ; load it into trisb to make portb all outputs

0005   1283           00034         bcf             status,RP0      ; reselect bank 0

0006   30FF           00035         movlw   0xff            ; put all ones into w

0007   0086           00036         movwf   portB           ; load w into portB put lines high to switch off display

00037

0008                  00038 display

0008   3040           00039         movlw   0x40            ; to display '0'

0009   0086           00040         movwf   portB           ; move w to port B to update the display

000A   2042           00041         call    delay1sec       ; delay 1 second

00042

000B   3079           00043         movlw   0x79            ; to display '1'

000C   0086           00044         movwf   portB           ; move w to port B to update the display

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000D   2042           00045         call    delay1sec       ; delay 1 second

00046

000E   3024           00047         movlw   0x24            ; to display '2'

000F   0086           00048         movwf   portB           ; move w to port B to update the display

0010   2042           00049         call    delay1sec       ; delay 1 second

00050

0011   3030           00051         movlw   0x30            ; to display '3'

0012   0086           00052         movwf   portB           ; move w to port B to update the display

0013   2042           00053         call    delay1sec       ; delay 1 second

00054

0014   3019           00055         movlw   0x19            ; to display '4'

0015   0086           00056         movwf   portB           ; move w to port B to update the display

0016   2042           00057         call    delay1sec       ; delay 1 second

00058

0017   3012           00059         movlw   0x12            ; to display '5'

0018   0086           00060         movwf   portB           ; move w to port B to update the display

0019   2042           00061         call    delay1sec       ; delay 1 second

00062

001A   3002           00063         movlw   0x02            ; to display '6'

001B   0086           00064         movwf   portB           ; move w to portB to update display

001C   2042           00065         call    delay1sec       ; delay 1 second

00066

001D   3078           00067         movlw   0x78            ; to display '7'

001E   0086           00068         movwf   portB           ; move w to portB to update display

001F   2042           00069         call    delay1sec       ; delay 1 second

00070

0020   3000           00071         movlw   0x00            ; to display '8'

0021   0086           00072         movwf   portB           ; move w to portB to update display

0022   2042           00073         call    delay1sec       ; delay 1 second

00074

0023   3018           00075         movlw   0x18            ; to display '9'

0024   0086           00076         movwf   portB           ; move w to portB to update display

0025   2042           00077         call    delay1sec       ; delay 1 second

00078

0026   3008           00079         movlw   0x08            ; to display 'A'

0027   0086           00080         movwf   portB           ; move w to portB to update display

0028   2042           00081         call    delay1sec       ; delay 1 second

00082

0029   3000           00083         movlw   0x00            ; to display 'B'

002A   0086           00084         movwf   portB           ; move w to portB to update display

002B   2042           00085         call    delay1sec       ; delay 1 second

00086

002C   3046           00087         movlw   0x46            ; to display 'C'

002D   0086           00088         movwf   portB           ; move w to portB to update display

002E   2042           00089         call    delay1sec       ; delay 1 second

00090

002F   3040           00091         movlw   0x40            ; to display 'D'

0030   0086           00092         movwf   portB           ; move w to portB to update display

0031   2042           00093         call    delay1sec       ; delay 1 second

00094

0032   3006           00095         movlw   0x06            ; to display 'E'

0033   0086           00096         movwf   portB           ; move w to portB to update display

0034   2042           00097         call    delay1sec       ; delay 1 second

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00098

0035   300E           00099         movlw   0x0E            ; to display 'F'

0036   0086           00100         movwf   portB           ; move w to portB to update display

0037   2042           00101         call    delay1sec       ; delay 1 second

00102

0038   2808           00103         goto    display         ; keep looping

00104

00105 ;--------------------------------------------------------------------

00106 ;subroutine pause: lets waste some time....

00107 ;

00108 ;inputs: nothing

00109 ;outputs: nothing

00110 ;

00111 ;does: nothing but waste some time

00112 ;corrupts: register W

00113 ;

00114 ;description: two loops using "mCount" and "nCount" register files

00115 ;                         "mCount" is the outer loop counter (255 down to zero)

00116 ;                         "nCount" is the inner loop counter (255 down to zero)

00117 ;

00118 ;--------------------------------------------------------------------

0039                  00119 pause

0039   30FF           00120         movlw   0xff            ; set w = (0xff) 255 decimal,(Altered to speed up simulation)

003A   008E           00121         movwf   mCount          ; mCount = w: outer loop counter

003B                  00122 loadN

003B   30FF           00123         movlw   0xff                    ; set w = (0xff) 255 decimal, (Altered to speed up simulation)

003C   008D           00124         movwf   nCount          ; nCount = w: inner loop counter

003D                  00125 decN

003D   0B8D           00126         decfsz  nCount, f       ; nCount--

003E   283D           00127         goto    decN            ; if nCount != 0 then repeat nCount--

00128

003F   0B8E           00129         decfsz  mCount, f       ; else decrement Count

0040   283B           00130         goto    loadN           ; if mCount != 0 then

00131                                                 ; reload nCount to 255 and decrement

0041   0008           00132         return                          ; else exit subroutine

00133

0042                  00134 delay1sec                               ; delay for 1 second

0042   2039           00135         call    pause           ; pause = 200ms

0043   2039           00136         call    pause

0044   2039           00137         call    pause

0045   2039           00138         call    pause

0046   2039           00139         call    pause

0047   0008           00140         return

00141

00142         end

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

decN                              0000003D

delay1sec                         00000042

display                           00000008

loadN                             0000003B

mCount                            0000000E

nCount                            0000000D

pause                             00000039

portB                             00000006

MPASM  5.51                        TASK-3.ASM   5-3-2018  11:57:59         PAGE  6

SYMBOL TABLE

LABEL                             VALUE

start                             00000000

status                            00000003

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX

0040 : XXXXXXXX-------- ---------------- ---------------- ----------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    72

Program Memory Words Free:   952

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 4.

Modify count.asm to enable the piezo buzzer to sound when the green button is pressed.

### Thought process.

For this buzzer to create a tone, the output pin on port B, bit 7 will need to be switched on and off at some frequency, here a loop is needed which checks for switch press and once it is pressed switch the buzzer on and off by calling the pause subroutine.

#### Pseudo-code.

- Header code for PIC 16F84

- Load w register with one’s and then move to portB to switch off display and sounder

- Start loop

- Check for switch press if not pressed keep looping

- If switch is pressed set portB bit 7 low

- Delay for certain time

- Set portB bit 7 high

- Delay for certain time

- Loop back to check switch condition

#### Listing file output.

MPASM  5.51                        TASK-4.ASM   5-3-2018  12:08:37         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-4.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 26th April 2018

00004 ; DESCRIPTION:  Press green button creates pwm for piezo buzzer make a tone

00005 ;                               frequency is approx 6.8Khz

00006

00007         list    p=16F84         ; select processor type

00008         radix   hex                     ; define radix as hexadecimal

00009         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00010         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00011 ;----------------------------------------------------------------------

00012 ;       cpu equates (memory map)

00000005            00013 portA   equ 0x05                ; port A address

00000006            00014 portB   equ     0x06            ; port B address

00000005            00015 trisa   equ 0x05                ; trisA address in registar bank 1

00000006            00016 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00017 status  equ 0x03                ; status register address

00000005            00018 RP0             equ 5                   ; bit 5 of status register

00000006            00019 RP1     equ     6                       ; bit 6 of status register

0000000C            00020 count   equ     0x0c            ; counter 0 to 255

0000000D            00021 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00022 mCount  equ     0x0e            ; pause routine outer loop counter

00023

00024 ;----------------------------------------------------------------------

00025

0000                  00026         org     0x00                    ; program start at this address

00027

0000                  00028 start

0000   1283           00029         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00030         bcf             status,RP1  ; unimplemented, maintain as 0

00031

0002   1683           00032         bsf             status,RP0      ; select bank 1

0003   3000           00033         movlw   0x00            ; load w with 0x00

0004   0086           00034         movwf   trisb           ; load it into trisb to make portb all outputs

0005   301F           00035         movlw   B'00011111' ; Load w with

0006   0085           00036         movwf   trisa           ; Load porta to have 5 inputs

0007   1283           00037         bcf             status,RP0      ; reselect bank 0

0008   30FF           00038         movlw   0xff

0009   0086           00039         movwf   portB           ; load w into portB put lines high to switch off display and buzzer

00040

000A                  00041 press

000A   1885           00042         btfsc   portA,1         ; check for green switch press skip if pressed

000B   280A           00043         goto    press           ; not pressed

000C   307F           00044         movlw   B'01111111'     ; load w with buzzer 'on' bit pattern

MPASM  5.51                        TASK-4.ASM   5-3-2018  12:08:37         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000D   0086           00045         movwf   portB           ; switch buzzer on

000E   2013           00046         call    pause           ; delay

000F   30FF           00047         movlw   B'11111111'     ; load w with buzzer 'off' bit pattern

0010   0086           00048         movwf   portB           ; switch off buzzer

0011   2013           00049         call    pause           ; delay

0012   280A           00050         goto    press

00051

00052 ;--------------------------------------------------------------------

00053 ;subroutine pause: lets waste some time....

00054 ;

00055 ;inputs: nothing

00056 ;outputs: nothing

00057 ;

00058 ;does: nothing but waste some time

00059 ;corrupts: register W

00060 ;

00061 ;description: two loops using "mCount" and "nCount" register files

00062 ;                         "mCount" is the outer loop counter (255 down to zero)

00063 ;                         "nCount" is the inner loop counter (255 down to zero)

00064 ;

00065 ;--------------------------------------------------------------------

0013                  00066 pause

0013   3004           00067         movlw   0x4                     ; set w

0014   008E           00068         movwf   mCount          ; mCount = w: outer loop counter

0015                  00069 loadN

0015   3004           00070         movlw   0x4                     ; set w

0016   008D           00071         movwf   nCount          ; nCount = w: inner loop counter

0017                  00072 decN

0017   0B8D           00073         decfsz  nCount, f       ; nCount--

0018   2817           00074         goto    decN            ; if nCount != 0 then repeat nCount--

00075

0019   0B8E           00076         decfsz  mCount, f       ; else decrement Count

001A   2815           00077         goto    loadN           ; if mCount != 0 then

00078                                                 ; reload nCount to 255 and decrement

001B   0008           00079         return                          ; else exit subroutine

00080

001C                  00081 delay1sec

001C   2013           00082         call    pause           ;pause = 200ms

001D   2013           00083         call    pause

001E   2013           00084         call    pause

001F   2013           00085         call    pause

0020   2013           00086         call    pause

0021   0008           00087         return

00088

00089         end

MPASM  5.51                        TASK-4.ASM   5-3-2018  12:08:37         PAGE  3

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-4.ASM   5-3-2018  12:08:37         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

count                             0000000C

decN                              00000017

delay1sec                         0000001C

loadN                             00000015

mCount                            0000000E

nCount                            0000000D

pause                             00000013

portA                             00000005

MPASM  5.51                        TASK-4.ASM   5-3-2018  12:08:37         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

portB                             00000006

press                             0000000A

start                             00000000

status                            00000003

trisa                             00000005

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XX-------------- ----------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    34

Program Memory Words Free:   990

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 5.

Modify count.asm so that the black button changes the frequency of the tone produced which is activated when the green button is pressed.

### Thought process.

This task asks that the buzzer is sounded when the green button is pressed, and then using the black button to alter the frequency of the tone. Two different frequencies are needed for this, so two delay subroutines which call the pause subroutine x number of times can be used here. A loop is needed to check for the green switch press which will then activate the buzzer at a frequency, then, whilst that is operating the buzzer it also needs to start checking the state of the black button and if it is pressed alter the frequency produced from the buzzer

#### Pseudo-code.

- Header code for PIC 16F84

- Load w register with one’s and then move to portB to switch off display and sounder

- Loop to check green button state

- If green button not pressed keep looping

- Loop for green button when pressed

- If green button is pressed set portB bit 7 to output PWM by calling a delay for sounder to operate

- Whilst green button is pressed check if black button is pressed if not pressed loop to green button state loop

- If black button is pressed alter PWM delay to change frequency of sounder

- Check state of black button if pressed keep looping

- If black button not pressed loop back to check green button state

#### Listing file output.

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-5.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 26th April 2018

00004 ; DESCRIPTION:  Pressing green button produces a tone from buzzer.

00005 ;                               Then pressing the black button alters the frequency / changes tone.

00006

00007         list    p=16F84         ; select processor type

00008         radix   hex                     ; define radix as hexadecimal

00009         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00010         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00011 ;----------------------------------------------------------------------

00012 ;       cpu equates (memory map)

00000005            00013 portA   equ 0x05                ; port A address

00000006            00014 portB   equ     0x06            ; port B address

00000005            00015 trisa   equ 0x05                ; trisA address in registar bank 1

00000006            00016 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00017 status  equ 0x03                ; status register address

00000005            00018 RP0             equ 5                   ; bit 5 of status register

00000006            00019 RP1     equ     6                       ; bit 6 of status register

0000000C            00020 count   equ     0x0c            ; counter 0 to 255

0000000D            00021 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00022 mCount  equ     0x0e            ; pause routine outer loop counter

00023

00024 ;----------------------------------------------------------------------

00025

0000                  00026         org     0x00                    ; program start at this address

00027

0000                  00028 start

0000   1283           00029         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00030         bcf             status,RP1  ; unimplemented, maintain as 0

00031

0002   1683           00032         bsf             status,RP0      ; select bank 1

0003   3000           00033         movlw   0x00            ; load w with 0x00

0004   0086           00034         movwf   trisb           ; load w into trisb to make portb all outputs

0005   301F           00035         movlw   B'00011111' ; load w fort porta

0006   0085           00036         movwf   trisa           ; load w into trisa to have 5 inputs

0007   1283           00037         bcf             status,RP0      ; reselect bank 0

0008   30FF           00038         movlw   0xff            ; load w with ones

0009   0086           00039         movwf   portB           ; load w into portB put lines high to switch off display and buzzer

00040

000A                  00041 press

000A   1C85           00042         btfss   portA,1         ; check for green switch press skip if not pressed

000B   200D           00043         call    green           ; calls green subroutine when switch is pressed

000C   280A           00044         goto    press           ; loop

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00045

000D                  00046 green

000D   307F           00047         movlw   B'01111111'     ; load w with buzzer on

000E   0086           00048         movwf   portB           ; switch buzzer on

000F   1C05           00049         btfss   portA,0         ; check for black switch press skip if not pressed

0010   2016           00050         call    black           ; call black if black switch is pressed

0011   2026           00051         call    delay1          ; delay

0012   30FF           00052         movlw   B'11111111'     ; load w with buzzer off

0013   0086           00053         movwf   portB           ; switch off buzzer

0014   2026           00054         call    delay1          ; delay

0015   0008           00055         return                          ; return back to press

00056

0016                  00057 black

0016   30FF           00058         movlw   B'11111111'     ; load w with buzzer off

0017   0086           00059         movwf   portB           ; swich buzzer off

0018   2028           00060         call    delay2          ; delay

0019   307F           00061         movlw   B'01111111'     ; load w with buzzer on

001A   0086           00062         movwf   portB           ; switch on buzzer

001B   2028           00063         call    delay2          ; delay

001C   0008           00064         return                          ; return back to green

00065

00066 ;--------------------------------------------------------------------

00067 ;subroutine pause: lets waste some time....

00068 ;

00069 ;inputs: nothing

00070 ;outputs: nothing

00071 ;

00072 ;does: nothing but waste some time

00073 ;corrupts: register W

00074 ;

00075 ;description: two loops using "mCount" and "nCount" register files

00076 ;                         "mCount" is the outer loop counter (255 down to zero)

00077 ;                         "nCount" is the inner loop counter (255 down to zero)

00078 ;

00079 ;--------------------------------------------------------------------

001D                  00080 pause

001D   3018           00081         movlw   0x18            ; set w

001E   008E           00082         movwf   mCount          ; mCount = w: outer loop counter

001F                  00083 loadN

001F   3018           00084         movlw   0x18            ; set w

0020   008D           00085         movwf   nCount          ; nCount = w: inner loop counter

0021                  00086 decN

0021   0B8D           00087         decfsz  nCount, f       ; nCount--

0022   2821           00088         goto    decN            ; if nCount != 0 then repeat nCount--

00089

0023   0B8E           00090         decfsz  mCount, f       ; else decrement Count

0024   281F           00091         goto    loadN           ; if mCount != 0 then

00092                                                 ; reload nCount to 255 and decrement

0025   0008           00093         return                          ; else exit subroutine

00094

0026                  00095 delay1

0026   201D           00096         call    pause

0027   0008           00097         return

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00098

0028                  00099 delay2

0028   201D           00100         call    pause

0029   201D           00101         call    pause

002A   0008           00102         return

00103

00104         end

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

black                             00000016

count                             0000000C

decN                              00000021

delay1                            00000026

delay2                            00000028

green                             0000000D

loadN                             0000001F

mCount                            0000000E

MPASM  5.51                        TASK-5.ASM   4-29-2018  19:08:25         PAGE  6

SYMBOL TABLE

LABEL                             VALUE

nCount                            0000000D

pause                             0000001D

portA                             00000005

portB                             00000006

press                             0000000A

start                             00000000

status                            00000003

trisa                             00000005

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXX----- ----------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    43

Program Memory Words Free:   981

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 6.

Modify count.asm so that the 7-segment display shows the number of times the green switch has been pressed, counting from 0 to F.

### Thought process.

This program could have been created by using code from the previous tasks, but whilst reading a book called “PIC in practice” it shows the use of look up tables and using the return with literal in W (RETLW) command, after studying and understanding the example given in the book[5 p119-121] it was decided to implement this technique to accomplish this task. What is needed here is a loop to check for the state of the green button, if it is pressed then add one to a counter, call the look up table and update the display with the relevant count, then it needs to check that the button has been released before the next press is initiated, once count has reached F, the counter will need to be reset to start again. A problem that may arise here is switch bounce causing inaccurate counting, to combat this the pause subroutine can be used.

#### Pseudo-code.

- Header code for PIC 16F84

- Load w register with one’s and then move to portB to switch off display

- Switch press loop

- Check condition of green switch press, if not pressed keep looping

- If switch is pressed call pause for switch bounce and then add 1 to count

- Call lookup table with value from count and return value from there using w register

- Put the returned value from w register on to portB to update display

- Check state of green switch keep looping until released

- If released call delay for switch bounce

- Loop back and check switch state for press

#### Listing file output.

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-6.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 27th April 2018

00004 ; DESCRIPTION:  Count the number of green switch presses from 0-F and reset (loop)

00005 ;                               pause routine used for switch bounce

00006

00007         list    p=16F84         ; select processor type

00008         radix   hex                     ; define radix as hexadecimal

00009         include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00010         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00011 ;----------------------------------------------------------------------

00012 ;       cpu equates (memory map)

00000005            00013 portA   equ 0x05                ; port A address

00000006            00014 portB   equ     0x06            ; port B address

00000005            00015 trisa   equ 0x05                ; trisA address in registar bank 1

00000006            00016 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00017 status  equ 0x03                ; status register

00000005            00018 RP0             equ 5                   ; bit 5 of status register

00000006            00019 RP1     equ     6                       ; bit 6 of status register

00000002            00020 pc              equ 0x02                ; Program counter address

00000002            00021 zbit    equ 2                   ; used for checking bit 2 of status register

0000000C            00022 count   equ     0x0c            ; counter 0 to 255

0000000D            00023 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00024 mCount  equ     0x0e            ; pause routine outer loop counter

00025

00026 ;----------------------------------------------------------------------

00027

0000                  00028         org     0x00                    ; program start at this address

00029

0000                  00030 start

0000   1283           00031         bcf             status,RP0      ; clear bank selection bits in the status reg

0001   1303           00032         bcf             status,RP1  ; unimplemented, maintain as 0

00033

0002   1683           00034         bsf             status,RP0      ; select bank 1

0003   3000           00035         movlw   0x00            ; load w with 0x00

0004   0086           00036         movwf   trisb           ; load w into trisb to make portb all outputs

0005   301F           00037         movlw   B'00011111' ; load w for porta

0006   0085           00038         movwf   trisa           ; load w into trisa to make 5 ports inputs

0007   1283           00039         bcf             status,RP0      ; reselect bank 0

0008   3040           00040         movlw   B'01000000'     ; pattern to display zero

0009   0086           00041         movwf   portB           ; display 0

000A   018C           00042         clrf    count           ; set count to 0

00043

000B                  00044 press

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000B   1885           00045         btfsc   portA,1         ; check for green switch press skip if pressed

000C   280B           00046         goto    press           ; not pressed

000D   201A           00047         call    pause           ; switch bounce delay

000E   0A8C           00048         incf    count,f         ; add 1 to count

000F   080C           00049         movf    count,w         ; move count to w

0010   3C10           00050         sublw   .16                     ; count-16, w is altered

0011   1903           00051         btfsc   status,zbit ; checks bit 2 of status register for result from 'sublw' if count minus 16

= 0 skip

0012   018C           00052         clrf    count           ; count = 16 make count = 0 to start again

0013   080C           00053         movf    count,w         ; put count in w

0014   2023           00054         call    table           ; look up table for bit pattern to display

0015   0086           00055         movwf   portB           ; output count to display

00056

0016                  00057 release

0016   1C85           00058         btfss   portA,1         ; is green switch released

0017   2816           00059         goto    release         ; not released

0018   201A           00060         call    pause           ; switch bounce delay

0019   280B           00061         goto    press           ; go back to press

00062

00063 ;--------------------------------------------------------------------

00064 ;subroutine pause: lets waste some time....

00065 ;

00066 ;inputs: nothing

00067 ;outputs: nothing

00068 ;

00069 ;does: nothing but waste some time

00070 ;corrupts: register W

00071 ;

00072 ;description: two loops using "mCount" and "nCount" register files

00073 ;                         "mCount" is the outer loop counter (255 down to zero)

00074 ;                         "nCount" is the inner loop counter (255 down to zero)

00075 ;

00076 ;--------------------------------------------------------------------

001A                  00077 pause

001A   3080           00078         movlw   0x80            ; set w = (0x80)

001B   008E           00079         movwf   mCount          ; mCount = w: outer loop counter

001C                  00080 loadN

001C   3080           00081         movlw   0x80            ; set w = (0x80)

001D   008D           00082         movwf   nCount          ; nCount = w: inner loop counter

001E                  00083 decN

001E   0B8D           00084         decfsz  nCount, f       ; nCount--

001F   281E           00085         goto    decN            ; if nCount != 0 then repeat nCount--

00086

0020   0B8E           00087         decfsz  mCount, f       ; else decrement Count

0021   281C           00088         goto    loadN           ; if mCount != 0 then

00089                                                 ; reload nCount to 255 and decrement

0022   0008           00090         return                          ; else exit subroutine

00091

0023                  00092 table

0023   0782           00093         addwf   pc,f            ; receives value from 'w' which causes program counter to skip x amount

of instructions

0024   3440           00094         retlw   B'01000000'     ; 0 bit pattern returned

0025   3479           00095         retlw   B'01111001'     ; 1 bit pattern returned

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

0026   3424           00096         retlw   B'00100100'     ; 2

0027   3430           00097         retlw   B'00110000'     ; 3

0028   3419           00098         retlw   B'00011001'     ; 4

0029   3412           00099         retlw   B'00010010'     ; 5

002A   3402           00100         retlw   B'00000010'     ; 6

002B   3478           00101         retlw   B'01111000'     ; 7

002C   3400           00102         retlw   B'00000000'     ; 8

002D   3418           00103         retlw   B'00011000'     ; 9

002E   3408           00104         retlw   B'00001000' ; A

002F   3400           00105         retlw   B'00000000'     ; B

0030   3446           00106         retlw   B'01000110'     ; C

0031   3440           00107         retlw   B'01000000'     ; D

0032   3406           00108         retlw   B'00000110'     ; E

0033   340E           00109         retlw   B'00001110'     ; F

00110

00111         end

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

count                             0000000C

decN                              0000001E

loadN                             0000001C

mCount                            0000000E

nCount                            0000000D

pause                             0000001A

pc                                00000002

portA                             00000005

MPASM  5.51                        TASK-6.ASM   5-3-2018  12:36:29         PAGE  6

SYMBOL TABLE

LABEL                             VALUE

portB                             00000006

press                             0000000B

release                           00000016

start                             00000000

status                            00000003

table                             00000023

trisa                             00000005

trisb                             00000006

zbit                              00000002

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXX------------

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    52

Program Memory Words Free:   972

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

## Task 7.

Write a piece of code that sends a fixed frequency pulse train to drive the buzzer, the pulse width must not be fixed.

### Thought process.

This task asks for a fixed frequency pulse train to the sounder, the pulse width should not be fixed it should be constantly changing like a music synthesiser. Initial thoughts were to use code from previous tasks and just alter the delay timings, but initial experimentation with this caused the frequency to alter.

Consulting the data sheet for the PIC 16F84 does not show a PWM module so some research was conducted on how to create PWM from this chip.

A tutorial online was found on how to create PWM, the example code[8] used in the tutorial does not quite accomplish the final outcome asked for in this task, but once an understanding of how the code worked and with a bit of thinking it can easily be adapted to achieve this.

To vary the duty cycle a counter is needed to be incremented with each iteration of on/off cycle, but the total on/off duty cycle cannot exceed 100%, for example first iteration on = 10 then off would need to = 90, next iteration on = 20 off would need to = 80 then on = 30 off = 70 etc, so the code will need to increment the on time and subsequently subtract that from 100% to attain the off time.

#### Pseudo-code.

- Header code for PIC 16F84

- Load w register with one’s and then move to portB to switch off display and sounder

- Set duty cycle counter to 0 which = 0%, 100% would = 255

- Add 1 to duty cycle counter then put it into a temporary register

- Check that duty cycle counter does not equal 255 if so then reset counter

- Switch on buzzer for x amount of time governed by duty cycle counter and loop until a temporary register is decremented to zero

- Setup ‘off’ duration by subtracting duty cycle from 255 put that result into temporary register

- If the result = 0 then reset program and start again

- If result is greater than 0 switch off buzzer for x amount of time determined by the result from previous calculation by looping until temporary register is decremented to zero

- Loop until duty ‘on’ cycle has reached 100% then reset and start again

#### Listing file output.

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 Task-7.asm

00002 ; AUTHOR:               Nicholas Jones

00003 ; DATE:                 27th April 2018

00004 ; DESCRIPTION:  Sounder produces a tone with a fixed frequency but altering PWM duty cycle

00005

00006         list    p=16F84         ; select processor type

00007         radix   hex                     ; define radix as hexadecimal

00008         #include "p16f84.inc"

00001         LIST

00002

00003 ;==========================================================================

00004 ;  MPASM PIC16F84 processor include

00005 ;

00006 ;  (c) Copyright 1999-2013 Microchip Technology, All rights reserved

00007 ;==========================================================================

00008

00206         LIST

2007   3FF1           00009         __CONFIG _CP_OFF & _WDT_OFF & _PWRTE_ON & _XT_OSC

00010 ;----------------------------------------------------------------------

00011 ;       cpu equates (memory map)

00000006            00012 portB   equ     0x06            ; port B address

00000006            00013 trisb   equ 0x06                ; trisB address in register bank 1

00000003            00014 status  equ 0x03                ; status register

00000005            00015 RP0             equ 5                   ; bit 5 of status register

00000006            00016 RP1     equ     6                       ; bit 6 of status register

00000002            00017 zbit    equ 2                   ; used for checking bit 2 of status register

0000000C            00018 duty    equ     0x0c            ; used for duty cycle counter

0000000D            00019 temp    equ     0x0d            ; for pwm on or pwm off time receives value from duty

00020

00021 ;----------------------------------------------------------------------

00022

0000                  00023         org     0x00                    ; program start at this address

00024

0000                  00025 start

0000   1283           00026         bcf             status,RP0      ; clear bank selection bits in the status reg (bit 5)

0001   1303           00027         bcf             status,RP1      ; unimplemented, maintain as 0

00028

0002   1683           00029         bsf             status,RP0      ; select bank 1

0003   3000           00030         movlw   0x00            ; load w with 0x00

0004   0086           00031         movwf   trisb           ; load it into trisb to make portb all outputs

0005   1283           00032         bcf             status,RP0      ; reselect bank 0

0006   30FF           00033         movlw   0xff            ; load w with ones

0007   0086           00034         movwf   portB           ; Set all lines high to switch off display and sounder.

0008   018C           00035         clrf    duty            ; set duty to 0

00036

0009                  00037 rstrt

0009   080C           00038         movf    duty,w          ; put duty into w

000A   3CFF           00039         sublw   .255            ; minus this from w to check if duty has reached maximum (alter this for

on cycle range 1 (1%) to 255 (100%)

000B   1903           00040         btfsc   status,zbit     ; if last is not = to 0 skip

000C   018C           00041         clrf    duty            ; reset duty counter to 0

000D   0A8C           00042         incf    duty,f          ; add 1 to duty

000E   080C           00043         movf    duty,w          ; put duty into w

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000F   008D           00044         movwf   temp            ; put w into temp

0010   1386           00045         bcf             portB,7         ; switch on buzzer

00046

0011                  00047 pwmON                                   ; duty cycle on

0011   0000           00048         nop                                     ; more nop's = lower frequency

0012   0000           00049         nop

0013   0000           00050         nop

0014   0000           00051         nop

0015   0000           00052         nop

0016   0000           00053         nop

0017   0000           00054         nop

0018   0000           00055         nop

0019   0000           00056         nop

001A   0000           00057         nop

001B   0000           00058         nop

001C   0000           00059         nop

001D   0000           00060         nop

001E   0000           00061         nop

001F   0000           00062         nop

0020   0B8D           00063         decfsz  temp,f          ; minus 1 from temp skip if zero

0021   2811           00064         goto    pwmON           ; loop until temp = 0

00065

00066 ;; this does some calculations with duty enabling freq to stay same

00067 ;; but duty cycle alters

0022   30FF           00068         movlw   .255            ; put 255 into w don't alter this

0023   008D           00069         movwf   temp            ; put w into temp

0024   080C           00070         movf    duty,w          ; put original duty count into w

0025   028D           00071         subwf   temp,f          ; sub w (duty) from temp

0026   3CFF           00072         sublw   .255            ; 255 minus w (duty)

0027   1903           00073         btfsc   status,zbit     ; check result from sublw

0028   2809           00074         goto    rstrt           ; duty has reached maximum value go back to restart

0029   1786           00075         bsf             portB,7         ; switch off buzzer

00076

002A                  00077 pwmOFF                                  ; duty cycle off

002A   0000           00078         nop                                     ; need to have same amount of nop's here as above to

002B   0000           00079         nop                                     ; keep frequency consistent

002C   0000           00080         nop

002D   0000           00081         nop

002E   0000           00082         nop

002F   0000           00083         nop

0030   0000           00084         nop

0031   0000           00085         nop

0032   0000           00086         nop

0033   0000           00087         nop

0034   0000           00088         nop

0035   0000           00089         nop

0036   0000           00090         nop

0037   0000           00091         nop

0038   0000           00092         nop

0039   0B8D           00093         decfsz  temp,f          ; minus 1 from temp skip if zero

003A   282A           00094         goto    pwmOFF          ; loop until temp = 0

003B   2809           00095         goto    rstrt           ; go back to restart

00096

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00097         end

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

C                                 00000000

DC                                00000001

EEADR                             00000009

EECON1                            00000088

EECON2                            00000089

EEDATA                            00000008

EEIE                              00000006

EEIF                              00000004

F                                 00000001

FSR                               00000004

GIE                               00000007

INDF                              00000000

INTCON                            0000000B

INTE                              00000004

INTEDG                            00000006

INTF                              00000001

IRP                               00000007

NOT_PD                            00000003

NOT_RBPU                          00000007

NOT_TO                            00000004

OPTION_REG                        00000081

PCL                               00000002

PCLATH                            0000000A

PORTA                             00000005

PORTB                             00000006

PS0                               00000000

PS1                               00000001

PS2                               00000002

PSA                               00000003

RA0                               00000000

RA1                               00000001

RA2                               00000002

RA3                               00000003

RA4                               00000004

RB0                               00000000

RB1                               00000001

RB2                               00000002

RB3                               00000003

RB4                               00000004

RB5                               00000005

RB6                               00000006

RB7                               00000007

RBIE                              00000003

RBIF                              00000000

RD                                00000000

RP0                               00000005

RP1                               00000006

STATUS                            00000003

T0CS                              00000005

T0IE                              00000005

T0IF                              00000002

T0SE                              00000004

TMR0                              00000001

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  5

SYMBOL TABLE

LABEL                             VALUE

TMR0IE                            00000005

TMR0IF                            00000002

TRISA                             00000085

TRISA0                            00000000

TRISA1                            00000001

TRISA2                            00000002

TRISA3                            00000003

TRISA4                            00000004

TRISB                             00000086

TRISB0                            00000000

TRISB1                            00000001

TRISB2                            00000002

TRISB3                            00000003

TRISB4                            00000004

TRISB5                            00000005

TRISB6                            00000006

TRISB7                            00000007

W                                 00000000

WR                                00000001

WREN                              00000002

WRERR                             00000003

Z                                 00000002

_CONFIG                           00002007

_CP_OFF                           00003FFF

_CP_ON                            0000000F

_FOSC_EXTRC                       00003FFF

_FOSC_HS                          00003FFE

_FOSC_LP                          00003FFC

_FOSC_XT                          00003FFD

_HS_OSC                           00003FFE

_IDLOC0                           00002000

_IDLOC1                           00002001

_IDLOC2                           00002002

_IDLOC3                           00002003

_LP_OSC                           00003FFC

_PWRTE_OFF                        00003FFF

_PWRTE_ON                         00003FF7

_RC_OSC                           00003FFF

_WDTE_OFF                         00003FFB

_WDTE_ON                          00003FFF

_WDT_OFF                          00003FFB

_WDT_ON                           00003FFF

_XT_OSC                           00003FFD

__16F84                           00000001

__DEBUG                           1

duty                              0000000C

portB                             00000006

pwmOFF                            0000002A

pwmON                             00000011

rstrt                             00000009

start                             00000000

status                            00000003

temp                              0000000D

MPASM  5.51                        TASK-7.ASM   5-3-2018  12:52:19         PAGE  6

SYMBOL TABLE

LABEL                             VALUE

trisb                             00000006

zbit                              00000002

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXX----

2000 : -------X-------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    60

Program Memory Words Free:   964

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

# Conclusion.

## Overall thoughts.

Writing code for any microcontroller can be done using a high-level language such as ‘C’ where by the compiler does a lot of the ‘housekeeping’ in the background, when writing code in assembly language more thought is needed with the logical order of each instruction and more code may be needed as one line of code with a high-level language can equate to multiple lines of instructions when written in assembly. Completion of this assignment has enabled a better understanding of, effectively how ‘dumb’ microcontrollers and microprocessors are and need instructions fed to them in a way that the human brain would not even think about. Relating directly to the PIC microcontrollers a better understanding of the use of registers and how to manipulate bit’s in each of these registers has been attained, experience whilst completing this assignment with consulting and interpreting datasheets for the PIC and other components has been gained, knowledge on how to interface components safely with a microcontroller and getting them to operate has been achieved.

## Future programming practices.

All tasks have been completed, as time has gone on more experience has been gained with assembly language and thinking logically, some of the tasks given more time and thought could possibly be optimised in a number of ways such as substituting some instructions for others that may be more efficient enabling less code needed there by saving on memory space, restructure the order of instruction execution enabling the program to flow quicker and more efficient also creating subroutines that could be used for all tasks attempted and any future projects.

# References.

[1]	“MPLAB- X IDE | Microchip Technology Inc.” [Online]. Available: http://www.microchip.com/mplab/mplab-x-ide. [Accessed: 22-Mar-2018].

[2]	Johansen Darrel, “An introduction to the MPLAB IDE.” .

[3]	Texas Instruments, “LM340, LM340A and LM7805 family wide Vin 1.5-A fixed voltage regulators,” Data Sheet. 2017.

[4]	“PIC16F84A Data Sheet.”

[5]	“Master Clear Pin External Reset - Developer Help.” [Online]. Available: http://microchipdeveloper.com/8bit:mclr. [Accessed: 02-Apr-2018].

[6]	TDK, “Piezoelectronic Buzzers ( without circuit ) PS Series ( Pin Terminal / Lead ),” no. July, pp. 1–7, 2008.

[7]	D. W. Smith, PIC in Practice Project-Based Approach., Second. Elsevier Ltd, 2006.

[8]	“PWM Signal with PIC 16F84.” [Online]. Available: http://www.pages.drexel.edu/~kws23/tutorials/PWM/PWM.html. [Accessed: 03-May-2018].