University of Wales Trinity Saint David

Motorsports Engineering Level 6

Analytical Assessment of Engine Internal Components

Preston Thomas

2305918

22/03/25

2550 Words

Contents

1.0	Introduction	2

2.0 Methodology	2

2.1 Pressure and Kinematics	2

2.2 Force Calculation at Each Component	4

2.3 Stress and Fatigue Modelling	5

3.0 Results (Base RPM Components)	6

3.1 Midpoint of Conrod	6

3.2 Gudgeon Pin	7

3.3 Conrod Bolt	8

3.4 Critical Analysis	9

4.0 9000 RPM Bolt Analysis	10

4.1 Interpolation to 9000 RPM	10

4.2 New Bolt Stresses and FSF	10

4.3 Critical Discussion	10

5.0 Bolt Redesign to Restore FSF	11

5.1 Redesign Approach	11

5.2 New Geometry and FSF	11

5.3 Discussion	13

6.0 Torque Analysis: Standard vs Cross-Plane	13

6.1 Instantaneous Torque Calculation	13

6.2 Torque Comparison	14

6.2.1 Standard Configuration	14

6.2.2 Cross plane Configuration	15

6.3 Suitability for Applications	16

6.3.1 Standard Configuration	16

6.3.2 Cross Plane Configuration	16

7.0 Conclusion	16

8.0 Extra Work	17

9.0 References	17

# Introduction

This report presents a comprehensive analysis of the forces, stresses and fatigue safety factors (FSFs) experienced by three key components in an engine: the connecting rod midpoint, the gudgeon pin and the connecting rod bolts. The forces acting on each component are calculated using pressure against crank angle data and kinematic equations. These are then used to determine stresses and evaluate fatigue performance using Goodman Diagrams.

The study assesses the impact of increasing engine speed to 9000 RPM, using interpolated pressure data to recalculate the FSF for the conrod bolts. Based on this analysis, a redesign of the bolt geometry is proposed to restore or improve the original FSF. The report then evaluates the instantaneous torque output of the engine, under both standard and cross plane crankshaft configurations, comparing their torque ripple characteristics and discussing their suitability for different applications.

This investigation aims to provide a thorough, data driven assessment of component durability and performance under varying operational conditions, with a focus on both mechanical integrity and dynamic torque behaviour (Heywood, 1988).

# 2.0 Methodology

## 2.1 Pressure and Kinematics

The analysis begins with a pressure against crank angle dataset for a single cylinder over a 720° cycle (-360° to 359°). The data was interpolated using excels FORCAST.LINEAR function to ensure a uniform 1° resolution. This allowed precise alignment between pressure and crank angle for subsequent force calculations.

The gas force on the piston was calculated as:

Fgasθ=Pθ∙Ap=Pθ∙πD24

Where P(θ) is the instantaneous pressure and D is the piston diameter.

Figure 1 Gas Force Vs Crank Angle Graph

Inertia forces were derived from the second derivative of piston displacement, using the slider crank mechanism.

Figure 2 Inertia Force Vs Crank Angle Graph

Finertia=mp∙r∙ω2(cosθ+rlcos2θ)

The net force transmitted through the connecting rod is:

Fnet=Fgas-Finertia

Figure 3 Net Force Vs Crank Angle Graph

## 2.2 Force Calculation at Each Component

- The connecting rod midpoint was modelled as a member in axial loading with force equal to Fnet.

Figure 4 Rod Force Vs Crank Angle Graph

Figure 5 Side Force Vs Crank Angle Graph

- The gudgeon pin was treated as a simply supported beam under central loading with maximum bending moment:

Mmax=Fgas∙lp4

- The conrod was analysed under peak tensile loading at top dead centre (TDC) where inertia forces are biggest:

Fbolt=mr∙r∙ω22

## 2.3 Stress and Fatigue Modelling

Stress was calculated using (Budynas & Nisbett, 2011):

- Axial: σ=FA

- Bending: σ=M∙cI

- Bearing: σb=Fd∙t

Fatigue safety was assessed using the Goodman relation (Budynas & Nisbett, 2011):

σaσe+σmσu=1n

Figure 6 Pressure Vs Crank Angle Graph

# 3.0 Results (Base RPM Components)

## 3.1 Midpoint of Conrod

The connecting rod midpoint experiences axial loading transmitted from the piston through the rod. Using the force against crank angle data, the resulting axial stress cycle at the rod midpoint ranges from 205.98 MPa to -30.29 MPa, with a mean stress of 87.84 MPa and a stress range of 236.27 MPa.

When these values are plotted on a Goodman diagram using the materials allowable endurance stress (820.94 MPa), the resulting fatigue safety factor is 3.47. An FSF above 3 indicates that the rod has a comfortable margin against failure. In practical terms, this means the rod is strong enough to handle the cyclic loading without being excessively overbuilt. It strikes a good balance between durability and weight.

Figure 7 Rod Stress Vs Crank Angle Graph

Figure 8 Goodman Diagram - Conrod Midpoint

## 3.2 Gudgeon Pin

The gudgeon pin carries the oscillating load between the piston and the small end of a connecting rod. At base RPM, the pin experiences relatively modest stresses: ranging from 42.21 MPa to -13.23 MPa, with a mean stress of 14.49 MPa and a range of 55.44 MPa.

With an allowable endurance of 886.96 MPa the FSF works out to 15.99. This means during the load cycle the gudgeon pin is unaffected. While this ensures reliability it also hints that the pin could potentially be lighter without compromising the safety of the component.

Figure 9 Pin Stress Vs Crank Angle Graph

Figure 10 Goodman Diagram - Gudgeon Pin

## 3.3 Conrod Bolt

The conrod bolt is the component that is most stressed by high speeds, especially during the exhaust TDC event where inertia forces peak. At base RPM the bolt sees a maximum tensile stress of 476.69 MPa and a minimum of -70.10 MPa giving a mean stress of 203.29 and a range of 546.79 MPa.

Using the allowable endurance stress of 807.04 MPa the FSF is 1.48. This is significantly lower than the rod and pin. An FSF around 1.5 means the bolt is operating close to its fatigue limit, even at the base RPM condition. Bolts are often the limiting factor as they carry large tensile loads and have relatively small cross-sectional areas.

Figure 11 Bolt Stress Vs Crank Angle Graph

FSF = 1.48

Figure 12 Goodman Diagram - Bolt at 6000 RPM

## 3.4 Critical Analysis

The data examined demonstrates how the three components have very different levels of fatigue robustness:

- Gudgeon Pin (FSF=15.99)

  - The pin is massively over-engineered for the loads it sees. Its stresses are tiny compared to its material capacity.

- Connecting Rod (FSF=3.47)

  - The rod is comfortably safe. It has a healthy fatigue margin without being excessively heavy.

- Conrod Bolt (FSF=1.48)

  - The bolt is the limiting factor. It’s close to its fatigue limit and will be the first component to suffer if RPM increases.

This creates a hierarchy which engineers will consider during the design process. Failures of the rod and pin are catastrophic whereas bolts are designed closer to their limits to save weight. Therefore, designers often prioritise the rod and pin to ensure these are less likely to fail. However, as engine speed increases, the bolt becomes the first component that needs redesign or material upgrade.

# 4.0 9000 RPM Bolt Analysis

## 4.1 Interpolation to 9000 RPM

To understand how the conrod bolt behaves at higher engine speeds the pressure against crank angle data was extended to 9000 RPM. As cylinder pressure is fundamentally tied to crank angle, rather than time the shape of the pressure, trace stays the same. What changes dramatically is the inertia force which increases with the square of engine speed.

At 9000 RPM the angular velocity is three times higher than at the base condition. Since inertia force scales with angular velocity squared (Heywood, 1988), the bolt experiences roughly nine times more inertia loading. This is why bolts are usually the first components to struggle as RPM increases, they carry the full tensile load when the piston decelerates sharply at TDC.

The same interpolation method used earlier was applied here, ensuring the pressure data aligned correctly with the crank angle domain. Once the pressure trace was mapped the inertia term was recalculated using the updated angular velocity, giving new stress cycle for the bolt at 9000 RPM.

## 4.2 New Bolt Stresses and FSF

At 9000 RPM the bolts stress cycle becomes significantly more severe. Using calculated values:

- Max Stress: 603.21 MPa

- Min Stress: -172 MPa

- Mean Stress: 215.6 MPa

- Range Stress: 775.21 MPa

- Allowable Stress: 795.96 MPa

- Fatigue Safety Factor: 1.03

This is a dramatic drop from the base RPM FSF of 1.48. An FSF of 1.03 means the bolt is operating right on the edge of its fatigue limit. The bolt would likely survive a short burst at 9000 RPM, but if this was sustained it would likely lead to fatigue cracking (Propulsion & Power Research, 2018).

## 4.3 Critical Discussion

The reduction in FSF at 9000 RPM highlights that the bolt is dominated by inertia forces and not by gas forces. While the pressure trace barely changes with RPM, the inertia term grows explosively. This is why the rod and pin remain safe at high speed, but the bolt quickly approaches its fatigue limit.

An FSF of 1.03 is concerning as it means the bolt is only just meeting the fatigue requirement with almost no margin for (typically a FSF of over 1.5 is ideal):

- Manufacturing variation

- Preload Inconsistency

- Lubrication Changes

- Thermal effects

- Material defects

- Real world dynamic loads

In a high performance or endurance engine this would be unacceptable. Even in short duration motorsport use, it would be risky unless the bolt is replaced frequently.

This analysis reinforces a common engineering reality, if you want an engine to rev higher, the bolts are usually the first components that need redesigning.

# 5.0 Bolt Redesign to Restore FSF

## 5.1 Redesign Approach

With the bolts fatigue safety factor dropping to 1.03 at 9000 RPM, it’s clear that the existing geometry is no longer adequate for high-speed operation. An FSF this low means the bolt is essentially sitting on the fatigue limit, any small variation in preload, lubrication or material quality, could push it into failure territory.

To bring the FSF back up to a safe level (ideally matching or exceeding the base RPM FSF of 1.48), the most direct and effective approach is to increase the bolts tensile load capacity. Since the bolt is dominated by axial tension the simplest way to achieve this is by increasing the diameter which increases the cross-sectional area and reduces stress.

σ=FA=4∙Fπ∙d2

To reduce the maximum stress from 603.21 MPA to a safer level, the diameter must be increased until the stress falls far enough below the allowable endurance stress of 795.96 MPa to restore the desired FSF.

## 5.2 New Geometry and FSF

With the new diameter and core area applied to the 9000 RPM loading conditions the bolts stress cycle improves noticeably:

- New max stress: 446.25 MPa

- New min stress: -127.25 MPa

- New mean stress:159.5 MPa

- New stress range: 573.5 MPa

- New allowable endurance stress: 846.45 MPa

These updated values produce a new fatigue safety factor of 1.48. This is almost identical to the bolts original FSF at base RPM (1.48), meaning the redesign successfully restores the bolt to its previous level of fatigue robustness. Using goal seek on excel it was also possible to attempt to improve the FSF to a higher level of 2.

| Case | Diameter (mm) | Max Stress (MPa) | FSF |
| --- | --- | --- | --- |
| Base RPM | 7.35 | 476.69 | 1.48 |
| 9000 RPM | 7.35 | 603.21 | 1.03 |
| 9000 RPM Redesigned | 8.55 | 446.25 | 1.48 |
| 9000 RPM 2 FSF | 9.76 | 342.33 | 2 |

Table 1 Bolt Redesign Comparison Table

Figure 13 Bolt FSF Comparison Across All Instances Bar Chart

Figure 14 Goodman Diagram - Bolt Redesign at 9000 RPM

## 5.3 Discussion

The redesigned bolt geometry achieves exactly what was needed: it restores the fatigue safety factor to a safe and familiar level without requiring a dramatic increase in size. This is a practical and efficient solution, especially for engines where packaging constraints limit how much the bolt can grow.

However, the redesign does come with a few considerations:

- Slightly increased mass: Even a small increase in bolt diameter adds weight which can influence engine balance and bearing loads at high RPM.

- Manufacturing implications: A new bolt size may require updated tooling or different thread-rolling dies.

- Material alternatives: In high-end engines, designers often combine modest geometric changes with stronger materials to achieve higher strength without adding mass.

Alternative approaches such as improving the material selection or optimising preload could help improve the fatigue performance without increasing the size.

# 6.0 Torque Analysis: Standard vs Cross-Plane

## 6.1 Instantaneous Torque Calculation

To compare how the engine behaves under different crankshaft layouts, the instantaneous torque was calculated for both a standard inline-four configuration and a cross plane configuration. The starting point for both cases was the single cylinder torque trace (Heywood, 1988), obtained by multiplying the net force at the crankpin by crank radius:

TθFnetθ∙r

Once the single cylinder torque curve was generated it was replicated and phase shifted to simulate a four cylinder engine. The phase offsets were:

- Standard configuration: 0°, 180°, 360°, 540°

- Cross plane configuration: 0°, 90°, 270°, 450°

These offsets were applied using Excels INDEX and MOD functions to wrap the 720 point torque array cleanly. Each shifted torque trace was then added together point by point to produce the total instantaneous torque over a full 720° cycle.

This method captures the real dynamic behaviour of the engine: the torque pulses, the gaps between firing events and the overall smoothness of the torque delivery.

## 6.2 Torque Comparison

When the total torque curves were plotted for both configurations, the differences were immediately clear.

### 6.2.1 Standard Configuration

The standard inline four layout fires every 180°, which produces four evenly spaced torque pulses. This results in a relatively smooth torque curve with predictable peaks and lows.

Figure 15 Base Cylinders Torque Vs Crank Angle Graph

Figure 16 9000 RPM Cylinder Torques Vs Crank Angle Graph

### 6.2.2 Cross plane Configuration

The cross plane layout used here introduces uneven firing intervals, most notably a 270° gap between two of the firing events. This creates a noticeable dip in torque before the next cylinder fires which increases the overall torque ripple.

Even though cross plane designs are often associated with smoother sound and better low end behaviour in V8 engines, the same benefits do not automatically carry over to an inline four. In this case the uneven spacing makes the torque delivery less smooth.

Figure 17 Instantaneous Torque Vs Crank Angle - Standard Vs Cross-Plane (Base RPM) Graph

Figure 18 Instantaneous Torque Vs Crank Angle - Standard Vs Cross-Plane (9000 RPM) Graph

## 6.3 Suitability for Applications

The results make the strengths and weaknesses of each configuration clear:

### 6.3.1 Standard Configuration

- More consistent torque delivery

- Better suited for high RPM performance where smooth torque is essential for drivetrain stability, traction and component longevity

This is why most high revving inline four engines (motorcycles, race engines and performance cars) use this layout.

### 6.3.2 Cross Plane Configuration

- Uneven firing intervals create a distinctive torque pattern

- More suited to applications where character, sound or low RPM behaviour matter more than absolute smoothness

In an inline four the cross plane layout doesn’t naturally deliver the smoothness it provides in a V8. Without counterbalancing or additional design changes, it tends to introduce more fluctuation rather than less.

# 7.0 Conclusion

This assignment looked at how the main parts of a high revving four stroke engine deal with load, stress and fatigue and how different crankshaft layouts affect torque. Using pressure against crank angle data and a full kinematic model, the forces on the connecting rod, gudgeon pin and conrod bolts were turned into realistic stress cycles and assessed with goodman fatigue diagrams.

The results showed a clear pattern: the gudgeon pin faces little stress, the connecting rod has a comfortable margin and the conrod bolt is the part that starts to struggle. At 9000 RPM the bolts FSF drops close to one, putting it right on the edge of fatigue failure. Increasing the bolts core diameter from around 7.35mm to 8.55mm brought the FSF back to the original level of 1.48, showing how even small geometry tweaks can make a big difference, though they come with the usual impacts like added mass and packaging limits.

The torque analysis added a broader view of engine behaviour. The standard crankshaft produced smoother more predictable torque, while the cross plane layout created uneven firing intervals and a noticeable torque dip caused by a 270° gap between combustion events. This led to less consistent torque values making the cross plane setup less suited to high RPM where stability really matters. Overall, the work highlights how combining mechanical modelling fatigue analysis and torque evaluation reveals which components are genuinely at risk, demonstrating how design choices shape durability and performance. It reinforces a familiar engineering truth which is that, when revs climb, inertia forces take over and the smallest stressed parts are usually the first ones to need redesigning.

# 8.0 Extra Work

Material choice makes a significant difference to how engine parts deal with fatigue, especially when they are operating at high RPM and sustaining loads every cycle. Titanium and aluminium alloys are known for coping well with tough fatigue conditions at higher temperatures mainly because their microstructure can be tuned to slow down crack growth and stop cracks forming too early (Ellard et al, 2023). Titanium alloys in general also hold up well in very high cycle fatigue but they can be quite sensitive to surface defects or small manufacturing issues, so even a small flaw can cut the life down more than you would expect (Wu et al, 2024). Aluminium alloys are lighter which helps reduce inertia forces, but they usually have lower fatigue limits and can degrade faster in harsh environments so they are not the best choice for heavily loaded parts, unless weight saving is the main priority (Ellard et al, 2023). To summarise, picking a material requires balance, where it must be considered whether strength or weight is more of a priority.

# 9.0 References

Budynas, R.G. and Nisbett, J.K. (2011) Shigley’s Mechanical Engineering Design. 9th edn. New York: McGraw-Hill. Available at: academia.edu

Ellard, J.J.M., et al. (2023) ‘Low cycle fatigue behaviour of titanium aluminium based intermetallic alloys: A short review’, Metals, 13(8), p. 1491. Available at: ResearchGate

Heywood, J.B. (2018) Internal Combustion Engine Fundamentals. New York: McGraw-Hill. Available at: UWTSD Library

Wu, X., et al. (2024) ‘Titanium alloy materials with very high cycle fatigue: A review’, Materials, 17(12), p. 2987. Available at: ResearchGate