University of Wales Trinity Saint David

School of Engineering

Vehicle Handling Optimisation Through Simulation

Lee Stewart 1700860

Automotive Engineering

03/05/2022

Advanced Vehicle Dynamic Behaviour

ACEA7003

Contents

Table List	2

1.0.	Introduction	3

2.0. Methodology	4

2.1. Design of Experiment – DOE (Box Behnken)	5

2.2. Specific Experimental Design	5

3.0. Simulation Results	8

4.0. Vehicle Optimisation & Performance Analysis/Comparison	9

4.1. Tyre Response Modelling	12

5.0 Conclusion	14

6.0. References	15

# Table List

Table 1: Table displaying the upper and lower limits of the variables chosen for optimisation.	7

Table 2: Table displaying the variables to be used in each simulation.	8

`Table 3: Table displaying the results from the default settings simulation run. (Results in red)	9

Table 4: Table displaying the minimum and maximum value of the chosen metrics for optimisation as well as the simulation run in which it was achieved.	10

Table 5: Table Displaying the setting used to produce the MIN & MAX values displayed in Table 4. (Red=Undesirable response, Green = Desirable response, Orange = Mixed response.)	10

Table 6: Table displaying the optimised suspension system values.	12

Table 7: Table comparing results from the default run to a run utilising the optimised suspension setting.	12

Table 8: Table displaying the maximum vertical load and maximum self-aligning torque seen in the default and optimised runs.	13

# Introduction

As the automotive and transport sectors evolve through time, the approach to develop new systems and increase the performance of existing components has changed dramatically. In the modern era of engineering simulation techniques have become paramount in the product design process and performance analysis. These simulations techniques allow engineers to analyse the likely performance of a system or component without undergoing a time consuming and often expensive physical experiment. This implementation of simulation techniques often results in a more streamlined and less time intensive design or system set up process to be achieved.  (Hindsbo, 2018)

This report will focus on the optimisation of a vehicle suspension system through simulation software. The software chosen to aid in the simulation process of the vehicle set up in this case was chosen to be the Car Real Time Simulator developed by VI-Grade. This software was chosen as it allows for a multibody simulation of the dynamic response of vehicles being subjected to specific manoeuvres dictated by the operator. (VI-Grade, 2020) This allows the response of a vehicle to be predicted analysed in depth. This approach to vehicle dynamic response analyse allows vehicle designers to predict the response of a vehicle with various set up configurations without the laborious process of manually adjusting the vehicle’s configuration and a time-consuming testing process to reach the desired results.

The parameters chosen to optimise the vehicle set up in this case are the spring rate of the front and rear suspension systems, the damping  rate at each corner of the vehicle, and the anti-roll bar stiffness. To determine the ideal set up, the vehicle will be subjected to a common dynamic manoeuvre used to determine the performance of a vehicle and the outlined parameters adjusted until the most appropriate vehicle response is reached. The vehicle used throughout the simulation and optimisation process will be a common sedan type vehicle already existing within the VI-Grade software. The reason for this is due to the vehicle type being an extremely common feature on roads within the UK and the need for these types of vehicles to be as comfortable as possible whilst maintaining the required performance characteristics to function safely in varying conditions. (Toogood, 2021)

# 2.0. Methodology

To optimise the suspension set up of a typical sedan type vehicle, various combinations of the parameters outlined in the previous section should be simulated and the performance of which compared and analysed. This will allow the best combination of spring rates, damping rates, and anti-roll bar stiffnesses to be determined for the vehicle’s specific application. To allow the determination of the ideal set up to be determined in a suitable amount of simulation runs, a Box Behnken type design will be used.

## 2.1. Design of Experiment – DOE (Box Behnken)

A Box Behnken experimental methodology is a design of experiment technique developed by  George Box and Donald Behnken in 1960 and if applied correctly allows an experiment to be completed in a more efficient manner. (Anup K.Das, 2018)  This is because a Box Behnken experimental design will set up an experiment which allows the effect of multiple variables to be determined an analysed. This is achieved by determining the upper and lower limits of the variables which are to be investigated and monitoring the effect of various combinations of values within these limits. The software package chosen to design an experiment which maps the effect of various suspension characteristics was chosen to be the Design Expert software available from Stat-Ease.

Applying this DOE process to an experiment such as the one outlined throughout this report will, allow the amount of simulation runs needed to yield the necessary results to be vastly reduced. This will in turn reduce the computational effort and time required to complete the necessary analysis. (National Institute of Standards and Technology, US, 2022)

## 2.2. Specific Experimental Design

In the case of the experiment to be performed to optimise the suspension set up of a common sedan type vehicle, the specific variables to be investigated are listed below.

- Front spring rate.

- Rear spring rate.

- Front damping rate.

- Rear damping rate.

- Front anti-roll bar stiffness.

- Rear anti-roll bar stiffness.

The response of the vehicle with various combinations of the listed variables will be determined through various metrics. The metrics in which vehicle permeance will be determined are listed below.

- Manoeuvre completion time.

- Maximum velocity achieved throughout the manoeuvre.

- Maximum lateral acceleration of the vehicle.

- Maximum longitudinal acceleration of the vehicle.

- Maximum steering angle needed to complete the manoeuvre.

- Vertical tyre loadings throughout the manoeuvre.

These metrics were chosen as part of the analysis because it was thought that they would allow the performance of the vehicle regarding occupant comfort and overall vehicle response throughout the manoeuvre to be determined and analysed. It should be noted the dynamic manoeuvre chosen for the investigation in this case is a 7-cone slalom that takes place over a distance of 30.5 metres. This manoeuvre was chosen as completing a manoeuvre such as this will produce relatively large lateral accelerations which could result in large amount of vehicle roll and a degree of pitching around the vehicles centre of gravity. (S.Rakheja, 2000) This is usually an unwanted response of a vehicle when subjecting to steering inputs. Reducing these accelerations when the vehicle is subjected to an extreme dynamic manoeuvre such as a slalom should result in a more comfortable ride for its occupants when utilised for its specific application. As well as this the completion time and maximum velocity reached are to be analysed to ensure the performance of the vehicle has not been comprised too much when determining the ideal set up for occupant comfort. It is also worth noted that the vehicle’s entrance speed to the manoeuvre and the initial selected gear remains constant regardless of the variable combination being analysed. The upper and lower limits of the variables chosen to optimise are listed below in Table 1.

| Component | Upper Limit | Lower Limit |
| --- | --- | --- |
| Front Spring Rate (N/mm) | 100 | 50 |
| Rear Spring Rate (N/mm) | 70 | 10 |
| Front Damping Rate (Ns/m) | 9000 | 2000 |
| Rear Damping Rate (Ns/m) | 5000 | 100 |
| Front Anti Roll Bar Stiffness (N/mm) | 75 | 5 |
| Rear Anti Roll Bar Stiffness (N/mm) | 25 | 5 |

Table 1: Table displaying the upper and lower limits of the variables chosen for optimisation.

These variables along with the upper and lower limits were then entered into the Design Expert software aforementioned to allow the experimental procedure to be designed and determined. The designed experimental procedure is shown below in Table 2.

Table 2: Table displaying the variables to be used in each simulation.

# 3.0. Simulation Results

To allow for a comparison and performance analysis of the sedan type vehicle after optimisation of the variable outlined in the previous sections, a simulation run was completed using the default setting provided by the VI-Grade software. The results gathered from the default setting run and the results gathered from the optimisation experiment are displayed in the table below.

| Run | Spring Rate (Front) | Damping rate (Front) | Anti-Roll spring rate (Front) | Spring Rate (Rear) | Damp Rate (Rear) | Anti-Roll spring rate (Rear) | Lateral Accel | Long Accel | Completion Time | Max Speed | Steering Input max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N/mm | Ns/m | N/mm | N/mm | Ns/m | N/mm | m/s^2 | m/s^2 | s | km/h | Degrees |
| Default | 75 | 3000 | 41 | 39.54 | 1500 | 11.36 | 8.93 | 1.28 | 11.3 | 89.57 | 280.8791 |
| 1 | 75 | 9000 | 75 | 40 | 5000 | 15 | 8.7309 | 1.1772 | 11.29 | 89.47 | 250.0317 |
| 2 | 75 | 2000 | 40 | 40 | 5000 | 25 | 9.2214 | 3.0411 | 11.08 | 92.39 | 193.3884 |
| 3 | 100 | 5500 | 40 | 70 | 100 | 15 | 9.3195 | 2.4525 | 11.1 | 91.49 | 216.2746 |
| 4 | 75 | 5500 | 40 | 40 | 2550 | 15 | 8.829 | 1.2753 | 11.29 | 87.51 | 245.4545 |
| 5 | 100 | 5500 | 5 | 40 | 2550 | 25 | 8.829 | 1.5696 | 11.26 | 89.61 | 213.9860 |
| 6 | 75 | 5500 | 75 | 70 | 2550 | 25 | 9.0252 | 1.3734 | 11.05 | 91.42 | 222.5683 |
| 7 | 75 | 5500 | 75 | 10 | 2550 | 5 | 8.3385 | 1.4715 | 11.8 | 85.5 | 346.1538 |
| 8 | 75 | 2000 | 75 | 40 | 5000 | 15 | 8.7309 | 1.1772 | 11.29 | 89.51 | 244.3102 |
| 9 | 100 | 5500 | 40 | 10 | 5000 | 15 | 8.9271 | 1.3734 | 11.26 | 89.61 | 222.5683 |
| 10 | 75 | 5500 | 40 | 40 | 2550 | 15 | 8.829 | 1.2753 | 11.29 | 89.51 | 245.4545 |
| 11 | 100 | 9000 | 40 | 10 | 2550 | 15 | 8.4366 | 1.2753 | 11.53 | 87.52 | 282.0724 |
| 12 | 50 | 5500 | 75 | 40 | 2550 | 25 | 9.0252 | 1.7658 | 11.1 | 91.18 | 258.6141 |
| 13 | 75 | 5500 | 5 | 10 | 2550 | 25 | 8.829 | 1.4715 | 11.26 | 89.65 | 209.9809 |
| 14 | 50 | 5500 | 40 | 10 | 100 | 15 | 8.5347 | 1.2753 | 11.54 | 87.5 | 291.2269 |
| 15 | 75 | 2000 | 40 | 40 | 100 | 5 | 8.9271 | 1.1772 | 11.54 | 87.68 | 272.3458 |
| 16 | 100 | 5500 | 40 | 70 | 5000 | 15 | 9.0252 | 2.4525 | 11.09 | 91.55 | 202.5429 |
| 17 | 50 | 2000 | 40 | 10 | 2550 | 15 | 9.3195 | 3.2373 | 11.08 | 92.54 | 190.5276 |
| 18 | 75 | 5500 | 5 | 70 | 2550 | 5 | 9.0252 | 3.2373 | 11.28 | 90.57 | 346.1538 |
| 19 | 75 | 2000 | 5 | 40 | 100 | 15 | 9.2214 | 1.4715 | 11.5 | 87.79 | 230.0063 |
| 20 | 100 | 5500 | 5 | 40 | 2550 | 5 | 9.4176 | 3.5316 | 11.34 | 90.16 | 346.7260 |
| 21 | 50 | 9000 | 40 | 70 | 2550 | 15 | 9.2214 | 3.2373 | 11.08 | 92.47 | 311.8245 |
| 22 | 100 | 5500 | 75 | 40 | 2550 | 25 | 9.2214 | 1.6677 | 11.1 | 91.13 | 264.3356 |
| 23 | 75 | 5500 | 5 | 70 | 2550 | 25 | 8.9271 | 1.5696 | 11.48 | 87.71 | 234.5835 |
| 24 | 50 | 5500 | 75 | 40 | 2550 | 5 | 8.5347 | 1.2753 | 11.54 | 87.56 | 277.4952 |
| 25 | 75 | 5500 | 40 | 40 | 2550 | 15 | 8.829 | 2.2563 | 12.01 | 89.51 | 645.9631 |
| 26 | 75 | 9000 | 5 | 40 | 5000 | 15 | 8.7309 | 1.5696 | 11.80 | 89.67 | 644.2466 |
| 27 | 75 | 9000 | 40 | 40 | 5000 | 25 | 9.2214 | 2.4525 | 11.82 | 91.56 | 633.9478 |
| 28 | 75 | 9000 | 40 | 40 | 5000 | 5 | 8.9271 | 1.5696 | 12.01 | 89.35 | 648.8239 |
| 29 | 50 | 2000 | 40 | 70 | 2550 | 15 | 9.2214 | 3.3354 | 11.98 | 92.31 | 633.9478 |
| 30 | 50 | 5500 | 5 | 40 | 2550 | 25 | 8.5347 | 1.5696 | 11.77 | 87.78 | 656.8340 |
| 31 | 50 | 9000 | 40 | 10 | 2550 | 15 | 8.829 | 1.5696 | 12.52 | 89.34 | 648.8239 |
| 32 | 75 | 9000 | 40 | 40 | 100 | 25 | 9.4176 | 3.1392 | 12.01 | 92.45 | 633.9478 |
| 33 | 75 | 2000 | 40 | 40 | 100 | 25 | 9.4176 | 3.1392 | 11.98 | 92.41 | 634.5200 |
| 34 | 100 | 2000 | 40 | 70 | 2550 | 15 | 9.3195 | 2.5506 | 12.01 | 91.61 | 634.5200 |
| 35 | 75 | 2000 | 75 | 40 | 100 | 15 | 8.7309 | 1.2753 | 12.25 | 89.5 | 646.5352 |
| 36 | 75 | 5500 | 40 | 40 | 2550 | 15 | 8.829 | 1.2753 | 10.76 | 89.51 | 645.9631 |
| 37 | 100 | 5500 | 75 | 40 | 2550 | 5 | 8.5347 | 1.2753 | 10.92 | 87.52 | 660.2670 |
| 38 | 75 | 5500 | 40 | 40 | 2550 | 15 | 8.829 | 1.2753 | 11.2 | 89.5 | 645.9631 |
| 39 | 100 | 9000 | 40 | 70 | 2550 | 15 | 9.2214 | 2.4525 | 11.2 | 91.51 | 634.5200 |
| 40 | 75 | 9000 | 5 | 40 | 100 | 15 | 9.4176 | 3.1392 | 10.75 | 90.58 | 645.9631 |
| 41 | 75 | 9000 | 75 | 40 | 100 | 15 | 8.7309 | 1.2753 | 10.74 | 89.46 | 646.5352 |
| 42 | 50 | 5500 | 40 | 70 | 100 | 15 | 9.4176 | 3.1392 | 10.94 | 92.56 | 633.9478 |
| 43 | 75 | 5500 | 5 | 10 | 2550 | 5 | 8.829 | 1.6677 | 11.16 | 87.66 | 662.5556 |
| 44 | 50 | 5500 | 40 | 70 | 5000 | 15 | 9.2214 | 3.2373 | 11 | 92.44 | 633.3757 |
| 45 | 75 | 2000 | 5 | 40 | 5000 | 15 | 8.9271 | 3.2373 | 10.74 | 90.48 | 644.8188 |
| 46 | 75 | 9000 | 40 | 40 | 100 | 5 | 9.0252 | 1.2753 | 10.76 | 87.59 | 657.4062 |
| 47 | 75 | 5500 | 75 | 70 | 2550 | 5 | 8.7309 | 1.2753 | 11.14 | 89.49 | 632.8035 |
| 48 | 100 | 5500 | 40 | 10 | 100 | 15 | 8.5347 | 1.2753 | 11.2 | 87.46 | 678.5759 |
| 49 | 75 | 5500 | 75 | 10 | 2550 | 25 | 8.6328 | 1.1772 | 11.76 | 89.48 | 665.4164 |
| 50 | 100 | 2000 | 40 | 10 | 2550 | 15 | 8.3385 | 1.2753 | 11.25 | 87.56 | 634.5200 |
| 51 | 75 | 2000 | 40 | 40 | 5000 | 5 | 8.9271 | 1.5696 | 11.78 | 89.43 | 662.5556 |
| 52 | 50 | 5500 | 40 | 10 | 5000 | 15 | 8.7309 | 1.1772 | 11.29 | 89.59 | 667.1328 |
| 53 | 50 | 5500 | 5 | 40 | 2550 | 5 | 8.6328 | 1.5696 | 11.97 | 87.84 | 631.0870 |

`Table 3: Table displaying the results from the default settings simulation run. (Results in red)

# 4.0. Vehicle Optimisation & Performance Analysis/Comparison

To optimise the sedan type vehicle’s suspension settings the settings which produced the best and worst results must first be analysed to determine why the vehicle is responding in the way the results dictate.

|  | Lat Acceleration (m/s^2) | Long Acceleration (m/s^2) | Time Taken (s) | Max Velocity (km/h) | Max Steering Input (Degrees) |
| --- | --- | --- | --- | --- | --- |
| Min | 8.3385 (Run 7) | 1.1772 (Run8) | 10.74 (Run 41) | 85.5 (Run 7) | 190.5276 (Run 17) |
| Max | 9.4176 (Run 20) | 3.5316 (Run 20) | 12.52 (Run 31) | 92.56 (Run 42) | 678.5759 (Run 48) |

Table 4: Table displaying the minimum and maximum value of the chosen metrics for optimisation as well as the simulation run in which it was achieved.

| Run | Spring Rate (Front) | Damping rate (Front) | Anti-Roll spring rate (Front) | Spring Rate (Rear) | Damp Rate (Rear) | Anti-Roll spring rate |
| --- | --- | --- | --- | --- | --- | --- |
| # | N/mm | Ns/m | N/mm | Ns/m | N/mm | N/mm |
| Run 7 | 75 | 5500 | 75 | 10 | 2550 | 5 |
| Run 8 | 75 | 2000 | 75 | 40 | 5000 | 15 |
| Run 17 | 50 | 2000 | 40 | 10 | 2550 | 15 |
| Run 41 | 75 | 9000 | 75 | 40 | 100 | 15 |
| Run 20 | 100 | 5500 | 5 | 40 | 2550 | 5 |
| Run 31 | 50 | 9000 | 40 | 10 | 2550 | 15 |
| Run 42 | 50 | 5500 | 40 | 70 | 100 | 15 |
| Run 48 | 100 | 5500 | 40 | 10 | 100 | 15 |

Table 5: Table Displaying the setting used to produce the MIN & MAX values displayed in Table 4. (Red=Undesirable response, Green = Desirable response, Orange = Mixed response.)

It is evident from the above tables that adjusting the outlined variables can have a dramatic effect on vehicle response. It is shown in the above tables that the settings used during Run 20 would result in the vehicle producing the greatest amount of lateral & longitudinal acceleration recorded throughout the experiment. This is to be expected as the there is a large spring rate seen in the front suspension with a relatively high value for spring rate seen in the rear suspension. As well as this, although there is a moderate amount of damping  present within the front and rear suspension systems, the small amount of stiffness present within the front and rear anti-roll bars exacerbates the amount of lateral acceleration experienced by the vehicle. (Sachin Sunil Kelkar, 2021) This is an unwanted phenomenon in automotive vehicle development as it will likely result in an exaggerated amount of vehicle roll which could cause the occupants of the vehicle to experience a reduced level of comfort during vehicle operation. (BlueStar Inspections, 2022) This is likely caused by the inconsistency of the settings used at the front and rear of the vehicle. It is shown that there is a high spring rate coupled with a large amount of damping present in the front suspension, but a small amount of damping coupled with a relatively low spring rate present in the rear suspension system. (Bangji Zhang, 2016)

As aforementioned, this inconsistency throughout the suspension systems will result is a vehicle with unfamiliar handling characteristics with poor control. It is also evident from analysis of the above tables that run 31 produced the slowest manoeuvre completion time. This is likely due to the small spring rate coupled with a moderate amount of damping seen in the rear suspension. This would cause a slow reaction time in the rear suspension and result in the rear of the vehicle feeling relatively soft. (Chua, 2019) From this, the vehicle would be quite difficult to control when subjected to a side to side loading present when completing a dynamic manoeuvre such as a slalom course. It is also shown that the amount of steering angle needed to complete the manoeuvre when utilising the settings used in run 48 is quite extreme. This would be an unwanted feature in road vehicles as the extreme amount of steering input needed to complete the manoeuvre would be difficult for most vehicle operators to achieve. (Moltke, 2022)

It is shown that the settings used throughout  run 8 would reduce the amount of longitudinal acceleration experienced by the vehicle. This would be desirable in a vehicle development aspect as the reduced amount of longitudinal acceleration will likely result in a reduced pitching moment around the vehicles centre of gravity. This would increase the vehicles occupant comfort characteristics. The reason for this reduced longitudinal acceleration seen when utilising the setting used throughout run 8 is due to the moderate spring rates within the front and rear suspension systems coupled with a relatively large amount of damping. This increased damping force present within the suspension systems would slow the pitching movement of the vehicle down. Therefore a reduction in the number of oscillations of the suspension systems will increase the speed in which the coupled systems return to rest and increase the driveability of the vehicle. (Dixon, 2007) It is evident from Tables 4 & 5 that the setting used during run 41 results in the vehicle completing the manoeuvre in the least amount t of time. This is due to the high level of stiffness seen throughout the vehicle due to the high spring rates seen in the front and rear suspension systems coupled with the large amount of stiffness present within the anti-roll bars. (Time Attack LTD, 2014) Although the settings outlined in Table 5 allow the vehicle to complete the manoeuvre in the least amount of time, this is not necessarily a necessary feature needed for a road going vehicle such as the one being analysed. This is because a vehicle with high levels of stiffness can feel harsh to its occupants and subsequently reduce the amount of comfort experienced.

It is apparent from table 5 that the settings utilised during run 7 resulted in the vehicle experiencing the least amount of lateral acceleration seen throughout the entirety of the experiment. This will in turn reduce the amount of vehicle roll around its roll centre and result in a comfortable ride. However, it should be noted that although the lateral acceleration seen from the vehicle was reduced, the settings used for run 7 also resulted in the vehicle producing the slowest maximum velocity seen throughout the experiment. Although this is not necessarily the most relevant metric when it comes to optimising a road going vehicles suspension settings, a reduction in velocity of over 7 km/h over a distance of only 30.5 meters is relatively significant. This relatively low maximum velocity is likely due to the softness of the rear suspension spring coupled with the small amount of stiffness seen in the rear anti roll bar. This softness experienced in the rear of the vehicle will result in the vehicle taking more time to return to its resting position after being subjected to a lateral or longitudinal acceleration. This will in turn result in an inconstant feeling when competing dynamic manoeuvres and likely result in a reduced maximum velocity experienced during the manoeuvre. (Time Attack LTD, 2014) It should also be noted the settings utilised during run 8 allows the vehicle to reach a maximum velocity of 89.51 km/h as well as producing the least amount of longitudinal acceleration. Based on this and analysis of the results shown above, it is the opinion of the author that the most optimum settings lie in between the settings used for runs 7 & 8.

| Run | Spring Rate (Front) | Damping rate (Front) | Anti-Roll spring rate (Front) | Spring Rate (Rear) | Damp Rate (Rear) | Anti-Roll spring rate (Rear) |
| --- | --- | --- | --- | --- | --- | --- |
| # | N/mm | Ns/m | N/mm | Ns/m | N/mm | N/mm |
| Run 7 | 75 | 5500 | 75 | 10 | 2550 | 5 |
| Run 8 | 75 | 2000 | 75 | 40 | 5000 | 15 |
| Optimum | 75 | 3750 | 75 | 25 | 3775 | 10 |

Table 6: Table displaying the optimised suspension system values.

It is shown in Table 6 that the values used from run 7 & 8 which produced the most desirable results have been used to determine the optimum suspension settings for the sedan type vehicle. The results from a simulation run utilising these optimum settings are shown below and compared to that of the run using the default settings provided by VI-Grade.

|  | Lat Acceleration (m/s^2) | Long Acceleration (m/s^2) | Time Taken (s) | Max Velocity (km/h) | Max Steering Input (Degrees) |
| --- | --- | --- | --- | --- | --- |
| Default | 8.93 | 1.28 | 11.3 | 89.57 | 280.8791 |
| Optimised | 8.39 | 1.21 | 11.04 | 89.42 | 226.6759 |

Table 7: Table comparing results from the default run to a run utilising the optimised suspension setting.

Although other set ups have yielded more desirable results for certain metrics, it is evident from analysis of the above table that compromising slightly on the performance of these metrics allows for a more well-rounded vehicle set up. It is shown in the above table that there has been an improvement in the performance of the vehicle in most of the metrics analysed with the only exception being a slightly reduced maximum velocity during the dynamic manoeuvre . From this, it is evident that when the sedan type vehicle is set up with the optimised suspension settings shown in table 6, the result is a vehicle with superior comfort and better performance in most metrics analysed.

## 4.1. Tyre Response Modelling

Although the tyres being used throughout the experiment and optimisation process remains constant, a Pajeka tyre curve was generated for the default run and the run utilising the optimum setting outlined in table 6. This is to allow for an analysis into the loading characteristics seen in the tyres with various suspension settings. A Pajeka tyre model allows for an analysis into the vertical load experienced in the tyre, the slip angle or slip ratio, and the output of force from the tyre at varying camber inclinations. (Pacejka, 1987) However developing a Pajeka model for tyres can often prove to be extremely difficult as many difficult to obtain variables are necessary to reach useable accurate results. These variables include tyre stiffness, rolling resistance, camber, and various coefficients which require applying iterative equations to reach the value. (Noble, 2022) It is apparent from this that modelling the response of a tyre can be a relatively difficult and laborious process, and due to this it would be better determined through computational methods rather than classic mathematical modelling techniques. Due to this, it was thought prudent to use the default tyre available through the VI-Grade software as the required variables are programmed into the software. This will allow for an analysis into the loading characteristics experienced when utilising the suspension setting outlined in the previous section. The results generated from this tyre testing are displayed below in Table 8.

| Tyre & Setup | Max Vertical Load | Max Self-Aligning Torque |
| --- | --- | --- |
| Front Left | 5600 N | 200 Nm |
| Rear Left (Default) | 6200 N | 120 Nm |
| Front Right (Default) | 5600 N | 200 Nm |
| Rear Right (Default) | 6200 N | 120 Nm |
| Front Left (Optimised) | 4800 N | 160 Nm |
| Rear Left (Optimised) | 5400 N | 100 Nm |
| Front Right (Optimised) | 4800 N | 160 Nm |
| Rear Right (Optimised) | 5400 N | 100 Nm |

Table 8: Table displaying the maximum vertical load and maximum self-aligning torque seen in the default and optimised runs.

It is evident from analysis of Table 8 that the maximum vertical loads experienced in the tyres have been reduced when implementing the optimised suspension settings outlined in previous sections. This is due to there being less load transfer and therefore less weight and subsequently a reduced vertical load being transferred into the tyre during the dynamic manoeuvre. This results in a reduced self-aligning torque experienced in the wheels and therefore a reduction in force seen at the steering wheel occurs as a result. This subsequently results in a more comfortable driving experience. (Mike Blundell, 2015)

# 5.0 Conclusion

It has been shown throughout this report that altering the suspension setting can have a dramatic effect on a vehicle’s response to a dynamic manoeuvres such as a slalom. This was discovered by applying a Box Behnken design of experiment approach to determining an ideal suspension set up for a common sedan type vehicle. To establish the most optimum setting, a simulation was set up to allow a simulated vehicle to negotiate a dynamic manoeuvre with various suspension set ups. The software chosen to complete these simulations had a common sedan type vehicle already built into the software. This allowed for easy adjustment of the settings being investigated and helped streamline the experimental process. It was found that the optimum setting was a mixture of some of the settings used within two of the simulation runs. The results gathered from the simulation utilising the determined optimum settings was then compared to that of a run utilising the default settings provided by the modelling software. It was found that these optimum setting resulted in an improvement in the majority of the metrics chosen for analysis. As well as this, the vertical loads and self-aligning torque seen at the vehicles wheels was also analysed when completing a slalom manoeuvre with the ideal set up and the default set up. The results of which were then compared. It was shown that the simulation where the ideal settings were used seen an improvement in each metric chosen for analysis. To allow for a more in-depth analysis and a better optimisation of the vehicle’s suspension system, it would be prudent to perform a different dynamic manoeuvre to determine if the ideal set up for a slalom manoeuvre would also be the ideal set up for other scenarios.

The most optimum suspension system settings found throughout the experiment are as follows.

- Spring rate of the front suspension – 75 N/mm

- Damping rate of the front suspension – 3750 Ns/m

- Spring rate of the front anti-roll bar – 75 N/mm

- Spring rate of the rear suspension – 25 N/mm

- Damping rate of the rear suspension – 3775 Ns/m

- Spring rate of the rear anti-roll bar – 10 N/mm

# 6.0. References

Anup K.Das, S. D., 2018. Optimization of Extraction Using Mathematical Models and Computation. Computational Phytochemistry, pp. 75-106.

Bangji Zhang, J. i. Z. J. Y. N. Z. Q. J., 2016. Modal and Dynamic Analysis of a Vehicle with Kinetic Dynamic Suspension System, Beijing, China: Hindawi.

BlueStar Inspections, 2022. Your Vehicle's Suspension System. [Online] 
Available at: https://www.bluestar.com/get_informed/article/your-vehicles-suspension-system
[Accessed 28th April 2022].

Chua, J., 2019. Soft suspension: Why keen drivers don’t like it. Torque, 28 April.

Dixon, J., 2007. The Shock Absorbers Handbook. Chichester: John Wiley & Sons LTD.

Hindsbo, M., 2018. Why Engineering Simulation is Becoming Increasingly Important. [Online] 
Available at: https://www.spotlightmetal.com/why-engineering-simulation-is-becoming-increasingly-important-a-762493/
[Accessed 18th April 2022].

Mike Blundell, D. H., 2015. Modelling and Analysis of Suspension Systems. In: The Multibody Systems Approach to Vehicle Dynamics (Second Edition). s.l.: Elsevier Ltd. , pp. 185-334.

Moltke, D. v., 2022. Turning the Steering Wheel Too Much?. [Online] 
Available at: https://blayze.io/blog/turn-the-steering-wheel-too-far
[Accessed 28th April 2022 ].

National Institute of Standards and Technology, US, 2022. Box-Behnken designs. [Online] 
Available at: https://www.itl.nist.gov/div898/handbook/pri/section3/pri3362.htm
[Accessed 23rd April 2022].

Noble, A., 2022. Tyres & Pajeka Models, Swansea: UWTSD.

Pacejka, H. B., 1987. Tyre Modelling for Use in Vehicle Dynamics Studies. SAE Transactions, Volume 96, pp. 190-204.

S.Rakheja, A. P. M., 2000. DYNAMIC RIDE PROPERTIES OF A ROLL-CONNECTED VEHICLE SUSPENSION. Cairo, Egypt, Science Direct.

Sachin Sunil Kelkar, P. G. S. S. P. S. A., 2021. A detailed study on design, fabrication, analysis, and testing of the anti-roll bar system for formula student cars, s.l.: SN Applied Sciences .

Time Attack LTD, 2014. Chassis Stiffness: The Basics. [Online] 
Available at: https://www.timeattack.co.uk/chassis-stiffening-basics/
[Accessed 30th April 2022].

Toogood, D., 2021. ARE UK DRIVERS CHOOSING TO PURCHASE SUVS AND CROSSOVERS OVER SEDANS AND CONVERTIBLES?. Island Echo, 21st June.

VI-Grade, 2020. Automotive solutions. [Online] 
Available at: https://www.vi-grade.com/en/products/automotive/
[Accessed 18th April 2022].