L4-Micros, peripherals and interfaces

Name: Sam Lamprey

Student ID: 2203378

Course: Electrical and electronic engineering

Module code: ACDA4004

Lecturer: Gareth jones

Micros and peripherals assignment

Section 1

Figure 1

The circuit shown in figure 1 is used to control the 7-segment display using the PIC16F84A chip. The integrated circuit is connected to two buttons which can be used to change the number that is displayed on the 7-segment display. The components that were used to create the circuit are a 7-segment display, a pic 16f84a, buttons and resistors. The 7-segment display is mainly used to display numbers counting up or down in a sequence that can be chosen depending on the code that is written. It can also work together with the buttons which can manually control when the number increases or decreases. All of this only works when the correct code is inputted onto the chip, otherwise it will not work. The PIC16F84A can also be used for a wide range of functions as in its programming set it has [4] 35 different instructions that it can carry out as shown in figure 3. The resistors are used in the circuit to help control the flow of current especially through the 7-segment display which uses LEDs so it is important that the current flowing through them is limited otherwise it could cause the LED to blow up. The reason that LEDs are used are because they have many advantages [1] “But the main advantage of light emitting diodes is that because of their small die size, several of them can be connected together within one small and compact package” which is what a 7-segment display is. They are also more energy efficient than more traditional bulbs which means that the amount of voltage running through the circuit is a lot less than would be needed if bulbs were used instead. In the circuit there are also two pull up resistors which are used to control the voltage between the button and chip when the button is activated. This makes sure that the chip does blow as too much voltage or current through it could cause damage to it or even cause it to break. There is also a crystal in the circuit which is used as a clock which could have different uses depending on what is trying to be achieved such as creating a PWM waveform that can be changed depending on the frequency of the clock.

Figure 2

Figure 3[1]

Section 2

To be able to display different numbers on the 7-segment display you must first work out which hex number correlates with each number on the display. Each number will have a specific hex number that will light the correct segments on the display to show each number. The simplest of these to work out is the number 8 on the display as this will show when all parts of the display are set to on. The other numbers can be worked out by trial and error or can be calculated using the spec sheet for the display as shown in figure 2 that shows which of the 7 segments correlates with each hex code which also correlates to a specific binary number as show in figure 3. Once all the hex numbers have been worked out then you can start inputting them into the code for the chip so that it will display them on the 7-segment display. To do this you need to use the specific hex numbers and make sure that the output pins of the chip are connected to the display with a resistor between them. Then you can move the hex numbers to the w register so that they can be outputted onto the display. To make the display count up you would need to display the first number then create a delay then output the second number and carry on doing this until you have counted from 0 to 9. Then use the loop command to make it go back to the start and then it will repeat.

Figure 4 [3]

Section 3

;====================================================================

; Main.asm file generated by New Project wizard

;

; Created: Mon Feb 12 2018

; Processor: PIC16F84A

; Compiler: MPASM (Proteus)

;====================================================================

;====================================================================

; DEFINITIONS

;====================================================================

#include p16f84a.inc ; Include register definition file

portB equ 0x06 ;port B address

trisb equ 0x06 ;trisB address in register bank 1

portA equ 0x05 ;port A address

trisa equ 0x05 ; trisa address in register bank

status equ 0x03 ;status register

RP0 equ 5 ;bit 5 of status register

RP1 equ 6 ;bit 6 of status register

count equ 0x0c ; counter 0 to 255

nCount equ 0x0d ; pause routine inner loop counter

mCount equ 0x0e ; pause routine outer loop counter

button1 equ 0x0F

;====================================================================

; VARIABLES

;====================================================================

;====================================================================

; RESET and INTERRUPT VECTORS

;====================================================================

; Reset Vector

RST code 0x0

goto Start

;====================================================================

; CODE SEGMENT

;====================================================================

PGM code

Start

bcf status,RP0 ; clear bank selection bits in the status reg

bcf status,RP1

bsf   status,RP0

movlw 0x00 ; load w with 0x00

movwf trisb ; load it into trisb to make portb all outputs

bcf status,RP0 ; reselect bank 0

clrf portB ; clear all lines low

clrf count ; initialize counter to 0

clrf button1 ; initially set button1 to 0

loop

BTFSC button1,0

goto butdisp1

BTFSC button1,1

call butdisp2

call checkbuttons ; check if buttons are working correctly

goto loop

butdisp1

movlw 0xF9 ;load w with 0xF9 to display 1 on counter

movwf portB ; move w to the output to update the display

return

butdisp2

movlw 0xA4 ;load w with 0x24

movwf portB ; move w to output to update the display

return

checkbuttons

BTFSS portA,0

call apressed ; move apressed to check if button A is working correctly

bsf button1,0

BTFSS portA,1

call bpressed ; move bpressed to check if button B is working correctly

bsf button1,1

return

apressed

bsf button1,0 ; set apressed to 0

bcf button1,1 ; clear apressed

return

bpressed

bsf button1,0 ; set bpressed to 0

bcf button1,1 ; clear bpressed

return

get_count

movf count, w ; move count to W

BTFSS portA,0 ;

goto display2

movlw 0xF9

movwf portB

goto get_count

display2

movlw 0xA4

movwf portB ; move W to port B to update the display

goto get_count

addlw 1 ; add +1 to W

movwf count ; save W in count

call pause ; delay 200ms by subroutine

call pause

call pause

call pause

call pause ; five pause executions equals ~ 1 second

;--------------------------------------------------------------------

;subroutine pause: lets waste some time....

;

;inputs: nothing

;outputs: nothing

;

;does: nothing but waste some time

;corrupts: register W

;

;description: two loops using "mCount" and "nCount" register files

; "mCount" is the outer loop counter (255 down to zero)

; "nCount" is the inner loop counter (255 down to zero)

;

;--------------------------------------------------------------------

pause

movlw 0xff ; set w = 255 decimal

movwf mCount ; mCount = w: outer loop counter

loadN

movlw 0xff ; set w = 255 decimal

movwf nCount ; nCount = w: inner loop counter

decN

decfsz nCount, f ; nCount--

goto decN ; if nCount != 0 then repeat nCount--

decfsz mCount, f ; else decrement Count

goto loadN ; if mCount != 0 then

; reload nCount to 255 and decrement

return ; else exit subroutine

Loop

goto Loop

;====================================================================

END

Reference list

[1] http://ww1.microchip.com/downloads/en/DeviceDoc/31029a.pdf

[2] https://www.electronics-tutorials.ws/blog/7-segment-display-tutorial.html  “But the main advantage of light emitting diodes is that because of their small die size, several of them can be connected together within one small and compact package producing what is generally called a 7-segment Display”

[3] https://components101.com/displays/7-segment-display-pinout-working-datasheet

[4] https://www.circuitstoday.com/pic16f84a-instruction-set “The instruction set of PIC16F84A has 35 instructions [The controller “understands” 35 words]”