Locost Dimensions

Ms     ( to be found prior to test )

Musf =  38 kg /wheel

Musr =   42 kg/wheel

Wheelbase 	2.351m

Rolling radius 0.275 m  for Avon Radial Slicks ( NB this is nominal figure – each tyre type  will be slightly different )

Front spring rate ( Nominal 325 lb/in)   left 55.62 N/mm   Right 57.49 N/mm

Rear spring rate ( Nominal 170 lb/in)    29.426 N/mm

Ratio of  front wheel  travel to spring travel  = 1.97

Ratio of  wheel  travel to pot travel   =  1.622

Rear Suspension leverage of wheel to spring = 1.0

Velocity  ratio of damper wheel to pot  = 1.0

Roll centre heights  and track to be found in WinGeo3 models

Calculation of CG Height in the workshop

Locost CG Height  workshop test results

The values you will need to calculate the CG are:

Measured Level

FL= 151kg  FR = 153kg   RL= 155kg  RR = 207kg

Car raised by 675 mm

RL = 167.45kg and RR = 201.9kg