EXAMINATION SOLUTIONS

(to accompany the examination paper sent to the external examiner

but NOT to be seen by students)

| Programme: |  | HND Applied Computing | Level: | 4 |  |
| --- | --- | --- | --- | --- | --- |
| Module: |  | Computer Architecture and Operating Systems |  |  |  |
| Author: |  | Paul John | Page: | 1 of 4 |  |
| Q 1 | Question |  |  |  | Marks |
| (a) (b) (c) (d) (e) (f) (g) | - 11100111 - 01100010 - 241 - 85 - 01111111 - 10111101 - 165 - 63 2^16 = 65536 - 1920x1080x4 = 8294,400 Bytes (or approx. 8.3MB) 66355200 - Approx. 414KB or 414720Bytes 3317760 - Image Size (in bits) 1920x1080x32 = 66,355,200 / 20Mb/s link (Approx. 3 seconds) - 600Mb/s - 9.4MB - 25MB Total |  |  |  | 2 2 2 2 2 2 2 2 2 3 2 4 1 1 1 30 |

##### Continuation Sheet

| Module/Unit: | Computer Architecture and Operating Systems | Page: | 2 of 4 |
| --- | --- | --- | --- |

| Q 2 | Question | Marks |
| --- | --- | --- |
| (a) (b) (c) | - Address Bus – The address bus carries an address generated by the CPU. Width of address bus determines amount of memory that can be installed - Data Bus – Carries data being exchanged by the various elements of the system. - Control Bus – Is a bi-directional bus meaning that signals can be carried in both directions - Data bus sends/retrieves information. - Control bus sets/monitor signals. Identification and discussion of each bullet point (2 marks, Max 4) Discussion could include: - Can support up to 127 devices - Hot swappable - Bandwidth between devices is shared - Can support a large number of devices - Can be expanded by the use of a hub - Devices can be sent into hibernation by the host - Backwards compatible with earlier versions - Other relevant points Note: each point raised = 2 marks (10 marks Max) Total | 2 2 2 4 10 20 |

##### Continuation Sheet

| Module/Unit: | Computer Architecture and Operating Systems | Page: | 3 of 4 |
| --- | --- | --- | --- |

| Q 3 | Question | Marks |
| --- | --- | --- |
| (a) (b) (c) | Discussion could include: - System board does not support 32GB memory, only supports max 8GB - 32 bit O/S installed, - Should install 64 bit O/S - Replace system board that has support for 32GB - Check system board manual for memory configuration - Other relevant points Note: each point raised = 1 marks, extension of point 2 marks (Max 3 per point, Max 15) - Extension of available memory that is a combination of physical/secondary storage (or similar description) - Occurs when a process tries to access memory that is not located in Physical Memory (or similar description) - The amount of time spent swapping pages to/from backing storage (or similar description) Discussion could include: - Shows running processes - Identifies which processes that slow down the operating systems - Uses to troubleshoot slow performance - Able to manage processes on start-up - Able to start/restart/stop services - Other relevant points Note: each point raised 1 mark, extension 1 mark (Max 2 per point, Max 4 marks) Total | 15 2 2 2 4 25 |

##### Continuation Sheet

| Module/Unit: | Computer Architecture and Operating Systems | Page: | 4 of 4 |
| --- | --- | --- | --- |

| Q 4 | Question | Marks |
| --- | --- | --- |
| (a) (b) (c) | Discussion could include: - Wireless security protocol - Network key - SSID - Securing the router (admin username/password) - Devices on the network - Writing network key on piece of paper or using the sticky label/card when router given to user - Anti-Virus on devices - Sharing of network information - Any other relevant point Discussion could include: - Follows dotted decimal format - Each octet ranges from 0 to 255 - Max 4 billion unique addresses - Cannot have devices with the same address - Difficult to implement on larger networks (chance of duplication is possible) - Any other relevant point Note: each point raised 1 mark – Max 5 Marks Discussion could include: - Stops duplication of IP Addresses - IP Address given out by DHCP server (or similar) - When a computer is switched on, if it doesn’t have a static IP address, it will send a request out on the network, the DHCP server then looks at its internal list, allocates a spare IP address to that machine, makes a note of it, alongside the MAC address of that computer. The computer then stores that IP address and uses it as its own unique ID. - Any other relevant discussion/point Total | 15 5 5 25 |