Pc BUILD STRUCTURE REPORT

HND Applied computing; Computer architecture and networks

February 1, 2020

Thomas James samuel Phillips

Phi17833288

Contents

No table of contents entries found.

# Scenario 1

For this scenario I was asked to create a pc for my elderly relative, this pc wouldn’t be used for gaming, would only have one monitor and would be used for basic things like word processing, watching YouTube and skype calling family. It shouldn’t be expensive.

Due to the person wanting the pc at low cost my goal was to keep it in the area of £500 to £550. However, I didn’t achieve this and it ended up costing £572.65, whoever this was with all the accessories as well

## CPU and Graphics

I went for a low-end CPU due to it not being used for tasks that was use all the CPU.

The CPU I chose was an intel core I3-8100. It has a clock speed of 3.6 GHz and has 4 cores. The L1 cache is 32kB instruction and 32 kb data for each core, the l2 cache is 256kB per core and the L3 cache is 6MB.

This CPU is perfect for this build, it wouldn’t make the pc slow while also not being more than they need, they will be able to skype call and work on word documents.

Another reason I chose this CPU is because it has internal graphics which will work for the uses for this pc. It uses Intel UHD Graphics 630.

## Motherboard and Ports

The motherboard I chose is a ASrock H310CM micro ATX, it has the socket required for the CPU, it has everything required for the PC, 2 ram slots, one expansion slot and also having the benefit of being small which means a small case can be used. Like the CPU it supports DDR4 ram, however, only supports up to 32gb of ram. However, there isn’t need for more than 16gbs.

As for the ports on it, it has a 1000Mbit/s ethernet port, giving the person a choice of having Ethernet or using the wireless LAN card that will be installed, 4 SATA 6 GB/s connectors, allowing them to add something like an optical drive or more storage space, and has 6 USB ports with 2 of them being USB 3. Most importantly it allows video to be supported by the CPU and doesn’t rely on a video card being installed. Because of this, it has a VGA port, DVI port and a HDMI port

## RAM

For the ram I chose to keep it simple, I chose crucial 16GB ddr4-2666. This is one memory stick and Is plenty of memory for the system the MHz of the memory doesn’t matter due to him not using it for any graphical developments/ editing

## Storage

For the storage device I chose a 1TB HDD as the price difference between a 500GB one was £10. The disk spin is 7200RPM which will make the system boot in a reasonable time. The hard drive also has 64 MB cache which will increase the boot speed of the programs he uses the most.

## Case

The case is a small case a Thermaltake Versa H17 MicroATX Mini Tower Case. Its dimensions are only 380 mm x 205 mm x 390 mm making it easy to fit anywhere. It also has more USB ports just in case the amount on the motherboard isn’t enough. It also has room for the wireless expansion card that I chose

## Power supply

For the power supply I got a be quiet pure power 11. It is rated 80+ bronze and wont make that much noise. It is a 300w power supply and although the system is only 180w altogether, I couldn’t find a reliable power supply that was lower than 300w.

## Monitor

For the monitor I got a 23 inch aoc 24b1xh. It is 1080p and will be suited for the use. They will be able to make the display big enough for them to read all the text and display video at 60 Hz

## Keyboard, mouse and other

For the keyboard and mouse, I kept it very simple, I got a v7 ku200 keyboard and a Targus amu81sz mouse. Both are wired connections. I also got speakers for them creative labs pebble, this is so he can hear the videos and skype calls.

Furthermore, to save them from needing to plug the pc into their router, I chose a TP-link wi-fi adapter. It is rated up to 802.11n and will fit into the expansion slot in the motherboard. Finally I chose windows 10 home as the operating system

| Part | name | amount | link |
| --- | --- | --- | --- |
| CPU | Intel core i3-8100 | £97.97 | Amazon |
| Motherboard | ASRock H310CM-HDV Micro ATX | £49.98 | Amazon |
| Memory | Crucial 16GB DDR4-2666 | £55.98 | Amazon |
| Storage | Seagate BarraCuda 1TB | £33.46 | Amazon |
| Case | Thermaltake Versa H17 MicroATX Mini Tower Case | £53.59 | Amazon |
| Power supply | be quiet pure power 11 | £44.99 | Amazon |
| OS | Windows 10 home | £103.54 | PC-World |
| Wireless network card | TP-link wi-fi adapter | £11.99 | Amazon |
| Monitor | AOC 24B1Xh 23 inch | £84.99 | PC-World |
| Keyboard | v7 ku200 keyboard | £10.28 | Amazon |
| Mouse | Targus amu81sz mouse. | £2.89 | Amazon |
| Speakers | Creative labs pebble | £22.99 | Amazon |
|  | Total | £572.65 |  |