Josh Gourlay (2111487)

2111487@student.uwtsd.ac.uk

Level 5 Systems Engineering

ACDA5021

List of Figures

Figure 1 – Comparison of the systems and traditional approach to design [1].	3

Figure 2 – Functional decomposition of 4.5” naval round.	4

Figure 3 – Critical design features from QFD2.	5

Figure 4 – ‘Cliff edge’ analogy to achieving a robust design [11].	6

Figure 5 – Critical control factors highlighted from What-Why table.	7

Figure 6 – Critical noise factors highlighted from What-Why table.	7

Figure 7 – Shell body manufacturing process flow.	9

Figure 8 – Manufacturing requirements tree diagram.	9

Figure 9 - Critical process functionality identified from QFD1.	10

Figure 10 – Process importance identified from QFD2.	12

Figure 11 – Screenshot of QFD2 matrix with potential emergence identified.	13

Figure 12 – Process importances identified from QFD3.	14

Figure 13 – Key controls identified in flow forming 4.5” shell bodies [7].	15

Figure 14 – Noise importances associated with flow forming 4.5” shell bodies.	15

Figure 15 – Results of QFD4.	17

List of Tables

Table 1 – Manufacturing Functional Means Analysis (FMA) [3].	11

Table of Contents

- Abstract3

Introduction3

Systems Engineering Toolset4

  - Functional Decomposition4

  - Quality Functional Deployment (QFD) 24

  - P-diagram5

  - What/Why Table6

  - Manufacturing Requirements8

  - Quality Functional Deployment (QFD) 110

  - Functional Means Analysis (FMA)10

  - Quality Functional Deployment (QFD) 212

  - Quality Functional Deployment (QFD) 313

  - Manufacturing P-diagram and What/Why Table14

  - Quality Functional Deployment (QFD) 416

Conclusion17

References18

- Annex A: Design QFD2 19

- Annex B: Design P-diagram20

- Annex C: Design What/Why Table21

- Annex D: Manufacturing QFD122

- Annex E: Manufacturing QFD223

- Annex F: QFD324

- Annex G: Manufacturing P-diagram25

- Annex H: Manufacturing What/Why table26

- Annex I: QFD427

Abstract

A 4.5” naval projectile has been used as a worked example to demonstrate the usefulness of systems engineering toolsets in ensuring a robust design including functional decomposition, design Quality Functional Deployment (QFD) 2, P-diagram and What/Why table. The systems engineering ethos was used to transform the design into the manufacturing stage, using the critical component identified from the design QFD2 process as an example. The manufacturing tools used included: QFD1, QFD2, Functional Means Analysis (FMA), QFD3, P-diagram, What/Why table and QFD4. This report demonstrates the usefulness of these tools in creating a robust design and manufacturing process, by analysing and understanding the challenges prior to spending time and resource on creating expensive prototypes.

# Introduction

Systems engineering is a fundamentally different approach to traditional design and manufacture. Traditional design involves a design team gathering requirements, designing, and manufacturing prototypes, followed by subsequent testing of this prototype. One major flaw with this method is that it is a costly and time-consuming process as emergent functionality, and characteristics present themselves after significant financial and resource investment. This then requires re-design and testing to rectify.

Systems engineering on the other hand is a more methodical approach whereby the gathered requirements are first analysed to ensure that the design proposal is actually fit for the intended purpose, and only once this is confirmed significant resource is spent on designing the system correctly. Essentially, systems engineering is all about designing the right system, subsystem, or component, then designing the system, subsystem, or component right. This ‘left shifting’ ethos allows emergent behaviour to be identified and dealt with earlier in the design stage, prior to financial investment into non-functioning prototypes.  Whilst the initial analysis phase in the systems engineering approach might seem more labour-intensive and lengthier, the ability to identify and remove emergent characteristics early, means that the systems approach is often quicker when compared to the traditional approach. Figure 1 summarises the two approaches.

Figure 1 – Comparison of the systems and traditional approach to design [1].

This report will demonstrate the use of tools fundamental to the system engineering approach using a 4.5” projectile as a case study.

# Systems Engineering Toolset

## Functional Decomposition

Functional decomposition is a useful tool in eliciting functionality at a subsystem level, in which the functional requirements can be derived from. It is a simple process in which each component of the subsystem under analysis is labelled describing its function and written in the required verb-noun relationship as expected of Functional requirements. Figure 2 demonstrates the functional decomposition carried out on a 4.5” naval round. For the purposes of this task, only the projectile subsystem was under investigation, therefore the propelling charge case, fuze, booster and basebleed subsystems were ignored.

Shell body

House HE fill

Fragment on detonation

Provide aerodynamic stability

Oxsilan coating

Protect shell from corrosion

Cannula

Allow crimping of case to shell body

Screw thread

Interface with Fuze

Prevent Fuze unscrewing under gun launch

Varnish

Prevent corrosion

Driving band

Engage with gun rifling

Seal propelling gases

Paint

Protect shell from corrosion

Identify shell

Basebleed igniter thread

Attach Basebleed igniter

Tail thread

Attach tail unit

Figure 2 – Functional decomposition of 4.5” naval round.

## Quality Functional Deployment (QFD) 2

QFD2 utilises the component functionality gathered from the functional decomposition described in Figure 2 and compares the relationship to how those functions are carried out. Prior to QFD2 a FMA can be used to compare different means of achieving the functions required when designing a new system, but for the purposes of this exercise, the existing means highlighted in Figure 2 are sufficient. The QFD2 matrix can be found in Annex A; the results of QFD2 are depicted in Figure 3.

Figure 3 – Critical design features from QFD2.

Typically, QFD1 would be completed in which the relationship between non-functional requirements are compared to the functional requirements, eliciting the critical functionality of the subsystem. These values determined from QFD1 would then be imported into the ‘Importance’ column in QFD2. As QFD1 was not carried out, an importance value was given scored as either 1, 3, 5, or 9 from least to most important.

Figure 3 highlights the importance of the shell body, the QFD2 process has identified it as the most critical feature. This component will continue the systems engineering process of which the next tool is a parameter diagram or P-diagram.

## P-Diagram

P-diagrams are essential in creating a robust design and can be used by engineers to identify and categorise variation within the design, known as noise factors. There are 3 types of noise considered in a P-diagram, external, unit to unit and deterioration. External noises are typically environmental weather conditions such as wind, rain, humidity etc. Deterioration noises are variations introduced to the system that are caused by components becoming progressively worse, for example, corrosion, metal fatigue, thermal degradation etc. Unit to unit variation occurs between individual components that are being controlled in the design. In the 4.5” shell body example, one of the factors that needs to be controlled to ensure aerodynamic stability in flight is the outer geometry. If, for example, the ogive geometry was too wide, the shell centre of gravity and moment of inertia would not be stable, potentially causing balloting mid-flight. This is a very dangerous and unpredictable scenario in which the shell could land well away from the intended target potentially causing harm to civilians or friendly forces. However, there will always be a tolerance associated, as it is not practical to manufacture shell bodies to the exact same outer geometry on every occasion. The tolerance needs to be carefully considered as a highly restrictive tolerance will lead to a considerable reject rate and will be extremely costly to implement a manufacturing process with extremely tight tolerance. Conversely, a relaxed tolerance will lead to reduced performance or more importantly a safety incident in which the weapon system and/or personnel/civilians could be affected.

The ‘Cliff edge’ analogy is often used to describe this relationship and is shown in Figure 4.

The P-diagram carried out on the 4.5” shell body can be found in Annex B.

Required customer performance

Figure 4 – ‘Cliff edge’ analogy to achieving a robust design [11].

Figure 4 highlights the importance of robust design. By tuning the design parameters to achieve the highest possible performance (blue highlighted point on Figure 4), the design will be too sensitive to noises causing it to ‘fall off the cliff’ and consistently fail to satisfy the required customer performance. Conversely, designing the system to cope with the noises and less sensitive to variation i.e. the red highlighted area in Figure 4, will ensure that the required customer performance is consistently achieved.

## What-Why Table

Once the noise factors have been identified using the P-diagram, a What-Why table was used to identify which noise factors are likely to affect which control factors and subsequently the output performance of the design feature. The relationship between how much variation a noise may cause to the control factor is scored 9, but tools such as a pairwise comparison could be used to determine this value. Figure 5 and Figure 6 highlights the critical control factors and noise parameters elicited from the What-Why table.

The completed What-Why table can be found in Annex C.

Figure 5 – Critical control factors highlighted from What-Why table.

Figure 6 – Critical noise factors highlighted from What-Why table.

As highlighted in Figure 5, weight has been identified as the most important factor to control in the design of the 4.5” shell body, suggesting that potentially multiple noises can affect the output by influencing this control factor. By highlighting the critical control factor, a design team can focus on where to apply finances and resource to ensure a robust design. Additionally, as described in Section 2.11, during the manufacturing phase, control measures can be introduced into the manufacturing process to reduce the emergence associated with this noise.

The What-Why table can also highlight which noise factors are consistently affecting multiple control factors in a design which in turn will affect the output performance. As shown in Figure 6, the variation in thickness has been identified as the critical noise factor. For example, in Figure 2, the base of the shell is much thicker than the ogive for a number of reasons. Firstly, it has to support the weight of the HE during gun launch but also provides structural integrity to prevent the body collapsing when the driving band is pressed on. If the thickness of this region of the shell body was too thin, it could collapse under setback forces or during driving band pressing. Conversely, if the ogive was too thick, the increased weight at the nose of the shell would affect the centre of gravity, thus leading to reduced range or aerodynamic instability.

At a design stage, identifying this noise, which is highly influential in many design parameters, will allow design engineers to specify much tighter tolerances on these parameters. Conversely, the variation in wavy band geometry is not as influential in terms of conformance to a build standard and overall shell performance, so a more relaxed tolerance is acceptable.

The noises described thus far are indirect noises; they affect the output of the component by affecting the control factors of said component. The external noises identified in Figure 5 (wind, rain, air pressure and humidity) are potentially direct noises in which these factors directly affect the output performance of the component, as opposed to affecting this via the control factors. Wind, for example, will directly affect the aerodynamics of the shell  body in flight; stronger cross winds will cause the shell to drift off target, therefore this noise will need to be accounted for via the fire control system of the weapon platform.

## Manufacturing Requirements

Similarly with designing a system, the manufacturing of that system, subsystem or component must also adhere to requirements. Typically, manufacturing engineers should be present during the design phase as it is possible that due to limited manufacturing capabilities, the design may need to be altered to accommodate. Using a different example of shaped charge liners, a design team may stipulate the use of a liner with an angle of 55° to achieve a required penetration characteristic, but due to non-ideal manufacturing capability, this penetration target may not be achieved. Therefore, investment would need to be made to improve the manufacturing method or a design iteration would need to be made to accommodate the non-ideal manufacturing method, such as by changing the liner angle to 45° and adding a plane wave generator. This example highlights the importance ensuring the right experienced personnel are present during this requirements phase.

To elicit the functional requirements from a manufacturing perspective, a process flow was used in which the generic manufacturing steps were placed in sequence, described in a verb-noun relationship. Importantly, these steps are solution independent at this stage. Figure 7 depicts the process flow for the manufacture of the 4.5” shell body.

Figure 7 – Shell body manufacturing process flow.

The functional requirements determined from the process flow in Figure 7 were used to populate the horizontal axis of QFD1.

Figure 8 summarises the manufacturing requirements in a tree diagram format [2], which were also populated into QFD1 on the vertical axis.

Figure 8 – Manufacturing requirements tree diagram.

## QFD1

Manufacturing QFD1 is compiled in the same way as a design QFD1 in which the relationship between functional and non-functional requirements are compared based on the how strongly the two correlate, either none, 1, 3 or 9 (weakest to strongest relationship). Similarly, the non-functional requirements are scored in terms of importance, either by an arbitrary scale e.g. 1 to 5 or using tools such as a Pairwise comparison. The QFD1 matrix can be found in Annex D, with results highlighted in Figure 9.

Figure 9 - Critical process functionality identified from QFD1.

The results from QFD1 allow a manufacturing team to understand which of the manufacturing processes are most important in satisfying the manufacturing requirements. Understanding this allows them to focus in on these most important processes and allocate more resource into means of achieving these most important functions. For example, forming of the internal and external case are the most important, therefore more budget and resource should be spent on equipment to ensure these functions are achieved accurately. Conversely, clean cavity is the least important function, suggesting the project team should not necessarily spend a large portion of the budget on equipment to carry out this functionality well, when there are more important processes to consider.

## FMA

A FMA was carried out whereby the functional requirements derived from Figure 7 were placed vertically on the left-hand side, and a selection of methods of carrying out the functions were suggested. The FMA is shown in Table 1, with selected means highlighted yellow.

Table 1 – Manufacturing Functional Means Analysis (FMA) [3].

Induction heating was selected as the means of heating the billet as it is an industry standard method with a number of advantages over the alternative means, such as: fast heating due to the heat being directly applied to the billet, rather than loss of energy to surroundings, increased efficiency thus reducing service/operating costs and improved safety [4].

Flow forming has been selected as it is also a widely used manufacturing method in the defence industry to produce highly precise components to extremely tight tolerances. There are further benefits compared to the alternative means identified including, less waste due to utilisation of a tubular blank (known as a preform) as opposed to a large block billet, thus reducing material costs. As flow forming uses a series of rollers to force material against a shaped mandrel, this process also causes elongation of the preform [5]. This is ideal as it means an additional capability specifically to elongate the shell body is no longer required, significantly reducing the equipment capital costs, and therefore one less piece of equipment that could potentially breakdown, holding up manufacture.

Quenching and tempering are heat treatment processes used industry wide to enhance the mechanical properties of materials, especially steels. Quenching is a process in which the steel is heated to extremely high temperatures, then rapidly cooled in a quenching medium such as oil or water to improve the hardness and strength of the material [6]. This quenching process makes the steel very brittle, so is usually followed by tempering whereby the steel is heated to lower temperatures (compared to quenching) and left to cool at a slower rate, typically in air. This ensures the steel is more malleable to improve material ductility. A combination of the two processes ensures that the required mechanical properties are achieved.

Similarly, machining has been selected to form the driving band recess and adjust final weight as it is also an industry standard practise and is currently used in the manufacture of 4.5” shell bodies.

## QFD2

Following the FMA, manufacturing QFD2 was carried out in which the relationship between the means selected in the FMA were compared to the manufacturing functionality determined from the process flow in Figure 7. The QFD2 matrix can be found in Annex E with results highlighted in Figure 10.

Figure 10 – Process importance identified from QFD2.

Figure 10 highlights the critical processes in the manufacture of 4.5” shell body, namely flow forming closely followed by induction heating. As is expected when carrying out QFD2 following a FMA, the matrix shown in Annex E and Figure 11 highlights the leading diagonal between functionality and feature relationships. However, it is also important to look at groups clustered away from the central diagonal, as this can identify emergent (unexpected) interactions that are not likely to have been considered, which are often detrimental to the overall system. Figure 11 highlights the leading diagonal in the QFD2 matrix, in addition to potential emergent interactions encircled.

Figure 11 – Screenshot of QFD2 matrix with potential emergence identified.

As previously mentioned, to provide the shell body with the necessary structural hardness and ductility, quenching and tempering heat treatment processes are carried out. In particular, tempering is the heating of the steel and subsequent air cooling to improve toughness and ductility. In Figure 11, the interaction encircled is also suggesting that tempering is taking place, as the billet (before it has been shaped) is being heating to 1000°C then allowed to cool in air to allow geometry shaping. Therefore, when the tempering process takes place later in the shell body manufacturing process, it is important that this initial tempering step is accounted for when determining the temperature and duration of the latter tempering process. If this initial tempering step was not accounted for, the shell body may not have the required properties, leading to poor fragmentation of the case when detonated as an emergent behaviour.

## QFD3

QFD3 is the tool used to identify the relationships between various important design features and how they interact with the identified manufacturing methods. It is essentially where the design engineers would hand over the design to the manufacturing engineers. It uses the same scoring method as described in QFD1 and QFD2 whereby the relationships are scored as either none, 1, 3 or 9. Similarly to manufacturing QFD2, the manufacturing functional requirements are placed horizontally in the matrix and scored against the most important control parameters and noises identified during the design P-Diagrams and What/Why Table. The QFD3 matrix can be found in Annex F with the critical process steps elicited from the process shown in Figure 12.

Figure 12 – Process importances identified from QFD3.

Flow forming has also come out as the most important process technique in both QFD2 and QFD3, suggesting that flow forming is a critical manufacturing process. This is useful information for the manufacturing engineers because it suggests that flow forming is vital to achieve the manufacturing functional requirements, but also is the means by which variation and unwanted emergent behaviour could be introduced if not tightly controlled.

## Manufacturing P-diagram and What/Why Table

To investigate how noises could affect the flow forming manufacturing process, firstly a P-diagram was carried out to identify the noises, then the relationship between these identified noises and the control factors were analysed via the What/Why table. The P-diagram and What/Why table can be found in Annex G and Annex H respectively.

The critical control factors and noises are presented in Figure 13 and Figure 14 respectively.

Figure 13 – Key controls identified in flow forming 4.5” shell bodies [7].

Figure 14 – Noise importances associated with flow forming 4.5” shell bodies.

Figure 14 shows that roller pressure, coolant flow rate and mandrel speed have been identified as the three most important control factors, as the What/Why process has identified that there are multiple noises affecting these controls. As a result, these noises could potentially indirectly affect the 4.5” shell body manufactured from this flow forming process. Therefore, additional means known as control measures (discussed further in Section 2.11), should be installed to ensure that no emergent behaviours occur affecting the conformance of the shell body to the build standard.

The What/Why process has identified variation in roller geometry, variation in number of rollers and variation in roller feed rate as the three most important noises during the flow forming process. This information tells manufacturing engineers that these noises have the most potential to affect the manufactured shell body and cause emergence, and therefore gives them a priority list in which control measures can be put in place to irradicate these noises. For example, the process building temperature was one of the lowest scored noises presented in Figure 14, as it does not affect that many control factors in the flow forming process. Therefore, it is unlikely that control measures to maintain building temperature such as climate control systems would be installed, as these would come at great expense not proportional to the problem caused by this noise. With that being said, if the process building temperature was left truly uncontrolled and was consistently reaching 40°C for example, this would likely induce operator fatigue and exhaustion, leading to mistakes resulting in non-conforming 4.5” shell bodies. A series of Taguchi experiments could be undertaken to understand the interaction between temperature and operator fatigue to ascertain what the optimum is, thus will inform what level of control measure would be required to achieve this temperature.

Conversely, variation in roller geometry is likely to affect multiple control factors and indirectly the output of the manufacturing process, therefore would be justifiable to spend project budget on control measures to irradicate or greatly reduce this noise.

## QFD4

QFD4 is a scoring matrix that utilises the control factors of the manufacturing process, identified from the P-diagram in Annex G and assigns an assessment of the ability to achieve the process parameters. Additionally, important noises identified from the What/Why process and important design features of the shell body can also be inputted into the QFD4 matrix. For each of the control factors, noises or design features listed, a corresponding control measure is suggested i.e. some form of process method to ensure that the target values are achieved. The relationship between the control measures suggested and the process parameters from the manufacturing P-diagram/design QFD2 are compared and scored in the same none, 1, 3, 9 scoring method as described in the previous QFD matrices. The QFD4 matrix can be found in Annex I, with the control importances summarised in Figure 15.

Figure 15 – Results of QFD4.

The QFD4 process has determined that planned maintenance is a vital control measure in ensuring that emergent features do not affect the shell bodies manufactured via the flow forming process. Planned maintenance will ensure that bolts, gaskets, joints etc involved in the positioning and application of pressure to the preform will be replaced periodically. Similarly, as the rollers are used frequently, they will also deteriorate causing variation of the roller geometry, so an effective maintenance schedule can ensure that they are replaced before non-conforming product is manufactured. Additionally, the process parameters that are reliant on pressure to carry out their function, such as roller pressure and coolant flow rate, a closed loop control system is suggested. If the roller pressure or coolant flow rate is too high, this negative feedback will be communicated to the controller, typically a PID (Proportional Integral Derivative) controller [8], causing a high-pressure control valve to open, reducing the pressure. Pressure gauges will supplement this control system to allow operators to monitor the pressure throughout the process.

Further quality control measures have also been suggested to ensure the critical design features identified in QFD2, namely shell weight and outer geometry, are achieving the target values stated in QFD4. These include weighing scales and dimensional checks utilising coordinate measuring machine (CMM) [9].

# Conclusion

This report has demonstrated the use of systems engineering tools applied to a 4.5” naval round example. Firstly the functional decomposition has elicited the component functionality, which was then inputted in the design QFD2. QFD2 defined the shell body as the critical feature, therefore this component was progressed through to the P-diagram and What/Why toolsets to identify the noises and controls and work out how these noises interact with the controls (indirect noises) or directly affect the component output (direct noises). Once this was established, the shell body was taken through the manufacturing stage, starting with gathering the manufacturing requirements. The manufacturing requirements and manufacturing functionality were compared in QFD1, with forming case internal and external identified as the critical functionality. Means were suggested to achieve these functionalities via the FMA, and these means were then compared against the functionality, which resulted in the flow forming being named as the critical manufacturing process. QFD3 was then carried out in which the design and manufacturing requirements combined, further identifying flow forming as a critical process. A P-diagram and What/Why table was duplicated to identify and quantify the critical noises and controls of the flow forming process. The final step was QFD4, in which the critical design features and identified noises and controls were assigned control measures to ensure the manufacturing process adhered to the established parameters, such that emergent characteristics do not filter into the manufactured component.

Using these tools demonstrates the usefulness of systems engineering in creating a robust design and manufacturing process by prioritising time and resource into analysing the requirements and ensuring that emergent functionality is managed prior to production of non-conforming components.

# References

| [1] | D. Holley, Lifecycle Managment and Systems Engineering Philosophy and Principles Lecture, Swansea: University of Wales Trinity St David, 2022. |
| --- | --- |
| [2] | D. Holley, Manufacturing System Design, Swansea: UWTSD, 2023. |
| [3] | R. E. a. W. W. F. Keith, “Electrical heating of forging billets. Final report,” Texas Univ., Austin (USA). Center for Electromechanics , Austin, Texas (USA) , 1982. |
| [4] | O. a. M. P. a. D. E. a. B. J. M. Lucia, “Induction Heating Technology and Its Applications: Past Developments, Current Technology and Future Challenges,” IEEE Transactions on Industrial Electronics, vol. 61, no. 5, pp. 2509-2520, 2013. |
| [5] | D. C. a. J. C. Daniele Marini, “A Review of Flow Forming Processes and Mechanisms,” Key Engineering Materials, 2015. |
| [6] | X. D. a. D. Ju, “Modeling and Simulation of Quenching and Tempering Process in Steels,” Physics Procedia, pp. 368-374, 2012. |
| [7] | B. B. P. R. K. K. e. a. Podder, “Flow forming of thin-walled precision shells,” Sadhana, vol. 43, 2018. |
| [8] | J. Gourlay, “Control Systems Design Assignment Report, ACDA5011: Instrumentation & Control,” Swansea, 2024. |
| [9] | E. Savio, Coordinate Measuring Machine, CIRP Encyclopedia of Production Engineering, Berlin: Springer, 2019. |
| [10] | T. K. N. R. D. F. Ebrahimi M, “Effect of flow-forming parameters on surface quality, geometrical precision and mechanical properties of titanium tube,” Journal of Process Mechanical Engineering , vol. 232, no. 6, pp. 702 - 708, 2018. |
| [11] | D. Holley, Subsytem Design Lecture Slides, Swansea, 2024. |

Annex A: Design QFD2

Annex B: Design P-diagram

Annex C: Design What-Why Table

Annex D: Manufacturing QFD1

Annex E: Manufacturing QFD2

Annex F: QFD3

Annex G: Manufacturing P-diagram

Annex H: Manufacturing What/Why Table

Annex I: QFD4 [10]