Switch S1 Configurations

Unauthorised Access Is Strictly Prohibited

User Access Verification

Password:

S1> en

Password:

S1#show running-config

Building configuration...

Current configuration : 1640 bytes

!

! Last configuration change at 11:13:04 UTC Tue Mar 27 2018

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname S1

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$a0z1$W2.bQy.slbpSjqeRZEWkv.

!

!

!

no aaa new-model

switch 1 provision ws-c3750-24p

system mtu routing 1500

--More--         no ip domain-lookup

!

!

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

interface FastEthernet1/0/1

!

interface FastEthernet1/0/2

!

interface FastEthernet1/0/3

!

--More--         interface FastEthernet1/0/4

!

interface FastEthernet1/0/5

!

interface FastEthernet1/0/6

!

interface FastEthernet1/0/7

!

interface FastEthernet1/0/8

!

interface FastEthernet1/0/9

!

interface FastEthernet1/0/10

!

interface FastEthernet1/0/11

!

interface FastEthernet1/0/12

!

interface FastEthernet1/0/13

!

interface FastEthernet1/0/14

!

interface FastEthernet1/0/15

--More--         !

interface FastEthernet1/0/16

!

interface FastEthernet1/0/17

!

interface FastEthernet1/0/18

!

interface FastEthernet1/0/19

!

interface FastEthernet1/0/20

!

interface FastEthernet1/0/21

!

interface FastEthernet1/0/22

!

interface FastEthernet1/0/23

!

interface FastEthernet1/0/24

!

interface GigabitEthernet1/0/1

!

interface GigabitEthernet1/0/2

!

--More--         interface Vlan1

ip address 172.16.2.2 255.255.255.192

!

ip default-gateway 172.16.2.1

ip classless

ip http server

ip http secure-server

!

!

!

!

banner motd ^C

Unauthorised Access Is Strictly Prohibited ^C

!

line con 0

password 7 14141B180F0B

login

line vty 0 4

password 7 14141B180F0B

login

line vty 5 15

login

!

--More--         end

S1#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  172.16.2.2      YES manual up                    up

FastEthernet1/0/1      unassigned      YES unset  up                    up

FastEthernet1/0/2      unassigned      YES unset  down                  down

FastEthernet1/0/3      unassigned      YES unset  down                  down

FastEthernet1/0/4      unassigned      YES unset  down                  down

FastEthernet1/0/5      unassigned      YES unset  down                  down

FastEthernet1/0/6      unassigned      YES unset  up                    up

FastEthernet1/0/7      unassigned      YES unset  down                  down

FastEthernet1/0/8      unassigned      YES unset  down                  down

FastEthernet1/0/9      unassigned      YES unset  down                  down

FastEthernet1/0/10     unassigned      YES unset  down                  down

FastEthernet1/0/11     unassigned      YES unset  down                  down

FastEthernet1/0/12     unassigned      YES unset  down                  down

FastEthernet1/0/13     unassigned      YES unset  down                  down

FastEthernet1/0/14     unassigned      YES unset  down                  down

FastEthernet1/0/15     unassigned      YES unset  down                  down

FastEthernet1/0/16     unassigned      YES unset  down                  down

FastEthernet1/0/17     unassigned      YES unset  down                  down

FastEthernet1/0/18     unassigned      YES unset  down                  down

FastEthernet1/0/19     unassigned      YES unset  down                  down

FastEthernet1/0/20     unassigned      YES unset  down                  down

FastEthernet1/0/21     unassigned      YES unset  down                  down

--More--         FastEthernet1/0/22     unassigned      YES unset  down                  down

FastEthernet1/0/23     unassigned      YES unset  down                  down

FastEthernet1/0/24     unassigned      YES unset  down                  down

GigabitEthernet1/0/1   unassigned      YES unset  down                  down

GigabitEthernet1/0/2   unassigned      YES unset  down                  down

S1#show ip route

Default gateway is 172.16.2.1

Host               Gateway           Last Use    Total Uses  Interface

ICMP redirect cache is empty

S1#