Cyber Security (CS)

Packet Tracer Activity – Server Firewalls and Router ACLs

Faculty of Architecture, Computing & Engineering

School of Applied Computing

Mount Pleasant Campus

Swansea

SA1 6ED

# Note: The password used in the Packet Tracer activities in this exercise is: PT_cyber1

# Addressing Table

| Device | Private IP Address | Public IP Address | Subnet Mask | Site |
| --- | --- | --- | --- | --- |
| Web Server | N/A | 209.165.201.10 | 255.255.255.0 | Internet |

# Objectives

Part 1: Connect to the Web Server

Part 2: Prevent Unencrypted HTTP Sessions

Part 3: Access the Firewall on the Email Server

# Background

In this activity, you will access a user within the Metropolis site and connect using HTTP and HTTPS to a remote Web Server. The IP addressing, network configuration, and service configurations are already complete. You will use a client device in the Metropolis site to test connectivity to a remote Web Server and then secure the Metropolis site by preventing unencrypted web sessions from connecting to the outside world.

- Connect to the Web Server

    - Access the HQ Internet Web Server on Sally’s PC using HTTP.

- Click the Metropolis Bank HQ site and then click the PC Sally.

- Click the Desktop tab and then click Web Browser.

- Enter the URL of http://www.cisco.corp and click Go.

- Click the link Login Page.

Why would a user be concerned when submitting information using this website?

____________________________________________________________________________________

    - Access the HQ Internet Web Server on Sally’s PC using HTTPS.

- Access the Web Browser on Sally’s computer.

- Enter the URL of https://www.cisco.corp and click Go.

- Click on the link Login Page.

Why would a user be less concerned when submitting information using this website?

____________________________________________________________________________________

- Close Sally’s computer.

- Prevent Unencrypted HTTP Sessions

    - Configure the HQ_Router.

- Within the Metropolis Bank HQ site, click the HQ_Router.

- Click the CLI tab and press Enter.

- Use the password cisco to login to the router.

- Use the enable command and then configure terminal command to access the global configuration mode.

- In order to prevent unencrypted HTTP traffic from traveling through the HQ router, network administrators can create and deploy access control lists (ACLs).

- The following commands are beyond this course but are used to demonstrate the ability to prevent unencrypted traffic from moving through the HQ_Router.

- Within the global configuration mode HQ_Router(config)# copy the following access-list configuration below and paste it into the HQ_Router.

!

access-list 101 deny tcp any any eq 80

access-list 101 permit ip any any

!

int gig0/0

ip access-group 101 in

!

end

- Close the HQ_Router.

    - Access the HQ Internet Web Server on Sally’s PC using HTTP.

- Within the Metropolis Bank HQ site, click the PC Sally.

- Click the Desktop tab and then click Web Browser.

- Enter the URL of http://www.cisco.corp and click Go.

Is Sally’s computer able to access the HQ Internet Web Server using HTTP?

____________________________________________________________________________________

    - Access the HQ Internet Web Server on Sally’s PC using HTTPS.

- Access the Web Browser on Sally’s computer.

- Enter the URL of https://www.cisco.corp and click Go.

Is Sally’s computer able to access the HQ Internet Web Server using HTTP?

____________________________________________________________________________________

- Close Sally’s computer.

- Access the Firewall on the Email Server

- Within the Metropolis Bank HQ site, click the Email server.

- Click the Desktop tab and then click on Firewall. There are no firewall rules implemented.

- In order to prevent non-email related traffic from being sent or received from the Email server, network administrators can create firewall rules directly on the server, or as previously shown, they can use access control lists (ACLs) on a network device like a router.