Imran Ali 1701017

Cardiff Router

User Access Verification

Password:

Cardiff>en

Password:

Cardiff#sh ru

Cardiff#sh running-config

Building configuration...

Current configuration : 1589 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname Cardiff

!

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

ip dhcp excluded-address 172.16.198.1 172.16.198.5

!

ip dhcp pool CardifFDHCP

network 172.16.198.0 255.255.255.192

default-router 172.16.198.1

dns-server 172.16.198.1

!

!

!

no ip cef

no ipv6 cef

!

!

!

username admin password 7 0822455D0A16

username cardiff password 7 08364D400C01041A405B5E54

!

!

license udi pid CISCO2901/K9 sn FTX1524J6F6-

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name wanexam2020.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Tunnel10

ip address 192.168.10.1 255.255.255.248

mtu 1476

tunnel source Serial0/3/0

tunnel destination 10.0.1.225

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 172.16.198.1 255.255.255.192

duplex auto

speed auto

!

interface Serial0/3/0

ip address 10.0.1.22 255.255.255.252

encapsulation ppp

ppp authentication chap

clock rate 2000000

!

interface Serial0/3/1

no ip address

encapsulation ppp

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router bgp 65025

bgp log-neighbor-changes

no synchronization

network 10.0.0.0

network 10.0.1.20 mask 255.255.255.252

!

ip classless

ip route 0.0.0.0 0.0.0.0 209.165.201.253

!

ip flow-export version 9

!

!

!

!

!

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

transport input ssh

!

!

!

end