M4X00386 RESIT EXAM 2017-18

SOLUTIONS

- QUESTION ONE / Closed System: (20 marks)

A rigid container equipped with a resistance heater contains 1.5 kg of air. The resistance heater is switched on for 20 minutes. Determine,

- The change of specific internal energy in kJ/kg during this time; knowing that the air container loses 2.37 MJ of heat while 2 kW of power is applied to the resistance heater.

- The final air temperature if the initial air temperature in the container is 12C.

AIR

m = 1.5 kg

t = 1200 s

T1 = 12C

Q=-2370 kJ

Wsw=-2 kW

∆u=Qm-WSD×tm

∴       ∆u=-23701.5--2 20×601.5

∴       ∆u=-1580+2×12001.5

∴       ∆u =-1580+2×800=-1580+1600

∴       ∆u =20 kJkg

(15 marks)

∆u =cvT2-T1

∴       T2 =∆ucv+T1

∴       T2 =200.718+12+273

∴       T2 =27.855+285=312.86K

∴       T2 =27.855+12=39.86℃

(5 marks)

- QUESTION TWO  / Open system, steady-flow process:  (15 marks)

Steam at 1.8 MPa and 400°C steadily enters a nozzle whose inlet area is 0.02 m2. The mass flow rate of steam through the nozzle is 5 kg/s. Steam leaves the nozzle at 1.4 MPa with a velocity of 275 m/s. Heat losses from the nozzle, per unit mass of the steam are estimated to be 64.57 kJ/kg.

Using the given table A-6, determine:

- The inlet velocity.

- The exit temperature of the steam.

qout

P1=1.8 MPa

T1=400°C

A1=0.02 m2

P2=1.4 MPa

c2=275 m/s

STEAM

m=5 kg/s

Steam enters a nozzle steadily at a specific flow rate and velocity. The inlet velocity of steam and the exit temperature are to be determined.

Assumptions:

- This is a steady-flow process since there is no change with time at any point and thus ∆mcv=0 and ∆Ecv=0.

- There are no work interactions.

- The potential energy change is zero, ∆pe=0.

We take the nozzle as the system. This is a control volume since mass crosses the system boundary during the process. We observe that there is only one inlet and one exit and thus:

m1=m2=m

2 marks

- The specific volume and enthalpy of steam at the nozzle inlet are:

P1=1.8MPaT1=400°C          v1=0.16849 m3/kg      h1=3251.6 kJ/kg         (Table A-6)

2 marks

Then,

m=1v1c1A1

∴     5 kg/s=10.16849 m3/kg×c1×0.02 m2

∴           c1=42.1 m/s

2 marks

- Under stated assumptions and observations, the energy balance for this steady-flow system can be expressed in the rate form as:

Q=mh2+12c22-mh1+12c12          since W=0, and ∆pe=0

∴q=h2-h1+12c22-12c12

3 marks

Dividing by the mass flow rate m and substituting, h2 is determined to be:

h2=h1+q-c22-c122

∴          h2=3251.6×103-64.57×103-2752-42.122

∴          h2=3150.1  kJ/kg

3 marks

Then,

P2=1.4MPah2=3150.1 kJ/kg    T2=350°C         (Table A-6)

3 marks

- QUESTION THREE  / Open system, unsteady-flow process:  (15 marks)

We take the tank as the system, which is a control volume since mass crosses the boundary.

This is an unsteady-flow process since the conditions within the device are changing during the process, but it can be analysed as a uniform-flow process since the state of fluid at the inlet remains constant.

The mass and energy balances for this uniform-flow process can be expressed as follows:

- Mass balance:

mi-me=m2-m1system

∴        mi=m2             since me=m1=0

4 marks

- Energy balance:

Q-W=outmh-inmh+m2e2-m1e1system

∴    0=-mihi+m2u2            since   Q≅W≅ke≅pe≅0

∴    mihi=m2u2

∴    hi=u2                                  since   mi=m2

5 marks

The change in internal energy or enthalpy for an ideal gas during a process are defined as:

u2-u1=cvT2-T1       kJkghe-hi=cpTe-Ti        (kJkg)

∴    u2=cv .T2     since u1=0hi=cp.Ti       since Te=0

2 marks

Therefore:

hi=u2

∴    cp.Ti=cv .T2

∴    T2=cpcv .Ti

2 marks

According to Table A-2:

cp = 1.005 kJ/Kg.K

cv = 0.718 kJ/Kg.K

Hence:

T2=cpcv .Ti=1.0050.718×17+273

∴      T2=405.92 K

∴      T2=405.92-273 °C

∴      T2=132.92°C

2 marks