Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

2 failed login attempts within 120 second

What does the block-for 30 mean in the above command?

It prevents logins to the identified account for 30 seconds to limit exposure to brute force attacks

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

‘username admin privilege 15 secret adminpass_’ the password is to short otherwise

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh

‘TRANSPORT input ssh’ as the above cmd fails.

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

The telnet protocol was disabled in the the above command

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection?

‘adminpass’ didn’t work as this password was two short adminpass_ did at it met the 10 character limit. Using a valid password  a connection was established and the motd was displayed.

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

The session was forcibly closed by the router. The account is locked for a short period of time any attempts to reconnect are rejected until the lockout period has passed.

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed?

Unable to login using this account as this account has not been created

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain.

Previous configuration settings have been put in place to inhibit the use of bruteforce attacks. By injecting a time out into an attack any bruteforce attack is only going to take longer before its effective.

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch?

It was possible to SSH to access the switch from the PC

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

The command ssh-v failed to work on the version of IOS ssh /?

Show ssh  as command displayed what connections if any were running on both SSHv1 and SSHv2

s1#show ssh %No SSHv2 server connections running. %No SSHv1 server connections running. s1#ssh -v % Incomplete command. s1#ssh -V % Incomplete command. s1#

How would you provide multiple users, each with their own username, access to a network device?

The following command modified:-

username <usename> privilege 15 secret <password>  can be used to create multiple administrative users each with a unique username and password.

To reduce exposure to limit access to least privilege we could simplify the command to

Username <username> secret <password>

Ideally in a larger organization an implementation of RADIUS or similar technology could be employed to maintain a single logon identity to managed across an large estate

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

Encryption Standards comparison.

Evidence of cryptography  or the process of converting plain text to cyphertext has been practiced by civilizations for roughly 4000 years. The Mesopotamians and ancient  were known to have used a simple substitution methods to protect information. While  Spartans in ancient Greece used a Scytale,  text written to a strip of material bound around a rod, the text was meaningless when unwound until it was rewound around a rod of the same dimensions.

Cryptography traditionally the preserve of governments  and the financial industry. For example the Enigma machine used to devastating effects by Nazi Germany during World War 2 was initially invented in 1918 to protect communication in the banking industry [1].

Todays interconnected networked society is massively dependant on strong, secure and private communication based on encryption technologies developed in the 1970s. Globally networks, computers, smart phones, IOT devices etc are thousands of times more powerful than the systems used develop the encryption techniques highlighted below.

DES or Data Encryption Standard is a symmetrical encryption process. Composed of a 64bit block of data, comprising of encrypted information across 56 bits of encrypted information with  8 bits of parity. Developed in 1970 at IBM by Horst Feistel [2] it was derived out of the earlier system, Lucifer a 128bit encryption process.

DES was considered secure for many years however as computing power has become even more effective it is no longer seen as secure for most recent processes. The relatively small bit length makes this standard susceptible to brute force attacks. Although not the only method available to break encryption brute force times for DES have dramatically decreased from days to hours. This led to the standard being dropped in favour of new revised encryption standards with longer bit keys lengths.

Seeing the weakness inherent with in DES, AES or Advance Encryption Standards began development in 1997 with the final published  Rijndael standard published in 2001 created by two Belgian researchers Joan Daement and  Vincent Rijmen  [2] . The standards were published royalty free for vendors to use. The bit count of encryption can be either 128, 192 or 256 bits this increased bit length reduced the threat of a brute force attack. In theory modern day computers would take an impractical length of time to brute force attack this encryption. However, there are tool sets that have been developed to recover AES-256 encryption keys. A Tempest [3] exploit [4] was found by monitoring the electro magnetic emissions from a device as it was processing data. This exploit permitted the team of researchers to decipher the key in less than 60 seconds.

Both the DES and AES are examples of a symmetrical cipher where the security key is known by both parties. Ron Rivest, Adi Shemir and Leonard Alderman devised the RSA cryptographic process in 1977 [2]. It became one of the first widely used asymmetric public key  cryptographic systems used for secure data transmission.

Asymmetric encryption permits entities to share data securely without the need to pre share encryption/decryption keys before exchanging data. In public key encryption each entity has two keys. A public key which is made available publicly and a private key that is never shared. Data sent to a recipient would be encrypted with the recipients public key the recipients private key is then used to decrypt the previously encrypted data.

Secure Hashing Algorithm commonly know as SHA unlike the above isn’t an encryption protocol. Data sets applied to the hashing algorithm will output a unique  160 bit digest of the data set. This is a repeatable process, provided the original data set has not been changed. Modifying the data set even by one bit will produce an entirely different hash value.  The published hash value of a dataset provides reassurance to the recipient that the original data has not been modified, tampered or corrupted.

SHA1 as an algorithm was known to have been exploitable ‘theoretically’ as early as 2005. With replacement algorithms SHA2 and SHA3  in place to resolve the weaknesses of the predecessor. Google in collaboration with the CWI institute in Amsterdam announced the first  practical demonstration of an SHA1 collision [5]. A collision is where two differing data sets produce the same hash value consider that SHA1 signatures have been used to validated the integrity of Digital Certificates, software updates , email signatures etc. the ramifications can be very severe.

| [1] | The British Museum, "teachinghistory100.org," The British Museum, [Online]. Available: http://www.teachinghistory100.org/objects/about_the_object/enigma_cipher_machine. [Accessed 8 December 2019]. |
| --- | --- |
| [2] | D. R. Stinson, "Block ciphers and the Advanced Encryption Standard," in Cryptography Theory and Practice, 3 ed., pp. 95-175. |
| [3] | C. Goodman, "An introduction to TEMPEST," SANS Institute, 2019. |
| [4] | "TEMPEST side-channel attacks recover AES-256 encryption keys," Rambus Press, 2017. [Online]. Available: https://www.rambus.com/blogs/tempest-side-channel-attacks-recover-aes-256-encryption-keys/. [Accessed 8 Dec 2019]. |
| [5] | "Annoncing the first SHA1 Collision," Google, [Online]. Available: https://security.googleblog.com/2017/02/announcing-first-sha1-collision.html. [Accessed 7 Dec 2019]. |