Section 1

Proposal For| OLIVIER THEATRE

Works| SMALL ISLAND

Client| NATIONAL THEATRE

Proposal prepared by| RICHE TARR

CONTENTS

Section 2

Provide here brief scope outline with date of works and anticipated duration.

Section 3

Amount of personnel involved and who the manager / supervisors are. Name of Theatre/Area First Aider also to be included in this section. Insert here also all site contact numbers.

Section 4

Attach A4/A3 drawings showing location(s) of works. If works are to take place on/public area, insert full details of pedestrian route including accurate layout of pedestrian diversion & all required signage.

Section 5

This section is to list all flown elements and brief description of the piece and any particulars that could affect its hanging or special considerations.

Section 6

Detailed breakdown of the scenic element and lifting equipment required.

Section 7

Step by step sequence of work on how the activity is to be carried out. To include but not limited to, materials/plant access, lifting activities, any special requirements e.g. mechanical lifting, power, man access to works, constraints, etc, and actual methodology for task. (METHOD STATEMENT)

Section 8

Details of competent and certified operatives and all certification to be included within this section, tools and special equipment to be listed

Section 9

Inductions, specific Work Permits and any other site specific information

which may impact these works and need to be considered.

Section 10 Risk assessments (e.g. manual handling, noise, working from heights, use of hazardous substances). Where a harness has to be utilised a rescue plan is to be included in this section. Lifting equipment kit list.

Section 2

# SCOPE OF THE LIFTING PLAN;

Fit-up of ‘SMALL ISLAND’ in the Olivier Theatre from 09/04/19 to 12/04/19

# LIMITS OF THE LIFTING PLAN;

This Method Statement relates to the activities of the members of The National Theatre’s Rigging and Automation Department’s crew and their sub-contractors whether directly or indirectly involved in the works detailed in this Method Statement.

No other persons come under the purlieu of this Method Statement and it is the responsibility of them and their supervisors and employers that they do not interfere with nor hinder the works herein described.

Section 3.

Rigging team including;

- Riche Tarr 3737 (Supervisor & First Aider)

- Phil Horsburgh (First Aider)

- James Goodsall

- Camille Lesaffre

- Jack Little

- Kelsey Smith

Section 4

Olivier Theatre including;

- Main stage

- Rear stage

- Side Stages

- Grid

- Fly tower galleries

- sub-stage (drum revolve)

- Paint frame

- Assembly Area

Section 5

Scenic elements listed below;

- Curved wall

- Windrush Cloth

- Windows x 5

- LX Trusses

- LX Practicals x 5

- Fogger Drop

- Blinder

- Gauzes x 2

- Border

- Clothes drops x 5

- Full black

Section 6

| Item Name | Curved Wall |  |
| --- | --- | --- |
| Position in stage | U/S/C |  |
| Weight | 4930kg |  |
| No of lifting points | 3 per piece (Splits in 3) |  |
| Lifting method | 6mtr SWR drifts @ 8mm to 3.25t shackles |  |
| Lifting equipment | 5 x 1tonne NAV Hoists |  |
| Position of lifting gear | Grid |  |
| Truss Required | None |  |
| Rep position | Fly tower |  |

| Item Name | Windrush Cloth |  |
| --- | --- | --- |
| Position in stage | Spanning M/S 8250 from setting line |  |
| No of lifting points | 2 on the cloth |  |
| Weight | 100kg max |  |
| Lifting method | Manually hauled |  |
| Lifting equipment | Rope and Pulleys |  |
| Position of lifting gear | On Stage / Fly floor |  |
| Truss Required | NO |  |
| Rep position | Struck when out of rep |  |

| Item Name | Windows x 5 |  |
| --- | --- | --- |
| Position in stage | Various surrounding the Drum revolve |  |
| Weight | Between 20 – 100kg |  |
| No of lifting points | 2 straight picks per window |  |
| Lifting method | 2 point hoists per window |  |
| Lifting equipment | 2 x 1.5t shackles, 2 x 6mm SWR drifts @ various lengths & 2 x 8mm maillions per window |  |
| Position of lifting gear | Grid |  |
| Truss Required | NO |  |
| Rep position | Fly Tower |  |

| Item Name | LX Trusses (share of Follies rig) |  |
| --- | --- | --- |
| Position in stage | Various |  |
| Weight | Various |  |
| No of lifting points | 4 x straight picks per truss |  |
| Lifting method | TBC |  |
| Lifting equipment | 2mtr EWL roundslings & 2tonne shackles |  |
| Position of lifting gear | Grid |  |
| Truss Required | 300 box truss x ??mtrs |  |
| Rep position | Fly tower |  |

| Item Name | LX Practicals x 5 |  |
| --- | --- | --- |
| Position in stage | Various |  |
| Weight | 20kg max |  |
| No of lifting points | 1x straight pick per lamp |  |
| Lifting method | Shackles, 10mtr SWR drifts @ 3mm, maillions |  |
| Lifting equipment | 1 x Point hoist per practical |  |
| Position of lifting gear | Grid |  |
| Truss Required | NO |  |
| Rep position | Fly Tower |  |

| Item Name | Fogger Drop |  |
| --- | --- | --- |
| Position in stage | D/S spanning full stage |  |
| Weight | 100kg max |  |
| No of lifting points | 2 with 3.5 mtr bridles |  |
| Lifting method | Track # 3? |  |
| Lifting equipment | Badgers & shackles |  |
| Position of lifting gear | Grid |  |
| Truss Required | 10mtrs H40L |  |
| Rep position | Fly tower |  |

| Item Name | Blinder |  |
| --- | --- | --- |
| Position in stage | U/S/C |  |
| Weight | TBC |  |
| No of lifting points | 3 |  |
| Lifting method | Track # 28 |  |
| Lifting equipment | 3 x Straight picks |  |
| Position of lifting gear | Grid |  |
| Truss Required | 10 mtrs of H40L |  |
| Rep position | Fly Tower |  |

| Item Name | Gauzes x 2 |  |
| --- | --- | --- |
| Position in stage | U/S/R&L in front of Shutters |  |
| Weight | TBC |  |
| No of lifting points | 2 Straight picks per Gauze |  |
| Lifting method | 2 x 0.5tonne Nav hoists per gauze |  |
| Lifting equipment | Bridles, Badgers & shackles |  |
| Position of lifting gear | Grid |  |
| Truss Required | 10mtrs of H40L per Gauze |  |
| Rep position | Fly tower |  |

| Item Name | Border |  |
| --- | --- | --- |
| Position in stage | U/S/C |  |
| Weight | TBC |  |
| No of lifting points | 3 x straight picks |  |
| Lifting method | Track # 27 |  |
| Lifting equipment | Badgers and 2tonne shackles |  |
| Position of lifting gear | Grid |  |
| Truss Required | 11 mtrs of H40L |  |
| Rep position | Fly tower |  |

| Item Name | Clothes drops x 5 |  |
| --- | --- | --- |
| Position in stage | Various |  |
| Weight | 10kg max |  |
| No of lifting points | 1 per drop |  |
| Lifting method | 1 x Point hoist per drop |  |
| Lifting equipment | Solonoids |  |
| Position of lifting gear | Grid |  |
| Truss Required | NO |  |
| Rep position | Fly tower |  |

| Item Name | Full Black |  |
| --- | --- | --- |
| Position in stage | Track # 24 |  |
| Weight | TBC |  |
| No of lifting points | 3 x straight picks |  |
| Lifting method | 3 x synchro discs |  |
| Lifting equipment | Badgers & 2tonne shackles |  |
| Position of lifting gear | Grid |  |
| Truss Required | 13mtrs H40L |  |
| Rep position | Fly tower |  |

Section 7

Method Statement for Curved wall build;

- Address all staff working on the project, and all staff working in and around the space with a work safety briefing

- Show the build team the drawings and this method statement

- Set out safety barriers and/or flickers if necessary

- Position S/R wall under build hoists x 3

- Connect build hoists to S/R wall via loose tackle

- Lift S/R wall to sufficient height to position ballast truck below

- Land S/R wall onto Ballast truck and bolt together

- Make sure the S/R wall is free standing (add weight to ballast truck and/or secure to the stage)

- Disconnect the build hoists x 3

- Position S/R sliding wall under build hoists x 3 and connect via loose tackle

- Offer S/R truck up to S/R sliding wall and connect to each other accordingly

- Perform full bolt check and disconnect build hoists

- Push fully built S/R truck with sliding door into desired position

- Repeat method for S/L Truck

- Push Header sections x 2 to C/S underneath build hoists x 3

- Lift the header sections to an upright position individually via loose tackle

- Bolt header sections together creating one full sized header piece

- Lift full sized header out to desired height to line up bolt holes with the S/R & S/L trucks

- Bolt both the S/R & S/L trucks to full header

- Perform full bolt check

- Clear tools and make area safe

- Remove safety barrier and/or flickers if necessary

Section 8

EQUIPMENT REQUIRED

All equipment will be chosen as appropriate to the task and its conformity to the relevant standard.

Rigging equipment

Personnel equipment;

- PPE inc. Hard hat and Steelies

- Working at height equipment

- Tool kits

Additional safety equipment in the event of rescue or during inspection;

- Restricted access signs

- Exclusion zone barriers

- First aid pack

- Lanyards for all tools.

- Other PPE, (safety glasses/gloves/head torch)

All of the above equipment will have an individual inspection record for it as standard procedure. However it is the responsibility of the supervisor to check all the equipment for any faults or defects prior to each use.

Section 9

Rigging Supervisor to liaise with Operations Manager & Production Manager to inform of works.

Section 10

For RISK ASSESSMENT please go to the SMARTSHEET

Alternatively, you can find the associated documents here;

Cross Dept. Drive > Rigging Dept. > Technical Resources info. > Shows > Olivier > SMALL ISLAND

~END~