Managing Health, Safety and Welfare in Construction

Managing Health and Safety in a Project

Lecturer: Dr Deborah Hughes

Institution: University of Wales Trinity St David

Index

Introduction

- Construction phase plan

| Company name | Davies Construction |
| --- | --- |
| Project description | Full construction of a single, two storey detached domestic building |
| Client | University of Wales Trinity St David |
| Start date | 06/03/2020 |
| Location | Adjacent to the new student accommodation on the SA1 site. |

Location of site

| Management |  |
| --- | --- |
| Site Rules | - Wear PPE whenever on site - Follow Safety signs and procedures - Report hazards and problems to site manager - Protection for the public around the site - Keep all facilities clean - All personnel should have undergone safety training prior to being on site. - Permission from the site manager must be given prior to any work taking place on site. - No one under the influence of drugs or alcohol are permitted to be on site |
| Health and safety aims | Ensure the safety of workers and members of the public |
| Arrangements to ensure cooperation between project team members and coordination of their work | - Frequent meetings between site manager and builders - Frequent meetings between Site manager and client - Project programme to set out goals of the project and key dates so that all members of the project know when everything should be happening |
| Arrangements for involving workers | - Create a transparent and open environment for workers to communicate |
| Site induction | - Induction must be site specific and not broad - Highlight risks at the site - Site inductions are needed for frequent visitors like architects and quantity surveyors |
| Fire and emergency procedures | - Fire alarms - Fire exit signs - Frequent fire drills - Creating a map highlighting hazardous locations |

Welfare facilities

The welfare facilities are provided by the contractor in most cases but the client is actually responsible for them.

What facilities are needed on site?

- Toilets

- Washing facilities

- Changing rooms

- Eating area

Details

- The toilets be flushable toilets with running water, connected to a drainage system

- The number of toilets depends on the number of staff working on site. British standards recommends 1 toilet for every 7 persons.

- Toilets must be kept clean

- Washing facilities require, a clean supply of hot and cold water, soap, drying equipment, sufficient ventilation

- Changing rooms should have men and women rooms separately

Location of facilities

The welfare facilities will be located on the right hand side of the site at the entrance. This is close enough to the site but also gives plenty of room for the proposed building and any building material and or equipment at the left hand side of the entrance as shown on the drawing below.  The toilets will be a row of portaloos and will be close to the washing facilities as recommended by HSE.

Method statements

Davies Construction

Construction of a single, two story dwelling near Swansea city centre at Kings Road

Client: UWTSD

- Scaffolding

Scaffolding assists in the building of the house by supporting the workers and equipment so that work can be done higher on the house.

Anyone who has attended a CISRS scaffold investigation course is deemed to be able to assess the scaffold on whether it is acceptable or not.

The main hazard is working from heights so there will always be a risk of falling. Barriers, stable ladders and timber beams and extra caution reduce this risk.

Control measures

- Scaffolding requires barriers around the edges so that the workers can walk and carry equipment without feeling at risk.

- Warning signs are to be put around the scaffold

- Visitors are prohibited from entering the scaffold unless the site manager has specifically said otherwise

- Workers must not attempt to lift above their threshold and anyone carrying any tools must be trained to do so

PPE

- Hard hats

- Safety boots

- Gloves

- High visibility vests

- Excavating

Excavating is used when digging out a manhole or pit to inspect the ground in the pre construction phase and is also used in the construction phase when creating room for the foundations. The foundations are said to be concrete strip foundations with a depth of 1m.

Excavations are to be assessed at least once in every day during which persons are at work there (Health and safety authority).

A hazard when excavating for a ground investigation is a trench collapse which are very dangerous when the excavation is deep enough that a worker is required to enter. A risk that is obvious when talking about any excavation is falling into it.

Control measures

- Yellow warning tape around any excavation to prevent anyone from falling into

- Supports must be put in place in order to prevent the pits from collapsing like timber walling in trenches

- Inspections of all excavations are vital after a rainstorm

- Do not work under raised loads

- Heavy equipment or vehicles should be kept away from the edge of the excavation to avoid collapse

- Complete a ground investigation where foundations are to be laid to ensure that there are no unknown underground utilities present

- Electrical

Electrical components are found frequently through houses. They are needed for lights, fridges, microwaves and many more household components.

The electricity will be installed as a distribution system.  A main circuit is drawn to a distribution centre in each room which are connected to the distribution boards.

| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Heavy Machinery |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | x | Employees |  | x | Contractor |  |  | x | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Noise complaints |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 1 |  |  |  |  | 3 |  |  |  |
| 2 | Hearing difficulty due to constant loud machinery |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 3 | Hand arm vibration system |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 12 |  |  |  |
| 4 | Self inflicted injuries such as crushed fingers |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 3 |  |  |  |  | 9 |  |  |  |
| 5 | Tripping over equipment |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Make suitable working hours on site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Workers to wear ear defenders when necessary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Mainly educationally based. Ensure that workers are holding powerful tools loosely, keep hands warm and take regular breaks from heavy use of powerful tools. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Use machinery when possible but use caution when workers are needed to lift or carry heavy materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Have regular site inspections to ensure the site is always kept tidy and objects such as cables or hand tools aren’t left out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Noise complaints |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 1 |  |  |  |  | 2 |  |  |  |
| 2 | Hearing difficulty due to constant loud machinery |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 4 |  |  |  |  | 4 |  |  |  |
| 3 | Hand arm vibration system |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 4 | Self inflicted injuries such as crushed fingers |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  | 6 |  |  |  |
| 5 | Tripping over equipment |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  | 2 |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Position |  |  |  |  |  |  | Signed |  |  |  |  |  |  |  |  |  | Date of Completion: |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |
| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| Working from heights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | x | Employees |  |  | Contractor |  |  | x | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Falling from a significant height |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 5 |  |  |  |  | 10 |  |  |  |
| 2 | Falling objects |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Secure Scaffolding. Avoid working from heights wherever possible such as assembling materials at ground level. Precautions such as safety nets, double guard rails and toe boards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Ensure all workers are wearing hard hats. Reduce the need for tools above ground level by assembling materials needed, on the floor and use caution when tools are needed at a hight. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Falling from a significant hight |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 5 |  |  |  |  | 5 |  |  |  |
| 2 | Falling objects |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Position |  |  |  |  |  |  | Signed |  |  |  |  |  |  |  |  |  | Date of Completion: |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |
| Activity to be Assessed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |  |
| Excavation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons undertaking or affected by the activity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | x | Employees |  | x | Contractor |  |  | x | Public |  |  |  |  | Service User |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Other ……………………………………………………………………………………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazards and Associated Risks |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Collapsing Trenches |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 2 | Falls into excavations |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  | 6 |  |  |  |
| 3 | Existing services |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 2 |  |  |  |  | 9 |  |  |  |
| 4 | Contaminated ground |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 4 |  |  |  |  | 8 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Timber walling to the sides of trenches to make the trench more stable. Excavation holes will be thoroughly inspected before anyone enters. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Apply warning signs and tape around the excavation. Cover trenches when not being used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Any cables found shall be treated as though they were live until proven otherwise. Where there has been services identified in the desk study, only safe digging will be done, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Appropriate PPE and washing facilities will be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  |  |  |  |  |  |  |  | Likelihood |  |  |  |  |  | Severity |  |  |  |  | Risk Level |  |  |  |
| 1 | Collapsing Trenches |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 2 | Falls into excavations |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 3 |  |  |  |  | 3 |  |  |  |
| 3 | Existing services |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  | 1 |  |  |  |  | 3 |  |  |  |
| 4 | Contaminated ground |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Position |  |  |  |  |  |  | Signed |  |  |  |  |  |  |  |  |  | Date of Completion: |  |  |  |  |  |  |  |  |
| Reviews |  |  |  | Key |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Review Date: |  |  |  | Likelihood |  |  |  | Severity |  |  | Severity of Injury |  | 5 | 5Y |  |  |  | 10R |  | 15 |  |  | 20 |  | 25 |
| Reviewed by: |  |  |  | 1 very unlikely 2 unlikely 3 likely 4 very likely 5 certainty |  |  |  | 1 nuisance 2 minor 3 medical treatment 4 major 5 fatal |  |  |  |  | 4 | 4 |  |  |  | 8 |  | 12 |  |  | 16 |  | 20 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 3 |  |  |  | 6G |  | 9 |  |  | 12 |  | 15 |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 2 |  |  |  | 4 |  | 6Y |  |  | 8 |  | 10Y |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5G |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  |  | 0 | 1 |  |  |  | 2 |  | 3 |  |  | 4 |  | 5 |
| Review Date : |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood of Injury |  |  |  |  |  |  |  |  |  |  |  |
| Reviewed by: |  |  |  |  |  |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  | Medium Risk |  |  |  |  | High Risk |  |