| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2024 | 2024 |

MoCK EXAMINATION- Question PAPer

MODULE CODE: ACDA4006

Electrical and Electronic Principles

TIME:  2 HOURS

Maximum Marks : 100

answer Any 4 questions

Equipment or materials required – Calculator, Graph sheet,

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ANY 4 QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig:1 below,

- Calculate the total circuit resistance ( RT ).				[3 marks]

- Calculate the supply current ( Is)						[2 marks]

- Calculate the total power dissipated. 					[2 marks]

Fig:1

- An indicator light requires a 330Ω resistor to limit current. The voltage across the current limiting resistor is 3V. What is current flowing through the resistor? Also, calculate how the current change if a 200Ω resistor is used instead?	[4 Marks]

- An oscilloscope display shows the below voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

2V/div

10ms/div

Fig:2

- Estimate the period of the waveform.					[1 mark]

- From the estimated period, determine its frequency.			[2 marks]

- Determine its peak-peak amplitude					[2 marks]

- Determine the r.m.s. value of the waveform.				[2 marks]

- Design an equivalent circuit with a single current source, for the circuit shown in fig.3,										[3 Marks]

Fig: 3

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)									[4 Marks]

Question 2 [25 Marks]

- For the circuit shown in Fig. 4, calculate:

  - The total resistance.							[3 Marks]

  - The total current if the battery voltage was 12V			[2 Marks]

  - The total power dissipated by the circuit				[2 Marks]

Fig: 4

- Three resistors (RA RB and Rc)are connected in series to a 250V Voltage Source. Where, Rc=50Ω, VB=80V and Total current = 2A. Calculate the following

- Voltage across RA 							[2 Marks]

- Voltage across RC 							[2 Marks]

- Resistance value of RA and RB						[4 Marks]

- Explain the following terms:

- Resistance									[2 Marks]

- Reactance									[2 Marks]

- Impedance									[2 Marks]

- List any four types of Transistors.						[4 marks]

Question 3 [25 Marks]

- A certain power supply requires 25W of input power and produces an output power of 20W. Calculate its efficiency in percentage and the total power loss?	[4 Marks]

- Determine the voltage across each resistor in the circuit shown in fig.5. 	[4 Marks]

Fig.5

- Determine the current across each resistor in the circuit shown in fig.6, using mesh analysis (Kirchhoff’s Voltage Law)					[17 Marks]

Fig.6

Question 4 [25 Marks]

- The alternating current is given by i=40 sin314t. Determine the following

- Maximum Value of current		[1 Mark]

- RMS Value of Current		[2 Marks]

- Frequency		[2 Marks]

- What is the use of the circuit shown in fig.7?  Describe the functions of each stage of the circuit with the necessary diagrams.		[12 marks]

Fig.7

- For the circuit shown in fig 8:

- Calculate the total impedance						[2 Marks]

- Calculate the circuit current and phase angle				[3 Marks]

- Calculate the voltage drop across the resistor				[1 Mark]

- Calculate the voltage drop across the inductor			[1 Mark]

- Calculate the voltage drop across the capacitor			[1 Mark]

Fig.8

Question 5 [25 Marks]

- Describe the construction and working of PN Junction diode with the aid of necessary diagrams.		[10 Marks]

- A 10 V Battery has an internal Resistance of 10Ω, Calculate the power dissipated by various load resistances given in the below table and complete the table.		[10 Marks]

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 5 |  |
| 10 |  |
| 15 |  |
| 20 |  |
| 25 |  |

- Plot a graph of the load Power against the load resistance,.		[5 Marks]

Diwedd/End