| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 7 | LEFEL 7 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| November 2019 | tachwedd 2019 |

MODULE CODE: ACCA7017

IMPLEMENTING IP ROUTING

PRACTICAL TEST 1

Name			:

Student ID		:

Date & Time		:

This is an individual practical lab-test, held under examination conditions. You will be assigned four Routers, you should configure these Routers based on the following specifications.

The routers that you will configure are: R1, R2, and R3.

This paper is marked out of 100.

You will have 2 hours to complete this task.

Final configuration

R1#sh run

Building configuration...

Current configuration : 1935 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R1

no ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

key chain R1chain

key 1

key-string FIRST-KEY

!

interface Loopback0

no ip address

ipv6 address 2001:DB8:CAFE:10::1/64

ipv6 eigrp 205

!

interface Loopback11

ip address 172.16.11.1 255.255.255.0

!

interface Loopback12

ip address 172.16.12.1 255.255.255.0

!

interface Loopback13

ip address 172.16.13.1 255.255.255.0

!

interface Loopback14

ip address 172.16.14.1 255.255.255.0

!

interface Loopback15

ip address 172.16.15.1 255.255.255.0

!

interface Loopback55

ip address 172.16.55.1 255.255.255.0

!

interface Loopback56

ip address 172.16.56.1 255.255.255.0

!

interface Loopback57

ip address 172.16.57.1 255.255.255.0

!

interface Loopback58

ip address 172.16.58.1 255.255.255.0

!

interface Loopback100

ip address 172.16.100.65 255.255.255.192

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

bandwidth 128

ip address 20.10.10.1 255.255.255.252

ip authentication mode eigrp 205 md5

ip authentication key-chain eigrp 205 R1chain

ip summary-address eigrp 205 172.16.8.0 255.255.248.0 5

ip summary-address eigrp 205 172.16.48.0 255.255.240.0 5

ipv6 address 2001:DB8:ACBD:10::1/64

ipv6 eigrp 205

clock rate 64000

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router eigrp 205

network 172.16.0.0

network 20.10.10.0 0.0.0.3

!

ipv6 router eigrp 205

eigrp router-id 1.1.1.1

no shutdown

!

ip classless

!

ip flow-export version 9

end

R2#sh run

Building configuration...

Current configuration : 1901 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R2

!

no ip cef

ipv6 unicast-routing

!

key chain R1chain

key 1

key-string FIRST-KEY

!

interface Loopback0

no ip address

ipv6 address 2001:DB8:CAFE:100::1/64

ipv6 eigrp 205

!

interface Loopback1

no ip address

ipv6 address 2001:DB8:ABCD:100::1/64

ipv6 eigrp 205

!

interface Loopback209

ip address 209.65.12.254 255.255.255.252

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

bandwidth 128

ip address 20.10.10.2 255.255.255.252

ip authentication mode eigrp 205 md5

ip authentication key-chain eigrp 205 R1chain

ipv6 address 2001:DB8:ACBD:10::2/64

ipv6 eigrp 205

!

interface Serial0/0/1

bandwidth 128

ip address 20.10.10.5 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 SECRETPASS

ipv6 address 2001:DB8:ACBD:20::1/64

ipv6 eigrp 205

clock rate 64000

!

interface Vlan1

no ip address

shutdown

!

router eigrp 205

redistribute ospf 1 metric 10000 100 255 1 1500

redistribute static

network 20.10.10.0 0.0.0.3

distribute-list OSPF26-FILTER out ospf 1

!

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

redistribute eigrp 205 subnets

network 20.10.10.4 0.0.0.3 area 0

default-information originate

!

ipv6 router eigrp 205

eigrp router-id 2.2.2.2

no shutdown

!

ip classless

ip route 0.0.0.0 0.0.0.0 Loopback209

!

ip flow-export version 9

!

!

ip access-list standard OSPF26-FILTER

remark Used with DList to filter OSPF Area 26 Network 172.26.20.0

deny 172.26.20.0 0.0.0.255

permit any

!

end

R3#sh run

Building configuration...

Current configuration : 2603 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R3

no ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

interface Loopback0

no ip address

ipv6 address 2001:DB8:9A:10AA::1/64

ipv6 eigrp 205

!

interface Loopback20

ip address 172.26.20.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback21

ip address 172.26.21.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback22

ip address 172.26.22.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback23

ip address 172.26.23.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback40

ip address 192.168.40.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback45

ip address 192.168.45.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback50

ip address 192.168.50.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback55

ip address 192.168.55.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback60

ip address 192.168.60.1 255.255.255.0

ip ospf network point-to-point

!

interface Loopback65

ip address 192.168.65.1 255.255.255.0

ip ospf network point-to-point

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

bandwidth 128

ip address 20.10.10.6 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 SECRETPASS

ipv6 address 2001:DB8:ACBD:20::2/64

ipv6 eigrp 205

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 3.3.3.3

log-adjacency-changes

area 175 range 192.168.0.0 255.255.128.0

passive-interface default

no passive-interface Serial0/0/1

network 20.10.10.4 0.0.0.3 area 0

network 192.168.40.0 0.0.0.255 area 175

network 192.168.45.0 0.0.0.255 area 175

network 192.168.50.0 0.0.0.255 area 175

network 192.168.55.0 0.0.0.255 area 175

network 192.168.60.0 0.0.0.255 area 175

network 192.168.65.0 0.0.0.255 area 175

network 172.26.20.0 0.0.0.255 area 26

network 172.26.21.0 0.0.0.255 area 26

network 172.26.22.0 0.0.0.255 area 26

network 172.26.23.0 0.0.0.255 area 26

!

ipv6 router eigrp 205

eigrp router-id 3.3.3.3

no shutdown

!

ip classless

end

R3#