// Caitlin Lonsdale (2306892) - Embedded C Assignment 12/2024

#include <main.h>											//Include main header file

#define LCD_ENABLE_PIN PIN_D2								//Name Pin D2 as LCD_ENABLE_PIN

#define LCD_RS_PIN PIN_D0									//Name Pin D0 as LCD_RS_PIN

#define LCD_RW_PIN PIN_D1									//Name Pin D1 as LCD_RW_PIN

#define LCD_DATA4 PIN_D4									//Name Pin D4 as LCD_DATA4

#define LCD_DATA5 PIN_D5									//Name Pin D5 as LCD_DATA5

#define LCD_DATA6 PIN_D6									//Name Pin D6 as LCD_DATA6

#define LCD_DATA7 PIN_D7									//Name Pin D7 as LCD_DATA7

#include <lcd.c>											//Include lcd c file

// Prototyping and Initialisation -----------------------------------------------------------------------------------------

void analog(void);											//Prototype function analog

void print_adc(void);										//Prototype function print_adc

void reset(void);											//Prototype function reset

void stop_motor(void);										//Prototype function stop_motor

void clear_leds(void);										//Prototype function clear_leds

void led_display(void);										//Prototype function led_display

void knightrider(void);										//Prototype function knightrider

unsigned int sw_previous=0;									//Initialise global integer variable sw_previous

unsigned int previous_but=2;								//Initialise global integer variable previous_but

unsigned int adc;											//Initialise global integer variable adc

unsigned int lights[16]={128,64,32,16,8,4,2,1,1,2,4,8,16,32,64,128};//Initialise global integer array variable lights for LEDs

// Main Function ----------------------------------------------------------------------------------------------------------

void main() {												//Run function main

setup_adc_ports(sAN0);									//Set ports AN0 as ADC port

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);			//Set internal ADC clock

setup_timer_2(T2_DIV_BY_16,249,15); 					//Set timer with 2.0 ms overflow and 30.0 ms interrupt

lcd_init();												//Initialise the LCD screen

while(TRUE) {											//Start infinite while loop

analog();											//Call function analog

// Task 1 -------------------------------------------------------------------------------------------------------------------

if(input(PIN_A2)) {									//Execute if input on Pin A2 from SW1-A

reset();										//Call function reset

if(adc<75) {									//Execute if ADC value less than 75

output_low(PIN_C0);							//Set Pin C0 low to turn motor anticlockwise

output_high(PIN_C1);						//Set Pin C1 high to turn motor anticlockwise

lcd_gotoxy(5,2);                    		//Set the position at the centre of line 2 of the LCD

printf(lcd_putc,"Anticlockwise"); 			//Print the string 'Anticlockwise' at the LCD position

} else if(adc>175) {							//Execute if ADC value greater than 175

output_high(PIN_C0);						//Set Pin C0 high to turn motor clockwise

output_low(PIN_C1);							//Set Pin C1 low to turn motor clockwise

lcd_gotoxy(5,2);                    		//Set the position at the centre of line 2 of the LCD

printf(lcd_putc,"  Clockwise  "); 			//Print the string 'Clockwise' at the LCD position

} else {										//Execute if all if statements untrue

stop_motor();								//Call function stop_motor

lcd_gotoxy(5,2);                    		//Set the position at the centre of line 2 of the LCD

printf(lcd_putc,"Motor Stopped"); 			//Print the string 'Clockwise' at the LCD position

}												//End if-else statement

print_adc();									//Call function print_adc

led_display();									//Call function led_display

} else {											//Execute if no input on Pin A2 from SW1-A

stop_motor();									//Call function stop_motor

}													//End if-else statement

// Task 2 --------------------------------------------------------------------------------------------------------------------

if(input(PIN_A3)) {									//Execute if input on Pin A3 from SW1-B

reset();										//Call function reset

analog();										//Call function analog

print_adc();									//Call function print_adc

led_display();									//Call function led_display

}													//End if statement

// Task 3 --------------------------------------------------------------------------------------------------------------------

if(input(PIN_A4)) { 								//Execute if input on Pin A4 from SW1-C

reset();										//Call function reset

clear_leds();									//Call function clear_leds

unsigned current_but = input(PIN_A1);			//Initialise local variable current_but equal to the input of Pin A1

if(previous_but == 2) {							//Execute if variable previous_but is equal to 2 from initialisation

printf("\f Button Not Pressed\r");			//Print the string 'Button Not Pressed' in the virtual terminal

previous_but = 0;							//Set the variable previous_but value to 0

}												//End if statement

if(current_but == 0) {							//Execute if variable current_but is equal to 0

output_low(PIN_C0);							//Set Pin C0 low to turn motor off

output_low(PIN_C1);							//Set Pin C1 low to turn motor off

lcd_gotoxy(22,1);							//Set the position at the centre of line 3 of the LCD

printf(lcd_putc,"Button Not Pressed");		//Print the string 'Button Not Pressed' at the LCD Position

if(current_but != previous_but) {			//Execute if current_but not equal to previous_but - prints once only per button state

printf("\f Button Not Pressed\r");		//Print the string 'Button Not Pressed' in the virtual terminal

}											//End if statement

} else {										//Execute if variable current_but is not equal to 0

output_high(PIN_C0);						//Set Pin C0 high to turn motor clockwise

output_low(PIN_C1);							//Set Pin C1 low to turn motor clockwise

lcd_gotoxy(22,1);							//Set the position at the centre of line 3 of the LCD

printf(lcd_putc,"  Button Pressed  "); 		//Print the string 'Button Pressed' at the LCD Position

if(current_but != previous_but) {			//Execute if current_but not equal to previous_but - prints once only per button state

printf("\f Button Pressed\r");			//Print the string 'Button Not Pressed' in the virtual terminal

}											//End if statement

}												//End if-else statement

previous_but = current_but;						//Set variable previous_but as current_but value for next cycle

}													//End if statement

// Task 4 ---------------------------------------------------------------------------------------------------------------------

if(input(PIN_A5)) {									//Execute if input on Pin A5 from SW1-D

reset();										//Call function reset

lcd_gotoxy(27,2);                    			//Set the position at the centre of line 4 of the LCD

printf(lcd_putc,"Kitt mode"); 					//Print the string 'Kitt mode' at the LCD position

knightrider();									//Call function knightrider

}													//End if statement

}														//End infinite while loop

}															//End function main

// Functions ------------------------------------------------------------------------------------------------------------------

void analog(void) {											//Run function analog - reads the ADC value of RV1

set_adc_channel(0);										//Select ADC channel 0

delay_us(30);											//Delay 30 us

adc = read_adc();										//Call conversion function and store value in integer value adc

}															//End function analog

void print_adc(void) {										//Run function print_adc - prints the ADC value to the LCD

lcd_gotoxy(10,1);                    					//Set the position at the centre of line 1 in the LCD

printf(lcd_putc,"%3u",adc);  							//Print the integer ADC value at the LCD position

printf("%u\r",adc);										//Print the integer ADC value in the virtual terminal

}															//End function print_adc

void reset(void) {											//Run function reset - clears displays after switch changes

unsigned int r = input_a();								//Initialise local integer variable r equal to the input of Ports A

r &= 0x3C;												//Mask off analog Ports A

r >>= 2;												//Shift remaining ports to postions 8,4,2,1

if(sw_previous != r) {									//Execute if sw_previous is not equal to variable r

sw_previous = r;									//Set variable sw_previous equal to r value

previous_but = 2;									//Set variable previous_but equal to 2 as in initialisation

lcd_gotoxy(1,1);									//Set the position at the start of line 1 in the LCD

printf(lcd_putc,"\f");								//Clear the LCD display

if(r == 8) {										//Execute if r equals 8 as input on SW1.4

printf("\f Kitt mode");							//Print the string 'Kitt mode' in the virtual terminal (Task 4)

}													//End if statement

}														//End if statement

}															//End function reset

void stop_motor(void) {										//Run function stop_motor - turns motor off

output_low(PIN_C0);										//Set Pin C0 low to turn motor off

output_low(PIN_C1);										//Set Pin C1 low to turn motor off

}															//End function stop_motor

void clear_leds(void) {										//Run function clear_leds

output_low(PIN_B7);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B6);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B5);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B4);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B3);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B2);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B1);										//Set Pin B7 low to turn LED 7 off

output_low(PIN_B0);										//Set Pin B7 low to turn LED 7 off

}															//End function clear_leds

void led_display(void) {									//Run function led_display - displays ADC value in binary on LEDs

if(adc>=128) { 											//If statement to check variable adc value and set 8th binary bit on LED 7

output_high(PIN_B7);								//Set Pin B7 high to turn LED 7 on to represent 1

adc-=128;											//Decrement variable adc value to check next bit

} else {												//Else statement to set 8th binary bit on LED 7

output_low(PIN_B7);									//Set Pin B7 low to turn LED 7 off to represent 0

}														//End if-else statement

if(adc>=64) { 											//If statement to check variable adc value and set 7th binary bit on LED 6

output_high(PIN_B6);								//Set Pin B6 high to turn LED 6 on to represent 1

adc-=64;											//Decrement variable adc value to check next bit

} else {												//Else statement to set 7th binary bit on LED 6

output_low(PIN_B6);									//Set Pin B6 low to turn LED 6 off to represent 0

}														//End if-else statement

if(adc>=32) { 											//If statement to check variable adc value and set 6th binary bit on LED 5

output_high(PIN_B5);								//Set Pin B5 high to turn LED 5 on to represent 1

adc-=32;											//Decrement variable adc value to check next bit

} else {												//Else statement to set 6th binary bit on LED 5

output_low(PIN_B5);									//Set Pin B5 low to turn LED 5 off to represent 0

}														//End if-else statement

if(adc>=16) { 											//If statement to check variable adc value and set 5th binary bit on LED 4

output_high(PIN_B4);								//Set Pin B4 high to turn LED 4 on to represent 1

adc-=16;											//Decrement variable adc value to check next bit

} else {												//Else statement to set 5th binary bit on LED 4

output_low(PIN_B4);									//Set Pin B4 low to turn LED 4 off to represent 0

}														//End if-else statement

if(adc>=8) { 											//If statement to check variable adc value and set 4th binary bit on LED 3

output_high(PIN_B3);								//Set Pin B3 high to turn LED 3 on to represent 1

adc-=8;												//Decrement variable adc value to check next bit

} else {												//Else statement to set 4th binary bit on LED 3

output_low(PIN_B3);									//Set Pin B3 low to turn LED 3 off to represent 0

}														//End if-else statement

if(adc>=4) { 											//If statement to check variable adc value and set 3rd binary bit on LED 2

output_high(PIN_B2);								//Set Pin B2 high to turn LED 2 on to represent 1

adc-=4;												//Decrement variable adc value to check next bit

} else {												//Else statement to set 3rd binary bit on LED 2

output_low(PIN_B2);									//Set Pin B2 low to turn LED 2 off to represent 0

}														//End if-else statement

if(adc>=2) { 											//If statement to check variable adc value and set 2nd binary bit on LED 1

output_high(PIN_B1);								//Set Pin B1 high to turn LED 1 on to represent 1

adc-=2;												//Decrement variable adc value to check next bit

} else {												//Else statement to set 2nd binary bit on LED 1

output_low(PIN_B1);									//Set Pin B1 low to turn LED 1 off to represent 0

}														//End if-else statement

if(adc>=1) { 											//If statement to check variable adc value and set 1st binary bit on LED 0

output_high(PIN_B0);								//Set Pin B0 high to turn LED 0 on to represent 1

adc-=1;												//Decrement variable adc value to check next bit

} else {												//Else statement to set 1st binary bit on LED 0

output_low(PIN_B0);									//Set Pin B0 low to turn LED 0 off to represent 0

}														//End if-else statement

}															//End function led_display

void knightrider(void) {									//Run function knightrider

unsigned int q;											//Initialise local integer variable q

for(q=0;q<16;q++) {										//For loop that increments q in steps of 1 from 0-15

output_b(lights[q]);								//Set LED on from the Pin B port according to the index value q of the variable array lights

delay_ms(200);										//Delay 200 ms

}														//End for loop

}															//End function knightrider

Comments

2.. 20   3.. 30    4.. 40   5.. 10

***************************************

2.. Your code has a number of functions (subroutines) and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%