| LOCATION: london | LLEOLIAD: llundain |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| September 2016 | medi 2016 |

MODULE CODE: accc5002

LOCAL AREA NETWORKS

TIME:  2 HOURS

SOLUTIONS

Local Area Networks

BSc Computer Networks (ACCC5002)

Practical Lab Assessment

Year 2/Level 5

September 2016

Name			:

Student ID		:

Date & Time		:

This is an individual practical lab-test, held under examination conditions. You will be assigned three Switches and a Router; you should configure these devices based on the given specifications.

This paper is marked out of 100.

You will have 2 hours to complete this task.

## TOPOLOGY DIAGRAM

VLAN 10 – Sales: 		192.168.10.0/24

VLAN 20 – HR: 		192.168.20.0/24

VLAN 99 – Management: 	192.168.99.0/24

## Addressing Table 									(8 marks)

## Choose the first address in each subnet for the Gateway.

## Choose the fifth address in each subnet for management VLAN 99 addresses.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Gateway | Ge0/0.10 |  |  | N/A |
|  | Ge0/0.20 |  |  | N/A |
|  | Ge0/0.99 |  |  | N/A |
|  | S0/0/0 |  |  | N/A |
| ISP | S0/0/0 |  |  | N/A |
| SW1 | VLAN99 |  |  |  |
| SW2 | VLAN99 |  |  |  |
| SW3 | VLAN99 |  |  |  |
| PC1 | NIC | DHCP | DHCP | DHCP |
| PC2 | NIC | DHCP | DHCP | DHCP |

## Port Assignments

## SW1

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/2, Fa0/3 & Fa0/5 | 802.1Q Trunks (Native VLAN 99) | 192.168.99.0 |
| Fa0/1 | VLAN 10 (Sales) | 192.168.10.0 |

## SW2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/4 – 0/5 | 802.1Q Trunks (Native VLAN 99) | 192.168.99.0 |
| Fa0/1 | VLAN 20 (HR) | 192.168.20.0 |

## SW3

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/2 – 0/4 | 802.1q Trunks (Native VLAN 99) | 192.168.99.0 |
| Fa0/1 | 802.1q Trunk |  |

Task 1: Prepare the Network 							(5 marks)

Step 1: Cable a network that is similar to the one in the topology diagram.

Step 2: Clear any existing configurations on the devices.

Task 2: Perform Basic Device Configurations 					(10 marks)

Configure the switches (SW1, SW2, and SW3) and routers (Gateway & ISP) according to the following guidelines:

- Configure the hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure synchronous logging for the console connection.

- Configure a password for vty connections.

Task 3: Router and Switch configuration instructions 				(64 marks)

- Configure SW3 as a VTP server and SW1 & SW2 as VTP clients in the class.com domain. Set the VTP password to ‘cisco’.

- Configure EtherChannel between the SW1 and SW3 as per the topology diagram.

- Create VLANs 10, 20 & 99, and name as required.

- Assign ports to appropriate VLANs as shown in the Port Assignments table (access and trunks).

  - Trunk ports with native VLAN 99 to be configured on links between switches.

  - Trunk on SW3 port F0/1 to be configured (without native VLAN 99).

- Configure Rapid PVST+ on all switches.

- Configure SW3 to be the primary root for all VLANs and SW1 to be secondary.

- Configure PortFast and BPDU Guard on the interface connected to PC1 and PC2.

- Configure SW3 as a DHCP server to provide IP addresses and default gateways to users on VLAN 10 & 20. Verify all PCs receive IP address details.

  - Don’t forget to configure the IP address helper on the Gateway router.

- Configure management IP addresses for all network switches in VLAN 99, and configure them for Telnet access (Configure default gateway for the switches on VLAN 99).

- Configure router-on-a-stick inter-VLAN routing.

  - Configure sub-interfaces on R1 to operate on VLAN 10, 20, and 99 with the address from the Address table.

- Configure the ISP router with appropriate IP addressing information (Refer to topology).

  - Clock rate for the DCE interface: 128000

- Configure a default route on the ISP with S0/0/0 as the exit interface.

- Configure NAT Overload and map both 192.168.10.0/24 and 192.168.20.0/24 to the S0/0/0 interface on the Gateway router.

Task 4: After configuring each host computer, record the host network settings with the ipconfig /all command.  								(4 marks)

| Host PC1 Network Configuration |  |  |
| --- | --- | --- |
| IP Address |  |  |
| Subnet Mask |  |  |
| Default Gateway |  |  |
| Host PC2 Network Configuration |  |  |
| IP Address |  |  |
| Subnet Mask |  |  |
| Default Gateway |  |  |

Instructor: 	  	Ping Successful? ________________			(4 marks)

Task 5: Document the Configurations 						(5 marks)

All documentation for your solution should be submitted. Your name should be clearly printed on the documents. 			Devicename_Surname_Pno.txt

At the end of the test a pen drive will be given to you, the configuration files for all routers should be copied to the media and handed in to your instructor.  It is you responsibility that you copy the files correctly.

| ROUTER | SWITCH |
| --- | --- |
| Show running-config | Show running-config |
| Show ip interface brief | Show ip interface brief |
| Show ip route | Show vlan |
| Show ip nat stat | Show interface trunk |
| Show ip nat translations | Show spanning-tree |
|  | Show etherchannel summary |
|  | Show vtp status |

SOLUTIONS:

################################################################

SW1:

SW1#sh run

Building configuration...

Current configuration : 2033 bytes

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname SW1

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$QGR2$2oi3IZQClGPCgWwHyV3./1

!

!

!

no aaa new-model

system mtu routing 1500

no ip domain-lookup

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

spanning-tree vlan 1,10,20,99 priority 28672

!

vlan internal allocation policy ascending

!

!

!

interface Port-channel1

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

channel-group 1 mode on

!

interface FastEthernet0/3

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

channel-group 1 mode on

!

interface FastEthernet0/4

!

interface FastEthernet0/5

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 192.168.99.1 255.255.255.0

!

ip classless

ip http server

ip http secure-server

!

!

!

line con 0

logging synchronous

line vty 0 4

password cisco

login

line vty 5 15

login

!

end

SW1#

################################################################

SW2:

SW2#

SW2#sh run

Building configuration...

Current configuration : 3383 bytes

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname SW2

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$efxQ$XOqxekx8nDOMm9rmBxlAF/

!

!

!

no aaa new-model

system mtu routing 1500

no ip domain-lookup

!

!

!

!

crypto pki trustpoint TP-self-signed-2649938816

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2649938816

revocation-check none

rsakeypair TP-self-signed-2649938816

!

!

crypto pki certificate chain TP-self-signed-2649938816

certificate self-signed 01

3082023C 308201A5 A0030201 02020101 300D0609 2A864886 F70D0101 04050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32363439 39333838 3136301E 170D3933 30333031 30303030

35375A17 0D323030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 36343939

33383831 3630819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100E279 269E36BA 08F28662 3E0A3046 63CCBE41 5FE24263 077DF2E0 F4E76D76

699AF9F3 BC673625 CEC7DCE5 7850953E 70EF00DE C6901BA2 1F7324A9 C67BF3EE

D9C2FEE5 96251A19 05C70FC2 A6B78AB8 8A675769 D135767F FD27C5E5 2FE8A7FC

296CFC2D A58947A8 E0A6C55C 3E83F8EA 5871B63C C60F0AFD 0A35AA11 45451B9A

8E950203 010001A3 64306230 0F060355 1D130101 FF040530 030101FF 300F0603

551D1104 08300682 04535732 2E301F06 03551D23 04183016 80147E60 657DDFB2

20F258A3 241E35EB 10AC01F8 1B10301D 0603551D 0E041604 147E6065 7DDFB220

F258A324 1E35EB10 AC01F81B 10300D06 092A8648 86F70D01 01040500 03818100

D87EB258 B98F32ED 115A1201 D59C4C14 8588BD11 BEB1C843 0654A403 184A7E27

CD90587C 8DE2142E CB2F1EB6 8BF60B0D 08DE9BC7 8FFD5D8A B53E0FF7 588E34D3

8AB26B55 3ED4D4AC 586994F8 7A876955 8915BE21 D04AC56E C96570BF C67CD113

0C39D5D4 60007174 E2B4827D 4424DF03 BD0EA35B 5937E8CD 7FD8F8D1 2D936DA6

quit

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

interface FastEthernet0/1

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 192.168.99.2 255.255.255.0

!

ip default-gateway 192.168.99.5

ip classless

ip http server

ip http secure-server

!

!

!

line con 0

logging synchronous

line vty 0 4

password cisco

login

line vty 5 15

login

!

end

SW2#

################################################################

SW3:

SW3#

SW3#sh run

Building configuration...

Current configuration : 2274 bytes

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname SW3

!

boot-start-marker

boot-end-marker

!

enable secret 5 $1$h2rA$aKy9cRH5k9oVd542gKIzg.

!

!

!

no aaa new-model

system mtu routing 1500

no ip domain-lookup

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

!

ip dhcp pool Sales

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

!

ip dhcp pool HR

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

spanning-tree vlan 1,10,20,99 priority 24576

!

vlan internal allocation policy ascending

!

!

!

interface Port-channel1

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

channel-group 1 mode on

!

interface FastEthernet0/3

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

channel-group 1 mode on

!

interface FastEthernet0/4

switchport trunk encapsulation dot1q

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 192.168.99.3 255.255.255.0

!

ip default-gateway 192.168.99.5

ip classless

ip http server

ip http secure-server

!

!

!

line con 0

logging synchronous

line vty 0 4

password cisco

login

line vty 5 15

login

!

end

SW3#

################################################################

Gateway:

Gateway#sh run

Building configuration...

Current configuration : 1677 bytes

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Gateway

!

boot-start-marker

boot-end-marker

!

logging message-counter syslog

enable secret 5 $1$Oum4$rPN8HK3s9/gcZUHjdubRk.

!

no aaa new-model

memory-size iomem 5

!

dot11 syslog

ip source-route

!

!

ip cef

!

!

no ip domain lookup

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

archive

log config

hidekeys

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/1.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.99.3

ip nat inside

ip virtual-reassembly

!

interface GigabitEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.99.3

ip nat inside

ip virtual-reassembly

!

interface GigabitEthernet0/1.99

encapsulation dot1Q 99

ip address 192.168.99.5 255.255.255.0

!

interface Serial0/0/0

ip address 10.0.0.1 255.255.255.0

ip nat outside

ip virtual-reassembly

no fair-queue

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list 1 interface Serial0/0/0 overload

!

access-list 1 permit 192.168.10.0 0.0.0.255

access-list 1 permit 192.168.20.0 0.0.0.255

!

!

!

!

!

!

control-plane

!

!

!

!

!

!

!

!

!

!

line con 0

logging synchronous

line aux 0

line vty 0 4

password cisco

login

!

scheduler allocate 20000 1000

end

Gateway#

################################################################

ISP:

ISP#sh run

Building configuration...

Current configuration : 1119 bytes

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname ISP

!

boot-start-marker

boot-end-marker

!

logging message-counter syslog

enable secret 5 $1$Z2av$o3AmcPDruDtYRiGbf/eBf0

!

no aaa new-model

memory-size iomem 5

!

dot11 syslog

ip source-route

!

!

ip cef

!

!

no ip domain lookup

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

voice-card 0

!

!

!

!

!

archive

log config

hidekeys

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial0/0/0

ip address 10.0.0.2 255.255.255.0

no fair-queue

clock rate 128000

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

ip forward-protocol nd

ip route 0.0.0.0 0.0.0.0 10.0.0.1

no ip http server

no ip http secure-server

!

!

!

!

!

!

!

!

!

control-plane

!

!

!

!

!

!

!

!

!

!

line con 0

logging synchronous

line aux 0

line vty 0 4

password cisco

login

!

scheduler allocate 20000 1000

end

ISP#

################################################################