# Networking Worksheet: Two-LAN Network Design and Default Gateway Investigation

Name: ___________________________

Date: ___________________________

Class / Section: __________________

OBJECTIVES

- Design a network with two LANs

- Recall and apply private Class A and Class C IP addressing

- Correctly select Ethernet cable types

- Configure IP addresses and subnet masks

- Understand the role of the default gateway

EQUIPMENT REQUIRED

- 1 Router (Cisco 4321)

- 3 Switches (Cisco 2960)

- 5 PCs

- Copper straight-through cables

- Copper crossover cable

- Cisco Packet Tracer

NETWORK TOPOLOGY

LAN A:

- Uses private Class C addressing

- Two switches

- Switch A1 connects PC1 and PC2

- Switch A2 connects PC3

- Switch A1 connects to Switch A2 using a crossover cable

- Switch A1 connects to the router

LAN B:

- Uses private Class A addressing

- One switch

- PC4 and PC5 connect to the switch

- Switch connects to the router

PART 1 – ADDRESS PLANNING (RECALL)

Write the private IP range for:

Class C: ___________________________

Class A: ___________________________

PART 2 – BUILD THE PHYSICAL NETWORK

- PCs to switches using straight-through cables

- Switch to router using straight-through cables

- Switch A1 to Switch A2 using a crossover cable

Do not configure the router yet.

PART 3 – CONFIGURE LAN A (CLASS C)

Configure IP address and subnet mask only. Do not configure a default gateway.

PC1 IP: ____________  Subnet Mask: ____________

PC2 IP: ____________  Subnet Mask: ____________

PC3 IP: ____________  Subnet Mask: ____________

PART 4 – CONFIGURE LAN B (CLASS A)

Configure IP address and subnet mask only. Do not configure a default gateway.

PC4 IP: ____________  Subnet Mask: ____________

PC5 IP: ____________  Subnet Mask: ____________

PART 5 – SAME-LAN CONNECTIVITY TEST

From PC1, ping PC2 and PC3.

From PC4, ping PC5.

Why are these pings successful without a router?

__________________________________________________

__________________________________________________

PART 6 – ROUTER CONFIGURATION

Enable G0/0/0 and G0/0/1 and assign valid IP addresses.

G0/0/0 IP: ____________  Subnet Mask: ____________

G0/0/1 IP: ____________  Subnet Mask: ____________

PART 7 – INTER-LAN PING TEST

From PC1, ping PC5.

Did the ping succeed? ____________

Why does the ping fail?

__________________________________________________

__________________________________________________

PART 8 – CONFIGURE DEFAULT GATEWAYS

PC1: ____________

PC2: ____________

PC3: ____________

PC4: ____________

PC5: ____________

PART 9 – FINAL TEST

Ping PC5 from PC1 again.

Why does the ping succeed now?

__________________________________________________

__________________________________________________

REFLECTION

What is the purpose of a default gateway?

__________________________________________________

__________________________________________________